| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, MA, 9–18 Jan. 2019 | Document: JVET-M0407-v2 |
| --- | --- |

| Title: | CE8: CPR reference memory reuse without increasing memory requirement (CE8.1.2a and CE8.1.2d) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu, Xiang Li and Shan Liu (Tencent) Eric Chai (Ubilinx) | Tel: Email: | xiaozhongxu@tencent.com eric.chai@ublnx.com |
| Source: | Tencent, Ubilinx |  |  |

_____________________________

# Abstract

This contribution reports the CE test results for CE8.1.2a and CE8.1.2d. In the tests in this proposal, in addition to the already reconstructed reference samples within the current CTU, search range in some parts of the left CTU is also allowed for CPR mode while the total required memory for performing CPR mode is unchanged (1 CTU size or 4 64x64 blocks). The allowed reference area in the left CTU should satisfy the condition that the corresponding memory that stores the reference samples from the left CTU has not yet been updated by the reconstructed samples from the current CTU. Specifically:

In CE8.1.2a, the CTU size reference sample memory is updated on a 64x64 block basis. Simulation results report that

- 42.96%/25.89%/15.71% AI/RA/LB BD rate reduction can be achieved for SCC 1080p class, when compared to VTM-3.0 anchor (CPR=0, PLT=0).

In CE8.1.2d, the CTU size reference sample memory is updated on a CU by CU basis. Simulation results report that

- 45.00%/27.25%/16.79% AI/RA/LB BD rate reduction can be achieved for SCC 1080p class, when compared to VTM-3.0 anchor (CPR=0, PLT=0)

# Introduction

In VTM-3.0 and current VVC WD [1], the search range for CPR mode is restricted inside the current CTU. In a worst case, a memory size equal to 1 CTU size of reference samples is required to store the reference samples for CPR mode. However, the 1 CTU size of memory, or four 64x64 blocks in VTM-3.0, is not fully utilized, especially at the beginning of decoding the current CTU. In this case, there are just few/no reference samples available inside the current CTU that can be used to predict a CPR coded block. At the meantime, some parts of the reference sample memory may still store the reconstructed samples from the left CTU, until they are updated by the already coded reference samples inside the current CTU. It is therefore desirable that when the part of reference sample memory has not yet been updated, its samples can be used for CPR compensation. This effectively extends the CPR search range from within the current CTU to some parts of the previous CTU.

# Proposed Solutions

In this document, it is firstly proposed to not allow CPR block size to be larger than 64x64 luma samples. The following discussions are based on this assumption.

Two methods are proposed to utilize the reference sample memory more efficiently such that the effective search range for CPR mode can be extended beyond the current CTU, which is the setting for CPR mode in VTM-3.0.

This means that once any of the 64x64 unit reference sample memory begins to update with the reconstructed samples from the current CTU, the previous stored reference samples (from the left CTU) in the whole 64x64 unit become unavailable for CPR reference purpose.

X

Curr

X

X

Curr

X

X

X

X

Curr

X

X

Curr

X

Fig. 1: current CTU processing order and its available reference samples in current and left CTU (when quad-tree split is enforced at 128x128 level)

X

X

Curr

X

X

X

Curr

Fig. 2: current CTU processing order and its available reference samples in current and left CTU when first level vertical binary-tree split is applied.

In CE8.1.2a, it is proposed to reuse the reference sample memory on 64x64 basis.

We consider each of the 64x64 blocks in the reference memory buffer as a whole, when some part of the 64x64 block has been updated with reconstructed samples from the current CTU, the reference samples from the left CTU in this whole 64x64 block cannot be used. More specifically, depending on the location of the current coding block relative to the current CTU, the follows apply:

•	If current block falls into the top-left 64x64 block of the current CTU, then in addition to the already reconstructed samples in the current CTU, it can also refer to the reference samples in the bottom-right 64x64 blocks of the left CTU, using CPR mode. The current block can also refer to the reference samples in the bottom-left 64x64 block of the left CTU and the reference samples in the top-right 64x64 block of the left CTU, using CPR mode.

•	If current block falls into the top-right 64x64 block of the current CTU, then in addition to the already reconstructed samples in the current CTU, if luma location (0, 64) relative to the current CTU has not yet been reconstructed, the current block can also refer to the reference samples in the bottom-left 64x64 block and bottom-right 64x64 block of the left CTU, using CPR mode; otherwise, the current block can also refer to reference samples in bottom-right 64x64 block of the left CTU.

•	If current block falls into the bottom-left 64x64 block of the current CTU, then in addition to the already reconstructed samples in the current CTU, if luma location (64, 0) relative to the current CTU has not yet been reconstructed, the current block can also refer to the reference samples in the top-right 64x64 block and bottom-right 64x64 block of the left CTU, using CPR mode. Otherwise, the current block can also refer to the reference samples in the bottom-right 64x64 block of the left CTU, using CPR mode.

•	If current block falls into the bottom-right 64x64 block of the current CTU, it can only refer to the already reconstructed samples in the current CTU, using CPR mode.

For complexity reduction purpose, in this test, the reference block should come from the same 64x64 block in reference sample memory.

In CE8.1.2d, it is proposed to reuse the reference sample memory on a CU-by-CU basis.

We consider a coding block in the current CTU and the block of same size and location in the left CTU as a pair of collocated blocks. For each reference block in the left CTU, if its entire collocated block in the current CTU has not yet been coded, the part for storing this reference block in the reference memory buffer has not yet updated with reference samples from the current CTU. This reference block can therefore be used for CPR prediction purpose. Different from in CE8.1.2a, where the availability of a reference block from the left CTU is decided on a 64x64 block basis, the availability check here is based on per CU.

# Experimental results

Tests were carried out by using the VTM3.0 software, and the common test conditions (CTC) defined in [2] and common test conditions in CE8 [3] are followed.

The following four tests are carried out:

Table 1: Summary results of Test 8.1.2a vs. VTM3.0+CPR=0 and VTM3.0+CPR=1

Table 2: Summary results of Test 8.1.2a vs. VTM3.0+CPR=0 and VTM3.0+CPR=1+PLT=1

Table 3: Summary results of Test 8.1.2d vs. VTM3.0+CPR=0 and VTM3.0+CPR=1

Table 4: Summary results of Test 8.1.2d vs. VTM3.0+CPR=0 and VTM3.0+CPR=1+PLT=1

# Recommendation

In this proposal, results of reusing reference sample memory to extend the CPR search range from within the current CTU to some parts of the left CTU are reported. It is asserted that by the proper availability check mechanism, the requirement for the size of reference sample memory is kept unchanged when compared with the anchor.

The authors would like to thank Huawei for cross-checking.

# References

[1] B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 3)”, JVET-L1001-v7, the 12th JVET meeting, the 12th JVET meeting, Oct. 2018, Macao, CN.

[2] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, the 12th JVET meeting, Oct. 2018, Macao, CN.

[3] X. Xu, Y.-C. Sun, Y.-H. Chao, J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding”, JVET-L1028, the 12th JVET meeting, Oct. 2018, Macao, CN.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ubilinx may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Spec Text Changes relative to JVET-L1001-v7

In section 8.3.2.1 General

……

If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_LUMA, it is a requirement of bitstream conformance that when the reference picture is the current picture, the luma motion vector mvLX shall obey the following constraints:

- When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xCb + ( mvLX[ 0 ] >> 4 ), yCb + ( mvLX[ 1 ] >> 4 ) ) as inputs, and the output the output shall be equal to TRUE.

- When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xCb + ( mvLX[ 0 ] >> 4 ) + cbWidth – 1, yCb + ( mvLX[ 1] >> 4 ) + cbHeight – 1 ) as inputs, and the output the output shall be equal to TRUE.

- One or both the following conditions shall be true:

- The value of ( mvLX[ 0 ] >> 4 ) + cbWidth is less than or equal to 0.

- The value of ( mvLX[ 1 ] >> 4 ) + cbHeight is less than or equal to 0.

- The following conditions shall be true:

( yCb + ( mvLX[ 1 ] >> 4 ) ) >> CtbLog2SizeY = yCb >> CtbLog2SizeY				(8-xxx)

( yCb + ( mvLX[ 1 ] >> 4 ) + cbHeight – 1) >> CtbLog2SizeY = yCb >> CtbLog2SizeY		(8-xxx)

( xCb + ( mvLX[ 0 ] >> 4 ) ) >> CtbLog2SizeY >= ( xCb >> CtbLog2SizeY ) – 1 		(8-xxx)

( xCb + ( mvLX[ 0 ] >> 4 ) + cbWidth – 1) >> CtbLog2SizeY >= ( xCb >> CtbLog2SizeY ) – 1	(8-xxx)

For CE8.1.2.a

- The following conditions shall be true:

- ( yCb + ( mvLX[ 1 ] >> 4 ) ) >> 64 = ( yCb + ( mvLX[ 1 ] >> 4 ) + cbHeight – 1) >> 64	(8-xxx)

- ( xCb + ( mvLX[ 0 ] >> 4 ) ) >> 64 = ( xCb + ( mvLX[ 0 ] >> 4 ) + cbWidth – 1) >> 64	(8-xxx)

- If ( xCb + ( mvLX[ 0 ] >> 4 ) ) >> CtbLog2SizeY is equal to ( xCb >> CtbLog2SizeY ) – 1, the following shall apply:

  - At least one of the following shall be true:

    - xCb % CtbLog2SizeY < 64

  - yCb % CtbLog2SizeY < 64

  - If xCb % CtbLog2SizeY is smaller than 64, and yCb % CtbLog2SizeY is smaller than 64, at least one of the two following shall be true:

    - ( yCb + ( mvLX[ 1 ] >> 4 ) ) % CtbLog2SizeY >= 64

    - ( xCb + ( mvLX[ 0 ] >> 4 ) ) % CtbLog2SizeY >= 64

  - Otherwise, if xCb % CtbLog2SizeY is greater than or equal to 64, and if yCb % CtbLog2SizeY is smaller than 64, the following applies:

    - If the block availability is FALSE according to clause 6.4.X with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and neighbouring block (xCb >> CtbLog2SizeY, yCb >> CtbLog2SizeY + 64) as inputs, the following shall be true:

      - ( yCb + ( mvLX[ 1 ] >> 4 ) ) % CtbLog2SizeY >= 64

    - Otherwise (the block availability is TRUE), both of the following shall be true:

      - ( yCb + ( mvLX[ 1 ] >> 4 ) ) % CtbLog2SizeY >= 64

      - ( xCb + ( mvLX[ 0 ] >> 4 ) ) % CtbLog2SizeY >= 64

  - Otherwise ( xCb % CtbLog2SizeY is smaller than 64 and yCb % CtbLog2SizeY is greater than or equal to 64 ), the following applies:

    - If the block availability is FALSE according to clause 6.4.X with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and neighbouring block (xCb >> CtbLog2SizeY + 64, yCb >> CtbLog2SizeY) as inputs, the following shall be true:

      - ( xCb + ( mvLX[ 0 ] >> 4 ) ) % CtbLog2SizeY >= 64

    - Otherwise (the block availability is TRUE), both of the following shall be true:

      - ( yCb + ( mvLX[ 1 ] >> 4 ) ) % CtbLog2SizeY >= 64

      - ( xCb + ( mvLX[ 0 ] >> 4 ) ) % CtbLog2SizeY >= 64

    - xCb % CtbLog2SizeY < 64

    - yCb % CtbLog2SizeY < 64

For CE8.1.2.d

- If ( xCb + ( mvLX[ 0 ] >> 4 ) ) >> CtbLog2SizeY is equal to ( xCb >> CtbLog2SizeY ) – 1, the following shall be true:

  - When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xCb + ( mvLX[ 0 ] >> 4 ) + CtbLog2SizeY, yCb + ( mvLX[ 1 ] >> 4 ) ) as inputs, and the output the output shall be equal to FALSE.

  - When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xCb + ( mvLX[ 0 ] >> 4 ) + cbWidth – 1 + CtbLog2SizeY, yCb + ( mvLX[ 1] >> 4 ) + cbHeight – 1 ) as inputs, and the output the output shall be equal to FALSE.