| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, MA, 9–18 Jan. 2019 | Document: JVET-M0408-v1 |
| --- | --- |

| Title: | CE8: CPR reference memory reuse with reduced memory requirement (CE8.1.2b and CE8.1.2c) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu, Xiang Li and Shan Liu (Tencent) Eric Chai (Ubilinx) | Tel: Email: | xiaozhongxu@tencent.com eric.chai@ublnx.com |
| Source: | Tencent, Ubilinx |  |  |

_____________________________

# Abstract

This contribution reports the CE test results for CE8.1.2b and CE8.1.2c. In the tests in this proposal, by properly managing the search range for a coding block in CPR mode, the requirement of reference sample buffer size can be reduced as compared to the VTM-3 anchor (1 CTU size or 4 64x64 blocks). The allowed reference area for a coding block in CPR mode depends on  the position of this coding block relative to the current CTU. Specifically:

In CE8.1.2b, the size of reference sample buffer is reduced from 4 64x64 blocks to 3 64x64 blocks. Simulation results report that

- 41.02%/24.55%/14.75% AI/RA/LB BD rate reduction can be achieved for SCC 1080p class, when compared to VTM-3.0 anchor (CPR=0, PLT=0).

In CE8.1.2c, the size of reference sample buffer is reduced from 4 64x64 blocks to 2 64x64 blocks. Simulation results report that

- 36.76%/21.43%/12.09% AI/RA/LB BD rate reduction can be achieved for SCC 1080p class, when compared to VTM-3.0 anchor (CPR=0, PLT=0)

# Introduction

In VTM-3.0 and VVC WD [1], the search range for CPR mode is restricted inside the current CTU. In a worst case, a memory size equal to 1 CTU size of reference samples is required to store the reference samples for CPR mode. However, the 1 CTU size of memory, or four 64x64 blocks in VTM-3.0, is not fully utilized, especially at the beginning of decoding the current CTU. In this case, there are just few/no reference samples available inside the current CTU that can be used to predict a CPR coded block. At the meantime, some parts of the reference sample buffer may still store the reconstructed samples from the left CTU, until they are updated by the already coded reference samples inside the current CTU. It is therefore desirable that when the part of reference sample memory has not yet been updated, its samples can be used for CPR compensation. This effectively extends the CPR search range from within the current CTU to some parts of the previous CTU.

Following the thoughts, the total memory size used for CPR mode can be reduced from 4 64x64 blocks. The details of the proposed methods are discussed in the following section.

# Proposed Solutions

In this document, it is firstly proposed to not allow CPR block size to be larger than 64x64 luma samples. The following discussions are based on this assumption.

Two methods are proposed to utilize the reference sample memory more efficiently such that the effective search range for CPR mode can be extended beyond the current CTU, which is the setting for CPR mode in VTM-3.0; at the meantime, the total memory requirement is reduced. The search range for a coding block in CPR mode depends on which of the 4 64x64 blocks of the current CTU it locates in and the partitioning at 128x128 level (horizontal binary/vertical binary/quad-tree partition).

In CE8.1.2b, the total of 3 64x64 blocks are used for CPR mode, which is 1 64x64 less when compared to the current VTM-3 anchor. This means that in addition to the 64x64 block unit for storing the reference samples of the current 64x64 block, additional 2 64x64 block size of memory can be used to store previously coded reference samples for CPR prediction. In Fig. 1, an illustration of the search ranges for each of the 64x64 block locations in the current CTU is shown.

Fig. 1: current CTU processing order and its available reference samples in current and left CTU. Total reference sample memory is 3 64x64 blocks (luma samples). Vertical strip block is the current 64x64 block; grey block is previously coded 64x64 blocks; dotted area encloses the available reference area for the current coding block. Top row: when quad-tree or horizontal binary-tree split is enforced at 128x128 level; Bottom row: when vertical ternary-tree split is enforced at 128x128 level.

The search ranges in Fig.1 can be explained in the following texts:

•	If current block falls into the top-left 64x64 block of the current CTU, then in addition to the already reconstructed samples in the current CTU, it can also refer to the reference samples in the top-right and bottom-right 64x64 blocks of the left CTU, using CPR mode.

•	If current block falls into the top-right 64x64 block of the current CTU, then in addition to the already reconstructed samples in the current CTU, if luma location (0, 64) relative to the current CTU has not yet been reconstructed, the current block can also refer to the reference samples in the top-right 64x64 block 64x64 block of the left CTU, using CPR mode.

•	If current block falls into the bottom-left 64x64 block of the current CTU, then in addition to the already reconstructed samples in the current CTU, if luma location (64, 0) relative to the current CTU has not yet been reconstructed, the current block can also refer to the reference samples in the bottom-right 64x64 block of the left CTU, using CPR mode.

•	If current block falls into the bottom-right 64x64 block of the current CTU, it can only refer to the already reconstructed samples in the current CTU except for the top-left 64x64 block, using CPR mode.

In CE8.1.2c, the total of 2 64x64 blocks are used for CPR mode, which is 2 64x64 less when compared to the current VTM-3 anchor. This means that in addition to the 64x64 block unit for storing the reference samples of the current 64x64 block, additional 1 64x64 block size of memory can be used to store previously coded reference samples for CPR prediction. In Fig. 2, an illustration of the search ranges for each of the 64x64 block locations in the current CTU is shown.

Fig. 2: current CTU processing order and its available reference samples in current and left CTU. Total reference sample memory is 2 64x64 blocks (luma samples). Vertical strip block is the current 64x64 block; grey block is previously coded 64x64 blocks; dotted area encloses the available reference area for the current coding block. Top row: when quad-tree or horizontal binary-tree split is enforced at 128x128 level; Bottom row: when vertical ternary-tree split is enforced at 128x128 level.

The search ranges in Fig.2 can be explained in the following texts:

•	If current block falls into the top-left 64x64 block of the current CTU, then in addition to the already reconstructed samples in the current CTU, it can also refer to the reference samples in the top-right 64x64 block of the left CTU, using CPR mode.

•	If current block falls into the top-right 64x64 block of the current CTU, then the current block can refer to the already reconstructed samples in same 64x64 block and the top-left 64x64 block of the current CTU, using CPR mode.

•	If current block falls into the bottom-left 64x64 block of the current CTU, then the current block can refer to the already reconstructed samples in same 64x64 block of the current CTU. In addition, if luma location (64, 0) relative to the current CTU has not yet been reconstructed, it can also refer to the reference samples in the top-left 64x64 block of the current CTU, using CPR mode; otherwise (luma location (64, 0) relative to the current CTU has been reconstructed), it can also refer to the reference samples in the top-right 64x64 block of the current CTU, using CPR mode.

•	If current block falls into the bottom-right 64x64 block of the current CTU, then the current block can refer to the already reconstructed samples in same 64x64 block and the top-right 64x64 block of the current CTU, using CPR mode.

For complexity reduction purpose, in these two tests, the reference block should come from the same 64x64 block in reference sample memory.

# Experimental results

Tests were carried out by using the VTM3.0 software, and the common test conditions (CTC) defined in [2] and common test conditions in CE8 [3] are followed.

The following four tests are carried out:

Table 1: Summary results of Test 8.1.2b vs. VTM3.0+CPR=0 and VTM3.0+CPR=1

Table 2: Summary results of Test 8.1.2b vs. VTM3.0+CPR=0 and VTM3.0+CPR=1+PLT=1

Table 3: Summary results of Test 8.1.2c vs. VTM3.0+CPR=0 and VTM3.0+CPR=1

Table 4: Summary results of Test 8.1.2c vs. VTM3.0+CPR=0 and VTM3.0+CPR=1+PLT=1

# Recommendation

In this proposal, results of reusing reference sample memory to extend the CPR search range from within the current CTU to some parts of the left CTU are reported. In addition, the requirement for reference sample memory is also reduced by choosing the search range according to the CU positions.

The authors would like to thank LGE for cross-checking.

# References

[1] B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 3)”, JVET-L1001-v7, the 12th JVET meeting, the 12th JVET meeting, Oct. 2018, Macao, CN.

[2] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, the 12th JVET meeting, Oct. 2018, Macao, CN.

[3] X. Xu, Y.-C. Sun, Y.-H. Chao, J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding”, JVET-L1028, the 12th JVET meeting, Oct. 2018, Macao, CN.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ubilinx may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Spec Text Changes relative to JVET-L1001-v8

In section 8.3.2.1 General

……

If treeType is equal to SINGLE_TREE or treeType is equal to DUAL_TREE_LUMA, it is a requirement of bitstream conformance that when the reference picture is the current picture, the luma motion vector mvLX shall obey the following constraints:

- When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xCb + ( mvLX[ 0 ] >> 4 ), yCb + ( mvLX[ 1 ] >> 4 ) ) as inputs, and the output the output shall be equal to TRUE.

- When the derivation process for block availability as specified in clause 6.4.X [Ed. (BB): Neighbouring blocks availability checking process tbd] is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and the neighbouring luma location ( xCb + ( mvLX[ 0 ] >> 4 ) + cbWidth – 1, yCb + ( mvLX[ 1] >> 4 ) + cbHeight – 1 ) as inputs, and the output the output shall be equal to TRUE.

- One or both the following conditions shall be true:

- The value of ( mvLX[ 0 ] >> 4 ) + cbWidth is less than or equal to 0.

- The value of ( mvLX[ 1 ] >> 4 ) + cbHeight is less than or equal to 0.

- The following conditions shall be true:

( yCb + ( mvLX[ 1 ] >> 4 ) ) >> CtbLog2SizeY = yCb >> CtbLog2SizeY				(8-xxx)

( yCb + ( mvLX[ 1 ] >> 4 ) + cbHeight – 1) >> CtbLog2SizeY = yCb >> CtbLog2SizeY		(8-xxx)

( xCb + ( mvLX[ 0 ] >> 4 ) ) >> CtbLog2SizeY >= ( xCb >> CtbLog2SizeY ) – 1 		(8-xxx)

( xCb + ( mvLX[ 0 ] >> 4 ) + cbWidth – 1) >> CtbLog2SizeY >= ( xCb >> CtbLog2SizeY ) – 1	(8-xxx)

For CE8.1.2.b

- The following conditions shall be true:

- ( yCb + ( mvLX[ 1 ] >> 4 ) ) >> 64 = ( yCb + ( mvLX[ 1 ] >> 4 ) + cbHeight – 1) >> 64	(8-xxx)

- ( xCb + ( mvLX[ 0 ] >> 4 ) ) >> 64 = ( xCb + ( mvLX[ 0 ] >> 4 ) + cbWidth – 1) >> 64	(8-xxx)

- If ( xCb + ( mvLX[ 0 ] >> 4 ) ) >> CtbLog2SizeY is equal to ( xCb >> CtbLog2SizeY ) – 1, the following shall apply:

  - At least one of the two following shall be true:

    - xCb % CtbLog2SizeY < 64

    - yCb % CtbLog2SizeY < 64

  - If xCb % CtbLog2SizeY is smaller than 64 and yCb % CtbLog2SizeY is smaller than 64, the following shall be true:

    - ( xCb + ( mvLX[ 0 ] >> 4 ) ) % CtbLog2SizeY >= 64

  - Otherwise, if xCb % CtbLog2SizeY is greater than or equal to 64, and yCb % CtbLog2SizeY is smaller than 64, the following shall be true:

    - The block availability shall be FALSE according to clause 6.4.X with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and neighbouring block (xCb >> CtbLog2SizeY, yCb >> CtbLog2SizeY + 64) as inputs.

    - ( xCb + ( mvLX[ 0 ] >> 4 ) ) % CtbLog2SizeY >= 64

    - ( yCb + ( mvLX[ 1 ] >> 4 ) ) % CtbLog2SizeY < 64

  - Otherwise (xCb % CtbLog2SizeY is smaller than 64 and yCb % CtbLog2SizeY is greater than or equal to 64), the following shall be true:

    - The block availability is FALSE according to clause 6.4.X with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and neighbouring block (xCb >> CtbLog2SizeY + 64, yCb >> CtbLog2SizeY) as inputs.

    - ( xCb + ( mvLX[ 0 ] >> 4 ) ) % CtbLog2SizeY >= 64

    - ( yCb + ( mvLX[ 1 ] >> 4 ) ) % CtbLog2SizeY >= 64

- If ( xCb + ( mvLX[ 0 ] >> 4 ) ) >> CtbLog2SizeY is equal to xCb >> CtbLog2SizeY, and xCb % CtbLog2SizeY is greater than or equal to 64 and yCb % CtbLog2SizeY is greater than or equal to 64, at least one of the two following shall be true:

  - ( xCb + ( mvLX[ 0 ] >> 4 ) ) % CtbLog2SizeY >= 64

  - ( yCb + ( mvLX[ 1 ] >> 4 ) ) % CtbLog2SizeY >= 64

For CE8.1.2.c

- The following conditions shall be true:

- ( yCb + ( mvLX[ 1 ] >> 4 ) ) >> 64 = ( yCb + ( mvLX[ 1 ] >> 4 ) + cbHeight – 1) >> 64	(8-xxx)

- ( xCb + ( mvLX[ 0 ] >> 4 ) ) >> 64 = ( xCb + ( mvLX[ 0 ] >> 4 ) + cbWidth – 1) >> 64	(8-xxx)

- If ( xCb + ( mvLX[ 0 ] >> 4 ) ) >> CtbLog2SizeY is equal to ( xCb >> CtbLog2SizeY ) – 1, the following shall apply:

  - The following shall be true:

    - xCb % CtbLog2SizeY < 64

    - yCb % CtbLog2SizeY < 64

    - ( xCb + ( mvLX[ 0 ] >> 4 ) ) % CtbLog2SizeY >= 64

    - ( yCb + ( mvLX[ 1 ] >> 4 ) ) % CtbLog2SizeY < 64

- If ( xCb + ( mvLX[ 0 ] >> 4 ) ) >> CtbLog2SizeY is equal to xCb >> CtbLog2SizeY, the following shall apply:

  - If xCb % CtbLog2SizeY is greater than or equal to 64, and yCb % CtbLog2SizeY is smaller than 64, the following shall be true:

    - ( yCb + ( mvLX[ 1 ] >> 4 ) ) % CtbLog2SizeY < 64

  - Otherwise, if xCb % CtbLog2SizeY is smaller than 64, and yCb % CtbLog2SizeY is greater than or equal to 64, and the block availability is FALSE according to clause 6.4.X with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and neighbouring block (xCb >> CtbLog2SizeY + 64, yCb >> CtbLog2SizeY) as inputs, the following shall be true:

    - ( xCb + ( mvLX[ 0 ] >> 4 ) ) % CtbLog2SizeY < 64

  - Otherwise, if xCb % CtbLog2SizeY is smaller than 64, and yCb % CtbLog2SizeY is greater than or equal to 64, and and the block availability is TRUE according to clause 6.4.X with the current luma location ( xCurr, yCurr ) set equal to ( xCb, yCb ) and neighbouring block (xCb >> CtbLog2SizeY + 64, yCb >> CtbLog2SizeY) as inputs, the following shall be true:

    - ( xCb + ( mvLX[ 0 ] >> 4 ) ) % CtbLog2SizeY >= 64

    - ( yCb + ( mvLX[ 1 ] >> 4 ) ) % CtbLog2SizeY < 64

  - Otherwise, if xCb % CtbLog2SizeY is greater than or equal to 64, and yCb % CtbLog2SizeY is greater than or equal to 64, the following shall be true:

    - ( yCb + ( mvLX[ 1 ] >> 4 ) ) % CtbLog2SizeY >= 64