| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, MA, 9–18 Jan. 2019 | Document: JVET-M0411-v1 |
| --- | --- |

| Title: | Non-CE8: Inter mode related flag signaling when current picture is the only reference picture |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu, Xiang Li and Shan Liu | Tel: Email: | xiaozhongxu@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes not to signal the following inter coding related flags in condition that the current picture is the only reference picture for a slice. Specifically, sub-block merge flag, affine flag, mmvd flag and intra-inter flag are not signaled. Simulation results report that BD rate changes are negligible (within +-0.1%) when compared to VTM-3.0 anchor (CPR=1).

# Introduction

In VTM-3.0 and current VVC WD [1], CPR mode is enabled by using the current picture is as a reference picture. The syntax structure for supporting CPR mode is aligned with regular inter coding.

However, combinations of CPR mode and some of the inter coding tools are currently not supported. More specifically, if any of the following flags is true, the reference picture cannot be the current picture.

- inter_affine_flag

- merge_ subblock_flag

- mmvd_flag

- mh_intra_flag

merge_triangle_flagFor a slice for which both temporal reference picture(s) and current decoded picture are available as its reference, motion vector prediction from neighboring blocks may contain either CPR candidates or regular inter candidates. Among those candidates, only the regular inter candidates can be used in the above mentioned inter coding tools. This is ensured by both of the following methods:

- excluding CPR candidates from motion vector derivation for these tools (subblock merge, mmvd, triangle partion)

- apply bitstream conformance check to disallow the combinations at the decoder side

In one particular case, if the current picture is the only reference picture for a slice, all motion vector predictors will exclude all CPR candidates, therefore none of the above coding modes can be used. The bitstream conformance check and decoder side operations seem to be unnecessary.

# Proposed Solutions

In this proposal, the values of the above-mentioned flags are not signaled, but inferred to be 0, when the current picture is the only reference picture in a slice. In this way, the bitstream conformance check is no longer needed.

# Experimental results

Tests are carried out by using the VTM3.0 software, and the common test conditions (CTC) defined in [2] and common test conditions in CE8 [3] are followed.

Table 1: Summary results of proposed method vs. VTM3.0+CPR=1

# Recommendation

In this proposal, it is proposed not to signal some of the inter related coding mode flags when current picture is the only reference picture. Simulation results show only minor impact on the proposed changes. It is recommended to include the proposed signaling change into next version VVC WD and VTM software.

The authors would like to thank xxx for cross-checking.

# References

[1] B. Bross, J. Chen and S. Liu, “Versatile Video Coding (Draft 3)”, JVET-L1001-v7, the 12th JVET meeting, the 12th JVET meeting, Oct. 2018, Macao, CN.

[2] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, the 12th JVET meeting, Oct. 2018, Macao, CN.

[3] X. Xu, Y.-C. Sun, Y.-H. Chao, J. Xu, “Description of Core Experiment 8 (CE8): Screen Content Coding”, JVET-L1028, the 12th JVET meeting, Oct. 2018, Macao, CN.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Spec Text Changes relative to JVET-L1001-v7

#### 8.3.4.6 Coding unit syntax

| } else { |  |
| --- | --- |
| if( slice_type = = B ) |  |
| inter_pred_idc[ x0 ][ y0 ] | ae(v) |
| if( sps_affine_enabled_flag && cbWidth >= 16 && cbHeight >= 16 && !CurrPicIsOnlyRef ) { |  |
| inter_affine_flag[ x0 ][ y0 ] | ae(v) |
| if( sps_affine_type_flag && inter_affine_flag[ x0 ][ y0 ] ) |  |
| cu_affine_type_flag[ x0 ][ y0 ] | ae(v) |
| } |  |

#### 8.3.4.8 Merge data syntax

| merge_data( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| If( !CurrPicIsOnlyRef ) { |  |
| mmvd_flag[ x0 ][ y0 ] | ae(v) |
| } |  |
| if( mmvd_flag[ x0 ][ y0 ] = = 1 ) { |  |
| mmvd_merge_flag[ x0 ][ y0 ] | ae(v) |
| mmvd_distance_idx[ x0 ][ y0 ] | ae(v) |
| mmvd_direction_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( MaxNumSubblockMergeCand > 0 && cbWidth >= 8 && cbHeight >= 8 && !CurrPicIsOnlyRef ) |  |
| merge_subblock_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_subblock_flag[ x0 ][ y0 ] = = 1 ) { |  |
| if( MaxNumSubblockMergeCand > 1 ) |  |
| merge_subblock_idx[ x0 ][ y0 ] | ae(v) |
| } else { |  |
| if( sps_mh_intra_enabled_flag && cu_skip_flag[ x0 ][ y0 ] = = 0 && ( cbWidth * cbHeight ) >= 64 && cbWidth < 128 && cbHeight < 128 && !CurrPicIsOnlyRef ) { |  |
| mh_intra_flag[ x0 ][ y0 ] | ae(v) |
| if( mh_intra_flag[ x0 ][ y0 ] ) { |  |
| if ( cbWidth <= 2 * cbHeight ¦ ¦ cbHeight <= 2 * cbWidth ) |  |
| mh_intra_luma_mpm_flag[ x0 ][ y0 ] | ae(v) |
| if( mh_intra_luma_mpm_flag[ x0 ][ y0 ] ) |  |
| mh_intra_luma_mpm_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| if( sps_triangle_enabled_flag && slice_type = = B && cbWidth * cbHeight >= 16 && !CurrPicIsOnlyRef ) |  |
| merge_triangle_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_triangle_flag[ x0 ][ y0 ] ) |  |
| merge_triangle_idx[ x0 ][ y0 ] | ae(v) |
| else if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| } |  |
| } |  |
| } |  |