| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0415 |
| --- | --- |

| Title: | Comments on High-Level Syntax of VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JCT-VC |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This document proposes comments on high-level syntax of VVC WD.  It includes following proposals:

- Modifications are proposed to the horizontal reference wrap-around signaling in VVC WD.

- It is proposed to define a NAL unit type which includes signaling information which is applicable to multiple  tile groups in a picture.

# Introduction

This document proposes comments on High-Level Syntax of VVC WD [1].  The following is proposed:

- Proposal 1: VVC WD includes horizontal reference wrap-around signaling. This allows specifying a horizontal offset which is used for horizontal wrap-around motion compensation for inter-prediction. We  propose modifications to the horizontal reference wrap-around signaling.

- Proposal 2: It is proposed to define a NAL unit type which includes signaling information which is applicable to multiple tile groups in a picture.

# Proposal

## Proposal 1

VVC WD [1] includes horizontal reference wrap-around signaling (via syntax elements sps_ref_wraparound_enabled_flag and sps_ref_wraparound_offset).

It is asserted that the current reference wrap-around signaling in Sequence Parameter Set (SPS) has following drawbacks:

- For sps_ref_wraparound_offset syntax element, a value of zero is not allowed and is thus wasted. Since sps_ref_around_offset signalling uses ue(v) coding where value of 0 takes the minimum number of bits this wastage is not warranted. To prevent this we propose two alternative options:

  - In option 1: The reference wrap-around offset is signalled with a minus1 encoding

  - In option 2: The syntax element sps_reference_wraparound_enabled_flag is not signalled, the value of zero is allowed to be signalled for reference wrap-around offset (i.e. for sps_ref_wraparound_offset syntax element) and semantics are defined such that the zero value specifies that horizontal wrap-around motion compensation is not used for inter-prediction.

- Current reference wrap-around offset signalling (i.e. sp_ref_wraparound_offset syntax element) wastes bits as the value signalled is required to be integer multiple of MinCbSizeY. We propose to signal the value for reference wrap-around offset signalling in units of MinCbSizeY. This is proposed for both option 1 and option 2 above.

The two alternative options are described below with highlights compared to VVC WD [1].

### Option 1

It is proposed to signal the reference wrap-around offset with a minus1 coding. Additionally we propose to signal the value for reference wrap-around offset signalling in units of MinCbSizeY.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_ref_wraparound_enabled_flag | u(1) |
| if( sps_ref_wraparound_enabled_flag ) |  |
| sps_ref_wraparound_offset_minus1 | ue(v) |
| … |  |
| } |  |

sps_ref_wraparound_enabled_flag equal to 1 specifies that horizontal wrap-around motion compensation is applied for inter prediction. sps_ref_wraparound_enabled_flag equal to 0 specifies that that horizontal wrap-around motion compensation is not applied.

sps_ref_wraparound_offset_minus1 plus 1 specifies the offset used for computing the horizontal wrap-around position in units of MinCbSizeY luma samples. sps_ref_wraparound_offset_minus1 shall be in the range of 0 to (pic_width_in_luma_samples / MinCbSizeY)-1, inclusive.  shall not be equal to 0, shall not be larger than pic_width_in_luma_samples, and shall be an integer multiple of MinCbSizeY.

Additionally the above change should be reflected in sections 8.3.6.3.2 and 8.3.6.3.3. as follows:

##### 8.3.6.3.2 Luma sample 8-tap interpolation filtering process

Inputs to this process are:

–	a luma location in full-sample units ( xIntL, yIntL ),

–	a luma location in fractional-sample units ( xFracL, yFracL ),

–	the luma reference sample array refPicLXL.

Output of this process is a predicted luma sample value predSampleLXL

The variables shift1, shift2 and shift3 are derived as follows:

–	The variable shift1 is set equal to Min( 4, BitDepthY − 8 ), the variable shift2 is set equal to 6 and the variable shift3 is set equal to Max( 2, 14 − BitDepthY ).

–	The variable picW is set equal to pic_width_in_luma_samples and the variable picH is set equal to pic_height_in_luma_samples.

The luma interpolation filter coefficients fL[ p ] for each 1/16 fractional sample position p equal to xFracL or  yFracL are specified in Table 8-9.

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..7:

xInti = sps_ref_wraparound_enabled_flag  ?  ClipH( (sps_ref_wraparound_offset_minus1+1)*MinCbSizeY, picW, ( xIntL + i − 3 ) )	(8-673) 
	:  Clip3( 0, picW − 1, xIntL + i − 3 )

yInti = Clip3( 0, picH − 1, yIntL + i − 3 )

…

##### Luma sample bilinear interpolation filtering process

Inputs to this process are:

- a luma location in full-sample units ( xIntL, yIntL ),

- a luma location in fractional-sample units ( xFracL, yFracL ),

- the luma reference sample array refPicLXL,

Output of this process is a predicted luma sample value predSampleLXL

The variable shift1 is set equal to Min( 4, BitDepthY − 8 ), the variable shift2 is set equal to 6 and the variable shift3 is set equal to Max( 2, 14 − BitDepthY ).

The variable picW is set equal to pic_width_in_luma_samples and the variable picH is set equal to pic_height_in_luma_samples.

The luma interpolation filter coefficients fL[ p ] for each 1/16 fractional sample position p equal to xFracL or  yFracL are specified in Table 8-10.

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..1:

xInti = sps_ref_wraparound_enabled_flag  ?  ClipH( (sps_ref_wraparound_offset_minus1+1)*MinCbSizeY, picW, ( xIntL + i − 3 ) )	(8-680)
	:  Clip3( 0, picW − 1, xIntL + i )

yInti = Clip3( 0, picH − 1, yIntL + i )

…

##### 8.3.6.3.4 Chroma sample interpolation process

Inputs to this process are:

–	a chroma location in full-sample units ( xIntC, yIntC ),

–	a chroma location in 1/32 fractional-sample units ( xFracC, yFracC ),

–	the chroma reference sample array refPicLXC.

Output of this process is a predicted chroma sample value predSampleLXC

The variables shift1, shift2 and shift3 are derived as follows:

–	The variable shift1 is set equal to Min( 4, BitDepthC − 8 ), the variable shift2 is set equal to 6 and the variable shift3 is set equal to Max( 2, 14 − BitDepthC ).

–	The variable picWC is set equal to pic_width_in_luma_samples / SubWidthC and the variable picHC is set equal to pic_height_in_luma_samples / SubHeightC.

The luma interpolation filter coefficients fC[ p ] for each 1/32 fractional sample position p equal to xFracC or  yFracC are specified in Table 8-11.

The variable xOffset is set equal to ((sps_ref_wraparound_offset_minus1+1)*MinCbSizeY)/ SubWidthC.

The chroma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..3:

xInti = sps_ref_wraparound_enabled_flag  ?  ClipH( xOffset, picWC, ( xIntC + i − 1 ) )	(8-687)
	:  Clip3( 0, picWC − 1, xIntC + i − 1 )

yInti = Clip3( 0, picHC − 1, yIntC + i − 1 )		(8-688)

### Option 2

It is proposed that the sps_reference_wraparound_enabled_flag is not signalled, the value of zero is allowed to be signalled for reference wrap-around offset (i.e. for sps_ref_wraparound_offset syntax element) and semantics are defined such that the zero value specifies that horizontal wrap-around motion compensation is not used for inter-prediction. Additionally we propose to signal the value for reference wrap-around offset signalling in units of MinCbSizeY.

The proposed changes are shown highlighted below compared to [1].

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_ref_wraparound_enabled_flag | u(1) |
| if( sps_ref_wraparound_enabled_flag ) |  |
| sps_ref_wraparound_offset | ue(v) |
| … |  |
| } |  |

sps_ref_wraparound_enabled_flag equal to 1 specifies that horizontal wrap-around motion compensation is applied for inter prediction. sps_ref_wraparound_enabled_flag equal to 0 specifies that that horizontal wrap-around motion compensation is not applied.

sps_ref_wraparound_offset specifies the offset used for computing the horizontal wrap-around position in units of MinCbSizeY luma samples. sps_ref_wraparound_offset shall be in the range of 0 to (pic_width_in_luma_samples / MinCbSizeY), inclusive. sps_ref_wraparound_offset equal to 0 specifies that horizontal wrap-around motion compensation is not used for inter-prediction.  shall not be equal to 0, shall not be larger than pic_width_in_luma_samples, and shall be an integer multiple of MinCbSizeY.

Additionally the above change should be reflected in sections 8.3.6.3.2 and 8.3.6.3.3. as follows:

##### 8.3.6.3.2 Luma sample 8-tap interpolation filtering process

Inputs to this process are:

–	a luma location in full-sample units ( xIntL, yIntL ),

–	a luma location in fractional-sample units ( xFracL, yFracL ),

–	the luma reference sample array refPicLXL.

Output of this process is a predicted luma sample value predSampleLXL

The variables shift1, shift2 and shift3 are derived as follows:

–	The variable shift1 is set equal to Min( 4, BitDepthY − 8 ), the variable shift2 is set equal to 6 and the variable shift3 is set equal to Max( 2, 14 − BitDepthY ).

–	The variable picW is set equal to pic_width_in_luma_samples and the variable picH is set equal to pic_height_in_luma_samples.

The luma interpolation filter coefficients fL[ p ] for each 1/16 fractional sample position p equal to xFracL or  yFracL are specified in Table 8-9.

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..7:

xInti = sps_ref_wraparound_enabled_flagsps_ref_wraparound_offset  ?  ClipH( sps_ref_wraparound_offset*MinCbSizeY, picW, ( xIntL + i − 3 ) )	(8-673) 
:  Clip3( 0, picW − 1, xIntL + i − 3 )

yInti = Clip3( 0, picH − 1, yIntL + i − 3 )

…

##### Luma sample bilinear interpolation filtering process

Inputs to this process are:

- a luma location in full-sample units ( xIntL, yIntL ),

- a luma location in fractional-sample units ( xFracL, yFracL ),

- the luma reference sample array refPicLXL,

Output of this process is a predicted luma sample value predSampleLXL

The variable shift1 is set equal to Min( 4, BitDepthY − 8 ), the variable shift2 is set equal to 6 and the variable shift3 is set equal to Max( 2, 14 − BitDepthY ).

The variable picW is set equal to pic_width_in_luma_samples and the variable picH is set equal to pic_height_in_luma_samples.

The luma interpolation filter coefficients fL[ p ] for each 1/16 fractional sample position p equal to xFracL or  yFracL are specified in Table 8-10.

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..1:

xInti = sps_ref_wraparound_enabled_flagsps_ref_wraparound_offset    ?  ClipH( sps_ref_wraparound_offset*MinCbSizeY, picW, ( xIntL + i − 3 ) )	(8-680)
:  Clip3( 0, picW − 1, xIntL + i )

yInti = Clip3( 0, picH − 1, yIntL + i )

…

##### 8.3.6.3.4 Chroma sample interpolation process

Inputs to this process are:

–	a chroma location in full-sample units ( xIntC, yIntC ),

–	a chroma location in 1/32 fractional-sample units ( xFracC, yFracC ),

–	the chroma reference sample array refPicLXC.

Output of this process is a predicted chroma sample value predSampleLXC

The variables shift1, shift2 and shift3 are derived as follows:

–	The variable shift1 is set equal to Min( 4, BitDepthC − 8 ), the variable shift2 is set equal to 6 and the variable shift3 is set equal to Max( 2, 14 − BitDepthC ).

–	The variable picWC is set equal to pic_width_in_luma_samples / SubWidthC and the variable picHC is set equal to pic_height_in_luma_samples / SubHeightC.

The luma interpolation filter coefficients fC[ p ] for each 1/32 fractional sample position p equal to xFracC or  yFracC are specified in Table 8-11.

The variable xOffset is set equal to (sps_ref_wraparound_offset*MinCbSizeY)/ SubWidthC.

The chroma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..3:

xInti = sps_ref_wraparound_enabled_flagsps_ref_wraparound_offset  ?  ClipH( xOffset, picWC, ( xIntC + i − 1 ) )	(8-687)
:  Clip3( 0, picWC − 1, xIntC + i − 1 )

yInti = Clip3( 0, picHC − 1, yIntC + i − 1 )		(8-688)

## Proposal 2

In current VVC WD number of syntax elements are signaled and repeated in each tile group header (e.g. ALF data).

It is proposed to define a new NAL unit type which includes signaling information which is applicable to multiple tile groups in a picture. The information signaled in this NAL unit type can be overridden by information signaled in a tile group header as required.

A picture header NAL unit type is proposed as shown below.

Table 7-1 – NAL unit type codes and NAL unit type classes

| Nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| 0 | NONIRAP_NUT | Coded tile group of a non-IRAP picture tile_group_layer_rbsp( ) | VCL |
| 1 | IRAP_NUT | Coded tile group of an IRAP picture tile_group_layer_rbsp( ) | VCL |
| 2 | PICH_NUT | Picture header of a picture pic_header() | VCL/ non-VCL |
| 23-15 | RSV_VCL_NUT | Reserved VCL NAL Units | VCL |
| 16 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 17 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 18 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 19 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) | non-VCL |
| 20, 21 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 22-26 | RSV_NVCL | Reserved | non-VCL |
| 27-31 | UNSPEC | Unspecified | non-VCL |

Picture header syntax and semantics

| pic_header( ) { | Descriptor |
| --- | --- |
| pic_parameter_set_id | ue(v) |
| pic_header_id | ue(v) |
| // picture level data |  |
| byte_alignment( ) |  |
| } |  |

pic_parameter_set_id identifies the PPS for reference by other syntax elements. The value of pps_pic_parameter_set_id shall be in the range of 0 to 63, inclusive.

pic_header_id identifies the picture header for reference by other syntax elements.

It is recommended that which picture level data should be signaled in picture header be discussed.

Tile group header syntax and semantics

| tile_group_header( ) { | Descriptor |
| --- | --- |
| tile_group_pic_header_id | ue(v) |
| // tile group level data |  |
| byte_alignment( ) |  |
| } |  |

tile_group_pic_header_id specifies the value of pic_header_id for the picture header that applies to this tile group. The value of tile_group_pic_header_id shall be in the range of 0 to 2^32-1, inclusive.

# Conclusion

This document proposes modifications to HLS signaling for VVC.

- Modifications are proposed to the horizontal reference wrap-around signaling in VVC WD.

- It is proposed to define a NAL unit type which includes signaling information which is applicable to multiple tile groups in a picture.

We propose to adopt these proposals in the next version of VVC WD.

# Patent rights declaration(s)

Sharp may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-L1001-v7, Versatile Video Coding (Draft 3), Macao, December 2018