| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0426-v1 |
| --- | --- |

| Title: | CE3-related: Improvement on the Intra Sub-Partitions Coding Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Santiago De-Luxán-Hernández, Valeri George, Jackie Ma, Tung Nguyen Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-511 santiago.de.luxan@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution presents an improvement of the Intra Sub-Partitions (ISP) coding mode proposed in the CE3 Tests 1.1.1 and 1.1.2 (JVET-M0102). Particularly, an improvement for Test CE3-1.1.2 is proposed. The changes in the design create a more symmetric and harmonic structure of the sub-partitions. Results show a gain of 0,06% for the AI configuration and a gain of 0,01% in the case of RA (with respect to Test CE3-1.1.2). There is no impact on the run-times in both configurations and both encoder and decoder. On the other hand, class F shows higher gains: 0.41% for AI and 0.30% for RA.

# Introduction

Document JVET-M0102 [1] presents two CE3 experiments on the Intra Sub-Partitions (ISP) coding mode, namely:

- Test CE3-1.1.1 : ISP with a better trade-off between coding gain and encoder run-time.

- Test CE3-1.1.2 : Test 1.1.1 with a restriction: the resulting partitions must have a width of at least 4 samples. This implies that 4×N blocks can no longer be divided vertically (and therefore no split type parameter signaling is required) and 8×N blocks are divided in 2 sub-partitions if the split is vertical (instead of 4).

The additional constraint implemented in Test 1.1.2 is responsible for the loss of Test 1.1.2 with respect to Test 1.1.1, given that ISP is not utilized vertically on 4×N blocks with N>4 and the number of sub-partitions in the vertical split of  8×N blocks with N>4 is constrained from 4 to 2. To improve this loss and get a more symmetrical design of the sub-partitions, this contribution proposes the following modification to the CE3-1.1.2 Test presented in [1]:

Given an input 4×N block with N>4, the split flag will be transmitted again to the decoder:

- If the value is 0, a horizontal split will be performed in the same way as the normal configuration of Test 1.1.2

- If the value is 1, the 4×N (with N>4) block will be predicted using the exactly vertical intra mode (50) and then a 1-D vertical transform will be performed on the generated residual signal. The resulting coefficients will be quantized and entropy-coded as independent 1-D arrays. In this case, the CBF of each line will be independently coded from the CBFs of other coefficient lines. Note that in this case the prediction stage is exactly the same as in normal intra prediction. The only difference lies in the transform and the entropy coding. Since there is no intra dependency, all lines can be transformed and entropy coded at the same time. Besides, only the above reference samples are needed to create the prediction. No signaling will be used to transmit the intra mode, since it is inferred to be 50.

# Experimental results

According to the common test conditions [2], the proposed method is evaluated for intra-only (AI) and random-access (RA) configuration with the VTM-3.0 software. The corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, AVX2 on, turbo boost off) with Linux OS and a GCC 7.2.1 compiler.

Table 1: Results with an AI configuration on top of VTM-3.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.13% | -0.12% | 114% | 103% |
| Class A2 | -0.28% | -0.19% | -0.19% | 113% | 102% |
| Class B | -0.50% | -0.41% | -0.42% | 113% | 104% |
| Class C | -0.76% | -0.64% | -0.79% | 112% | 105% |
| Class E | -0.82% | -0.52% | -0.44% | 109% | 104% |
| Overall | -0.52% | -0.40% | -0.42% | 112% | 104% |
| Class D | -0.63% | -0.53% | -0.62% | 112% | 104% |
| Class F (optional) | -2.07% | -1.44% | -1.42% | 103% | 104% |

Table 2: Results with a RA configuration on top of VTM-3.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.14% | -0.08% | 102% | 105% |
| Class A2 | -0.20% | -0.17% | 0.03% | 102% | 101% |
| Class B | -0.27% | -0.13% | -0.22% | 102% | 102% |
| Class C | -0.34% | -0.53% | -0.27% | 102% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.25% | -0.25% | -0.16% | 102% | 102% |
| Class D | -0.31% | -0.55% | -0.26% | 103% | 100% |
| Class F (optional) | -1.63% | -0.99% | -1.04% | 100% | 102% |

Table 3: Results with an AI configuration on top of CE3-1.1.2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.02% | -0.09% | 100% | 102% |
| Class A2 | 0.01% | 0.02% | 0.04% | 100% | 100% |
| Class B | -0.01% | -0.04% | -0.01% | 100% | 99% |
| Class C | -0.07% | -0.09% | -0.19% | 100% | 101% |
| Class E | -0.24% | -0.18% | -0.16% | 100% | 100% |
| Overall | -0.06% | -0.05% | -0.08% | 100% | 100% |
| Class D | -0.15% | -0.13% | -0.21% | 101% | 101% |
| Class F (optional) | -0.41% | -0.27% | -0.40% | 100% | 99% |

Table 4: Results with a RA configuration on top of CE3-1.1.2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.18% | 0.11% | 100% | 104% |
| Class A2 | 0.00% | -0.02% | -0.01% | 100% | 99% |
| Class B | -0.01% | 0.13% | 0.15% | 100% | 100% |
| Class C | 0.00% | -0.17% | -0.15% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.03% | 0.03% | 100% | 100% |
| Class D | -0.09% | -0.11% | -0.18% | 100% | 100% |
| Class F (optional) | -0.30% | -0.16% | -0.26% | 100% | 100% |

# Cross-Check

Cross-checks for the tests have been performed by ITRI.

# Conclusion

The changes in the design create a more symmetric and harmonic structure of the sub-partitions. Results show a gain of 0,06% for the AI configuration and a gain of 0,01% in the case of RA (with respect to Test CE3-1.1.2). There is no impact on the run-times in both configurations and both encoder and decoder. On the other hand, class F shows higher gains: 0.41% for AI and 0.30% for RA. This is probably due to the fact screen content is typically richer in sharp edges, which can be better modelled with smaller partitions.

# References

| [1] | S. De-Luxán-Hernández, V. George, J. Ma, T. Nguyen, H. Schwarz, D. Marpe and T. Wiegand, "CE3: Intra Sub-Partitions Coding Mode (Tests 1.1.1 and 1.1.2)," Document JVET-M0102, Marrakech, MA., 2018. |
| --- | --- |
| [2] | F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring (editors), "JVET common test conditions and software reference configurations for SDR video," Document JVET-L1010, Macao, CN, 2018. |

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).