| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0433 |
| --- | --- |

| Title: | CE4-related: Constraint on GBi index inheritance in Merge Mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Guichun Li, Xiaozhong Xu, Xiang Li, Shan Liu 2747 Park Boulevard, Palo Alto, CA 94306, U.S.A. | Tel: Email: | +1-650-798-3300 guichunli@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to limit the GBi index inheritance to only allow the inheritance from regular spatial merge candidates. The proposed method was implemented on VTM-3.0. It was reported that by applying the proposed constraint, the overall BD-rate impact is 0.03% for both Random Access and Low Delay B configurations.

# Introduction

Generalized Bi-prediction (GBi) was adopted in VVC and included in VTM-3.0. When a block is coded in inter mode and is bi-predicted, its GBi weighting is indicated by the GBi index. In current GBi design, there are 5 weights {-2/8, 3/8, 4/8, 5/8, 10/8} available for low-delay pictures and 3 weights {3/8, 4/8, 5/8} for non-low-delay pictures.

In merge mode, when the merge candidate is bi-predicted, its GBi weighting may be inherited to the current block. In VTM-3.0, both spatial merge candidates and model inherited affine merge candidates have GBi index inheritance enabled.

# Proposed

In the proposed method, a constraint is applied on the GBi index inheritance in merge mode, so that only spatial merge candidates with bi-prediction can have its GBi index inherited. All other type of merge candidates, including sub-block merge will use the default GBi index (1/2 weighting). It is asserted that this simplifies the derivation process as well as saving the storage. Sub-block based merge candidates don’t need to store GBi index anymore. And blocks coded in affine merge mode can save the operations related to GBi.

# Experimental Results

The proposed method is implemented based on VTM-3.0. The simulations were conducted following JVET common test conditions [2]. VTM-3.0 is used as anchor for the test.

Table 3.1 Simulation Result

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.06% | 0.05% | 101% | 101% |  |  |  |  |  |
| Class A2 | 0.00% | 0.15% | 0.14% | 103% | 102% |  |  |  |  |  |
| Class B | 0.04% | 0.15% | 0.05% | 100% | 100% | 0.08% | -0.23% | -0.09% | 103% | 102% |
| Class C | 0.03% | 0.04% | 0.00% | 100% | 99% | 0.04% | 0.31% | 0.35% | 101% | 103% |
| Class E |  |  |  |  |  | -0.05% | -0.08% | 0.01% | 100% | 100% |
| Overall | 0.03% | 0.10% | 0.06% | 101% | 101% | 0.03% | -0.01% | 0.08% | 102% | 102% |
| Class D | -0.02% | -0.05% | -0.03% | 100% | 100% | 0.03% | -0.05% | 0.08% | 103% | 103% |
| Class F | 0.03% | 0.00% | 0.03% | 100% | 100% | 0.08% | -0.16% | -0.53% | 100% | 102% |

# Conclusion

It was observed that by applying the proposed constraint of enabling GBi index inheritance only for spatial merge candidates, the overall BD-rate impact is minor, at 0.03% for both Random Access and Low Delay B configurations. It is proposed to adopt the proposed constraint on GBi index inheritance to VVC and VTM.

# Reference

- J. Chen, Y. Ye, S. Kim, “Algorithm description for Versatile Video Coding and Test Model 3 (VTM 3),” JVT-L1002, Macao, CN, Oct 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-L1010 Macao, CN, Oct 2018.

- Y.-C. Su, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek), Y. He, J. Luo, X. Xiu, Y. Ye, “CE4-related: Generalized bi-prediction improvements combined from JVET-L0197 and JVET-L0296”, JVET-L0646, Macao, CN, Oct 2018

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).