| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0444-v2 |
| --- | --- |

| Title: | CE4-related: Simplified symmetric MVD based on CE4.4.3 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) Jiancong Luo Yuwen He | Tel: Email: | +1-609-436-0885 Daniel.Luo@InterDigital.com Yuwen.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a simplified symmetric MVD mode for bi-prediction based on the corresponding CE test CE4.4.3. With the symmetric MVD mode, MVD of list 1 is not signaled but derived from the signaled MVD of list 0, and reference picture indices of both list 0 and list 1 are not signaled but derived based on the POC of reference pictures and the current picture. Two items are proposed in this contribution: (1) Disabling bi-directional optical flow (BDOF) for symmetric MVD mode;(2) encoder optimizations to improve the performance. The simulation results reportedly show on average 0.33% BD rate saving with 105% encoding time for RA configuration over VTM 3.0.

# Introduction

A symmetric MVD mode for bi-prediction is proposed in JVET-L0370 [2]. With the symmetric MVD mode, motion information including reference picture indices of both list-0 and list-1 and MVD of list-1 are not signaled but derived. Based on this work, it is proposed to disable BDOF when symmetric MVD mode is applied.

The decoding process of the proposed symmetric MVD mode is described below:

Firstly, in slice level, variables BiDirPredFlag, RefIdxSymL0 and RefIdxSymL1 are derived as follows:

- If mvd_l1_zero_flag is 1, BiDirPredFlag is set equal to 0.

- Otherwise, if the nearst reference picture in list-0 and the nearst reference picture in list-1 form a forward and backward pair of reference pictures or a backward and forward pair of reference pictures, BiDirPredFlag is set to 1. Otherwise BiDirPredFlag is set to 0.

Secondly, in CU level, a symmetrical mode flag indicating whether symmetrical mode is used or not is explicitly signaled if the CU is bi-prediction coded and BiDirPredFlag is equal to 1.

When the symmetrical mode flag is true, only mvp_l0_flag, mvp_l1_flag and MVD0 are explicitly signaled. The reference indices for list-0 and list-1 are set equal to the pair of reference pictures, respectively. MVD1 is set equal to (–MVD0). The final motion vectors are shown in below formula.

mvx0, mvy0=(mvpx0+mvdx0,mvpy0+mvdy0) mvx1, mvy1=(mvpx1-mvdx0,mvpy1-mvdy0)

Thirdly, when symmetric mode flag is true, BDOF is not applied to the prediction to reduce the complexity.

Figure 1 Illustration for symmetrical mode

In the encoder, symmetric MVD motion estimation starts with initial MV evaluation. A set of initial MV candidates comprising of the MV obtained from uni-prediction search, the MV obtained from bi-prediction search and the MVs from the AMVP list. The one with the lowest rate-distortion cost is chosen to be the initial MV for the symmetric MVD motion search.

# Simulation results

The contribution has been implemented on top of VTM 3.0 based on CE4.4.3 in CE4 [1]. The test condition described in JVET-L1024 [1] is used to verify the contribution. The results are summarized in Table 1.

Table 1 Coding performance of proposed symmetric MVD over VTM3.0

|  | Random Acess Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.34% | -0.44% | -0.46% | 105% | 99% |
| Class A2 | -0.42% | -0.41% | -0.41% | 105% | 99% |
| Class B | -0.34% | -0.32% | -0.33% | 105% | 99% |
| Class C | -0.25% | -0.27% | -0.22% | 104% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.33% | -0.35% | -0.34% | 105% | 99% |
| Class D | -0.17% | -0.29% | -0.18% | 104% | 99% |
| Class F (optional) | -0.33% | -0.33% | -0.29% | 105% | 99% |

Additionally, the performance of this contribution is compared with CE4.4.3 in Table 2.

Table 2 Coding performance of proposed symmetric MVD over CE4.4.3

|  | Random Acess Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4.4.3 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.29% | -0.25% | 101% | 100% |
| Class A2 | -0.22% | -0.27% | -0.26% | 101% | 99% |
| Class B | -0.19% | -0.23% | -0.16% | 102% | 98% |
| Class C | -0.12% | -0.16% | -0.12% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.17% | -0.23% | -0.18% | 101% | 99% |
| Class D | -0.09% | -0.21% | -0.14% | 100% | 100% |
| Class F (optional) | -0.21% | -0.21% | -0.18% | 100% | 100% |

# Conclusions

This contribution proposes a simplified symmetric MVD mode for bi-prediction coding. The proposed mode derives the reference picture indices for both list 0 and list 1 based on POC of reference pictures and current picture. The proposed mode derives the MVD for the list 1 from list 0 based on the assumption of linear motion. When the proposed symmetric MVD mode is applied, the BDOF is not applied on the prediction. Simulation results reportedly shows on average 0.33% BD-rate saving with 105% encoding time for RA configuration over VTM-3.0.

# References

- H. Yang, S. Liu, K. Zhang, “Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding”, JVET-L1024, the 12th JVET meeting, Oct 2018, Macao, CN.

- H. Chen, H. Yang, J. Chen, “CE4: Symmetrical MVD mode (Test 4.5.1)”, JVET-L0370, the 12th JVET meeting, Oct. 2018, Macao, CN.

# Patent rights declaration(s)

InterDitigal may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).