| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0461 |
| --- | --- |

| Title: | CE2-related: Alternate ALF filter shapes for luma |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Daniel Socek Atul Puri | Tel: Email: | daniel.socek@intel.com atul.puri@intel.com |
| Source: | Intel Corp. |  |  |

_____________________________

# Abstract

In the contribution, it is proposed to modify luma filter shape for adaptive loop filter (ALF). In VTM3.0, the ALF shape for luma is fixed to 7x7 diamond (25 taps and 13 unique coefficients). Two modified luma ALF shapes are considered: (1) combined 7x7 diamond and 9x9 cross filter with fully symmetric outermost cross coefficient (29 taps and 14 unique coefficients, Figure 1a), and (2) combined 7x7 diamond and 9x9 cross filter with fully symmetric two outermost cross coefficients (29 taps and 13 unique coefficients).  Alternate shapes are combined with TC offsets of -2 for LB and RA, and -6 for AI. It is reported that, compared to VTM3.0, the first alternate ALF luma shape (1) yields -0.57%, -0.17%, and -0.36% luma BD-rates for AI, RA, and LB, respectively, while shape (2) yields -0.57%, and -0.34% luma BD-rates for AI and LB, respectively.  There are minor runtime changes in the performed tests.

# Proposed methods

In VTM3.0, there is a single luma ALF filter used (7x7 diamond with 25 taps and 13 unique coefficients). In this contribution two tests are evaluated and presented. In the first test, the reference luma shape is replaced with modified luma shape: combined 7x7 diamond and 9x9 cross filter with fully symmetric outermost cross coefficient, 29 taps and 14 unique coefficients (Figure 1a). In the next test, reference shape is replaced with the second modified luma shape: combined 7x7 diamond and 9x9 cross filter with fully symmetric two outermost cross coefficients, 29 taps and 13 unique coefficients (Figure 1b).

Figure 1. The proposed alternate ALF filter shapes for luma

The for the above experiments TC offsets of -2 for LB and RA, and -6 for AI were used, as in [1].  Table 1 shows comparison between the tests.

Table 1. Summary of the performed tests

|  | Number of taps in luma ALF filter | Number of unique coefficients in luma ALF filter |
| --- | --- | --- |
| VTM3.0 | 25 | 13 |
| Test 1 | 29 | 14 |
| Test 2 | 29 | 13 |

# Experimental results

The proposed methods have been implemented on top of VTM3.0 [2].  Table 2 and Table 3 show results of the two alternate ALF luma shapes, with 14 and 13 unique coefficients, respectively.

# Conclusions

In this contribution, changing the shape of ALF filter for luma to filter from Figure 1a achieves -0.57%, -0.17%, and -0.36% luma BD-rates for AI, RA, and LB, respectively, while replacing the shape of ALF filter for luma to filter from Figure 1b yields -0.57%, and -0.34% luma BD-rates for AI and LB, respectively.  There is no major runtime change.

# Patent rights declaration

Intel Corp. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] M. Karczewicz, N. Shlyakhov, N. Hu, V. Seregin, W.-J. Chien, “CE2.4.1.4: Reduced filter shape size for ALF,” Document of Joint Video Experts Team, JVET-K0371.

[2] VTM3.0 reference software, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

Table 2. Results for alternate luma ALF filter shape from Figure 1a

Table 2. Results for alternate luma ALF filer shape from Figure 1b