| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0470 |
| --- | --- |

| Title: | CE7-related: Golomb-Rice/exponential Golomb coding for abs_remainder and dec_abs_level syntax elements |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Marta Karczewicz | Tel: Email: | +1 858 658 3973 mcoban@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a constant Golomb-Rice to exponential Golomb transition point unary code length for all Rice parameters and HEVC-RExt style binarization for exponential Golomb part of the abs_remainder and dec_abs_level syntax elements to limit the worst-case code length to 32 bits. The simulations results show 0.00% AI, 0.01% RA, 0.02% LB BD-Rate versus the VTM-3.0 anchor for common test conditions.

# Introduction

In VTM-3.0 escape codes (any part of a coefficient’s magnitude not described by a context coded syntax element) i.e. abs_remainder and dec_abs_level syntax elements, are coded using a Golomb-Rice/exponential-Golomb method [6]. The transition point between Golomb-Rice and exponential Golomb coding prefix code length minus 1 is dependent on the Rice parameter and defined by the following LUT

g_auiGoRiceRange[MAX_GR_ORDER_RESIDUAL] ={6, 5, 6, 3}.

The corresponding coefficient value threshold for transitioning from Golomb-Rice to exponential Golomb is

threshold   = g_auiGoRiceRange[ m_goRicePar ] << m_goRicePar.

Using a constant prefix code length for all rice parameters eliminates a look-up table and aligns it with HEVC.

In HEVC the length of the escape codes is restricted to 32 bits in the worst-case. This is considered desirable for implementation in hardware as well as in software [2]. In VVC transform coefficients are restricted to be in a 16-bit range (MAX_ MAX_TR_DYNAMIC_RANGE=15) [1].  For coding of abs_remainder syntax element, taking into account sig_coeff_flag, abs_level_gt1_flag, par_level_flag and abs_level_gt3_flag, the largest possible value for abs_remainder syntax element is 16381. Using worst-case Rice parameter 0, the length of abs_remainder syntax element is 33 bits. For coding of dec_abs_level syntax element, the larget possible value is 215. Using worst-case Rice parameter 0, the length of dec_abs_level syntax element is 35 bits. The worst case can be reduced to 32 bits by using the HEVC RExt scheme that limits the prefix length (and introducing a corresponding maximum suffix length) in the exponential-Golomb part of the escape coding scheme [3][4].

..

# Proposal

## Fixed transition point prefix code length

Prefix code length of 6 is proposed to define the transition point between Golomb-Rice and exponential Golomb codes for representing escape codes for rice parameter (rParam) range of 0-3. Corresponding threshold value of coefficients for the transition is

threshold   = 5 << m_goRicePar.

## Limiting worst-case escape code length to 32 bits

HEVC RExt scheme for extended precision processing with corresponding transition point in section 2.1 is used for coding of escape codes. The maximum prefix code length is given by

maximumPrefixLength = 32 – (5 + MAX_TR_DYNAMIC_RANGE)

When this prefix length is reached, the corresponding suffix length is then given by:

suffixLength = MAX_TR_DYNAMIC_RANGE – rParam

This scheme limits the worst-case escape code length to 32 bits for coding of abs_remainder and dec_abs_level syntax elements.

# Results

The proposed scheme is implemented on top of VTM-3.0 software [2].  The simulations were carried out under the JVET common test conditions [5] [7].

The test results for JVET common test conditions is tabulated in Table 1.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.05% | 0.07% | 101% | 107% |
| Class A2 | 0.00% | 0.03% | -0.02% | 102% | 108% |
| Class B | 0.00% | -0.02% | 0.02% | 84% | 91% |
| Class C | 0.00% | -0.03% | -0.08% | 84% | 94% |
| Class E | 0.00% | -0.03% | 0.16% | 84% | 94% |
| Overall | 0.00% | -0.02% | 0.02% | 90% | 97% |
| Class D | 0.00% | 0.07% | 0.00% | 85% | 98% |
| Class F | 0.00% | -0.16% | 0.09% | 83% | 96% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.03% | 0.14% | 103% | 109% |
| Class A2 | -0.03% | 0.15% | 0.16% | 105% | 109% |
| Class B | 0.01% | 0.05% | -0.12% | 84% | 96% |
| Class C | 0.02% | 0.12% | 0.14% | 85% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.09% | 0.06% | 92% | 102% |
| Class D | 0.00% | -0.18% | -0.09% | 83% | 95% |
| Class F | -0.07% | 0.02% | -0.01% | 84% | 101% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.28% | -0.11% | 80% | 84% |
| Class C | 0.01% | 0.05% | 0.03% | 82% | 98% |
| Class E | 0.05% | -0.01% | 0.22% | 85% | 97% |
| Overall | 0.02% | -0.10% | 0.02% | 82% | 92% |
| Class D | 0.07% | -0.24% | 0.44% | 81% | 90% |
| Class F | 0.09% | -0.17% | -0.33% | 84% | 96% |

Table 1: Average Bjøntegaard delta rate savings of the proposed method relative to VTM-3.0.

# Conclusion

In this document a fixed prefix length transition point between Golomb Rice and exponential Golomb for coding of escape codes and limiting the worst-case code length for coding of escape codes to 32 bits using the HEVC-RExt extended precision processing scheme is proposed.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 3)”, JVET-L1001-v7, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao, CN, October 2018.

- M. Budagavi, V. Sze, “coeff_abs_level_remaining maximum codeword length reduction”, JCTVC-J0142, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Stockholm, SE, 11–20 July 2012.

- K. Sharman, N. Saunders, J. Gamei, “AHG18: Worst-case Escape Code Length Mitigation”, JCTVC-Q0073, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Valencia, ES, 27 March – 4 April 2014.

- M. Karczewicz, R. Joshi “AHG18: Limiting the worst-case length for coeff_abs_level_remaining syntax element to 32 bits”, JCTVC-Q0132, Joint Collaborative Team on Video Coding (JCT-VC) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Valencia, ES, 27 March – 4 April 2014.

- H. Schwarz, M. Coban, C. Auyeung, “Description of Core Experiment 7 (CE 7): Quantization and coefficient coding,” JVET-L1027, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao, CN, October 2018.

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM.git

- “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao, CN, October 2018.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).