| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0494_v3 |
| --- | --- |

| Title: | CE3-related: Modifications on MPM list generation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Liang Zhao, Xin Zhao, Xiang Li, and Shan Liu | Email: | {leolzhao, xinzzhao, xlxiangli, shanl}@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In this contribution, modifications on MPM list generation are proposed. Firstly, for non-zero reference lines, three branches for MPM derivation are removed from working draft when the left and above neighboring modes are different angular modes. Secondly, for zero reference line, if one neighboring mode is Planar or DC mode, and the other neighboring mode is angular mode, it is proposed to firstly insert Planar mode into the MPM list. Thirdly, on top of the second test, it is proposed that, Planar and DC modes are always put as the first two MPM candidates, and zero line and non-zero lines share the same MPM list generation process which is used for zero-line in VTM-3.0. Planar and DC modes in zero reference line MPM list are excluded for the non-zero reference lines MPM list, and all other angular modes in zero reference line MPM list are used for MPM list of non-zero reference lines with the same order. It is reportedly that, the proposed method achieved -0.05% and -0.01% luma BD-rates with around 100% encoding and decoding time for All Intra (AI) and Random Access (RA) configurations, respectively.

Introduction

In VTM3.0 [1], two neighbors are used to derive the MPM list, and the location of the neighbors is shown in Figure 1, wherein left and above neighboring modes are denoted as leftIntraDir and aboveIntraDir respectively.

A

L

Figure 1: Location of neighbors for MPM list generation

Because Planar and DC modes are not allowed for non-zero reference lines, MPM list generation process is different between the zero reference line and non-zero reference lines. For non-zero reference lines, if the left and above neighboring modes are both angular modes but they are not equal, the MPM list generation part is categorized into 4 cases, which is shown below:

maxAB = max (leftIntraDir, aboveIntraDir)

minAB = min (leftIntraDir, aboveIntraDir)

candModeList[ 0 ] = leftIntraDir

candModeList[ 1 ] = aboveIntraDir

  - If maxAB − minAB is equal to 1, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB + 61 ) % 64 )

candModeList[ 3 ] = 2 + ( ( maxAB − 1 ) % 64 )

candModeList[ 4 ] = 2 + ( ( minAB + 60 ) % 64 )

candModeList[ 5 ] = 2 + ( maxAB % 64 )

  - Otherwise if maxAB − minAB is equal to 2, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB − 1 ) % 64 )

candModeList[ 3 ] = 2 + ( ( minAB + 61 ) % 64 )

candModeList[ 4 ] = 2 + ( ( maxAB − 1 ) % 64 )

candModeList[ 5 ] = 2 + ( ( minAB + 60 ) % 64 )

  - Otherwise if maxAB − minAB is greater than 61, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB − 1 ) % 64 )

candModeList[ 3 ] = 2 + ( ( maxAB + 61 ) % 64 )

candModeList[ 4 ] = 2 + ( minAB % 64 )

candModeList[ 5 ] = 2 + ( ( maxAB + 60 ) % 64 )

  - Otherwise, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB + 61 ) % 64 )

candModeList[ 3 ] = 2 + ( ( minAB − 1 ) % 64 )

candModeList[ 4 ] = 2 + ( ( maxAB + 61 ) % 64 )

candModeList[ 5 ] = 2 + ( ( maxAB − 1 ) % 64 )

For zero reference line, if one neighboring mode is Planar or DC mode, and the other neighboring mode is angular mode, left neighboring mode is always inserted into MPM list before above neighboring mode, and MPM list generation process is shown below:

candModeList[ 0 ] = leftIntraDir

candModeList[ 1 ] = aboveIntraDir

candModeList[ 2 ] = 1 − minAB

candModeList[ 3 ] = 2 + ( ( maxAB + 61 ) % 64 )

candModeList[ 4 ] = 2 + ( ( maxAB − 1 ) % 64 )

candModeList[ 5 ] = 2 + ( ( maxAB + 60 ) % 64 )

Proposed method

Simplification for non-zero reference lines MPM list generation

It is proposed that, for non-zero reference lines, if the left and above neighboring modes are different angular modes, MPM list is derived using only one branch, described as follows (namely Method #1):

candModeList[ 0 ] = leftIntraDir

candModeList[ 1 ] = aboveIntraDir

candModeList[ 2 ] = 2 + ( ( leftIntraDir + 61 ) % 64 )

candModeList[ 3 ] = 2 + ( ( leftIntraDir − 1 ) % 64 )

candModeList[ 4 ] = 2 + ( ( aboveIntraDir + 61 ) % 64 )

candModeList[ 5 ] = 2 + ( (aboveIntraDir − 1) % 64 )

As the 4-case branches are avoided, the text is simplified a lot.

Modification for zero reference line MPM list generation

For zero reference line, if one neighboring mode is Planar or DC mode, and the other neighboring mode is angular mode, it is proposed that, Planar mode is firstly inserted into the MPM list, and MPM list is derived as follows (namely Method #2):

candModeList[ 0 ] = PLANAR_IDX

candModeList[ 1 ] = maxAB

candModeList[ 2 ] = DC_IDX

candModeList[ 3 ] = 2 + ( ( maxAB + 61 ) % 64 )

candModeList[ 4 ] = 2 + ( ( maxAB − 1 ) % 64 )

candModeList[ 5 ] = 2 + ( ( maxAB + 60 ) % 64 )

Alternatively, it is also proposed that, if the angular neighboring mode is vertical or horizontal mode, angular neighboring mode is firstly inserted into the MPM list. Otherwise, Planar mode is firstly inserted into the MPM list. Therefore, MPM list is derived as follows (namely Method #3):

candModeList[ 0 ] = (maxAB == VER_IDX || maxAB == HOR_IDX) ? maxAB: PLANAR_IDX

candModeList[ 1 ] = (maxAB == VER_IDX || maxAB == HOR_IDX) ? PLANAR_IDX: maxAB

candModeList[ 2 ] = DC_IDX

candModeList[ 3 ] = 2 + ((maxAB + offset) % mod)

candModeList[ 4 ] = 2 + (( maxAB − 1 ) % mod )

candModeList[ 5 ] = 2 + ((maxAB + offset - 1) % mod)

Unification of MPM list generation

It is proposed that, zero reference line and non-zero lines share the same MPM list generation process, which is the zero-line MPM list generation process in VTM3.0. The size of MPM list of non-zero reference lines is reduced from 6 to 4, Planar and DC modes in zero reference line MPM list are excluded for the non-zero reference line MPM list, and all other angular modes in zero reference line MPM list are used for MPM list of non-zero reference lines with the same order. (namely Method #4)

Alternatively, for the zero-reference line, on top of the method in Section 2.2. which prioritize Planar and DC modes, it is further proposed that Planar and DC modes are always placed as the first two MPM candidates, and the 4 remaining angular MPM candidates are derived in the same way of VTM3.0 for zero reference line. For non-zero reference lines, only the 4 angular MPMs can be applied such that only one MPM list is generated for zero and nonzero reference lines, and only 4 angular MPMs need to be derived instead of 6 in VTM-3.0. (namely Method #5)

Experimental results

The proposed method is implemented on top of VTM3.0 software [1]. The simulations are performed on Linux cluster using AI and RA configurations under the common test condition described in JVET-L1010 [2]. Seven tests are conducted to verify the performance of the proposed method, which is listed below:

- Test 1: Simplification for non-zero lines MPM list generation (Method #1)

- Test 2: Modification for zero reference line MPM list generation (Method #2)

- Test 3: Alternative modification for zero reference line MPM list generation (Method #3)

- Test 4: Unification of MPM list generation (Method #4)

- Test 5: Unification of MPM list generation (Method #5)

- Test 6: Test 1 + Test 2

- Test 7: Test 1 + Test 3

Test results are shown in Table 1 ~ Table 7.

Table 1: Coding performance of Test 1 over VTM-3.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.03% | 0.04% | 98% | 94% | 0.01% | 0.06% | 0.05% | 100% | 99% |
| Class A2 | 0.01% | 0.09% | -0.03% | 98% | 96% | -0.03% | -0.03% | 0.12% | 99% | 98% |
| Class B | 0.01% | -0.02% | 0.00% | 99% | 97% | -0.01% | -0.01% | 0.06% | 100% | 100% |
| Class C | 0.02% | -0.03% | -0.06% | 100% | 101% | 0.00% | -0.07% | 0.14% | 101% | 104% |
| Class E | 0.03% | 0.03% | -0.04% | 100% | 99% |  |  |  |  |  |
| Overall | 0.02% | 0.00% | -0.02% | 99% | 97% | -0.01% | -0.02% | 0.09% | 100% | 100% |
| Class D | 0.01% | 0.04% | 0.03% | 99% | 101% | -0.01% | -0.30% | -0.03% | 101% | 103% |
| Class F | 0.04% | -0.01% | 0.09% | 99% | 101% | 0.00% | -0.05% | 0.10% | 100% | 101% |

Table 2: Coding performance of Test 2 over VTM-3.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.14% | -0.01% | 100% | 96% | -0.04% | 0.11% | 0.03% | 99% | 98% |
| Class A2 | -0.05% | -0.05% | -0.04% | 98% | 96% | -0.04% | 0.07% | 0.09% | 100% | 99% |
| Class B | -0.05% | -0.02% | 0.04% | 100% | 98% | -0.03% | 0.13% | -0.24% | 101% | 100% |
| Class C | -0.02% | -0.02% | -0.03% | 102% | 104% | 0.00% | -0.07% | 0.14% | 102% | 104% |
| Class E | -0.07% | -0.08% | 0.02% | 101% | 100% |  |  |  |  |  |
| Overall | -0.06% | -0.06% | 0.00% | 100% | 99% | -0.03% | 0.06% | -0.02% | 100% | 100% |
| Class D | -0.05% | -0.10% | 0.04% | 101% | 106% | -0.05% | -0.29% | -0.13% | 101% | 103% |
| Class F | 0.15% | 0.07% | 0.21% | 100% | 102% | 0.11% | 0.21% | 0.14% | 101% | 103% |

Table 3: Coding performance of Test 3 over VTM-3.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.17% | -0.06% | 98% | 94% | -0.02% | 0.05% | -0.03% | 99% | 98% |
| Class A2 | -0.04% | -0.01% | -0.03% | 98% | 95% | -0.01% | 0.08% | 0.22% | 99% | 98% |
| Class B | -0.04% | -0.05% | 0.01% | 100% | 97% | -0.02% | 0.01% | -0.02% | 100% | 99% |
| Class C | -0.02% | 0.00% | -0.03% | 100% | 101% | 0.00% | 0.11% | 0.10% | 101% | 104% |
| Class E | -0.07% | -0.12% | -0.09% | 100% | 99% |  |  |  |  |  |
| Overall | -0.05% | -0.06% | -0.03% | 99% | 97% | -0.01% | 0.06% | 0.06% | 100% | 100% |
| Class D | -0.03% | -0.08% | -0.07% | 102% | 105% | -0.04% | -0.21% | -0.10% | 101% | 106% |
| Class F | 0.01% | 0.01% | 0.03% | 99% | 100% | -0.03% | -0.01% | 0.22% | 101% | 102% |

Table 4: Coding performance of Test 4 over VTM-3.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.03% | -0.01% | 98% | 94% | 0.02% | -0.04% | 0.03% | 99% | 99% |
| Class A2 | 0.04% | 0.05% | 0.01% | 98% | 97% | 0.02% | 0.12% | 0.13% | 99% | 99% |
| Class B | 0.04% | 0.03% | 0.04% | 100% | 96% | 0.03% | 0.14% | -0.10% | 101% | 98% |
| Class C | 0.08% | -0.03% | -0.12% | 100% | 101% | 0.03% | 0.04% | 0.00% | 102% | 105% |
| Class E | 0.06% | 0.03% | 0.15% | 101% | 99% |  |  |  |  |  |
| Overall | 0.05% | 0.01% | 0.01% | 99% | 97% | 0.03% | 0.07% | 0.00% | 100% | 100% |
| Class D | 0.03% | -0.06% | -0.02% | 100% | 103% | -0.01% | -0.20% | -0.03% | 102% | 107% |
| Class F | 0.06% | 0.08% | 0.06% | 100% | 101% | 0.02% | -0.05% | 0.03% | 101% | 103% |

Table 5: Coding performance of Test 5 over VTM-3.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.30% | -0.37% | -0.19% | 99% | 99% | -0.09% | -0.05% | -0.02% | 96% | 97% |
| Class A2 | 0.00% | 0.02% | 0.01% | 99% | 98% | 0.01% | 0.21% | 0.16% | 97% | 99% |
| Class B | -0.04% | 0.02% | 0.00% | 101% | 101% | -0.02% | 0.14% | 0.04% | 96% | 100% |
| Class C | 0.08% | 0.10% | 0.09% | 101% | 102% | 0.04% | -0.09% | -0.04% | 97% | 101% |
| Class E | 0.05% | 0.06% | 0.26% | 100% | 101% |  |  |  |  |  |
| Overall | -0.04% | -0.02% | 0.03% | 100% | 100% | -0.01% | 0.05% | 0.03% | 96% | 99% |
| Class D | 0.02% | -0.09% | 0.08% | 101% | 101% | -0.01% | -0.22% | -0.12% | 97% | 100% |
| Class F | 0.39% | 0.31% | 0.48% | 101% | 102% | 0.19% | 0.18% | 0.34% | 96% | 100% |

Table 6: Coding performance of Test 6 over VTM-3.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.09% | 0.00% | 98% | 94% | -0.04% | -0.13% | 0.06% | 97% | 97% |
| Class A2 | -0.05% | -0.02% | -0.06% | 98% | 95% | -0.04% | 0.16% | 0.08% | 98% | 97% |
| Class B | -0.04% | -0.02% | -0.05% | 99% | 97% | -0.02% | 0.06% | 0.07% | 97% | 98% |
| Class C | 0.00% | -0.02% | -0.05% | 99% | 99% | 0.04% | -0.03% | 0.00% | 98% | 99% |
| Class E | -0.07% | -0.08% | 0.07% | 99% | 96% |  |  |  |  |  |
| Overall | -0.05% | -0.04% | -0.02% | 99% | 96% | -0.01% | 0.02% | 0.05% | 97% | 98% |
| Class D | -0.02% | 0.02% | -0.02% | 99% | 99% | -0.07% | -0.09% | -0.04% | 98% | 101% |
| Class F | 0.16% | 0.06% | 0.13% | 99% | 99% | 0.08% | 0.12% | 0.12% | 96% | 97% |

Table 7: Coding performance of Test 7 over VTM-3.0

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.16% | -0.06% | 102% | 96% | -0.02% | 0.06% | 0.11% | 96% | 96% |
| Class A2 | -0.03% | 0.00% | -0.02% | 98% | 95% | -0.02% | -0.01% | 0.20% | 97% | 96% |
| Class B | -0.03% | -0.02% | -0.04% | 99% | 95% | -0.02% | 0.21% | 0.03% | 97% | 97% |
| Class C | -0.01% | -0.01% | -0.06% | 99% | 100% | 0.04% | -0.11% | 0.11% | 97% | 98% |
| Class E | -0.06% | -0.09% | -0.06% | 99% | 96% |  |  |  |  |  |
| Overall | -0.04% | -0.05% | -0.05% | 99% | 96% | -0.01% | 0.05% | 0.10% | 97% | 97% |
| Class D | -0.01% | -0.04% | -0.11% | 99% | 96% | -0.04% | -0.09% | 0.22% | 97% | 96% |
| Class F | 0.02% | 0.02% | 0.03% | 98% | 98% | -0.01% | 0.06% | 0.07% | 96% | 96% |

Spec text changes

The following text describes test 4 of the proposed method. It is based on VVC draft3 v7 [3] section 8.2.2 The added texts are highlighted with in green, and the deleted texts are marked with strikethrough.

8.2.2	Derivation process for luma intra prediction mode

Input to this process are:

–	a luma location ( xCb , yCb ) specifying the top-left sample of the current luma coding block relative to the top left luma sample of the current picture,

–	a variable cbWidth specifying the width of the current coding block in luma samples,

–	a variable cbHeight specifying the height of the current coding block in luma samples.

In this process, the luma intra prediction mode IntraPredModeY[ xCb ][ yCb ] is derived.

……

- The candModeList[ x ] with x = 0..5 is derived as follows:

- If candIntraPredModeB is equal to candIntraPredModeA and candIntraPredModeA is greater than INTRA_DC, candModeList[ x ] with x = 0..5 is derived as follows:

  - If IntraLumaRefLineIdx[ xCb ][ yCb ] is equal to 0, the following applies:

candModeList[ 0 ] = candIntraPredModeA	(8-4)

candModeList[ 1 ] = INTRA_PLANAR	(8-5)

candModeList[ 2 ] = INTRA_DC		(8-6)

candModeList[ 3 ] = 2 + ( ( candIntraPredModeA + 61 ) % 64 )	(8-7)

candModeList[ 4 ] = 2 + ( ( candIntraPredModeA − 1 ) % 64 )	(8-8)

candModeList[ 5 ] = 2 + ( ( candIntraPredModeA + 60 ) % 64 )	(8-9)

  - Otherwise (IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0), the following applies:

candModeList[ 0 ] = candIntraPredModeA	(8-10)

candModeList[ 1 ] = 2 + ( ( candIntraPredModeA + 61 ) % 64 )	(8-11)

candModeList[ 2 ] = 2 + ( ( candIntraPredModeA − 1 ) % 64 )	(8-12)

candModeList[ 3 ] = 2 + ( ( candIntraPredModeA + 60 ) % 64 )	(8-13)

candModeList[ 4 ] = 2 + ( candIntraPredModeA % 64 )	(8-14)

candModeList[ 5 ] = 2 + ( ( candIntraPredModeA + 59 ) % 64 )	(8-15)

- Otherwise if candIntraPredModeB is not equal to candIntraPredModeA and candIntraPredModeA or candIntraPredModeB is greater than INTRA_DC, the following applies:

  - The variables minAB and maxAB are derived as follows:

minAB = Min( candIntraPredModeA, candIntraPredModeB )	(8-16)

maxAB = Max( candIntraPredModeA, candIntraPredModeB )	(8-17)

  - If candIntraPredModeA and candIntraPredModeB are both greater than INTRA_DC, candModeList[ x ] with x = 0..5 is derived as follows:

candModeList[ 0 ] = candIntraPredModeA	(8-18)

candModeList[ 1 ] = candIntraPredModeB	(8-19)

  - If IntraLumaRefLineIdx[ xCb ][ yCb ] is equal to 0, the following applies:

candModeList[ 2 ] = INTRA_PLANAR	(8-20)

candModeList[ 3 ] = INTRA_DC		(8-21)

  - If maxAB − minAB is in the range of 2 to 62, inclusive, the following applies:

candModeList[ 4 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-22)

candModeList[ 5 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-23)

  - Otherwise, the following applies:

candModeList[ 4 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-24)

candModeList[ 5 ] = 2 + ( ( maxAB ) % 64 )	(8-25)

  - Otherwise (IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0), the following applies:

  - If maxAB − minAB is equal to 1, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB + 61 ) % 64 )	(8-26)

candModeList[ 3 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-27)

candModeList[ 4 ] = 2 + ( ( minAB + 60 ) % 64 )	(8-28)

candModeList[ 5 ] = 2 + ( maxAB % 64 )	(8-29)

  - Otherwise if maxAB − minAB is equal to 2, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB − 1 ) % 64 )	(8-30)

candModeList[ 3 ] = 2 + ( ( minAB + 61 ) % 64 )	(8-31)

candModeList[ 4 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-32)

candModeList[ 5 ] = 2 + ( ( minAB + 60 ) % 64 )	(8-33)

  - Otherwise if maxAB − minAB is greater than 61, the following applies:

candModeList[ 2 ] = 2 + ( ( minAB − 1 ) % 64 )	(8-34)

candModeList[ 3 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-35)

candModeList[ 4 ] = 2 + ( minAB % 64 )	(8-36)

candModeList[ 5 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-37)

  - Otherwise, the following applies:

candModeList[ 2 ] = 2 + ( ( candIntraPredModeA minAB + 61 ) % 64 )	(8-38)

candModeList[ 3 ] = 2 + ( ( candIntraPredModeA minAB − 1 ) % 64 )	(8-39)

candModeList[ 4 ] = 2 + ( ( candIntraPredModeB maxAB + 61 ) % 64 )	(8-40)

candModeList[ 5 ] = 2 + ( ( candIntraPredModeB maxAB − 1 ) % 64 )	(8-41)

  - Otherwise (candIntraPredModeA or candIntraPredModeB is greater than INTRA_DC), candModeList[ x ] with x = 0..5 is derived as follows:

  - If IntraLumaRefLineIdx[ xCb ][ yCb ] is equal to 0, the following applies:

candModeList[ 0 ] = INTRA_PLANARcandIntraPredModeA	(8-42)

candModeList[ 1 ] = maxABcandIntraPredModeB	(8-43)

candModeList[ 2 ] = INTRA_DC1 − minA	(8-44)

candModeList[ 3 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-45)

candModeList[ 4 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-46)

candModeList[ 5 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-47)

  - Otherwise (IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0), the following applies:

candModeList[ 0 ] = maxAB		(8-48)

candModeList[ 1 ] = 2 + ( ( maxAB + 61 ) % 64 )	(8-49)

candModeList[ 2 ] = 2 + ( ( maxAB − 1 ) % 64 )	(8-50)

candModeList[ 3 ] = 2 + ( ( maxAB + 60 ) % 64 )	(8-51)

candModeList[ 4 ] = 2 + ( maxAB % 64 )	(8-52)

candModeList[ 5 ] = 2 + ( ( maxAB + 59 ) % 64 )	(8-53)

- Otherwise, the following applies:

  - If IntraLumaRefLineIdx[ xCb ][ yCb ] is equal to 0, the following applies:

candModeList[ 0 ] = candIntraPredModeA	(8-54)

candModeList[ 1 ] = ( candModeList[0]  = =  INTRA_PLANAR )  ?  INTRA_DC  :  	(8-55) 
		INTRA_PLANAR

candModeList[ 2 ] = INTRA_ANGULAR50	(8-56)

candModeList[ 3 ] = INTRA_ANGULAR18	(8-57)

candModeList[ 4 ] = INTRA_ANGULAR46	(8-58)

candModeList[ 5 ] = INTRA_ANGULAR54	(8-59)

  - Otherwise (IntraLumaRefLineIdx[ xCb ][ yCb ] is not equal to 0), the following applies:

candModeList[ 0 ] = INTRA_ANGULAR50	(8-60)

candModeList[ 1 ] = INTRA_ANGULAR18	(8-61)

candModeList[ 2 ] = INTRA_ANGULAR2	                                        (8-62)

candModeList[ 3 ] = INTRA_ANGULAR34	(8-63)

candModeList[ 4 ] = INTRA_ANGULAR66	(8-64)

candModeList[ 5 ] = INTRA_ANGULAR26	(8-65)

Conclusion

In this contribution, modifications on MPM list generation are proposed, including one simplification for non-zero reference line MPM list generation, one modification for the MPM list generation of zero reference line, and one unification for zero line and non-zero lines MPM list generation. It is reportedly shown that, -0.05% and -0.01% BD-rates with around 100% encoding and decoding time is achieved in AI and RA configuration respectively. Therefore, it is proposed to further study the proposed methods in CE.

References

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin “JVET common test conditions and software reference configurations,” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-L1010, Macao, CN, 3–12 Oct. 2018.

- B. Bross, J. Chen, and S. Liu “Versatile Video Coding (Draft 2),” Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-L1001, Macao, CN, 3–12 Oct. 2018.

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).