| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0496 |
| --- | --- |

| Title: | CE6: Compound Orthonormal Transform (Test 6.1.1 a/b/c/d) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao, Xiang Li, Shan Liu | Tel: Email: | {xinzzhao, xlxiangli, shanl} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution reports the test results of CE6-1.1 a/b/c/d on Compound Orthonormal Transform, which includes two aspects: 1) Use DST-4/DCT-4 to replace DST-7/DCT-8 for 4-point and 8-point transforms used in MTS, and 2) embeds 16-point and 32-point DST-7/DCT-8 into 64-point DCT-2. With the proposed method, it is reported that all the transform types used in VVC can be extracted from one single 64x64 matrix so that 33% transform core storage, i.e., 2.7 KB memory, can be saved when compared to VTM-3.0. It is reportedly shown that, on top of VTM-3.0, by replacing 4-point DST-7/DCT-8 with DST-4/DCT-4, average -0.17% and -0.06% luma BD-Rate is achieved under All Intra (AI) and Random Access (RA) configurations, respectively. By enabling COT, average -0.03% and 0.07% luma BD-Rate is achieved under AI and RA configurations, respectively. There is almost no run-time impact by using the proposed method.

# Introduction

In MTS as described in VVC Draft 3 [1], besides DCT-II, two additional primary transform types, including DST-7 and DCT-8 are used. The transform size of DST7 and DCT-8 is applicable for 4-point, 8-point, 16-point and 32-point. Separate transform cores require additional memory for storing the transform cores, and totally 2.7 KB memory are needed. In JVET-K0291 [2] and JVET-L0287 [3], a method of embedding 16-point and 32-point DST-7 and DCT-8 into 64-point DCT-2 are proposed. In this contribution, the methods proposed in JVET-L0287 are further tested on top of VTM-3.0.

# Proposed method

A 64-point primary transform core, which is called Compound Orthonormal Transform (COT) core, is proposed as shown below (only the left half of the transform matrix is shown, the right half can be derived according to the symmetry/anti- symmetry property of DCT-2 core), where

{aa, ab, ac, ad, ae, af, ag, ah, ai, aj, ak, al, am, an, ao, ap, aq, ar, as, at, au, av, aw, ax, ay, az, ba, bb, bc, bd, be, bf, bg, bh, bi, bj, bk, bl, bm, bn, bo, bp, bq, br, bs, bt, bu, bv, bw, bx, by, bz, ca, cb, cc, cd, ce, cf, cg, ch, ci, cj, ck} =

{ 64, 83, 36, 89, 75, 50, 18, 90, 87, 80, 70, 57, 43, 25, 9, 90, 89, 87, 83, 81, 77, 72, 66, 62, 56, 49, 41, 33, 25, 17,  9, 90, 90, 89, 88, 88, 86, 85, 84, 82, 80, 78, 77, 74, 72, 68, 66, 63, 60, 56, 53, 50, 45, 42, 38, 34, 30, 26, 21, 17, 13,  9,  4}.

With the above proposed 64-point COT core, the 2-point, 4-point, 8-point 16-point DCT-2 and 32-point COT can be extracted from the even rows, and the 4-point, 8-point DST-4/DCT-4, 16-point and 32-point DST-7/DCT-8 can be extracted the odd rows.

Compared to VTM-3.0, when MTS is used (MTS flag is 1), for 4-point and 8-point transforms, DST-4/DCT-4 is used instead of DST-7/DCT-8. When MTS is not used (MTS flag is 0), for 32-point and 64-point transforms, the proposed COT transform is used instead of DCT-2.

# Results

In this section, the results of four CE tests related to this proposed method, including CE6-1.1 a/b/c/d, are reported. Descriptions on the four tests are listed in below table,

Table 1: Descriptions on CE6 Test 6.1.1 a/b/c/d

| Test # | Description |
| --- | --- |
| 6-1.1a | JVET-L0287: Replacing 4-pt DST-7 / DCT-8 by 4-pt DST-4 / DCT-4 used in MTS |
| 6-1.1b | JVET-L0287: Replacing 4-pt and 8-pt DST-7 / DCT-8 by 4-pt and 8-pt DST-4 / DCT-4 used in MTS, respectively. |
| 6-1.1c | JVET-L0287: Compound Orthonormal Transform |
| 6-1.1d | 6-1.1c + 6.2.3a |

For each of the above test, four experiments are conducted [4]. The first test reports the coding performance under CTC condition [5], the second test reports the coding performance under CTC condition with inter MTS enabled, the third test reports the coding performance using Low QP condition, the fourth test reports the coding performance using Low QP condition with inter MTS enabled.

Table 2: Summary on the coding performance of Test 6.1.1a/b/c/d

Table 3: Coding performance of Test 6.1.1a

Table 4: Coding performance of Test 6.1.1b

Table 5: Coding performance of Test 6.1.1c

Table 6: Coding performance of Test 6.1.1d

# Complexity Analysis

## Memory requirements for storing transform cores

Comparisons on the memory requirement for storing primary transform cores are described in Table 6. Detailed calculation on the storage cost of MTS is described in Table 7.

Table 6: Memory requirements for storing memory transform cores

|  | DCT-2 | MTS | Total |
| --- | --- | --- | --- |
| VTM-3.0 | 5460 bytes | 2720 bytes | 8180 bytes |
| CE6-1.1a | 5460 bytes | 2720 bytes | 8180 bytes |
| CE6-1.1b | 5460 bytes | 2720 bytes | 8180 bytes |
| CE6-1.1c | 5460 bytes | 0 bytes | 5460 bytes |
| CE6-1.1d | 5460 bytes | 0 bytes | 5460 bytes |

Table 6: Memory requirements for storing memory transform cores of MTS

| Transform size | Number of elements | Number of transform types | Bits per element | Total |
| --- | --- | --- | --- | --- |
| 4-point | 16 | 2 | 8 bit | 32 bytes |
| 8-point | 64 | 2 | 8 bit | 128 bytes |
| 16-point | 256 | 2 | 8 bit | 512 bytes |
| 32-point | 1024 | 2 | 8 bit | 2048 bytes |
| All |  |  |  | 2720 bytes |

## Other metrics

| Metrics | Reported results |
| --- | --- |
| Operation counts | Unchanged |
| Minimum bit-precision | Unchanged |
| Specify if a transform requires multiple iterations where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output | No |
| Other operations and memory requirements relevant to the proposed tool | No |
| Encoder and decoder times obtained on the same computing platform for both reference and proposed tools should be provided | Reported in Section 3 |
| List of all combinations of transforms block size and transform type used for secondary transformation. | Same with VTM-3.0 |
| Provide analysis of implementation and arithmetic commonalities of proposed transforms | Implementation and arithmetic commonality unchanged |
| If the proposal requires additional computations at the encoder or decoder | No |

# Proposed text changes

The following proposed text modifications are based on JVET-K1001-v5. The added texts are highlighted in green, and deleted texts are marked by red strikethrough. The text changes of CE6-1.1a and CE6-1.1c are provided in section 5.1 and 5.2, respectively.

## Text changes of CE6-1.1a

#### 8.4.4.3	Transformation matrix derivation process

Inputs to this process are:

- a variable nTbS specifying the horizontal sample size of scaled transform coefficients,

- the transformation kernel type trType.

Output of this process is the transformation matrix transMatrix.

The transformation matrix transMatrix is derived based on trType and nTbs as follows:

- If trType is equal to 0, the following applies:

……

- Otherwise, if trType is equal to 1 and nTbs is equal to 4, the following applies:

transMatrix[ m ][ n ] = 	(8-821)

{

{  29   55   74   84 }

{  74   74    0  −74 }

{  84  −29  −74   55 }

{  55  −84   74  −29 }

{  18   50   75   89 }

{  50   89   18  −75 }

{  75   18  −89   50 }

{  89  −75   50  −18 }

},

- Otherwise, if trType is equal to 1 and nTbs is equal to 8, the following applies:

……

- Otherwise, if trType is equal to 2 and nTbs is equal to 4, the following applies:

transMatrix[ m ][ n ] = 	(8-828)

{

{  84   74   55   29 }

{  74    0  −74  −74 }

{  55  −74  −29   84 }

{  29  −74   84  −55 }

{  89   75   50   18 }

{  75  -18  -89  −50 }

{  50  -89   18   75 }

{  18  −50   75  −89 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 8, the following applies:

……

## Text changes of CE6-1.1c

8.4.4.2	Transformation process

Inputs to this process are:

- a variable nTbS specifying the horizontal sample size of transformed samples,

- a variable nonZeroS specifying the horizontal sample size of non-zero scaled transform coefficients,

- a list of scaled transform coefficients x[ j ] with j = 0..nonZeroS − 1,

- a transform kernel type variable trType.

Output of this process is the list of transformed samples y[ i ] with i = 0..nTbS − 1.

The transformation matrix derivation process as specified in clause 8.4.4.3 in invoked with the transform size nTbS and the transform kernel Type trType as inputs, and the transformation matrix transMatrix as output.

Depending on the value of trType, the following applies:, the list of transformed samples y[ i ] with i = 0..nTbS − 1 is derived as follows:

- If trType is equal to 0, the following transform matrix multiplication applies:

y[i]= j=0nonZeroS-1transMatrix[i][j*26-Log2(nTbS) ]*x[j]  with i = 0..nTbS − 1	(8-813)

- Otherwise , if(trType is equal to 1 or trType is equal to 2), the following transform matrix multiplication applies:

y[i]= j=0nonZeroS-1transMatrix[i][j]*x[(2*j+1)*25-Log2nTbS]  with i = 0..nTbS − 1	(8-814)

- Otherwise (trType is equal to 2), the following transform matrix multiplication applies:

y[i]= j=0nTbs-1transMatrix[i+nTbs][(2*j+1)*25-Log2nTbS]*-1j+1*x[j]  with i = 0..nTbS − 1	(8-815)

8.4.4.3	Transformation matrix derivation process

Inputs to this process are:

- a variable nTbS specifying the horizontal sample size of scaled transform coefficients,

- the transformation kernel type trType.

Output of this process is the transformation matrix transMatrix.

The transformation matrix transMatrix is derived based on trType and nTbs as follows:

- If trType is equal to 0, the following applies:

transMatrix[ m ][ n ] = transMatrixCol0to15[ m ][ n ] with m = 0..15, n = 0..63	(8-815)

transMatrixCol0to15 =	(8-816)

{

{  64   64   64   64   64   64   64   64   64   64   64   64   64   64   64   64 }

{  90   90   89   88   88   86   85   84   82   80   78   77   74   72   68   66 }

{  90   89   87   83   81   77   72   66   62   56   49   41   33   25   17    9 }

{  90   88   84   78   72   63   53   42   30   17    4   -9  -21  -34  -45  -56 }

{  91   90   90   90   88   87   86   84   83   81   79   77   73   71   69   65 }

{  90   90   88   85   82   78   73   67   61   54   46   38   31   22   13    4 }

{  90   88   84   79   71   62   52   41   28   15    2  −11  −24  −37  −48  −59 }

{  90   87   80   70   57   43   25    9   −9  −25  −43  −57  −70  −80  −87  −90 }

{  89   84   74   60   42   21    0  -21  -42  -60  -74  -84  -89  -89  -84  -74 }

{  89   81   66   49   25    0  -25  -49  -66  -81  -89  -89  -81  -66  -49  -25 }

{  88   78   60   34    4  -26  -53  -74  -86  -90  -82  -66  -42  -13   17   45 }

{  90   84   73   59   41   20   −2  −24  −44  −62  −77  −86  −90  −90  −83  −71 }

{  90   82   67   46   22   −4  −31  −54  −73  −85  −90  −88  −78  −61  −38  −13 }

{  90   79   59   33    2  −28  −56  −77  −88  −90  −81  −62  −37   −7   24   52 }

{  89   75   50   18  −18  −50  −75  −89  −89  −75  −50  −18   18   50   75   89 }

{  88   72   42    4  -34  -66  -85  -89  -77  -50  -13   26   60   82   90   80 }

{  87   66   33   -9  -49  -77  -90  -81  -56  -17   25   62   83   89   72   41 }

{  86   63   21  -26  -66  -88  -85  -60  -17   30   68   88   84   56   13  -34 }

{  88   71   41    2  −37  −69  −87  −90  −73  −44   −7   33   65   86   90   77 }

{  88   67   31  −13  −54  −82  −90  −78  −46   −4   38   73   90   85   61   22 }

{  87   62   20  −28  −69  −90  −84  −56  −11   37   73   90   81   48    2  −44 }

{  87   57    9  −43  −80  −90  −70  −25   25   70   90   80   43   −9  −57  −87 }

{  85   53    0  -53  -85  -85  -53    0   53   85   85   53    0  -53  -85  -85 }

{  83   49   -9  -62  -89  -77  -33   25   72   90   66   17  -41  -81  -87  -56 }

{  84   42  -21  -74  -89  -60    0   60   89   74   21  -42  -84  -84  -42   21 }

{  86   52   −2  −56  −87  −84  −48    7   59   88   83   44  −11  −62  −90  −81 }

{  85   46  −13  −67  −90  −73  −22   38   82   88   54   −4  −61  −90  −78  −31 }

{  84   41  −24  −77  −90  −56    7   65   91   69   11  −52  −88  −79  −28   37 }

{  83   36  −36  −83  −83  −36   36   83   83   36  −36  −83  −83  −36   36   83 }

{  82   30  -42  -86  -77  -17   53   89   68    4  -63  -90  -60    9   72   88 }

{  81   25  -49  -89  -66    0   66   89   49  -25  -81  -81  -25   49   89   66 }

{  80   17  -60  -90  -50   30   85   74    4  -68  -88  -38   42   88   66   -9 }

{  83   28  −44  −88  −73  −11   59   91   62   −7  −71  −90  −48   24   81   84 }

{  82   22  −54  −90  −61   13   78   85   31  −46  −90  −67    4   73   88   38 }

{  81   15  −62  −90  −44   37   88   69   −7  −77  −84  −24   56   91   52  −28 }

{  80    9  −70  −87  −25   57   90   43  −43  −90  −57   25   87   70   −9  −80 }

{  78    4  -74  -82  -13   68   85   21  -63  -88  -30   56   89   38  -50  -90 }

{  77    0  -77  -77    0   77   77    0  -77  -77    0   77   77    0  -77  -77 }

{  77   -9  -84  -66   26   88   53  -42  -90  -38   56   88   21  -68  -82   -4 }

{  79    2  −77  −81   −7   73   83   11  −71  −84  −15   69   86   20  −65  −87 }

{  78   −4  −82  −73   13   85   67  −22  −88  −61   31   90   54  −38  −90  −46 }

{  77  −11  −86  −62   33   90   44  −52  −90  −24   69   83    2  −81  −71   20 }

{  75  −18  −89  −50   50   89   18  −75  −75   18   89   50  −50  −89  −18   75 }

{  74  -21  -89  -42   60   84    0  -84  -60   42   89   21  -74  -74   21   89 }

{  72  -25  -90  -33   66   77  -17  -89  -41   62   81   -9  -87  -49   56   83 }

{  72  -34  -89  -13   82   56  -53  -84    9   88   38  -68  -74   30   90   17 }

{  73  −24  −90  −37   65   81  −11  −88  −48   56   86    2  −84  −59   44   90 }

{  73  −31  −90  −22   78   67  −38  −90  −13   82   61  −46  −88   −4   85   54 }

{  71  −37  −90   −7   86   48  −62  −79   24   91   20  −81  −59   52   84  −11 }

{  70  −43  −87    9   90   25  −80  −57   57   80  −25  −90   −9   87   43  −70 }

{  68  -45  -84   17   90   13  -85  -42   72   66  -50  -82   21   90    9  -86 }

{  66  -49  -81   25   89    0  -89  -25   81   49  -66  -66   49   81  -25  -89 }

{  66  -56  -74   45   80  -34  -85   21   88   -9  -90   -4   89   17  -86  -30 }

{  69  −48  −83   24   90    2  −90  −28   81   52  −65  −71   44   84  −20  −90 }

{  67  −54  −78   38   85  −22  −90    4   90   13  −88  −31   82   46  −73  −61 }

{  65  −59  −71   52   77  −44  −81   37   84  −28  −87   20   90  −11  −90    2 }

{  64  −64  −64   64   64  −64  −64   64   64  −64  −64   64   64  −64  −64   64 }

{  63  -66  -60   68   56  -72  -53   74   50  -77  -45   78   42  -80  -38   82 }

{  62  -66  -56   72   49  -77  -41   81   33  -83  -25   87   17  -89   -9   90 }

{  60  -74  -42   84   21  -89    0   89  -21  -84   42   74  -60  -60   74   42 }

{  62  −69  −56   73   48  −79  −41   83   33  −86  −24   88   15  −90   −7   91 }

{  61  −73  −46   82   31  −88  −13   90   −4  −90   22   85  −38  −78   54   67 }

{  59  −77  −37   87   11  −91   15   86  −41  −73   62   56  −79  −33   88    7 }

{  57  −80  −25   90   −9  −87   43   70  −70  −43   87    9  −90   25   80  −57 }

{  56  -80  -21   90  -17  -82   53   60  -78  -26   90  -13  -84   50   63  -77 }

{  56  -81  -17   90  -25  -77   62   49  -83   -9   89  -33  -72   66   41  -87 }

{  53  -85    0   85  -53  -53   85    0  -85   53   53  -85    0   85  -53  -53 }

{  56  −83  −15   90  −28  −77   65   44  −87   −2   88  −41  −69   73   33  −90 }

{  54  −85   −4   88  −46  −61   82   13  −90   38   67  −78  −22   90  −31  −73 }

{  52  −87    7   83  −62  −41   90  −20  −77   71   28  −91   33   69  −79  −15 }

{  50  −89   18   75  −75  −18   89  −50  −50   89  −18  −75   75   18  −89   50 }

{  50  -88   21   72  -78   -9   85  -60  -38   90  -34  -63   84   -4  -80   68 }

{  49  -89   25   66  -81    0   81  -66  -25   89  -49  -49   89  -25  -66   81 }

{  45  -90   42   50  -90   38   53  -89   34   56  -88   30   60  -88   26   63 }

{  48  −90   28   65  −84    7   79  −73  −15   87  −59  −37   91  −41  −56   88 }

{  46  −90   38   54  −90   31   61  −88   22   67  −85   13   73  −82    4   78 }

{  44  −91   48   41  −90   52   37  −90   56   33  −90   59   28  −88   62   24 }

{  43  −90   57   25  −87   70    9  −80   80   −9  −70   87  −25  −57   90  −43 }

{  42  -89   60   21  -84   74    0  -74   84  -21  -60   89  -42  -42   89  -60 }

{  41  -89   62   17  -81   77   -9  -66   87  -33  -49   90  -56  -25   83  -72 }

{  38  -86   74   -9  -63   90  -53  -21   80  -82   26   50  -89   66    4  -72 }

{  41  −90   65   11  −79   83  −20  −59   90  −48  −33   87  −71   −2   73  −86 }

{  38  −88   73   −4  −67   90  −46  −31   85  −78   13   61  −90   54   22  −82 }

{  37  −86   79  −20  −52   90  −69    2   65  −90   56   15  −77   87  −41  −33 }

{  36  −83   83  −36  −36   83  −83   36   36  −83   83  −36  −36   83  −83   36 }

{  34  -82   84  -38  -30   80  -85   42   26  -78   86  -45  -21   77  -88   50 }

{  33  -81   83  -41  -25   77  -87   49   17  -72   89  -56   -9   66  -90   62 }

{  30  -77   89  -63    9   50  -85   84  -45  -13   66  -90   74  -26  -34   78 }

{  33  −81   87  −48  −15   71  −90   62   −2  −59   90  −73   20   44  −86   83 }

{  31  −78   90  −61    4   54  −88   82  −38  −22   73  −90   67  −13  −46   85 }

{  28  −73   91  −71   24   33  −77   90  −69   20   37  −79   90  −65   15   41 }

{  25  −70   90  −80   43    9  −57   87  −87   57   −9  −43   80  −90   70  −25 }

{  26  -68   89  -80   45    4  -53   84  -88   63  -17  -34   74  -90   77  -38 }

{  25  -66   89  -81   49    0  -49   81  -89   66  -25  -25   66  -89   81  -49 }

{  21  -60   84  -89   74  -42    0   42  -74   89  -84   60  -21  -21   60  -84 }

{  24  −65   88  −86   59  −15  −33   71  −90   83  −52    7   41  −77   91  −79 }

{  22  −61   85  −90   73  −38   −4   46  −78   90  −82   54  −13  −31   67  −88 }

{  20  −56   81  −91   83  −59   24   15  −52   79  −90   84  −62   28   11  −48 }

{  18  −50   75  −89   89  −75   50  −18  −18   50  −75   89  −89   75  −50   18 }

{  17  -50   74  -88   88  -77   53  -21  -13   45  -72   86  -89   78  -56   26 }

{  17  -49   72  -87   89  -77   56  -25   -9   41  -66   83  -90   81  -62   33 }

{  13  -38   60  -77   86  -90   85  -74   56  -34    9   17  -42   63  -78   88 }

{  15  −44   69  −84   91  −86   71  −48   20   11  −41   65  −83   90  −87   73 }

{  13  −38   61  −78   88  −90   85  −73   54  −31    4   22  −46   67  −82   90 }

{  11  −33   52  −69   81  −88   91  −87   79  −65   48  −28    7   15  −37   56 }

{   9  −25   43  −57   70  −80   87  −90   90  −87   80  −70   57  −43   25   −9 }

{   9  -26   42  -56   68  -78   85  -89   90  -86   80  -72   60  -45   30  -13 }

{   9  -25   41  -56   66  -77   83  -89   90  -87   81  -72   62  -49   33  -17 }

{   4  -13   21  -30   38  -45   53  -60   66  -72   77  -80   84  -86   88  -90 }

{   7  −20   33  −44   56  −65   73  −81   86  −90   91  −90   87  −83   77  −69 }

{   4  −13   22  −31   38  −46   54  −61   67  −73   78  −82   85  −88   90  −90 }

{   2   −7   11  −15   20  −24   28  −33   37  −41   44  −48   52  −56   59  −62 }

},

transMatrix[ m ][ n ] = transMatrixCol16to31[ m − 16 ][ n ] with m = 16..31, n = 0..63	(8-817)

transMatrixCol16to31 =	(8-818)

{

{  64   64   64   64   64   64   64   64   64   64   64   64   64   64   64   64 }

{  63   60   56   53   50   45   42   38   34   30   26   21   17   13    9    4 }

{  -9  -17  -25  -33  -41  -49  -56  -62  -66  -72  -77  -81  -83  -87  -89  -90 }

{ -66  -74  -80  -85  -88  -90  -89  -86  -82  -77  -68  -60  -50  -38  -26  -13 }

{  62   59   56   52   48   44   41   37   33   28   24   20   15   11    7    2 }

{  −4  −13  −22  −31  −38  −46  −54  −61  −67  −73  −78  −82  −85  −88  −90  −90 }

{ −69  −77  −83  −87  −90  −91  −90  −86  −81  −73  −65  −56  −44  −33  −20   −7 }

{ −90  −87  −80  −70  −57  −43  −25   −9    9   25   43   57   70   80   87   90 }

{ -60  -42  -21    0   21   42   60   74   84   89   89   84   74   60   42   21 }

{  25   49   66   81   89   89   81   66   49   25    0  -25  -49  -66  -81  -89 }

{  68   84   90   85   72   50   21   -9  -38  -63  -80  -89  -88  -77  -56  -30 }

{ −56  −37  −15    7   28   48   65   79   87   91   88   81   69   52   33   11 }

{  13   38   61   78   88   90   85   73   54   31    4  −22  −46  −67  −82  −90 }

{  73   87   90   83   65   41   11  −20  −48  −71  −86  −91  −84  −69  −44  −15 }

{  89   75   50   18  −18  −50  −75  −89  −89  −75  −50  −18   18   50   75   89 }

{  56   21  -17  -53  -78  -90  -84  -63  -30    9   45   74   88   86   68   38 }

{ -41  -72  -89  -83  -62  -25   17   56   81   90   77   49    9  -33  -66  -87 }

{ -72  -89  -82  -53   -9   38   74   90   80   50    4  -42  -77  -90  -78  -45 }

{  48   11  −28  −62  −84  −90  −79  −52  −15   24   59   83   91   81   56   20 }

{ −22  −61  −85  −90  −73  −38    4   46   78   90   82   54   13  −31  −67  −88 }

{ −79  −91  −77  −41    7   52   83   90   71   33  −15  −59  −86  −88  −65  −24 }

{ −87  −57   −9   43   80   90   70   25  −25  −70  −90  −80  −43    9   57   87 }

{ -53    0   53   85   85   53    0  -53  -85  -85  -53    0   53   85   85   53 }

{  56   87   81   41  -17  -66  -90  -72  -25   33   77   89   62    9  -49  -83 }

{  74   89   60    0  -60  -89  -74  -21   42   84   84   42  -21  -74  -89  -60 }

{ −41   15   65   90   79   37  −20  −69  −90  −77  −33   24   71   91   73   28 }

{  31   78   90   61    4  −54  −88  −82  −38   22   73   90   67   13  −46  −85 }

{  83   86   44  −20  −73  −90  −59    2   62   90   71   15  −48  −87  −81  −33 }

{  83   36  −36  −83  −83  −36   36   83   83   36  −36  −83  −83  −36   36   83 }

{  50  -21  -78  -85  -38   34   84   80   26  -45  -88  -74  -13   56   90   66 }

{ -66  -89  -49   25   81   81   25  -49  -89  -66    0   66   89   49  -25  -81 }

{ -77  -84  -26   53   90   56  -21  -82  -78  -13   63   89   45  -34  -86  -72 }

{  33  −41  −87  −77  −15   56   90   65   −2  −69  −90  −52   20   79   86   37 }

{ −38  −88  −73   −4   67   90   46  −31  −85  −78  −13   61   90   54  −22  −82 }

{ −86  −73   −2   71   87   33  −48  −90  −59   20   83   79   11  −65  −90  −41 }

{ −80   −9   70   87   25  −57  −90  −43   43   90   57  −25  −87  −70    9   80 }

{ -45   42   90   53  -34  -88  -60   26   86   66  -17  -84  -72    9   80   77 }

{  77   77    0  -77  -77    0   77   77    0  -77  -77    0   77   77    0  -77 }

{  78   74  -13  -85  -63   30   89   50  -45  -90  -34   60   86   17  -72  -80 }

{ −24   62   88   28  −59  −90  −33   56   90   37  −52  −90  −41   48   91   44 }

{  46   90   38  −54  −90  −31   61   88   22  −67  −85  −13   73   82    4  −78 }

{  88   56  −41  −91  −37   59   87   15  −73  −79    7   84   65  −28  −90  −48 }

{  75  −18  −89  −50   50   89   18  −75  −75   18   89   50  −50  −89  −18   75 }

{  42  -60  -84    0   84   60  -42  -89  -21   74   74  -21  -89  -42   60   84 }

{ -83  -56   49   87    9  -81  -62   41   89   17  -77  -66   33   90   25  -72 }

{ -80  -60   50   85   -4  -88  -42   66   77  -26  -90  -21   78   63  -45  -86 }

{  15  −79  −69   33   91   28  −71  −77   20   90   41  −62  −83    7   87   52 }

{ −54  −85    4   88   46  −61  −82   13   90   38  −67  −78   22   90   31  −73 }

{ −90  −33   73   69  −41  −88   −2   87   44  −65  −77   28   90   15  −83  −56 }

{ −70   43   87   −9  −90  −25   80   57  −57  −80   25   90    9  −87  −43   70 }

{ -38   74   63  -53  -80   26   89    4  -88  -34   77   60  -56  -78   30   88 }

{  89   25  -81  -49   66   66  -49  -81   25   89    0  -89  -25   81   49  -66 }

{  82   42  -77  -53   68   63  -60  -72   50   78  -38  -84   26   88  -13  -90 }

{  −7   88   33  −79  −56   62   73  −41  −86   15   91   11  −87  −37   77   59 }

{  61   73  −46  −82   31   88  −13  −90   −4   90   22  −85  −38   78   54  −67 }

{  91    7  −90  −15   88   24  −86  −33   83   41  −79  −48   73   56  −69  −62 }

{  64  −64  −64   64   64  −64  −64   64   64  −64  −64   64   64  −64  −64   64 }

{  34  -84  -30   85   26  -86  -21   88   17  -88  -13   89    9  -90   -4   90 }

{ -90    9   89  -17  -87   25   83  -33  -81   41   77  -49  -72   56   66  -62 }

{ -84  -21   89    0  -89   21   84  -42  -74   60   60  -74  -42   84   21  -89 }

{  −2  −90   11   90  −20  −87   28   84  −37  −81   44   77  −52  −71   59   65 }

{ −67  −54   78   38  −85  −22   90    4  −90   13   88  −31  −82   46   73  −61 }

{ −90   20   84  −44  −71   65   52  −81  −28   90    2  −90   24   83  −48  −69 }

{ −57   80   25  −90    9   87  −43  −70   70   43  −87   −9   90  −25  −80   57 }

{ -30   89   -9  -85   45   66  -74  -34   88   -4  -86   42   68  -72  -38   88 }

{  87  -41  -66   72   33  -89    9   83  -49  -62   77   25  -90   17   81  -56 }

{  85    0  -85   53   53  -85    0   85  -53  -53   85    0  -85   53   53  -85 }

{  11   84  −52  −59   81   20  −91   24   79  −62  −48   86    7  −90   37   71 }

{  73   31  −90   22   78  −67  −38   90  −13  −82   61   46  −88    4   85  −54 }

{  90  −44  −59   84    2  −86   56   48  −88   11   81  −65  −37   90  −24  −73 }

{  50  −89   18   75  −75  −18   89  −50  −50   89  −18  −75   75   18  −89   50 }

{  26  -89   45   53  -88   17   74  -77  -13   86  -56  -42   90  -30  -66   82 }

{ -81   66   25  -89   49   49  -89   25   66  -81    0   81  -66  -25   89  -49 }

{ -86   21   66  -85   17   68  -84   13   72  -82    9   74  -80    4   77  -78 }

{ −20  −71   81    2  −83   69   24  −90   52   44  −90   33   62  −86   11   77 }

{ −78   −4   82  −73  −13   85  −67  −22   88  −61  −31   90  −54  −38   90  −46 }

{ −87   65   20  −86   69   15  −84   71   11  −83   73    7  −81   77    2  −79 }

{ −43   90  −57  −25   87  −70   −9   80  −80    9   70  −87   25   57  −90   43 }

{ -21   84  -74    0   74  -84   21   60  -89   42   42  -89   60   21  -84   74 }

{  72  -83   25   56  -90   49   33  -87   66    9  -77   81  -17  -62   89  -41 }

{  88  -42  -34   85  -77   13   60  -90   56   17  -78   84  -30  -45   88  -68 }

{  28   52  −91   56   24  −84   77   −7  −69   88  −37  −44   90  −62  −15   81 }

{  82  −22  −54   90  −61  −13   78  −85   31   46  −90   67    4  −73   88  −38 }

{  84  −81   24   48  −90   71   −7  −62   91  −59  −11   73  −88   44   28  −83 }

{  36  −83   83  −36  −36   83  −83   36   36  −83   83  −36  −36   83  −83   36 }

{  17  -74   88  -53  -13   72  -89   56    9  -68   90  -60   -4   66  -90   63 }

{ -62   90  -66    9   56  -89   72  -17  -49   87  -77   25   41  -83   81  -33 }

{ -88   60   -4  -53   86  -82   42   17  -68   90  -72   21   38  -80   88  -56 }

{ −37  −28   79  −88   52   11  −69   91  −65    7   56  −90   77  −24  −41   84 }

{ −85   46   13  −67   90  −73   22   38  −82   88  −54   −4   61  −90   78  −31 }

{ −81   90  −62   11   44  −83   88  −59    7   48  −84   87  −56    2   52  −86 }

{ −25   70  −90   80  −43   −9   57  −87   87  −57    9   43  −80   90  −70   25 }

{ -13   60  -86   85  -56    9   42  -78   90  -72   30   21  -66   88  -82   50 }

{  49  -81   89  -66   25   25  -66   89  -81   49    0  -49   81  -89   66  -25 }

{  89  -74   42    0  -42   74  -89   84  -60   21   21  -60   84  -89   74  -42 }

{  44    2  −48   81  −90   73  −37  −11   56  −84   90  −69   28   20  −62   87 }

{  88  −67   31   13  −54   82  −90   78  −46    4   38  −73   90  −85   61  −22 }

{  77  −90   86  −65   33    7  −44   73  −90   87  −69   37    2  −41   71  −88 }

{  18  −50   75  −89   89  −75   50  −18  −18   50  −75   89  −89   75  −50   18 }

{   9  -42   68  -85   90  -80   60  -30   -4   38  -66   84  -90   82  -63   34 }

{ -33   62  -81   90  -83   66  -41    9   25  -56   77  -89   87  -72   49  -17 }

{ -90   84  -72   53  -30    4   21  -45   66  -80   88  -89   82  -68   50  -26 }

{ −52   24    7  −37   62  −81   90  −88   77  −56   28    2  −33   59  −79   90 }

{ −90   82  −67   46  −22   −4   31  −54   73  −85   90  −88   78  −61   38  −13 }

{ −71   83  −90   90  −86   77  −62   44  −24    2   20  −41   59  −73   84  −90 }

{  −9   25  −43   57  −70   80  −87   90  −90   87  −80   70  −57   43  −25    9 }

{  -4   21  -38   53  -66   77  -84   88  -90   88  -82   74  -63   50  -34   17 }

{  17  -33   49  -62   72  -81   87  -90   89  -83   77  -66   56  -41   25   -9 }

{  90  -89   88  -85   82  -78   74  -68   63  -56   50  -42   34  -26   17   -9 }

{  59  −48   37  −24   11    2  −15   28  −41   52  −62   71  −79   84  −88   90 }

{  90  −90   88  −85   82  −78   73  −67   61  −54   46  −38   31  −22   13   −4 }

{  65  −69   71  −73   77  −79   81  −83   84  −86   87  −88   90  −90   90  −91 }

},

transMatrix[ m ][ n ] = ( n & 1 ? −1 : 1 ) * transMatrixCol16to31[47 −m ][ n ]	(8-819)
		 with m =32..47, n = 0..63

transMatrix[ m ][ n ] = ( n & 1 ? −1 : 1 ) * transMatrixCol0to15[63 −m ][ n ]	(8-820)
		with m =48..63, n = 0..63

- Otherwise, if trType is equal to 1 and nTbs is equal to 4, the following applies:

transMatrix[ m ][ n ] = 	(8-821)

{

{  29   55   74   84 }

{  74   74    0  −74 }

{  84  −29  −74   55 }

{  55  −84   74  −29 }

},

- Otherwise, if trType is equal to 1 and nTbs is equal to 8, the following applies:

transMatrix[ m ][ n ] = 	(8-822)

{

{  17   32   46   60   71   78   85   86 }

{  46   78   86   71   32  −17  −60  −85 }

{  71   85   32  −46  −86  −60   17   78 }

{  85   46  −60  −78   17   86   32  −71 }

{  86  −17  −85   32   78  −46  −71   60 }

{  78  −71  −17   85  −60  −32   86  −46 }

{  60  −86   71  −17  −46   85  −78   32 }

{  32  −60   78  −86   85  −71   46  −17 }

},

- Otherwise, if trType is equal to 1 and nTbs is equal to 16, the following applies:

transMatrix[ m ][ n ] = 	(8-823)

{

{   9   17   25   33   41   49   56   62   66   72   77   81   83   87   89   90 }

{  25   49   66   81   89   89   81   66   49   25    0  −25  −49  −66  −81  −89 }

{  41   72   89   83   62   25  −17  −56  −81  −90  −77  −49   −9   33   66   87 }

{  56   87   81   41  −17  −66  −90  −72  −25   33   77   89   62    9  −49  −83 }

{  66   89   49  −25  −81  −81  −25   49   89   66    0  −66  −89  −49   25   81 }

{  77   77    0  −77  −77    0   77   77    0  −77  −77    0   77   77    0  −77 }

{  83   56  −49  −87   −9   81   62  −41  −89  −17   77   66  −33  −90  −25   72 }

{  89   25  −81  −49   66   66  −49  −81   25   89    0  −89  −25   81   49  −66 }

{  90   −9  −89   17   87  −25  −83   33   81  −41  −77   49   72  −56  −66   62 }

{  87  −41  −66   72   33  −89    9   83  −49  −62   77   25  −90   17   81  −56 }

{  81  −66  −25   89  −49  −49   89  −25  −66   81    0  −81   66   25  −89   49 }

{  72  −83   25   56  −90   49   33  −87   66    9  −77   81  −17  −62   89  −41 }

{  62  −90   66   −9  −56   89  −72   17   49  −87   77  −25  −41   83  −81   33 }

{  49  −81   89  −66   25   25  −66   89  −81   49    0  −49   81  −89   66  −25 }

{  33  −62   81  −90   83  −66   41   −9  −25   56  −77   89  −87   72  −49   17 }

{  17  −33   49  −62   72  −81   87  −90   89  −83   77  −66   56  −41   25   −9 }

},

- Otherwise, if trType is equal to 1 and nTbs is equal to 32, the following applies:

transMatrix[ m ][ n ] = transMatrixCol0to15[ m ][ n ] with m = 0..15, n = 0..31	(8-824)

transMatrixCol0to15 =	(8-825)

{

{   4    9   13   17   21   26   30   34   38   42   45   50   53   56   60   63 }

{  13   26   38   50   60   68   77   82   86   89   90   88   85   80   74   66 }

{  21   42   60   74   84   89   89   84   74   60   42   21    0  −21  −42  −60 }

{  30   56   77   88   89   80   63   38    9  −21  −50  −72  −85  −90  −84  −68 }

{  38   68   86   88   74   45    9  −30  −63  −84  −90  −78  −53  −17   21   56 }

{  45   78   90   77   42   −4  −50  −80  −90  −74  −38    9   53   82   89   72 }

{  53   85   85   53    0  −53  −85  −85  −53    0   53   85   85   53    0  −53 }

{  60   89   74   21  −42  −84  −84  −42   21   74   89   60    0  −60  −89  −74 }

{  66   90   56  −13  −74  −88  −45   26   80   84   34  −38  −85  −78  −21   50 }

{  72   86   34  −45  −89  −63   13   78   82   21  −56  −90  −53   26   84   77 }

{  77   80    9  −72  −84  −17   66   86   26  −60  −88  −34   53   90   42  −45 }

{  80   72  −17  −86  −60   34   90   45  −50  −89  −30   63   85   13  −74  −78 }

{  84   60  −42  −89  −21   74   74  −21  −89  −42   60   84    0  −84  −60   42 }

{  86   45  −63  −78   21   90   26  −77  −66   42   88    4  −85  −50   60   80 }

{  88   30  −78  −56   60   77  −34  −88    4   89   26  −80  −53   63   74  −38 }

{  90   13  −88  −26   84   38  −78  −50   72   60  −63  −68   53   77  −42  −82 }

{  90   −4  −90    9   89  −13  −88   17   88  −21  −86   26   85  −30  −84   34 }

{  89  −21  −84   42   74  −60  −60   74   42  −84  −21   89    0  −89   21   84 }

{  88  −38  −72   68   42  −86   −4   88  −34  −74   66   45  −85   −9   89  −30 }

{  85  −53  −53   85    0  −85   53   53  −85    0   85  −53  −53   85    0  −85 }

{  82  −66  −30   90  −42  −56   86  −13  −77   74   17  −88   53   45  −89   26 }

{  78  −77   −4   80  −74   −9   82  −72  −13   84  −68  −17   85  −66  −21   86 }

{  74  −84   21   60  −89   42   42  −89   60   21  −84   74    0  −74   84  −21 }

{  68  −88   45   30  −84   78  −17  −56   90  −60  −13   77  −85   34   42  −88 }

{  63  −90   66   −4  −60   90  −68    9   56  −89   72  −13  −53   88  −74   17 }

{  56  −88   80  −38  −21   72  −90   68  −17  −42   82  −86   53    4  −60   88 }

{  50  −82   88  −66   21   30  −72   90  −78   42    9  −56   85  −86   60  −13 }

{  42  −74   89  −84   60  −21  −21   60  −84   89  −74   42    0  −42   74  −89 }

{  34  −63   82  −90   84  −66   38   −4  −30   60  −80   90  −85   68  −42    9 }

{  26  −50   68  −82   89  −88   80  −66   45  −21   −4   30  −53   72  −84   90 }

{  17  −34   50  −63   74  −82   88  −90   88  −84   77  −66   53  −38   21   −4 }

{   9  −17   26  −34   42  −50   56  −63   68  −74   78  −82   85  −88   89  −90 }

},

transMatrix[ m ][ n ] = transMatrixCol16to31[ m − 16 ][ n ] with m = 16..31, n = 0..31	(8-826)

transMatrixCol16to31 =	(8-827)

{

{  66   68   72   74   77   78   80   82   84   85   86   88   88   89   90   90 }

{  56   45   34   21    9   −4  −17  −30  −42  −53  −63  −72  −78  −84  −88  −90 }

{ −74  −84  −89  −89  −84  −74  −60  −42  −21    0   21   42   60   74   84   89 }

{ −45  −17   13   42   66   82   90   86   74   53   26   −4  −34  −60  −78  −88 }

{  80   90   82   60   26  −13  −50  −77  −89  −85  −66  −34    4   42   72   88 }

{  34  −13  −56  −84  −88  −68  −30   17   60   85   88   66   26  −21  −63  −86 }

{ −85  −85  −53    0   53   85   85   53    0  −53  −85  −85  −53    0   53   85 }

{ −21   42   84   84   42  −21  −74  −89  −60    0   60   89   74   21  −42  −84 }

{  88   72    9  −60  −90  −63    4   68   89   53  −17  −77  −86  −42   30   82 }

{   9  −66  −88  −42   38   88   68   −4  −74  −85  −30   50   90   60  −17  −80 }

{ −90  −50   38   89   56  −30  −88  −63   21   85   68  −13  −82  −74    4   78 }

{   4   82   68  −21  −88  −56   38   90   42  −53  −88  −26   66   84    9  −77 }

{  89   21  −74  −74   21   89   42  −60  −84    0   84   60  −42  −89  −21   74 }

{ −17  −90  −30   74   68  −38  −88   −9   84   53  −56  −82   13   89   34  −72 }

{ −86    9   90   21  −82  −50   66   72  −42  −85   13   90   17  −84  −45   68 }

{  30   86  −17  −89    4   90    9  −88  −21   85   34  −80  −45   74   56  −66 }

{  82  −38  −80   42   78  −45  −77   50   74  −53  −72   56   68  −60  −66   63 }

{ −42  −74   60   60  −74  −42   84   21  −89    0   89  −21  −84   42   74  −60 }

{ −77   63   50  −84  −13   90  −26  −78   60   53  −82  −17   90  −21  −80   56 }

{  53   53  −85    0   85  −53  −53   85    0  −85   53   53  −85    0   85  −53 }

{  68  −80   −4   84  −63  −34   90  −38  −60   85   −9  −78   72   21  −88   50 }

{ −63  −26   88  −60  −30   88  −56  −34   89  −53  −38   90  −50  −42   90  −45 }

{ −60   89  −42  −42   89  −60  −21   84  −74    0   74  −84   21   60  −89   42 }

{  72   −4  −66   89  −50  −26   82  −80   21   53  −90   63    9  −74   86  −38 }

{  50  −88   77  −21  −45   86  −78   26   42  −85   80  −30  −38   84  −82   34 }

{ −78   34   26  −74   90  −66   13   45  −84   85  −50   −9   63  −89   77  −30 }

{ −38   77  −90   74  −34  −17   63  −88   84  −53    4   45  −80   89  −68   26 }

{  84  −60   21   21  −60   84  −89   74  −42    0   42  −74   89  −84   60  −21 }

{  26  −56   78  −89   86  −72   45  −13  −21   53  −77   88  −88   74  −50   17 }

{ −88   78  −63   42  −17   −9   34  −56   74  −85   90  −86   77  −60   38  −13 }

{ −13   30  −45   60  −72   80  −86   90  −89   85  −78   68  −56   42  −26    9 }

{  90  −88   86  −84   80  −77   72  −66   60  −53   45  −38   30  −21   13   −4 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 4, the following applies:

transMatrix[ m ][ n ] = 	(8-828)

{

{  84   74   55   29 }

{  74    0  −74  −74 }

{  55  −74  −29   84 }

{  29  −74   84  −55 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 8, the following applies:

transMatrix[ m ][ n ] = 	(8-829)

{

{  86   85   78   71   60   46   32   17 }

{  85   60   17  −32  −71  −86  −78  −46 }

{  78   17  −60  −86  −46   32   85   71 }

{  71  −32  −86  −17   78   60  −46  −85 }

{  60  −71  −46   78   32  −85  −17   86 }

{  46  −86   32   60  −85   17   71  −78 }

{  32  −78   85  −46  −17   71  −86   60 }

{  17  −46   71  −85   86  −78   60  −32 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 16, the following applies:

transMatrix[ m ][ n ] = 	(8-830)

{

{  90   89   87   83   81   77   72   66   62   56   49   41   33   25   17    9 }

{  89   81   66   49   25    0  −25  −49  −66  −81  −89  −89  −81  −66  −49  −25 }

{  87   66   33   −9  −49  −77  −90  −81  −56  −17   25   62   83   89   72   41 }

{  83   49   −9  −62  −89  −77  −33   25   72   90   66   17  −41  −81  −87  −56 }

{  81   25  −49  −89  −66    0   66   89   49  −25  −81  −81  −25   49   89   66 }

{  77    0  −77  −77    0   77   77    0  −77  −77    0   77   77    0  −77  −77 }

{  72  −25  −90  −33   66   77  −17  −89  −41   62   81   −9  −87  −49   56   83 }

{  66  −49  −81   25   89    0  −89  −25   81   49  −66  −66   49   81  −25  −89 }

{  62  −66  −56   72   49  −77  −41   81   33  −83  −25   87   17  −89   −9   90 }

{  56  −81  −17   90  −25  −77   62   49  −83   −9   89  −33  −72   66   41  −87 }

{  49  −89   25   66  −81    0   81  −66  −25   89  −49  −49   89  −25  −66   81 }

{  41  −89   62   17  −81   77   −9  −66   87  −33  −49   90  −56  −25   83  −72 }

{  33  −81   83  −41  −25   77  −87   49   17  −72   89  −56   −9   66  −90   62 }

{  25  −66   89  −81   49    0  −49   81  −89   66  −25  −25   66  −89   81  −49 }

{  17  −49   72  −87   89  −77   56  −25   −9   41  −66   83  −90   81  −62   33 }

{   9  −25   41  −56   66  −77   83  −89   90  −87   81  −72   62  −49   33  −17 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 32, the following applies:

transMatrix[ m ][ n ] = transMatrixCol0to15[ m ][ n ] with m = 0..15, n = 0..31	(8-831)

transMatrixCol0to15 =	(8-832)

{

{  90   90   89   88   88   86   85   84   82   80   78   77   74   72   68   66 }

{  90   88   84   78   72   63   53   42   30   17    4   −9  −21  −34  −45  −56 }

{  89   84   74   60   42   21    0  −21  −42  −60  −74  −84  −89  −89  −84  −74 }

{  88   78   60   34    4  −26  −53  −74  −86  −90  −82  −66  −42  −13   17   45 }

{  88   72   42    4  −34  −66  −85  −89  −77  −50  −13   26   60   82   90   80 }

{  86   63   21  −26  −66  −88  −85  −60  −17   30   68   88   84   56   13  −34 }

{  85   53    0  −53  −85  −85  −53    0   53   85   85   53    0  −53  −85  −85 }

{  84   42  −21  −74  −89  −60    0   60   89   74   21  −42  −84  −84  −42   21 }

{  82   30  −42  −86  −77  −17   53   89   68    4  −63  −90  −60    9   72   88 }

{  80   17  −60  −90  −50   30   85   74    4  −68  −88  −38   42   88   66   −9 }

{  78    4  −74  −82  −13   68   85   21  −63  −88  −30   56   89   38  −50  −90 }

{  77   −9  −84  −66   26   88   53  −42  −90  −38   56   88   21  −68  −82   −4 }

{  74  −21  −89  −42   60   84    0  −84  −60   42   89   21  −74  −74   21   89 }

{  72  −34  −89  −13   82   56  −53  −84    9   88   38  −68  −74   30   90   17 }

{  68  −45  −84   17   90   13  −85  −42   72   66  −50  −82   21   90    9  −86 }

{  66  −56  −74   45   80  −34  −85   21   88   −9  −90   −4   89   17  −86  −30 }

{  63  −66  −60   68   56  −72  −53   74   50  −77  −45   78   42  −80  −38   82 }

{  60  −74  −42   84   21  −89    0   89  −21  −84   42   74  −60  −60   74   42 }

{  56  −80  −21   90  −17  −82   53   60  −78  −26   90  −13  −84   50   63  −77 }

{  53  −85    0   85  −53  −53   85    0  −85   53   53  −85    0   85  −53  −53 }

{  50  −88   21   72  −78   −9   85  −60  −38   90  −34  −63   84   −4  −80   68 }

{  45  −90   42   50  −90   38   53  −89   34   56  −88   30   60  −88   26   63 }

{  42  −89   60   21  −84   74    0  −74   84  −21  −60   89  −42  −42   89  −60 }

{  38  −86   74   −9  −63   90  −53  −21   80  −82   26   50  −89   66    4  −72 }

{  34  −82   84  −38  −30   80  −85   42   26  −78   86  −45  −21   77  −88   50 }

{  30  −77   89  −63    9   50  −85   84  −45  −13   66  −90   74  −26  −34   78 }

{  26  −68   89  −80   45    4  −53   84  −88   63  −17  −34   74  −90   77  −38 }

{  21  −60   84  −89   74  −42    0   42  −74   89  −84   60  −21  −21   60  −84 }

{  17  −50   74  −88   88  −77   53  −21  −13   45  −72   86  −89   78  −56   26 }

{  13  −38   60  −77   86  −90   85  −74   56  −34    9   17  −42   63  −78   88 }

{   9  −26   42  −56   68  −78   85  −89   90  −86   80  −72   60  −45   30  −13 }

{   4  −13   21  −30   38  −45   53  −60   66  −72   77  −80   84  −86   88  −90 }

},

transMatrix[ m ][ n ] = transMatrixCol16to31[ m − 16 ][ n ] with m = 16..31, n = 0..31	(8-833)

transMatrixCol16to31 =	(8-834)

{

{  63   60   56   53   50   45   42   38   34   30   26   21   17   13    9    4 }

{ −66  −74  −80  −85  −88  −90  −89  −86  −82  −77  −68  −60  −50  −38  −26  −13 }

{ −60  −42  −21    0   21   42   60   74   84   89   89   84   74   60   42   21 }

{  68   84   90   85   72   50   21   −9  −38  −63  −80  −89  −88  −77  −56  −30 }

{  56   21  −17  −53  −78  −90  −84  −63  −30    9   45   74   88   86   68   38 }

{ −72  −89  −82  −53   −9   38   74   90   80   50    4  −42  −77  −90  −78  −45 }

{ −53    0   53   85   85   53    0  −53  −85  −85  −53    0   53   85   85   53 }

{  74   89   60    0  −60  −89  −74  −21   42   84   84   42  −21  −74  −89  −60 }

{  50  −21  −78  −85  −38   34   84   80   26  −45  −88  −74  −13   56   90   66 }

{ −77  −84  −26   53   90   56  −21  −82  −78  −13   63   89   45  −34  −86  −72 }

{ −45   42   90   53  −34  −88  −60   26   86   66  −17  −84  −72    9   80   77 }

{  78   74  −13  −85  −63   30   89   50  −45  −90  −34   60   86   17  −72  −80 }

{  42  −60  −84    0   84   60  −42  −89  −21   74   74  −21  −89  −42   60   84 }

{ −80  −60   50   85   −4  −88  −42   66   77  −26  −90  −21   78   63  −45  −86 }

{ −38   74   63  −53  −80   26   89    4  −88  −34   77   60  −56  −78   30   88 }

{  82   42  −77  −53   68   63  −60  −72   50   78  −38  −84   26   88  −13  −90 }

{  34  −84  −30   85   26  −86  −21   88   17  −88  −13   89    9  −90   −4   90 }

{ −84  −21   89    0  −89   21   84  −42  −74   60   60  −74  −42   84   21  −89 }

{ −30   89   −9  −85   45   66  −74  −34   88   −4  −86   42   68  −72  −38   88 }

{  85    0  −85   53   53  −85    0   85  −53  −53   85    0  −85   53   53  −85 }

{  26  −89   45   53  −88   17   74  −77  −13   86  −56  −42   90  −30  −66   82 }

{ −86   21   66  −85   17   68  −84   13   72  −82    9   74  −80    4   77  −78 }

{ −21   84  −74    0   74  −84   21   60  −89   42   42  −89   60   21  −84   74 }

{  88  −42  −34   85  −77   13   60  −90   56   17  −78   84  −30  −45   88  −68 }

{  17  −74   88  −53  −13   72  −89   56    9  −68   90  −60   −4   66  −90   63 }

{ −88   60   −4  −53   86  −82   42   17  −68   90  −72   21   38  −80   88  −56 }

{ −13   60  −86   85  −56    9   42  −78   90  −72   30   21  −66   88  −82   50 }

{  89  −74   42    0  −42   74  −89   84  −60   21   21  −60   84  −89   74  −42 }

{   9  −42   68  −85   90  −80   60  −30   −4   38  −66   84  −90   82  −63   34 }

{ −90   84  −72   53  −30    4   21  −45   66  −80   88  −89   82  −68   50  −26 }

{  −4   21  −38   53  −66   77  −84   88  −90   88  −82   74  −63   50  −34   17 }

{  90  −89   88  −85   82  −78   74  −68   63  −56   50  −42   34  −26   17   −9 }

},

# Conclusions

This proposal provides a method which embeds DST-7 and DCT-8 bases into DCT-2. With the proposed fast method, the additional memory requirement of MTS for storing primary transform cores becomes 0. The experimental results show that, when MTS is enabled for inter prediction residuals, average -0.03%, 0.03% and -0.03% luma BD-Rate is achieved under All Intra (AI), Random Access (RA) and Low Delay B (LDB) configurations, respectively.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 3),” JVET-L1001, 12th meeting, Macao, CN, 3–12 Oct. 2018.

- X. Zhao, X. Li, S. Liu, “CE6-related: Compound orthonormal transform,” JVET-K0292, 11th meeting, Ljubljana, SI, 10–18 July 2018.

- X. Zhao, X. Li, S. Liu, “CE6: Compound Orthonormal Transform (Test 6.1.7 f/g),” JVET-L0287, 12th meeting, Macao, CN, 3–12 Oct. 2018.

- A. Said, X. Zhao, “Description of Core Experiment 6 (CE6): Transforms and transform signalling,” JVET-L1026, 12th meeting, Macao, CN, 3–12 Oct. 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, 12th meeting, Macao, CN, 3–12 Oct. 2018.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).