| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0497 |
| --- | --- |

| Title: | CE6: Fast DST-7/DCT-8 with dual implementation support (Test 6.2.3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xin Zhao, Xiang Li, Yi Luo, Shan Liu | Tel: Email: | {xinzzhao, xlxiangli, juvenalluo, shanl} @tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution reports the test results of CE6-2.3 on a fast DST-7/DCT-8 method with dual implementation support, i.e., identical results between full matrix multiplication and fast method. Compared to full matrix multiplication, the proposed method reportedly reduces the multiplication operation counts by 50% and 39% for 16- and 32-point DST-7/DCT-8, respectively. It is reportedly shown that, on top of VTM-3.0, average 0.01%, -0.01% and 0.00% luma BD-Rate is achieved under All Intra (AI), Random Access (RA) and Low Delay B (LDB) configurations, respectively, and an average of 4% and 9% overall run-time can be saved for encoding and decoding under AI test condition, respectively.

# Introduction

In VVC Draft 3 [1], DST-7 and DCT-8 are used in Multiple Transform Selection (MTS). However, for 8-point, 16-point and 32-point DST-7/DCT-8, the transforms are implemented as full matrix multiplication, which is costly in terms of arithmetic operation counts. In JVET-K0291 [2] and JVET-L0286 [3], fast methods for 16-point and 32-point DST-7/DCT-8 is proposed, which reduced the arithmetic operations with almost no impact on coding performance. The proposed methods support dual implementation, i.e., DST-7/DCT-8 can be implemented by either fast method or matrix multiplication, and both ways lead to identical results. In this contribution, the methods proposed in JVET-L0286 are further tested on top of VTM-3.0.

# Proposed method

In an N-point DST-7/DCT-8 transform core, there are only N distinct numbers without considering sign changes, or additional zeros. Three features on the transform cores have been exploited to implement the fast method.

Feature #1: In some basis vectors which contain N distinct numbers without considering the sign changes, it is observed that the sum of several (2 or 3) numbers equals to the sum of another several (1 or 2) numbers.

Feature #2: Replicate patterns with symmetric or anti-symmetric characteristics can be observed in some basis vectors.

Feature #3: There is another one or two basis vector(s) which only contain(s) very few (1 or 2) distinct number(s) without considering the sign changes.

To explain the fast methods utilizing these features, an example from 16-point DST-7 transform using each of the three features is discussed. Denote the input vector for a 16-point transform as x={x0, x1, x2, …, x15}, and the output transform coefficient vector as y={y0, y1, y2, …, y15}. The transform core in the forward transform is shown as below:

{ a  b  c  d  e  f  g  h  i  j  k  l  m  n  o  p }

{ c  f  i  l  o  o  l  i  f  c  0 -c -f -i -l -o }

{ e  j  o  m  h  c -b -g -l -p -k -f -a  d  i  n }

{ g  n  l  e -b -i -p -j -c  d  k  o  h  a -f -m }

{ i  o  f -c -l -l -c  f  o  i  0 -i -o -f  c  l }

{ k  k  0 -k -k  0  k  k  0 -k -k  0  k  k  0 -k }

{ m  g -f -n -a  l  h -e -o -b  k  i -d -p -c  j }

{ o  c -l -f  i  i -f -l  c  o  0 -o -c  l  f -i }

{ p -a -o  b  n -c -m  d  l -e -k  f  j -g -i  h }

{ n -e -i  j  d -o  a  m -f -h  k  c -p  b  l -g }

{ l -i -c  o -f -f  o -c -i  l  0 -l  i  c -o  f }

{ j -m  c  g -p  f  d -n  i  a -k  l -b -h  o -e }

{ h -p  i -a -g  o -j  b  f -n  k -c -e  m -l  d }

{ f -l  o -i  c  c -i  o -l  f  0 -f  l -o  i -c }

{ d -h  l -p  m -i  e -a -c  g -k  o -n  j -f  b }

{ b -d  f -h  j -l  n -p  o -m  k -i  g -e  c -a }

where {a,b,c,d,e,f,g,h,i,j,k,l,m,n,o,p} are the N unique numbers which are specified by the formulation of DST-7.

Example of Feature #1:

It is noted that the element values have the following characteristics.

a + j = l

b + i = m

c + h = n

d + g = o

e + f = p

Therefore, to calculate y0, instead of doing the following vector-by-vector multiplication:

y0 = a·x0 + b·x1 + … + p·x15

which requires 16 multiplications. The following alternative implementation can be done to derive the same results:

y0 = a·(x0+x11) + b·(x1+x12) + … + j·(x9+x11) + k·x10

which requires 10 multiplications. It is observed that the number of multiplications is reduced.

In addition, while calculating y2, y3, y6, y8, y9, y11, y12, y14, y15, similar implementation can be done, and the intermediate results (x0+x11), (x1+x12), (x2+x13), (x3+x14), (x4+x15), (x5+x15), (x6+x14), (x7+x13), (x8+x12), (x9+x11) and k·x10 can be re-used. It is observed that the number of additions is reduced.

Example of Feature #2:

The second basis vector is

which can be divided into three segments, and they are replicate with sign changes, or flipped version of each other. Based on Feature #2, when calculating y1, instead of doing the following vector-by-vector multiplication

y1 = c·x0 + f·x1 + … - o·x15

which requires 16 multiplications. The following alternative implementation can be used to derive the same results.

y1 = c·(x0+x9-x11) + f·(x1+x8-x12) + i·(x2+x7-x13) + l·(x3+x6-x14) + o·(x4+x5-x15)

which only requires 5 multiplications. It is reported the number of multiplications is reduced.

In addition, when calculating y1, y4, y7, y10, y13, the implementation can be done in similar way, therefore, the intermediate results of (x0+x9-x11), (x1+x8-x12, (x2+x7-x13), (x3+x6-x14) and (x4+x5-x15) can be re-used. It is observed the number of additions is reduced.

Example of Feature #3:

It is observed that the 6th basis vector only contains 1 distinct number without considering the sign changes as shown below.

{k, k, 0, -k, -k, 0, k, k, 0, -k, -k, 0, k, k, 0, -k}

Instead of taking the vector-by-vector multiplication of

y5 = k·x0 + k·x1 + … - k·x15

which requires 11 multiplications, y5 can also be achieved by the following operations:

y5 = k·(x0 + x1 - … - x15)

which only requires 1 multiplication.

The abovementioned three features apply to forward 16- and 32-point DST-7/DCT-8 transforms. Similar fast methods also apply to backward 16- and 32-point DST-7/DCT-8 transforms.

# Results

Four experiments [4] are conducted to evaluate the proposed methods, the first test reports the coding performance under CTC condition [5], the second test reports the coding performance when inter MTS is enabled, the third test reports the coding performance under Low QP condition, and the fourth test reports the coding performance under Low QP condition when inter MTS is enabled. The anchor of first and third tests are VTM-3.0, and the anchor of second and fourth tests are VTM-3.0 with inter MTS enabled,

Table 1: Descriptions on CE6 Test 6.2.3

| Test # | Description |
| --- | --- |
| 6-2.3a | JVET-L0286: Fast DST-7/DCT-8 with dual implementation support |

Detailed results for each test are reported in Table 2.

Table 2: Coding performance of Test 6.2.3

# Complexity Analysis

## Arithmetic operations

The numbers of arithmetic operations required for a 1D N-point transform of matrix multiplication and the proposed fast method are tabulated in Table I. Theoretically, 50.4%, 39.5% and 46.1% total number of multiplications have been saved for 16-point, 32-point and 64-point transform respectively, and 35.4%, 27.6% and 42.2% add/subs have been saved for 16-point, 32-point and 64-point transform respectively.

Table 4: The number of arithmetic operations for a 1D transform

|  | Matrix multiplication |  |  | Proposed fast DST-7/DCT-8 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Transform size | Mult | Add/Sub | Shift | Mult | Add/Sub | Shift |
| 16 | 256 | 240 | 16 | 127 | 155 | 16 |
| 32 | 1024 | 992 | 32 | 620 | 718 | 32 |
| 64 | 4096 | 4032 | 64 | 2207 | 2331 | 64 |

## Software execution speed

In this test, a standalone test program is used to measure the run-time (in seconds) of individual transform functions by repeating a large amount of times. The test program is written in C, and auto-vectorization is disabled (#pragma loop(no_vector)) for compiling to provide fair comparisons.

More detailed descriptions on the test environment are available below:

- CPU: i7-6600U CPU @ 2.6 GHz

- OS: Windows 10 Pro, 64-bit

- Memory (RAM): 16 GB

- Compiler: Visual Studio 2017

Table 5: Comparisons on software execution speed (seconds)

|  |  | Matrix multiplication |  | Proposed fast method |  |
| --- | --- | --- | --- | --- | --- |
| Transform size | No. of iterations | Forward | Inverse | Forward | Inverse |
| 16-point DCT-8 | 2^23 | 11.3 | 15.2 | 4.8 | 4.9 |
| 32-point DCT-8 | 2^20 | 11.1 | 14.8 | 5.7 | 5.5 |
| 64-point DCT-8 | 2^17 | 11.0 | 14.2 | 5.2 | 5.1 |

## Other metrics

| Metrics | Reported results |
| --- | --- |
| Additional memory requirements for storing transform cores | No |
| Minimum bit-precision | Unchanged |
| Specify if a transform requires multiple iterations where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output | No |
| Other operations and memory requirements relevant to the proposed tool | No |
| Encoder and decoder times obtained on the same computing platform for both reference and proposed tools should be provided | Reported in Section 3 |
| List of all combinations of transforms block size and transform type used for secondary transformation. | Same with VTM-3.0 |
| Provide analysis of implementation and arithmetic commonalities of proposed transforms | Proposed method supports both partial butterfly and matrix multiplications |
| If the proposal requires additional computations at the encoder or decoder | No |

# Proposed text changes

The following proposed text modifications are based on JVET-K1001-v5. The added texts are highlighted in green, and deleted texts are marked by red strikethrough.

#### 8.4.4.3	Transformation matrix derivation process

Inputs to this process are:

- a variable nTbS specifying the horizontal sample size of scaled transform coefficients,

- the transformation kernel type trType.

Output of this process is the transformation matrix transMatrix.

The transformation matrix transMatrix is derived based on trType and nTbs as follows:

- If trType is equal to 0, the following applies:

……

- Otherwise, if trType is equal to 1 and nTbs is equal to 4, the following applies:

transMatrix[ m ][ n ] = 	(8-821)

{

{  29   55   74   84 }

{  74   74    0  −74 }

{  84  −29  −74   55 }

{  55  −84   74  −29 }

},

- Otherwise, if trType is equal to 1 and nTbs is equal to 8, the following applies:

transMatrix[ m ][ n ] = 	(8-822)

{

{  17   32   46   60   71   78   85   86 }

{  46   78   86   71   32  −17  −60  −85 }

{  71   85   32  −46  −86  −60   17   78 }

{  85   46  −60  −78   17   86   32  −71 }

{  86  −17  −85   32   78  −46  −71   60 }

{  78  −71  −17   85  −60  −32   86  −46 }

{  60  −86   71  −17  −46   85  −78   32 }

{  32  −60   78  −86   85  −71   46  −17 }

},

- Otherwise, if trType is equal to 1 and nTbs is equal to 16, the following applies:

transMatrix[ m ][ n ] = 	(8-823)

{

{   98   17   25   33   4140   4948   5655   62   6668   7273   77   81   8385   87   8988   9088 }

{  25   4948   6668   81   8988   8988   81   6668   4948   25    0  −25  −4948  −6668  −81  −8988 }

{  4140   7273   8988   8385   62   25  −17  −5655  −81  −9088  −77  −4948   −98   33   6668   87 }

{  5655   87   81   4140  −17  −6668  −9088  −7273  −25   33   77   8988   62    98  −4948  −8385 }

{  6668   8988   4948  −25  −81  −81  −25   4948   8988   6668    0  −6668  −8988  −4948   25   81 }

{  77   77    0  −77  −77    0   77   77    0  −77  −77    0   77   77    0  −77 }

{  83   5655  −4948  −87   −98   81   62  −4140  −8988  −17   77   6668  −33  −9088  −25   7273 }

{  89   25  −81  −4948   6668   6668  −4948  −81   25   8988    0  −8988  −25   81   4948  −6668 }

{  9088   −98  −8988   17   87  −25  −8385   33   81  −4140  −77   4948   7273  −5655  −6668   62 }

{  87  −4140  −6668   7273   33  −8988    98   8385  −4948  −62   77   25  −9088   17   81  −56 }

{  81  −6668  −25   8988  −4948  −4948   8988  −25  −6668   81    0  −81   6668   25  −8988   4948 }

{  7273  −8385   25   5655  −9088   4948   33  −87   6668    98  −77   81  −17  −62   8988  −4140 }

{  62  −9088   6668   −98  −5655   8988  −7273   17   4948  −87   77  −25  −4140   8385  −81   33 }

{  4948  −81   8988  −6668   25   25  −6668   8988  −81   4948    0  −4948   81  −8988   6668  −25 }

{  33  −62   81  −9088   8385  −6668   4140   −98  −25   5655  −77   8988  −87   7273  −4948   17 }

{  17  −33   4948  −62   7273  −81   87  −9088   8988  −8385   77  −6668   5655  −4140   25   −98 }

},

- Otherwise, if trType is equal to 1 and nTbs is equal to 32, the following applies:

transMatrix[ m ][ n ] = transMatrixCol0to15[ m ][ n ] with m = 0..15, n = 0..31	(8-824)

transMatrixCol0to15 =	(8-825)

{

{   4    9   13   17   21   26   30   34   38   42   4546   50   53   56   60   63 }

{  13   26   38   50   60   68   77   82   86   89   90   88   85   80   74   66 }

{  21   42   60   74   84   89   89   84   74   60   42   21    0  −21  −42  −60 }

{  30   56   77   8887   89   80   63   38    9  −21  −50  −72  −85  −90  −84  −68 }

{  38   68   86   88   74   4546    9  −30  −63  −84  −90  −78  −53  −17   21   56 }

{  4546   78   90   77   42   −4  −50  −80  −90  −74  −38    9   53   82   89   72 }

{  53   85   85   53    0  −53  −85  −85  −53    0   53   85   85   53    0  −53 }

{  60   89   74   21  −42  −84  −84  −42   21   74   89   60    0  −60  −89  −74 }

{  66   90   56  −13  −74  −8887  −4546   26   80   84   34  −38  −85  −78  −21   50 }

{  72   86   34  −4546  −89  −63   13   78   82   21  −56  −90  −53   26   84   77 }

{  77   80    9  −72  −84  −17   66   86   26  −60  −88  −34   53   90   42  −4546 }

{  80   72  −17  −86  −60   34   90   4546  −50  −89  −30   63   85   13  −74  −78 }

{  84   60  −42  −89  −21   74   74  −21  −89  −42   60   84    0  −84  −60   42 }

{  86   4546  −63  −78   21   90   26  −77  −66   42   8887    4  −85  −50   60   80 }

{  88   30  −78  −56   60   77  −34  −8887    4   89   26  −80  −53   63   74  −38 }

{  90   13  −8887  −26   84   38  −78  −50   72   60  −63  −68   53   77  −42  −82 }

{  90   −4  −90    9   89  −13  −88   17   8887  −21  −86   26   85  −30  −84   34 }

{  89  −21  −84   42   74  −60  −60   74   42  −84  −21   89    0  −89   21   84 }

{  8887  −38  −72   68   42  −86   −4   88  −34  −74   66   4546  −85   −9   89  −30 }

{  85  −53  −53   85    0  −85   53   53  −85    0   85  −53  −53   85    0  −85 }

{  82  −66  −30   90  −42  −56   86  −13  −77   74   17  −8887   53   4546  −89   26 }

{  78  −77   −4   80  −74   −9   82  −72  −13   84  −68  −17   85  −66  −21   86 }

{  74  −84   21   60  −89   42   42  −89   60   21  −84   74    0  −74   84  −21 }

{  68  −88   4546   30  −84   78  −17  −56   90  −60  −13   77  −85   34   42  −8887 }

{  63  −90   66   −4  −60   90  −68    9   56  −89   72  −13  −53   88  −74   17 }

{  56  −8887   80  −38  −21   72  −90   68  −17  −42   82  −86   53    4  −60   88 }

{  50  −82   88  −66   21   30  −72   90  −78   42    9  −56   85  −86   60  −13 }

{  42  −74   89  −84   60  −21  −21   60  −84   89  −74   42    0  −42   74  −89 }

{  34  −63   82  −90   84  −66   38   −4  −30   60  −80   90  −85   68  −42    9 }

{  26  −50   68  −82   89  −88   80  −66   4546  −21   −4   30  −53   72  −84   90 }

{  17  −34   50  −63   74  −82   8887  −90   88  −84   77  −66   53  −38   21   −4 }

{   9  −17   26  −34   42  −50   56  −63   68  −74   78  −82   85  −8887   89  −90 }

},

transMatrix[ m ][ n ] = transMatrixCol16to31[ m − 16 ][ n ] with m = 16..31, n = 0..31	(8-826)

transMatrixCol16to31 =	(8-827)

{

{  66   68   72   74   77   78   80   82   84   85   86   8887   88   89   90   90 }

{  56   4546   34   21    9   −4  −17  −30  −42  −53  −63  −72  −78  −84  −8887  −90 }

{ −74  −84  −89  −89  −84  −74  −60  −42  −21    0   21   42   60   74   84   89 }

{ −4546  −17   13   42   66   82   90   86   74   53   26   −4  −34  −60  −78  −88 }

{  80   90   82   60   26  −13  −50  −77  −89  −85  −66  −34    4   42   72   8887 }

{  34  −13  −56  −84  −88  −68  −30   17   60   85   8887   66   26  −21  −63  −86 }

{ −85  −85  −53    0   53   85   85   53    0  −53  −85  −85  −53    0   53   85 }

{ −21   42   84   84   42  −21  −74  −89  −60    0   60   89   74   21  −42  −84 }

{  88   72    9  −60  −90  −63    4   68   89   53  −17  −77  −86  −42   30   82 }

{   9  −66  −88  −42   38   8887   68   −4  −74  −85  −30   50   90   60  −17  −80 }

{ −90  −50   38   89   56  −30  −8887  −63   21   85   68  −13  −82  −74    4   78 }

{   4   82   68  −21  −8887  −56   38   90   42  −53  −88  −26   66   84    9  −77 }

{  89   21  −74  −74   21   89   42  −60  −84    0   84   60  −42  −89  −21   74 }

{ −17  −90  −30   74   68  −38  −88   −9   84   53  −56  −82   13   89   34  −72 }

{ −86    9   90   21  −82  −50   66   72  −42  −85   13   90   17  −84  −4546   68 }

{  30   86  −17  −89    4   90    9  −88  −21   85   34  −80  −4546   74   56  −66 }

{  82  −38  −80   42   78  −4546  −77   50   74  −53  −72   56   68  −60  −66   63 }

{ −42  −74   60   60  −74  −42   84   21  −89    0   89  −21  −84   42   74  −60 }

{ −77   63   50  −84  −13   90  −26  −78   60   53  −82  −17   90  −21  −80   56 }

{  53   53  −85    0   85  −53  −53   85    0  −85   53   53  −85    0   85  −53 }

{  68  −80   −4   84  −63  −34   90  −38  −60   85   −9  −78   72   21  −88   50 }

{ −63  −26   8887  −60  −30   88  −56  −34   89  −53  −38   90  −50  −42   90  −4546 }

{ −60   89  −42  −42   89  −60  −21   84  −74    0   74  −84   21   60  −89   42 }

{  72   −4  −66   89  −50  −26   82  −80   21   53  −90   63    9  −74   86  −38 }

{  50  −8887   77  −21  −4546   86  −78   26   42  −85   80  −30  −38   84  −82   34 }

{ −78   34   26  −74   90  −66   13   4546  −84   85  −50   −9   63  −89   77  −30 }

{ −38   77  −90   74  −34  −17   63  −8887   84  −53    4   4546  −80   89  −68   26 }

{  84  −60   21   21  −60   84  −89   74  −42    0   42  −74   89  −84   60  −21 }

{  26  −56   78  −89   86  −72   4546  −13  −21   53  −77   88  −8887   74  −50   17 }

{ −8887   78  −63   42  −17   −9   34  −56   74  −85   90  −86   77  −60   38  −13 }

{ −13   30  −4546   60  −72   80  −86   90  −89   85  −78   68  −56   42  −26    9 }

{  90  −88   86  −84   80  −77   72  −66   60  −53   4546  −38   30  −21   13   −4 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 4, the following applies:

transMatrix[ m ][ n ] = 	(8-828)

{

{  84   74   55   29 }

{  74    0  −74  −74 }

{  55  −74  −29   84 }

{  29  −74   84  −55 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 8, the following applies:

transMatrix[ m ][ n ] = 	(8-829)

{

{  86   85   78   71   60   46   32   17 }

{  85   60   17  −32  −71  −86  −78  −46 }

{  78   17  −60  −86  −46   32   85   71 }

{  71  −32  −86  −17   78   60  −46  −85 }

{  60  −71  −46   78   32  −85  −17   86 }

{  46  −86   32   60  −85   17   71  −78 }

{  32  −78   85  −46  −17   71  −86   60 }

{  17  −46   71  −85   86  −78   60  −32 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 16, the following applies:

transMatrix[ m ][ n ] = 	(8-830)

{

{  9088   8988   87   8385   81   77   7273   6668   62   5655   4948   4140   33   25   17    98 }

{  8988   81   6668   4948   25    0  −25  −4948  −6668  −81  −8988  −8988  −81  −6668  −4948  −25 }

{  87   6668   33   −98  −4948  −77  −9088  −81  −5655  −17   25   62   8385   8988   7273   4140 }

{  8385   4948   −98  −62  −8988  −77  −33   25   7273   9088   6668   17  −4140  −81  −87  −5655 }

{  81   25  −4948  −8988  −6668    0   6668   8988   4948  −25  −81  −81  −25   4948   8988   6668 }

{  77    0  −77  −77    0   77   77    0  −77  −77    0   77   77    0  −77  −77 }

{  7273  −25  −9088  −33   6668   77  −17  −8988  −4140   62   81   −98  −87  −4948   5655   8385 }

{  6668  −4948  −81   25   8988    0  −8988  −25   81   4948  −6668  −6668   4948   81  −25  −8988 }

{  62  −6668  −5655   7273   4948  −77  −4140   81   33  −8385  −25   87   17  −8988   −98   9088 }

{  5655  −81  −17   9088  −25  −77   62   4948  −8385   −98   8988  −33  −72   6668   4140  −87 }

{  4948  −8988   25   6668  −81    0   81  −6668  −25   8988  −4948  −4948   8988  −25  −6668   81 }

{  4140  −8988   62   17  −81   77   −98  −6668   87  −33  −4948   9088  −5655  −25   8385  −7273 }

{  33  −81   8385  −4140  −25   77  −87   4948   17  −7273   8988  −5655   −98   6668  −9088   62 }

{  25  −6668   8988  −81   4948    0  −4948   81  −8988   6668  −25  −25   6668  −8988   81  −4948 }

{  17  −4948   7273  −87   8988 −77   5655  −25   −98   4140  −6668   8385  −9088   81  −62   33 }

{   98  −25   4140  −5655   6668  −77   8385  −8988   9088  −87   81  −7273   62  −4948   33  −17 }

},

- Otherwise, if trType is equal to 2 and nTbs is equal to 32, the following applies:

transMatrix[ m ][ n ] = transMatrixCol0to15[ m ][ n ] with m = 0..15, n = 0..31	(8-831)

transMatrixCol0to15 =	(8-832)

{

{  90   90   89   88   8887   86   85   84   82   80   78   77   74   72   68   66 }

{  90   8887   84   78   72   63   53   42   30   17    4   −9  −21  −34  −4546  −56 }

{  89   84   74   60   42   21    0  −21  −42  −60  −74  −84  −89  −89  −84  −74 }

{  88   78   60   34    4  −26  −53  −74  −86  −90  −82  −66  −42  −13   17   4546 }

{  8887   72   42    4  −34  −66  −85  −89  −77  −50  −13   26   60   82   90   80 }

{  86   63   21  −26  −66  −8887  −85  −60  −17   30   68   88   84   56   13  −34 }

{  85   53    0  −53  −85  −85  −53    0   53   85   85   53    0  −53  −85  −85 }

{  84   42  −21  −74  −89  −60    0   60   89   74   21  −42  −84  −84  −42   21 }

{  82   30  −42  −86  −77  −17   53   89   68    4  −63  −90  −60    9   72   88 }

{  80   17  −60  −90  −50   30   85   74    4  −68  −8887  −38   42   88   66   −9 }

{  78    4  −74  −82  −13   68   85   21  −63  −8887  −30   56   89   38  −50  −90 }

{  77   −9  −84  −66   26   88   53  −42  −90  −38   56   8887   21  −68  −82   −4 }

{  74  −21  −89  −42   60   84    0  −84  −60   42   89   21  −74  −74   21   89 }

{  72  −34  −89  −13   82   56  −53  −84    9   88   38  −68  −74   30   90   17 }

{  68  −4546  −84   17   90   13  −85  −42   72   66  −50  −82   21   90    9  −86 }

{  66  −56  −74   4546   80  −34  −85   21   88   −9  −90   −4   89   17  −86  −30 }

{  63  −66  −60   68   56  −72  −53   74   50  −77  −4546   78   42  −80  −38   82 }

{  60  −74  −42   84   21  −89    0   89  −21  −84   42   74  −60  −60   74   42 }

{  56  −80  −21   90  −17  −82   53   60  −78  −26   90  −13  −84   50   63  −77 }

{  53  −85    0   85  −53  −53   85    0  −85   53   53  −85    0   85  −53  −53 }

{  50  −88   21   72  −78   −9   85  −60  −38   90  −34  −63   84   −4  −80   68 }

{  4546  −90   42   50  −90   38   53  −89   34   56  −88   30   60  −8887   26   63 }

{  42  −89   60   21  −84   74    0  −74   84  −21  −60   89  −42  −42   89  −60 }

{  38  −86   74   −9  −63   90  −53  −21   80  −82   26   50  −89   66    4  −72 }

{  34  −82   84  −38  −30   80  −85   42   26  −78   86  −4546  −21   77  −88   50 }

{  30  −77   89  −63    9   50  −85   84  −4546  −13   66  −90   74  −26  −34   78 }

{  26  −68   89  −80   4546    4  −53   84  −8887   63  −17  −34   74  −90   77  −38 }

{  21  −60   84  −89   74  −42    0   42  −74   89  −84   60  −21  −21   60  −84 }

{  17  −50   74  −8887   88  −77   53  −21  −13   4546  −72   86  −89   78  −56   26 }

{  13  −38   60  −77   86  −90   85  −74   56  −34    9   17  −42   63  −78   8887 }

{   9  −26   42  −56   68  −78   85  −89   90  −86   80  −72   60  −4546   30  −13 }

{   4  −13   21  −30   38  −4546   53  −60   66  −72   77  −80   84  −86   88  −90 }

},

transMatrix[ m ][ n ] = transMatrixCol16to31[ m − 16 ][ n ] with m = 16..31, n = 0..31	(8-833)

transMatrixCol16to31 =	(8-834)

{

{  63   60   56   53   50   4546   42   38   34   30   26   21   17   13    9    4 }

{ −66  −74  −80  −85  −88  −90  −89  −86  −82  −77  −68  −60  −50  −38  −26  −13 }

{ −60  −42  −21    0   21   42   60   74   84   89   89   84   74   60   42   21 }

{  68   84   90   85   72   50   21   −9  −38  −63  −80  −89  −8887  −77  −56  −30 }

{  56   21  −17  −53  −78  −90  −84  −63  −30    9   4546   74   88   86   68   38 }

{ −72  −89  −82  −53   −9   38   74   90   80   50    4  −42  −77  −90  −78  −4546 }

{ −53    0   53   85   85   53    0  −53  −85  −85  −53    0   53   85   85   53 }

{  74   89   60    0  −60  −89  −74  −21   42   84   84   42  −21  −74  −89  −60 }

{  50  −21  −78  −85  −38   34   84   80   26  −4546  −8887  −74  −13   56   90   66 }

{ −77  −84  −26   53   90   56  −21  −82  −78  −13   63   89   4546  −34  −86  −72 }

{ −4546   42   90   53  −34  −88  −60   26   86   66  −17  −84  −72    9   80   77 }

{  78   74  −13  −85  −63   30   89   50  −4546  −90  −34   60   86   17  −72  −80 }

{  42  −60  −84    0   84   60  −42  −89  −21   74   74  −21  −89  −42   60   84 }

{ −80  −60   50   85   −4  −8887  −42   66   77  −26  −90  −21   78   63  −4546  −86 }

{ −38   74   63  −53  −80   26   89    4  −8887  −34   77   60  −56  −78   30   88 }

{  82   42  −77  −53   68   63  −60  −72   50   78  −38  −84   26   8887  −13  −90 }

{  34  −84  −30   85   26  −86  −21   88   17  −8887  −13   89    9  −90   −4   90 }

{ −84  −21   89    0  −89   21   84  −42  −74   60   60  −74  −42   84   21  −89 }

{ −30   89   −9  −85   4546   66  −74  −34   88   −4  −86   42   68  −72  −38   8887 }

{  85    0  −85   53   53  −85    0   85  −53  −53   85    0  −85   53   53  −85 }

{  26  −89   4546   53  −8887   17   74  −77  −13   86  −56  −42   90  −30  −66   82 }

{ −86   21   66  −85   17   68  −84   13   72  −82    9   74  −80    4   77  −78 }

{ −21   84  −74    0   74  −84   21   60  −89   42   42  −89   60   21  −84   74 }

{  8887  −42  −34   85  −77   13   60  −90   56   17  −78   84  −30  −4546   88  −68 }

{  17  −74   88  −53  −13   72  −89   56    9  −68   90  −60   −4   66  −90   63 }

{ −88   60   −4  −53   86  −82   42   17  −68   90  −72   21   38  −80   8887  −56 }

{ −13   60  −86   85  −56    9   42  −78   90  −72   30   21  −66   88  −82   50 }

{  89  −74   42    0  −42   74  −89   84  −60   21   21  −60   84  −89   74  −42 }

{   9  −42   68  −85   90  −80   60  −30   −4   38  −66   84  −90   82  −63   34 }

{ −90   84  −72   53  −30    4   21  −4546   66  −80   88  −89   82  −68   50  −26 }

{  −4   21  −38   53  −66   77  −84   88  −90   8887  −82   74  −63   50  −34   17 }

{  90  −89   8887  −85   82  −78   74  −68   63  −56   50  −42   34  −26   17   −9 }

},

# Conclusions

This proposal provides a fast DST-7/DCT-8 method which supports dual implementations (identical output between matrix multiplication and fast method). With the proposed fast method, it is reported that the number of multiplication operations is reduced by 50% and 40% for 16-point and 32-point DST-7/DCT-8, respectively. The experimental results show reduced encoding and decoding time, with almost no change on BD-Rate. It is recommended to adopt this proposed scheme into VTM.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 3),” JVET-L1001, 12th meeting, Macao, CN, 3–12 Oct. 2018.

- Z. Zhang, X. Zhao, X. Li, S. Liu, “CE6-related: Fast DST-7/DCT-8 with dual implementation support,” JVET-K0291, 11th meeting, Ljubljana, SI, 10–18 July 2018.

- X. Zhao, X. Li, Y. Luo, S. Liu, “CE6: Fast DST-7/DCT-8 with dual implementation support (Test 6.1.4),” JVET-L0286, 12th meeting, Macao, CN, 3–12 Oct. 2018.

- A. Said, X. Zhao, “Description of Core Experiment 6 (CE6): Transforms and transform signalling,” JVET-L1026, 12th meeting, Macao, CN, 3–12 Oct. 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, 12th meeting, Macao, CN, 3–12 Oct. 2018.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).