| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0500 |
| --- | --- |

| Title: | CE10-realated: Unidirectional illumination compensation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin, Wei-Jung Chien, Ted Hsieh, Nan Hu, Marta Karczewicz | Tel: Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents test results for local illumination compensation (LIC) tool, which was implemented in the BMS software and is also studied in CE10. In the proposal, LIC is applied to AMVP, merge and MMVD tools, and it is disabled for other inter tools and bi-directional prediction. Test results show luma BD rate reduction of -0.51% with 116% encoder and 97% decoder runtime for RA configuration.

# Introduction

Local illumination compensation (LIC) was implemented in BMS software and detailed description of the method can be found in JVET-G1001 section 2.3.5. In this method, the linear model parameters are estimated based on the reconstructed neighbor samples of the current and reference CUs.

# Proposed method

In the proposed local illumination compensation, the linear model parameters derivation is kept unchanged and the LIC is applied on a CU basis. The proposed LIC is not applied to sub-block based inter prediction, such as ATMVP or affine, triangular partition, multi hypothesis intra inter and bi-directional prediction.

It is not applied to bi-directional prediction, since the reconstructed neighboring samples of the current block are not required to perform inter prediction in the inter pipeline and thus are not available for each uni-directional inter prediction, which otherwise would be required for LIC since the weighted average for bi-prediction is applied after deriving uni-directional predictors. Also, having LIC applied to bi-directional prediction introduces an additional stage due to performing LIC process before the weighting.

Figure 1. LIC with bi-prediction

For the same reasoning, LIC is not applied for the multi hypothesis intra inter because LIC is applied after inter prediction and weighting between intra and inter would be delayed by the LIC process.

Figure 2. LIC with multi hypothesis intra inter

LIC flag is included as a part of motion information in addition to MVs and reference indices. However, when merge candidate list is constructed, LIC flag is inherited from the neighbor blocks for merge candidates, but LIC is not used for motion vector pruning for simplification purpose.

LIC flag is not stored in the motion vector buffer of the reference picture, so LIC flag is always set equal to false for TMVP. LIC flag is also set equal to false for bi-directional merge candidates, such as par-wise average candidate, and zero motion candidates.

When LIC tool is not applied, LIC flag is not signaled.

# Test results

The proposed method was implemented on top of VTM-3.0 software and tested under common test conditions described in JVET-L1010.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.82% | -0.41% | -0.59% | 120% | 98% |
| Class A2 | -0.37% | -0.24% | -0.30% | 111% | 99% |
| Class B | -0.51% | -0.32% | -0.46% | 114% | 96% |
| Class C | -0.39% | -0.47% | -0.30% | 120% | 96% |
| Class E |  |  |  |  |  |
| Overall | -0.51% | -0.36% | -0.41% | 116% | 97% |
| Class D | -0.24% | -0.24% | -0.35% | 119% | 95% |
| Class F | -0.90% | -0.64% | -0.57% | 112% | 95% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.46% | -0.40% | -0.52% | 123% | 97% |
| Class E | 0.08% | 0.07% | -0.03% | 99% | 98% |
| Overall |  |  |  |  |  |
| Class D | -0.18% | 0.01% | -1.01% | 120% | 97% |
| Class F | -2.53% | -1.79% | -2.13% | 116% | 95% |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).