| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0513-v5 |
| --- | --- |

| Title: | CE7-related: Context modeling of pred_mode_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yin Zhao Seungwook Hong Haitao Yang Jianle Chen | Email: | yin.zhao@huawei.com Seungwook.Hong@huawei.com haitao.yang@huawei.com Jianle.Chen@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

This contribution proposes 5 methods to improve the context modeling of CU-level intra/inter coding mode flag pred_mode_flag by referring the prediction mode of its neighbor CU. The method1 refers prediction mode in above and left CU. Method 2 further takes into account the size of CU when both the neighbor CU and the current CU are not available or in inter mode. Method3 excludes the above CU from the context modeling when it is not in the same CTU as the current CU. It could be also applicable to other syntax elements which include the above CU in the context modeling so that the dependency of the syntax elements on the line buffer could be removed. Method4 uses a context modeling with 4 contexts and finally method 5 adds on the case to consider the above CU. All 5 methods have been tested. The result of method1 shows {-0.09%/-0.02%/-0.05%} BD-rate gain, method2 shows {-0.09%/-0.03%/-0.10%} BD-rate gain,  method3 shows {-0.02%/XXX/-0.01%} BD-rate gain, method4 shows {-0.09%/-0.04%/-0.06%} BD-rate gain, and method 5 shows {-0.03%/YYY/ZZZ%} BD-rate gain, respectively for RA, LDB and LDP configurations over VTM-3.0.

# Introduction

The pred_mode_flag is per CU syntax element specifies whether the current coding unit is coded as intra/inter prediction mode. pred_mode_flag equals to 1 indicates that the current coding unit is coded in intra prediction mode and 0 indicates inter prediction mode or not available[1]. The pred_mode_flag uses a single context using regular coding.

# Proposed Method

Method1 uses 3 contexts bins for coding pred_mode_flag depending on the prediction mode of above and left CU.

ctxId = (a available and intra?1:0)+(b available and intra?1:0)

Method2 further classifies the case by the size of CU when the ctxId is zero.

sizeC = widthCU * heightCU

ctxId = (a available and flag?1:0)+(b available and flag?1:0)

ctxId += (ctxId == 0) ? (sizeC > 256) ? 4 : ((sizeC > 64) ? 3 : 0) : 0

Table 1 shows the summary of each methods 1 and 2

Table 1: Context Modeling Method of pred_mode_flag. (a) method 1 (b) method 2

| Left | Above | CtxIdx |
| --- | --- | --- |
| 0 | 0 | 0 |
| 0 | 1 | 1 |
| 1 | 0 | 1 |
| 1 | 1 | 2 |
|  | (a) |  |

| Left | Above | sizeC | CtxIdx |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 0 | > 64 | 3 |
| 0 | 0 | > 256 | 4 |
| 0 | 1 |  | 1 |
| 1 | 0 |  | 1 |
| 1 | 1 |  | 2 |
| (b) |  |  |  |

Method3 extends method2 to exclude the above CU from the context modeling when it is out of CTU boundary. It could be applied not only to the pred_mode_flag but also to the other syntax elements listed below so that the dependency of these syntax elements on the line buffer could be removed.

- Affine flag

- Skip flag

- IMV(Adaptive MV precision Mode) flag

- Triangle flag

Then the new context modeling for pred_mode_flag including the four other syntax elements from above could be written as

sizeC = widthCU * heightCU

ctxId = (a available and flag?1:0) + (b available and flag ? (b in same CTU ? 1 : 0) : 0)

ctxId += (ctxId == 0) ? (sizeC > 256) ? 4 : ((sizeC > 64) ? 3 : 0) : 0

Method4 uses slightly different context modeling from Method 3 which categorize the size of CU into 2.

sizeC = widthCU * heightCU

ctxId = (a available and flag?1:0)+(b available and flag?1:0)

ctxId = ( ctxId == 0 && cuSizeC >= 256 ) ? 3 : ctxId

Method5 uses same context modeling as Method4 by checking the above CU in the same CTU boundary.

Table 2: Summary of each method

| Method |  | Method1 |  | Method2 |  | Method3 |  | Method4 |  | Method5 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| #Contexts for Context Modeling | 3 |  | 5 |  | 5 |  | 4 |  | 4 |  |
| #of cases when CtxId=0 | 0 |  | 3 |  | 3 |  | 2 |  | 2 |  |
| Check Above CU is in the same CTU boundary | No |  | No |  | Yes |  | No |  | Yes |  |

# Result

All proposed methods were implemented in VTM 3.0 and the results are showing in table 3, 4, 5, 6 and 7.

Table 3: performance of method1 over VTM 3.0

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 95% | 89% |
| Class A2 | 0.00% | 0.00% | 0.00% | 94% | 87% |
| Class B | 0.00% | 0.00% | 0.00% | 94% | 87% |
| Class C | 0.00% | 0.00% | 0.00% | 93% | 86% |
| Class E | 0.00% | 0.00% | 0.00% | 95% | 84% |
| Overall | 0.00% | 0.00% | 0.00% | 94% | 87% |
| Class D | 0.00% | 0.00% | 0.00% | 93% | 85% |
| Class F | #DIV/0! | #DIV/0! | #DIV/0! | #NUM! | #NUM! |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.08% | -0.03% | 100% | 100% |
| Class A2 | -0.09% | 0.08% | 0.08% | 100% | 100% |
| Class B | -0.09% | 0.09% | -0.07% | 100% | 100% |
| Class C | -0.08% | -0.05% | -0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | 0.02% | -0.02% | 100% | 100% |
| Class D | -0.03% | -0.01% | -0.06% | 100% | 100% |
| Class F | #DIV/0! | #DIV/0! | #DIV/0! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.11% | 0.09% | 100% | 100% |
| Class C | -0.01% | 0.29% | 0.14% | 100% | 99% |
| Class E | -0.03% | 0.41% | -0.13% | 100% | 99% |
| Overall | -0.02% | 0.15% | 0.05% | 100% | 100% |
| Class D | 0.00% | 0.19% | 0.16% | 100% | 99% |
| Class F | #DIV/0! | #DIV/0! | #DIV/0! | #NUM! | #NUM! |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | 0.15% | 0.42% | 100% | 99% |
| Class C | -0.08% | 0.08% | 0.08% | 100% | 101% |
| Class E | 0.00% | -0.22% | -0.64% | 100% | 102% |
| Overall | -0.05% | 0.03% | 0.04% | 100% | 100% |
| Class D | 0.01% | -0.27% | 0.30% | 100% | 101% |
| Class F | #DIV/0! | #DIV/0! | #DIV/0! | #NUM! | #NUM! |

Table 4: performance of method2 over VTM 3.0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.06% | 0.01% | 100% | 100% |
| Class A2 | -0.08% | 0.03% | 0.09% | 100% | 100% |
| Class B | -0.10% | 0.16% | 0.03% | 100% | 100% |
| Class C | -0.08% | -0.01% | 0.12% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | 0.04% | 0.06% | 100% | 100% |
| Class D | -0.02% | -0.02% | 0.04% | 100% | 100% |
| Class F | #DIV/0! | #DIV/0! | #DIV/0! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | -0.24% | -0.10% | 100% | 100% |
| Class C | -0.06% | 0.25% | -0.04% | 100% | 98% |
| Class E | 0.06% | 0.05% | -0.29% | 100% | 99% |
| Overall | -0.03% | -0.01% | -0.13% | 100% | 99% |
| Class D | -0.04% | 0.00% | 0.30% | 100% | 100% |
| Class F | #DIV/0! | #DIV/0! | #DIV/0! | #NUM! | #NUM! |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.08% | 0.24% | 0.06% | 100% | 99% |
| Class C | -0.05% | 0.28% | 0.03% | 101% | 101% |
| Class E | -0.20% | -0.56% | -0.95% | 100% | 102% |
| Overall | -0.10% | 0.05% | -0.20% | 100% | 100% |
| Class D | -0.01% | -0.08% | 0.12% | 100% | 101% |
| Class F | #DIV/0! | #DIV/0! | #DIV/0! | #NUM! | #NUM! |

Table 5: performance of method3 over VTM 3.0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.18% | 0.04% | 100% | 99% |
| Class A2 | -0.02% | 0.19% | 0.15% | 99% | 100% |
| Class B | -0.02% | 0.21% | 0.06% | 100% | 99% |
| Class C | -0.04% | 0.15% | 0.12% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.19% | 0.09% | 100% | 99% |
| Class D | -0.06% | -0.11% | -0.01% | 100% | 99% |
| Class F | #DIV/0! | #DIV/0! | #DIV/0! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | 0.01% | 0.16% | 0.20% | 99% | 95% |
| Class E | 0.17% | -0.29% | 0.10% | 99% | 94% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.03% | 0.12% | 0.21% | 100% | 98% |
| Class F | #DIV/0! | #DIV/0! | #DIV/0! | #NUM! | #NUM! |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.25% | 0.08% | 100% | 98% |
| Class C | -0.06% | 0.17% | 0.06% | 100% | 97% |
| Class E | 0.05% | -0.36% | -0.44% | 99% | 95% |
| Overall | -0.01% | 0.07% | -0.05% | 99% | 97% |
| Class D | 0.02% | -0.13% | 0.21% | 100% | 101% |
| Class F | #DIV/0! | #DIV/0! | #DIV/0! | #NUM! | #NUM! |

Table 6: performance of method4 over VTM 3.0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | 0.07% | -0.07% | 100% | 100% |
| Class A2 | -0.08% | 0.05% | 0.08% | 100% | 100% |
| Class B | -0.10% | 0.18% | -0.04% | 99% | 99% |
| Class C | -0.08% | 0.07% | 0.11% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | 0.11% | 0.02% | 100% | 99% |
| Class D | -0.08% | -0.09% | -0.06% | 100% | 99% |
| Class F | #DIV/0! | #DIV/0! | #DIV/0! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | -0.17% | 0.08% | 100% | 101% |
| Class C | -0.02% | 0.21% | 0.25% | 100% | 99% |
| Class E | -0.07% | 0.05% | 0.14% | 100% | 99% |
| Overall | -0.04% | 0.01% | 0.15% | 100% | 100% |
| Class D | -0.02% | 0.13% | 0.36% | 100% | 99% |
| Class F | #DIV/0! | #DIV/0! | #DIV/0! | #NUM! | #NUM! |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.06% | 0.31% | 0.33% | 100% | 100% |
| Class C | -0.05% | 0.32% | 0.06% | 100% | 101% |
| Class E | -0.10% | 0.03% | -0.78% | 100% | 100% |
| Overall | -0.06% | 0.24% | -0.04% | 100% | 100% |
| Class D | -0.04% | 0.04% | 0.16% | 100% | 101% |
| Class F | #DIV/0! | #DIV/0! | #DIV/0! | #NUM! | #NUM! |

Table 7: performance of method5 over VTM 3.0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.15% | 0.07% | 99% | 99% |
| Class A2 | 0.00% | 0.16% | 0.17% | 99% | 99% |
| Class B | -0.04% | 0.18% | 0.09% | 99% | 97% |
| Class C | -0.05% | 0.06% | 0.24% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | 0.14% | 0.15% | 99% | 99% |
| Class D | -0.04% | -0.08% | 0.03% | 100% | 100% |
| Class F | #DIV/0! | #DIV/0! | #DIV/0! | #NUM! | #NUM! |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | 0.01% | 0.25% | -0.07% | 99% | 95% |
| Class E | 0.12% | 0.53% | 0.04% | 99% | 94% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.00% | 0.30% | -0.20% | 100% | 97% |
| Class F | #DIV/0! | #DIV/0! | #DIV/0! | #NUM! | #NUM! |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | 0.00% | 0.15% | -0.02% | 100% | 97% |
| Class E | 0.06% | 0.05% | -0.66% | 99% | 95% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.00% | 0.00% | -0.28% | 100% | 99% |
| Class F | #DIV/0! | #DIV/0! | #DIV/0! | #NUM! | #NUM! |

# References

[1] J. Chen, Y. Ye, S.H. Kim, “Algorithm description for Versatile Video Coding and Test Model 3 (VTM 3)”, JVET-L1002-v1, 12th JVET Meeting, Macau, China, 3–12 Oct, 2018.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).