| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0519-v1 |
| --- | --- |

| Title: | Non-CE: Context modeling for coding the prediction mode flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a modified context modeling scheme for entropy coding the syntax element pred_mode_flag. In the proposed method, additional context variables are employed to further exploit contextual information. The proposed method reportedly generates -0.11% and -0.07% luma BD-rates for the RA and LB settings, respectively, under the VTM3.0 CTCs, while increasing the number of context variables by 3.

# Introduction

In VVC working draft (WD) 3 as specified in [1], the syntax element pred_mode_flag is entropy coded using a single context variable. In this contribution, additional context variables are employed.

# Proposed method

In the proposed method, context modeling for entropy coding the syntax element pred_mode_flag is conditioned on the coding unit (CU) prediction modes of the two neighbouring CUs and further conditioned on the size of the current CU using four context variables. The context index increment ctxInc is derived as follows:

ctxInc = ( ((predModeA == MODE_INTRA )? 1:0) + ((predModeL == MODE_INTRA)? 1:0)),

ctxInc += ( ctxInc > 0 || cuSizeC < 256 )? 1 : 0,

where the variables predModeA and predModeL represent the CU prediction modes of the above neighbouring CU and left neighbouring CU, respectively, and the variable cuSizeC represents the CU size in the number of luma samples.

# Experimental results

The proposed method was integrated into the VTM3.0 reference software [2]. The coding results generated by the proposed method were evaluated against the anchor results generated by the unmodified VTM3.0 reference software. Table 1 summarizes the experimental results conducted under the CTCs specified in [3]. The detailed simulation results can be found in the attached spreadsheets.

Table 1:  Summary of the average BD bitrate and run time results

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.11% | -0.07% | -0.01% | 100% | 101% |
| Class A2 | -0.10% | 0.08% | 0.06% | 100% | 99% |
| Class B | -0.10% | 0.07% | 0.07% | 100% | 100% |
| Class C | -0.13% | -0.07% | 0.04% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | 0.00% | 0.04% | 100% | 100% |
| Class D | -0.05% | -0.03% | 0.09% | 100% | 100% |
| Class F (optional) | -0.10% | -0.03% | -0.01% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.05% | -0.12% | -0.10% | 100% | 99% |
| Class C | -0.12% | 0.14% | 0.25% | 101% | 101% |
| Class E | -0.04% | 0.05% | 0.31% | 100% | 101% |
| Overall | -0.07% | 0.01% | 0.12% | 100% | 100% |
| Class D | -0.07% | 0.42% | -0.13% | 101% | 100% |
| Class F (optional) | 0.10% | 0.17% | 0.04% | 101% | 99% |

# Conclusions

The proposed method increases the number of context variables by three to further exploit contextual information. The proposed method reportedly generates -0.11% and -0.07% luma BD-rates for the RA and LB settings, respectively, under the VTM3.0 CTCs.

# References

- B. Bross, J. Chen, and S. Liu “Versatile Video Coding (Draft 3),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L1001, 12th Meeting: Macao, CN, 3–12 Oct. 2018.

- The software VTM3.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0 .

- F. Bossen, J. Boyce, X. Li, and V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-L1010, 12th Meeting: Macao, CN, 3–12 Oct. 2018.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).