| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, MA, 9–18 Jan. 2019 | Document: JVET-M0523-v1 |
| --- | --- |

| Title: | CE6: RQT-like transform partitioning for inter blocks (Test 6.4.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hilmi E. Egilmez, Vadim Seregin, Amir Said, Marta Karczewicz 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Tel: Email: | hegilmez@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents the results of CE6.4.2, where a transform unit (TU) partitioning method for inter blocks is tested. The proposed method allows partitioning as an additional feature for transformation of inter predicted blocks, and it uses the current inter MTS design in VTM-3.0 when no TU partitioning is used. Over VTM-3.0 (CTC conditions), the proposed method achieves

- 0.36% RA and 0.59% LDB average luma coding gains;

- 0.62% RA and 0.94% LDB average luma coding gains when Inter MTS is also enabled.

# Introduction

In VTM-3.0, pairs of DCT-2, DST-7 or DCT-8 are used for separable transformation of residual block, where DCT-2 is used up to size 64x64 and DST-7 and DCT-8 are allowed up to size 32x32. When MTS  flag is 0, DCT-2 is used for both horizontal and vertical transformation. If the MTS flag is 1, one of the four separable transform options derived from DST-7 and DCT-8 is signalled and used to transform a block.

Currently in VTM, no TU partitions are allowed when MTS is used. In order to achieve a better compression, partitioning of TUs may be useful as it provides split options other than signaling 5 different transform options offered in MTS (one from MTS flag = 0 and other four from MTS flag = 1). This proposal introduces a TU partitioning method as an additional option for transforming inter predicted blocks. When there is no TU split, the proposed design uses the same MTS design in VTM-3.0.

# Proposed method

The proposed partition method uses a quad-tree based splits for the inter predicted blocks. Specifically, a given TU can be partitioned into 4 sub-TUs. This partitioning is not only used for square TUs but also for non-square TUs as shown in Fig.1.

Figure 1: Quad-tree partitioning of a (left) square and (right) non-square blocks

In the proposed design, for a given CU, one level of TU partitioning is allowed only for luma blocks so that TU splits can have the maximum depth of 1. A flag is used to determine whether split is applied or not. Splits are allowed if both width and height of a given TU is larger than 4. If a split is used, for each sub-TU, a cbf flag is signaled as in VTM-3.0 signaled for TUs.

The separable transforms used for are inferred from the position of the subblock (i.e., no additional signaling is used to determine the transforms used in horizontal and vertical directions). Specifically, if transform is not skipped, for the subblocks P0, P1, P2 and P3 (as illustrated in Fig.1):

- For sub-block P0, DCT8 is used in both horizontal and vertical directions.

- For sub-block P3, DST7 is used in both horizontal and vertical directions.

- For sub-block P1, DST7 is used in horizontal direction, and DCT8 is used in vertical direction.

- For sub-block P2, DCT8 is used in horizontal direction, and DST7 is used in vertical direction.

In a nutshell, the proposed method can be viewed as a simple extension of RQT in HEVC with non-square block and DST7/DCT8 transforms.

# Experimental Results

## Results of CE 6.4.2

The proposed method is implemented based on VTM-3.0 software and tested (in CE 6.4.2) under CTC and by enabling the inter MTS (Inter AMT) via EMT=3 and EMTFast =3.

Table 1 shows the performance of the proposed method over VTM-3.0 (CTC), and Table 2 shows the gains obtained by the proposed method and Inter MTS over VTM-3.0 (CTC). In both tables, the all-intra (AI) results are omitted since the method does not propose any changes for coding of intra blocks and produces identical results to VTM-3.0 CTC results.

Table 1: Results of the proposed method (CE6.4.2) over VTM-3.0 (CTC).

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.19% | 0.19% | 0.30% | 119% | 102% |
| Class A2 | -0.37% | -0.18% | 0.38% | 121% | 102% |
| Class B | -0.43% | 0.10% | 0.39% | 122% | 103% |
| Class C | -0.38% | 0.20% | 0.33% | 125% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.36% | 0.09% | 0.35% | 122% | 102% |
| Class D | -0.55% | 0.21% | 0.39% | 126% | 102% |
| Class F | -0.29% | 0.06% | 0.12% | 118% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.62% | -1.42% | 0.67% | 131% | 104% |
| Class C | -0.63% | -0.52% | 0.26% | 131% | 100% |
| Class E | -0.49% | 0.46% | 0.41% | 121% | 98% |
| Overall | -0.59% | -0.65% | 0.47% | 129% | 101% |
| Class D | -0.76% | -1.14% | 1.06% | 132% | 102% |
| Class F | -1.20% | -0.60% | -0.10% | 124% | 98% |

Table 2: Results of the proposed method (CE6.4.2) with Inter MTS enabled over VTM-3.0 (CTC).

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.33% | 0.45% | 0.69% | 137% | 103% |
| Class A2 | -0.85% | 0.43% | 0.68% | 141% | 105% |
| Class B | -0.72% | 0.69% | 0.79% | 147% | 106% |
| Class C | -0.54% | 0.75% | 0.99% | 155% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.62% | 0.61% | 0.80% | 146% | 105% |
| Class D | -0.77% | 0.71% | 0.92% | 157% | 105% |
| Class F | -0.34% | 0.31% | 0.34% | 137% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.16% | 0.07% | 1.43% | 165% | 109% |
| Class C | -1.04% | 0.78% | 1.31% | 172% | 107% |
| Class E | -0.44% | 1.55% | 1.54% | 142% | 103% |
| Overall | -0.94% | 0.68% | 1.41% | 161% | 107% |
| Class D | -1.11% | 0.22% | 1.65% | 173% | 108% |
| Class F | -1.16% | 0.44% | 0.42% | 153% | 101% |

# Complexity metrics

## Operation and memory counts: The proposed method tested in CE6.4.2 introduces no additional computational complexity and memory requirements over VTM-3.0.

## The bit-precision required by the transforms: The same precision in the current VTM-3.0 is used.

## Specify if a transform requires multiple iterations: No additional iteration is required from VTM-3.0.

## Encoder and decoder times: These measurements are presented in Section 3.

# References

- A. Said and X. Zhao, “Description of Core Experiment 6: Transforms and Transform signaling,” JVET-L1026, 12th meeting, Macao, CN, Oct. 2018.

- F. Bossen, J.Boyce, X. Li, V. Seregin and K. Suehring,  “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, 12th meeting, Macao, CN, Oct. 2018.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).