| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, MA, 9–18 Jan. 2019 | Document: JVET-M0524-v1 |
| --- | --- |

| Title: | CE3/6-related: Unification of RQT-like transform partitioning for intra and inter blocks |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hilmi E. Egilmez, Vadim Seregin, Amir Said, Marta Karczewicz 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Tel: Email: | hegilmez@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents two unified extensions of the transform unit (TU) partitioning method, tested in CE6.4.2 for inter blocks. The first unification is a trivial extension of the method in CE6.4.2, which is applied to both intra and inter predicted blocks. The second unified design is a more flexible variation of the same method allowing smaller TU partitions.

Over VTM-3.0, the first extension provides

- 0.xx% AI, 0.xx% RA and 0.xx% LDB average luma coding gains;

- 0.19% AI, 0.74% RA and 0.xx% LDB average luma coding gains when Inter MTS is also enabled.

The second extension achieves

- 0.xx% AI, 0.xx% RA and 0.xx% LDB average luma coding gains;

- 0.29% AI, 0.xx% RA and 0.xx% LDB average luma coding gains when Inter MTS is also enabled.

# Introduction

In VTM-3.0, pairs of DCT-2, DST-7 or DCT-8 are used for separable transformation of residual block, where DCT-2 is used up to size 64x64 and DST-7 and DCT-8 are allowed up to size 32x32. When MTS  flag is 0, DCT-2 is used for both horizontal and vertical transformation. If the MTS flag is 1, one of the four separable transform options derived from DST-7 and DCT-8 is signalled and used to transform a block.

Currently in VTM, no TU partitions are allowed when MTS is used. In order to achieve a better compression, partitioning of TUs may be useful as it provides split options other than signaling 5 different transform options offered in MTS (one from MTS flag = 0 and other four from MTS flag = 1). This proposal introduces a TU partitioning method as an additional option for transforming inter predicted blocks. When there is no TU split, the proposed design uses the same MTS design in VTM-3.0.

In CE6.4.2, the proposed partition method uses a quad-tree based splits for the inter predicted blocks. Specifically, a given TU can be partitioned into 4 sub-TUs. This partitioning is not only used for square TUs but also for non-square TUs as shown in Fig.1.

Figure 1: Quad-tree partitioning of a (left) square and (right) non-square blocks

In CE6.4.2, for a given CU, one level of TU partitioning is allowed only for luma blocks so that TU splits can have the maximum depth of 1. A flag is used to determine whether split is applied or not. Splits are allowed if both width and height of a given TU is larger than 4. If a split is used, for each sub-TU, a cbf flag is signaled as in VTM-3.0 signaled for TUs.

The separable transforms used for are inferred from the position of the subblock (i.e., no additional signaling is used to determine the transforms used in horizontal and vertical directions). Specifically, if transform is not skipped, for the subblocks P0, P1, P2 and P3 (as illustrated in Fig.1):

- For sub-block P0, DCT8 is used in both horizontal and vertical directions.

- For sub-block P3, DST7 is used in both horizontal and vertical directions.

- For sub-block P1, DST7 is used in horizontal direction, and DCT8 is used in vertical direction.

- For sub-block P2, DCT8 is used in horizontal direction, and DST7 is used in vertical direction.

# Proposed methods

## Method I

The method tested in CE6.4.2 is extended to be applied intra predicted blocks. While it uses exactly the same method in CE6.4.2 for inter predicted blocks, the following simple modification is made to define separable transforms for intra predicted blocks:

- DST7 is used in both horizontal and vertical directions for all sub-TUs after splitting (i.e., DST7 is used for P0, P1, P2 and P3 for all sub-blocks illustrated in Fig.1).

## Method II

In CE6.4.2 (as well as in Method I), TU splits are allowed if both width and height of a given TU is larger than 4. In this second extension, TU splits are further allowed to be partitioned if either width or height of a TU is larger than 4. In addition to partitions in Method I, the following two cases are also included in Method II:

- A 4 x 8 or 8 x 4 TU can be partitioned into two sub-TUs of size 4 x 4.

- A 4 x N (resp. N x 4) TU for N > 8 can be partitioned into four sub-TUs of size 4 x (N/4) (resp. (N/4) x 4).

# Experimental Results

The proposed method is implemented based on VTM-3.0 software and tested under CTC and by enabling the inter MTS (Inter AMT) via EMT=3 and EMTFast =3.

## Results of Method I

Table 1 shows the gains obtained by the proposed method with Inter MTS over VTM-3.0 (CTC), and Table 2 shows the performance of the proposed method over VTM-3.0 (CTC).

Table 1: Results of Method I with Inter MTS enabled over VTM-3.0 (CTC).

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.08% | -0.05% | 126% | 99% |
| Class A2 | -0.14% | -0.10% | -0.12% | 124% | 101% |
| Class B | -0.19% | -0.13% | -0.15% | 124% | 100% |
| Class C | -0.22% | -0.23% | -0.31% | 122% | 101% |
| Class E | -0.32% | -0.28% | -0.08% | 125% | 104% |
| Overall | -0.19% | -0.16% | -0.15% | 124% | 101% |
| Class D | -0.10% | -0.15% | -0.10% | 121% | 103% |
| Class F | -0.63% | -0.34% | -0.28% | 122% | 103% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.42% | -0.06% | 0.35% | 138% | 102% |
| Class A2 | -0.94% | 0.13% | 0.63% | 142% | 103% |
| Class B | -0.85% | 0.32% | 0.45% | 145% | 101% |
| Class C | -0.69% | 0.39% | 0.69% | 155% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.74% | 0.22% | 0.53% | 146% | 101% |
| Class D | -0.85% | 0.18% | 1.08% | 155% | 100% |
| Class F | -0.80% | -0.12% | -0.13% | 140% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -1.05% | 0.57% | 1.39% | 169% | 105% |
| Class E | -0.52% | 2.32% | 1.89% | 138% | 97% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -1.16% | -0.18% | 1.38% | 167% | 99% |
| Class F | -1.58% | 0.05% | -0.18% | 150% | 97% |

Table 2: Results of Method I without Inter MTS over VTM-3.0 (CTC).

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

Table 3: Results of Method II with Inter MTS enabled over VTM-3.0 (CTC).

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.12% | -0.10% | -0.04% | 138% | 101% |
| Class A2 | -0.17% | -0.11% | -0.13% | 139% | 103% |
| Class B | -0.26% | -0.17% | -0.16% | 144% | 102% |
| Class C | -0.43% | -0.32% | -0.45% | 148% | 102% |
| Class E | -0.42% | -0.26% | -0.17% | 147% | 100% |
| Overall | -0.29% | -0.20% | -0.20% | 143% | 102% |
| Class D | -0.29% | -0.10% | -0.15% | 148% | 101% |
| Class F | -1.77% | -1.23% | -1.21% | 154% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.44% | -0.15% | 0.47% | 141% | 102% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.94% | 0.26% | 0.38% | 151% | 102% |
| Class C | -0.89% | 0.15% | 0.42% | 162% | 99% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -1.07% | 0.00% | 0.72% | 166% | 97% |
| Class F | -1.91% | -0.93% | -0.89% | 149% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -1.21% | 0.56% | 1.14% | 178% | 102% |
| Class E | -0.65% | 1.71% | 1.88% | 138% | 93% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -1.48% | -0.30% | 1.93% | 181% | 99% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 4: Results of Method II without Inter MTS over VTM-3.0 (CTC).

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

# References

- A. Said and X. Zhao, “Description of Core Experiment 6: Transforms and Transform signaling,” JVET-L1026, 12th meeting, Macao, CN, Oct. 2018.

- F. Bossen, J.Boyce, X. Li, V. Seregin and K. Suehring,  “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, 12th meeting, Macao, CN, Oct. 2018.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).