| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0528_r1 |
| --- | --- |

| Title: | Non-CE3: A unified luma intra mode list construction process |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Bossen1, K. Misra2 | Tel: Email: | +1 360 817 8451 misrak@sharplabs.com frank@bossentech.com |
| Source: | 1Sharp Electronics of Canada, 2Sharp Labs of America |  |  |

_____________________________

# Abstract

In the current VVC WD there exists three different luma intra mode list construction process: one for zero reference line index, a second for non-zero reference line index and a third for multi-hypothesis intra. This contribution proposes the use of a single list construction process. The contribution achieves this by first signaling if a luma intra mode is angular, and for angular mode constructing lists comprising only of angular modes (which is the same as currently being used for non-zero reference line index). For non-angular luma intra mode (i.e. PLANAR, DC) an additional flag is signaled that indicates the selected mode. It is asserted that the proposed modification simplifies the luma intra mode list construction process and on average has no significant coding efficiency impact. The proposed change is implemented in VTM-3.0 and evaluated under common test conditions. Note, that the unified list construction is used only for entropy coding and the encoder mode search is the same as in VTM-3.0. The proposed modification reportedly provides an average luma BD rate performance of -0.07% (all-intra), -0.03% (random-access), 0.05% (low delay B). It is also reported that for class A1 sequences in all-intra configuration the proposed scheme provides an average BD rate of -0.29%.

In revision 1 of the contribution results are reported for additional encoder-only changes that improves the average luma BD rate performance to -0.25% (all-intra), -0.09% (random-access), 0.02% (low delay B).

# Introduction

In the current VVC WD there exists three different luma intra mode list construction process: one for zero reference line index, a second for non-zero reference line index and a third for multi-hypothesis intra. The contribution proposes the use of a single list construction process comprising solely of angular modes. This process is the same as the one currently being used for non-zero reference line index. Since the list no longer contains non-angular modes (PLANAR, DC modes) the luma intra mode signaling is modified to first signal a flag indicating if the selected mode is angular. For non-angular mode a further flag is received indicating if the mode is PLANAR or DC. The modified pseudo-syntax is shown below:

if (!intra_luma_ref_idx)
	intra_luma_angular_mode_flag
	if (!intra_luma_angular_mode_flag)
		intra_luma_planar_flag // Planar if true, DC otherwise
if (intra_luma_ref_idx || intra_luma_angular_mode_flag)
	if (intra_luma_ref_idx)
		intra_luma_mpm_flag=1
	else
		intra_luma_mpm_flag
	if (intra_luma_mpm_flag)
		intra_luma_mpm_idx
	else
		intra_luma_mpm_remainder

For multi-hypothesis intra the only angular modes allowed are: vertical and horizontal modes. For the above listed approach, a single flag needs to be signaled to distinguish between the two angular modes (no list construction is needed). The detailed syntax and semantics modifications to VVC WD [1] can be found in attached document.

# Results

The proposed change is implemented in VTM-3.0 [3] and evaluated under common test conditions [2]. Note, that the unified list construction is used only for entropy coding and the encoder mode search is the same as in VTM-3.0. The summary results are listed below. Detailed results can be found in excel file attached to this contribution.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.29% | -0.33% | -0.20% | 101% | 105% |
| Class A2 | -0.04% | -0.10% | -0.07% | 99% | 103% |
| Class B | -0.08% | -0.10% | -0.06% | 102% | 102% |
| Class C | 0.03% | 0.01% | 0.03% | 102% | 101% |
| Class E | -0.01% | -0.03% | 0.01% | 101% | 102% |
| Overall | -0.07% | -0.10% | -0.05% | 101% | 102% |
| Class D | 0.04% | 0.19% | 0.06% | 102% | 100% |
| Class F (optional) | 0.21% | 0.15% | 0.15% | 101% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | 0.02% | 0.02% | 101% | 101% |
| Class A2 | -0.02% | -0.03% | 0.08% | 100% | 101% |
| Class B | -0.03% | 0.06% | 0.00% | 98% | 101% |
| Class C | 0.01% | 0.11% | 0.22% | 98% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | 0.05% | 0.08% | 99% | 101% |
| Class D | 0.01% | -0.33% | -0.27% | 98% | 100% |
| Class F (optional) | 0.09% | 0.14% | 0.11% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.28% | 0.07% | 97% | 102% |
| Class C | 0.06% | 0.38% | 0.28% | 99% | 99% |
| Class E | 0.08% | 1.10% | 0.56% | 96% | 101% |
| Overall | 0.05% | 0.28% | 0.26% | 97% | 101% |
| Class D | 0.03% | 0.07% | 0.27% | 100% | 100% |
| Class F (optional) | 0.06% | -0.38% | -0.48% | 101% | 101% |

# Additional encoder-only changes were also tested that adds modes with N lowest bit cost for mode signaling to R-D testing. This improves the average luma BD rate performance to:

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.47% | -0.48% | -0.35% | 101% | 106% |
| Class A2 | -0.20% | -0.12% | -0.17% | 103% | 102% |
| Class B | -0.25% | -0.20% | -0.15% | 104% | 103% |
| Class C | -0.16% | -0.24% | -0.15% | 107% | 100% |
| Class E | -0.21% | -0.14% | -0.02% | 97% | 100% |
| Overall | -0.25% | -0.23% | -0.16% | 103% | 102% |
| Class D | -0.18% | 0.01% | -0.12% | 108% | 99% |
| Class F (optional) | 0.01% | 0.03% | -0.03% | 106% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.13% | 0.00% | 100% | 98% |
| Class A2 | -0.05% | 0.17% | 0.18% | 97% | 98% |
| Class B | -0.11% | 0.22% | 0.02% | 97% | 98% |
| Class C | -0.04% | -0.09% | 0.17% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.09% | 0.06% | 0.09% | 98% | 98% |
| Class D | -0.06% | -0.41% | -0.15% | 100% | 100% |
| Class F (optional) | -0.01% | -0.03% | 0.02% | 99% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.12% | -0.21% | 97% | 97% |
| Class C | 0.07% | 0.20% | 0.30% | 94% | 95% |
| Class E | 0.01% | 1.04% | -0.12% | 89% | 94% |
| Overall | 0.02% | 0.27% | -0.02% | 94% | 96% |
| Class D | 0.02% | 0.01% | 0.59% | 97% | 94% |
| Class F (optional) | 0.08% | -0.31% | -0.16% | 97% | 97% |

# Recommendation

In the current VVC WD there exists three different luma intra mode list construction process: one for zero reference line index, a second for non-zero reference line index and a third for multi-hypothesis intra. This contribution proposes the use of a single list construction process. The proposed modification reportedly provides an average luma BD rate performance of -0.07% (all-intra), -0.03% (random-access), 0.05% (low delay B). It is also reported that for class A1 sequences in all-intra configuration the proposed scheme provides an average BD rate of -0.29%.

Further results are presented for encoder-only changes that adds modes with N lowest bit cost for mode signaling to R-D testing. This improves the average luma BD rate performance to -0.25% (all-intra), -0.09% (random-access), 0.02% (low delay B).

It is recommended that the approach be adopted in the next version of the VVC WD and VTM.

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 3),” JVET-L1001 12th Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao, CN, 3–12 Oct. 2018

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010 12th Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao, CN, 3–12 Oct. 2018

[3] https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-3.0 VTM-3.0