| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0538-v1 |
| --- | --- |

| Title: | CE6-1.4: Efficient Implementations of MTS with Transform Adjustments |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Amir Said, Hilmi E. Egilmez, Yung-Hsuan Chao, Vadim Seregin, Marta Karczewicz 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Email: | asaid@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports results of CE6.1.4, where a unified transformation approach is proposed to replace the DCT-8 and DST-7 used as primary transforms in MTS of VTM-3.0. It describes modifications and improvements made in response to concerns and suggestions mentioned during a BoG discussion on  primary transforms. The new method addresses all important issues, including only using the inverse DCT-2 at the decoder, and preserving low arithmetic bit precision. In addition, it needs a lower number of arithmetic operations, and the total memory (beyond required for DCT-2) for computing transforms of dimension 16, 32, and 64, is only 64 bytes. VTM-3-based throughput measurements show the advantage over other proposals, and a SIMD implementation is provided as example of how the simple computational structure can be easily and efficiently optimized in software and hardware. Simulation results show that under CTC, for MTS transforms of dimension 32 the B-D rates are 0.00%, -0.01%, -0.02%, and -0.04% for AI, RA, LB, and LP tests. Combining MTS transforms of dimension 32 and 64 yield, without increase on encoding or decoding times, gains of about 0.45% for class A2, and averages of -0.07%, -0.12%, 0.01%, and -0.01%.

# Introduction

Transform “adjustments” were first proposed in JVET-J0066 as a method for replacing computationally costly trigonometric transforms, like the DST-7, and DCT-8, with approximations that could

- Provide the codec with the same type of transform diversity as that provided by the trigonometric transforms, and thus yield practically the same coding gains;

- Could be computed using an “adjustment” of very low complexity, plus the DCT-2 as the “main computational engine,” since it is the transform already used in HEVC, has been widely studied and optimized, and enables re-using previous hardware and software implementations.

The method defined in JVET-L0386 used adjustment filters before the DCT-3 (inverse DCT-2) at the forward transform. At the primary transform BoG discussions during the Macao meeting [JVET-L0685], this was considered a problem, since it would require implementing the forward DCT-2 at decoders, while currently some implementations only have the inverse DCT-2 at the decoder.

To address this problem a new implementation is proposed, as shown in Fig. 1, where the adjustments are computed after the forward DCT-2, and their inverse before the inverse DCT-2. In this new version those adjustments correspond to simply multiplying the vector with the 8 lowest-frequency transform coefficients by an 8×8 matrix of dimension. The same matrix values are used for transforms of dimension 16, 32, and 64, and for the DST-7 and DCT-8 adjustments.

Fig. 1 – New implementation of transform adjustment that needs only the inverse DCT-2 at the decoder. For transforms of dimension 16, 32, and 64, the adjustment blocks correspond to multiplying the 8 lowest frequency transform coefficients by the same 8×8 matrix.

The simulation results show that that this alternative approach produced coding results similar or better than the previous, with very small loss or small gains, compared to the true DST-7 and DCT-8 transforms.

More details about the implementation are provided in Section 3, and in the next section a summary of the features provided by this approach and its implementation are presented.

# Summary of features

As mentioned in the introduction, the new implementation addresses several issues discussed in the primary transform BoG [JVET-L0685]. The following is a summary.

- With the adoption of 8-bit coefficients for primary transforms in VTM-3, the use of the DCT-2 in this proposal now corresponds to current HEVC implementations, and previously optimized transform SW and HW can be readily re-used.

- No additional HW is needed for supporting the forward DCT at decoder-only implementations.

- Arithmetic precision is maintained:

  - The coefficients of the 8×8 adjustment matrix are represented with 8 bits.

  - After the adjustment stages the computed values are re-normalized and “clamped” to guarantee that inputs to the inverse DCT-2 are limited to 16-bit integers.

- The band-matrix previously used for adjustment computations is replaced by single matrix, which was indicated as the preferred form of computation.

- All computations of the DCT-2 and the adjustment have very regular patterns, greatly simplifying efficient parallel implementation using SIMD instructions or custom HW (a SIMD-based code is shared as example).

- The additional memory (beyond that used for the DCT-2) needed for computing the MTS transforms of dimensions 16, 32, and 64, is only 64 bytes. This represents a very significant reduction compared to the memory required to store the DST-7 (or DCT-8) matrices.

- Due to the “zero-out” technique used in DCT-2 of dimension 64, the worst case transform throughput in VTM-3, which defines the custom HW costs, corresponds to the matrix-based DST-7 and DCT-8 on blocks of size 32×32. This proposal practically doubles the throughput for that worst case, reducing implementation costs.

- MTS transforms of dimension 64 now have computational complexity practically equal to the DCT-2 transform of dimension 64 (both including “zero-out”), so additional gains from extending MTS to larger blocks do not increase encoding or decoding times.

# Implementation

For a transform of dimension N = 16, 32, or 64, if c(DCT-2) is the N-dimensional vector with coefficients of the DCT-2 transform (in increasing frequency), we can split this vector as

c(DCT-2) = [  x | y ],

where x has dimension 8 (i.e., the coefficients of the 8 lowest frequencies) and y has dimension N – 8. The transform adjustments corresponding to the DCT-8 and DST-7 MTS options are defined as

ĉ(DCT-8) = [  A(DCT-8) x | y ],

ĉ (DST-7) = [  A(DST-7) x | y ].

where A is an 8×8 matrix (symbol ĉ is used instead of c to indicate that those are not the true DCT-8 and DST-7 coefficients).

Matrix A should be orthogonal and designed to maximize coding gain. In this proposal the matrix used to compute the orthogonal transform associated with the DST-7 is scaled by 128, and defined by the C++ code

const TMatrixCoeff g_TAF_DST7[8][8] =

{

{ 118,-48,-12, -2, -2, -1, -1,  0 },

{  38,108,-54,-15, -8, -6, -3, -2 },

{  24, 36,107,-51,-16, -9, -4, -4 },

{  14, 23, 34,111,-44,-13, -8, -5 },

{  10, 16, 19, 29,114,-38,-14, -8 },

{   8, 13, 13, 14, 26,118,-33,-12 },

{   6,  9,  9, 11, 15, 24,120,-28 },

{   5,  8,  8,  8, 11, 14, 23,124 }

};

while the matrix corresponding to the DCT-8 computation is

const TMatrixCoeff g_TAF_DCT8[8][8] =

{

{ 118, 48,-12,  2, -2,  1, -1,  0 },

{ -38,108, 54,-15,  8, -6,  3, -2 },

{  24,-36,107, 51,-16,  9, -4,  4 },

{ -14, 23,-34,111, 44,-13,  8, -5 },

{  10,-16, 19,-29,114, 38,-14,  8 },

{  -8, 13,-13, 14,-26,118, 33,-12 },

{   6, -9,  9,-11, 15,-24,120, 28 },

{  -5,  8, -8,  8,-11, 14,-23,124 }

};

It can be observed that the elements of the two matrices have same magnitudes, but different signs, according to a chessboard pattern. This is not a coincidence and is a consequence of properties of the DCT-2, DCT-8 and DST-7. Thus, only one of these matrices need to be stored, and used in the matrix-vector computations (as shown in the code for this CE). For some implementations it may be more convenient to store both versions, but this represents only 64 additional bytes of memory. Since the matrices above are scaled versions of orthogonal matrices, their inverses are defined by simply transposing (or left-side multiplication.

Note that in this proposal the same matrix coefficients are used for transforms of dimensions 16, 32, and 64, and thus the total memory requirement, beyond the DCT-2, is only 64 bytes.

# Complexity measures

## Operation counts for individual transforms

The operation counts for 1D forward/inverse transforms in this CE are shown in Table I, and include all arithmetic operations needed for implementing DCT-2 and adjustments. Note that counts for dimension 64 include the “zero-out” technique used in VTM-3.0.

Table I: Operation counts for 1-D forward/inverse transforms

|  | Number of Multiplications |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Size | 4 | 8 | 16 | 32 | 64 |
| DST7 | 8 | 64 | 150 | 406 | 747 |
| DCT8 | 8 | 64 | 150 | 406 | 747 |
| DCT2 | 8 | 24 | 86 | 342 | 683 |

|  | Number of Additions |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Size | 4 | 8 | 16 | 32 | 64 |
| DST7 | 15 | 65 | 177 | 465 | 871 |
| DCT8 | 15 | 65 | 177 | 465 | 871 |
| DCT2 | 13 | 37 | 113 | 401 | 807 |

|  | Number of Shifts |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Size | 4 | 8 | 16 | 32 | 64 |
| DST7 | 5 | 9 | 25 | 41 | 41 |
| DCT8 | 5 | 9 | 25 | 41 | 41 |
| DCT2 | 5 | 9 | 17 | 33 | 33 |

## Memory requirements for storing transform cores

The amount of memory needed for transforms are shown in Table II.

Table II: Number of bytes for each transform matrix

|  | Number of bytes in transform matrices |  |  |  |
| --- | --- | --- | --- | --- |
| Size | 16 | 32 | 64 | Summary (for each transform) |
| DST7 + DCT8 | 64 |  |  | 64 |
| DCT2 | 86 | 342 | 1366 | 1874 |

# Throughput measurements

Using the method of JVET-M0540, the throughput of the proposed method was measured, using 20 frames of the sequence “Tango2,” AI configuration, and QP = 32. Results are listed in Table III.

Table III: Transform computation throughput (106 coefficients per second)

| Transform method | Encoder |  |  |  |  |  | Decoder |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Forward |  |  | Inverse |  |  | Inverse |  |  |
|  | 16 | 32 | 64 | 16 | 32 | 64 | 16 | 32 | 64 |
| VTM (matrix) | 170 | 125 | 208 | 80 | 49 | 67 | 81 | 49 | 66 |
| CE6-1.4d | 205 | 173 | 336 | 180 | 212 | 342 | 214 | 257 | 361 |
| CE6-1.4d SIMD | 222 | 200 | 389 | 280 | 334 | 723 | 351 | 429 | 838 |
| JVET-M0539 | 409 | 346 | 1278 | 184 | 219 | 339 | 213 | 256 | 360 |
| JVET-M0539 SIMD | 422 | 386 | 1370 | 282 | 338 | 750 | 373 | 449 | 876 |

efficiently optimized in software and hardware.

# Simulation results

## CTC (inter MTS disabled)

### CE6-14a (dimension 32)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.04% | -0.04% | 97% | 90% |
| Class A2 | 0.08% | 0.23% | 0.14% | 98% | 94% |
| Class B | 0.04% | 0.14% | 0.11% | 99% | 95% |
| Class C | 0.01% | 0.01% | -0.07% | 98% | 97% |
| Class E | 0.00% | -0.01% | 0.13% | 97% | 94% |
| Overall | 0.00% | 0.07% | 0.05% | 98% | 94% |
| Class D | 0.02% | 0.02% | -0.01% | 98% | 98% |
| Class F (optional) | 0.01% | 0.07% | 0.02% | 98% | 98% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | 0.02% | 0.09% | 99% | 98% |
| Class A2 | -0.01% | 0.03% | 0.28% | 100% | 99% |
| Class B | -0.01% | 0.21% | -0.01% | 101% | 99% |
| Class C | 0.03% | 0.00% | 0.21% | 98% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.08% | 0.13% | 100% | 99% |
| Class D | 0.02% | -0.36% | -0.11% | 100% | 100% |
| Class F (optional) | 0.02% | -0.01% | 0.03% | 98% | 98% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.19% | 0.00% | 100% | 101% |
| Class C | 0.04% | 0.19% | 0.24% | 99% | 99% |
| Class E | -0.12% | 0.44% | 0.14% | 98% | 94% |
| Overall | -0.02% | 0.09% | 0.11% | 99% | 99% |
| Class D | 0.02% | 0.35% | 0.11% | 99% | 101% |
| Class F (optional) | -0.08% | -0.05% | 0.08% | 97% | 95% |

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.33% | 0.13% | 100% | 102% |
| Class C | 0.03% | 0.17% | 0.09% | 100% | 100% |
| Class E | -0.17% | 0.56% | -0.26% | 98% | 100% |
| Overall | -0.04% | 0.33% | 0.02% | 99% | 101% |
| Class D | -0.03% | 0.12% | 0.17% | 100% | 101% |
| Class F (optional) | -0.02% | -0.27% | -0.47% | 99% | 98% |

### CE6-14b (dimensions 32 and 64)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.45% | -0.29% | -0.26% | 98% | 90% |
| Class A2 | 0.02% | 0.12% | 0.14% | 98% | 94% |
| Class B | -0.01% | 0.09% | 0.13% | 99% | 97% |
| Class C | 0.02% | 0.02% | -0.10% | 98% | 95% |
| Class E | -0.02% | 0.03% | 0.02% | 98% | 91% |
| Overall | -0.07% | 0.01% | 0.00% | 98% | 94% |
| Class D | 0.02% | 0.01% | 0.00% | 98% | 96% |
| Class F (optional) | 0.00% | -0.03% | -0.01% | 98% | 95% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.44% | -0.85% | -0.75% | 102% | 99% |
| Class A2 | -0.07% | -0.02% | -0.01% | 101% | 99% |
| Class B | -0.07% | 0.17% | -0.23% | 101% | 99% |
| Class C | 0.01% | 0.00% | 0.05% | 97% | 96% |
| Class E |  |  |  |  |  |
| Overall | -0.12% | -0.12% | -0.22% | 100% | 98% |
| Class D | 0.02% | -0.36% | -0.13% | 98% | 96% |
| Class F (optional) | -0.04% | 0.01% | 0.00% | 97% | 95% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.18% | -0.13% | 101% | 102% |
| Class C | 0.04% | 0.20% | 0.35% | 98% | 94% |
| Class E | 0.05% | 1.14% | -0.21% | 96% | 91% |
| Overall | 0.01% | 0.28% | 0.01% | 99% | 97% |
| Class D | 0.04% | 0.29% | 0.69% | 98% | 95% |
| Class F (optional) | 0.04% | -0.24% | -0.34% | 97% | 95% |

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | 0.04% | -0.14% | 101% | 97% |
| Class C | 0.01% | 0.21% | 0.16% | 98% | 96% |
| Class E | -0.01% | 0.30% | -0.40% | 97% | 98% |
| Overall | -0.01% | 0.16% | -0.11% | 99% | 97% |
| Class D | -0.06% | -0.02% | 0.13% | 98% | 96% |
| Class F (optional) | 0.14% | -0.23% | 0.05% | 98% | 96% |

### CE6-14c (dimension 16 and 32)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | 0.02% | 0.08% | 93% | 88% |
| Class A2 | 0.25% | 0.26% | 0.26% | 95% | 91% |
| Class B | 0.20% | 0.32% | 0.23% | 96% | 91% |
| Class C | 0.11% | 0.11% | 0.01% | 96% | 93% |
| Class E | 0.08% | 0.07% | 0.18% | 95% | 90% |
| Overall | 0.12% | 0.17% | 0.15% | 95% | 91% |
| Class D | 0.07% | 0.07% | -0.02% | 96% | 96% |
| Class F (optional) | 0.08% | 0.08% | 0.03% | 96% | 94% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.13% | 0.05% | 99% | 98% |
| Class A2 | 0.05% | 0.08% | 0.12% | 100% | 99% |
| Class B | 0.08% | 0.29% | 0.25% | 101% | 100% |
| Class C | 0.08% | 0.01% | 0.22% | 98% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.09% | 0.18% | 99% | 99% |
| Class D | -0.01% | -0.17% | -0.25% | 99% | 99% |
| Class F (optional) | 0.06% | 0.12% | 0.11% | 97% | 97% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.04% | 0.37% | 100% | 102% |
| Class C | 0.08% | 0.12% | 0.22% | 99% | 100% |
| Class E | 0.03% | 0.57% | -0.15% | 97% | 96% |
| Overall | 0.04% | 0.20% | 0.19% | 99% | 100% |
| Class D | 0.08% | -0.13% | 0.42% | 99% | 99% |
| Class F (optional) | 0.06% | 0.33% | -0.43% | 97% | 95% |

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.15% | 0.09% | 98% | 98% |
| Class C | 0.02% | 0.09% | -0.01% | 99% | 98% |
| Class E | 0.01% | 0.35% | -0.05% | 98% | 102% |
| Overall | 0.01% | 0.18% | 0.02% | 98% | 99% |
| Class D | 0.04% | -0.48% | 0.78% | 99% | 100% |
| Class F (optional) | 0.11% | -0.10% | -0.74% | 97% | 98% |

### CE6-14d (dimension 16, 32, and 64)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.38% | -0.20% | -0.28% | 94% | 86% |
| Class A2 | 0.18% | 0.21% | 0.21% | 95% | 91% |
| Class B | 0.16% | 0.22% | 0.26% | 96% | 91% |
| Class C | 0.10% | 0.03% | -0.01% | 95% | 92% |
| Class E | 0.07% | 0.09% | 0.15% | 95% | 89% |
| Overall | 0.05% | 0.09% | 0.08% | 95% | 90% |
| Class D | 0.06% | 0.03% | 0.06% | 96% | 92% |
| Class F (optional) | 0.06% | 0.09% | 0.15% | 96% | 92% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.43% | -0.90% | -0.89% | 101% | 99% |
| Class A2 | 0.00% | -0.09% | 0.01% | 100% | 99% |
| Class B | 0.03% | 0.16% | -0.09% | 100% | 98% |
| Class C | 0.07% | 0.12% | 0.25% | 97% | 95% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | -0.11% | -0.14% | 99% | 98% |
| Class D | -0.03% | -0.17% | -0.17% | 97% | 95% |
| Class F (optional) | 0.04% | 0.08% | 0.11% | 96% | 95% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.30% | -0.21% | 101% | 99% |
| Class C | 0.04% | 0.00% | 0.33% | 97% | 96% |
| Class E | -0.01% | 0.39% | -0.05% | 96% | 93% |
| Overall | 0.01% | -0.03% | 0.01% | 98% | 96% |
| Class D | 0.09% | -0.30% | 0.30% | 98% | 95% |
| Class F (optional) | 0.06% | -0.41% | -0.28% | 97% | 95% |

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | 0.19% | -0.06% | 100% | 99% |
| Class C | 0.00% | -0.06% | 0.02% | 98% | 95% |
| Class E | -0.06% | 0.41% | -0.47% | 97% | 99% |
| Overall | -0.03% | 0.16% | -0.14% | 98% | 98% |
| Class D | 0.04% | -0.25% | 1.13% | 98% | 96% |
| Class F (optional) | 0.09% | -0.24% | -0.07% | 98% | 94% |

## CTC and inter MTS enabled

### CE6-14a (dimension 32)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.04% | -0.04% | 97% | 90% | -0.14% | -0.04% | -0.04% | 97% | 90% |
| Class A2 | 0.08% | 0.23% | 0.14% | 98% | 94% | 0.08% | 0.23% | 0.14% | 98% | 94% |
| Class B | 0.04% | 0.14% | 0.11% | 99% | 95% | 0.04% | 0.14% | 0.11% | 99% | 95% |
| Class C | 0.01% | 0.01% | -0.07% | 98% | 97% | 0.01% | 0.01% | -0.07% | 98% | 97% |
| Class E | 0.00% | -0.01% | 0.13% | 97% | 94% | 0.00% | -0.01% | 0.13% | 97% | 94% |
| Overall | 0.00% | 0.07% | 0.05% | 98% | 94% | 0.00% | 0.07% | 0.05% | 98% | 94% |
| Class D | 0.02% | 0.02% | -0.01% | 98% | 98% | 0.02% | 0.02% | -0.01% | 98% | 98% |
| Class F | 0.01% | 0.07% | 0.02% | 98% | 98% | 0.01% | 0.07% | 0.02% | 98% | 98% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.27% | 0.49% | 0.59% | 114% | 100% | -0.06% | 0.04% | 0.10% | 97% | 97% |
| Class A2 | -0.57% | 0.59% | 0.73% | 118% | 101% | 0.03% | 0.04% | 0.20% | 99% | 100% |
| Class B | -0.40% | 0.91% | 0.63% | 119% | 101% | 0.02% | 0.24% | 0.11% | 98% | 98% |
| Class C | -0.25% | 0.59% | 0.86% | 122% | 99% | 0.00% | 0.09% | 0.20% | 97% | 97% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.37% | 0.67% | 0.70% | 119% | 100% | 0.00% | 0.12% | 0.15% | 98% | 98% |
| Class D | -0.34% | 0.33% | 0.47% | 125% | 101% | 0.06% | -0.24% | -0.04% | 99% | 100% |
| Class F | -0.04% | 0.34% | 0.35% | 113% | 102% | 0.04% | -0.02% | 0.08% | 96% | 101% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.67% | 0.69% | 0.97% | 128% | 102% | 0.09% | 0.22% | -0.02% | 97% | 96% |
| Class C | -0.48% | 1.04% | 1.20% | 132% | 100% | 0.04% | -0.18% | 0.12% | 96% | 97% |
| Class E | -0.20% | 1.61% | 1.16% | 113% | 96% | 0.02% | 0.38% | -0.14% | 95% | 93% |
| Overall | -0.49% | 1.04% | 1.09% | 125% | 100% | 0.05% | 0.13% | -0.01% | 97% | 96% |
| Class D | -0.52% | 1.35% | 1.46% | 131% | 102% | 0.10% | 0.27% | 0.08% | 97% | 98% |
| Class F | -0.19% | 0.75% | 0.88% | 120% | 98% | -0.01% | 0.02% | 0.15% | 96% | 97% |

|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.71% | 0.96% | 1.15% | 136% | 100% | 0.09% | -0.20% | -0.12% | 96% | 92% |
| Class C | -0.46% | 1.14% | 1.02% | 141% | 102% | 0.08% | 0.11% | -0.02% | 96% | 97% |
| Class E | -0.33% | 1.52% | 0.82% | 122% | 101% | -0.02% | 0.72% | -0.29% | 96% | 97% |
| Overall | -0.53% | 1.16% | 1.03% | 134% | 101% | 0.06% | 0.13% | -0.13% | 96% | 95% |
| Class D | -0.61% | 1.45% | 1.31% | 142% | 103% | 0.06% | 0.22% | 0.72% | 99% | 100% |
| Class F | -0.18% | 0.34% | 0.12% | 129% | 100% | 0.19% | 0.31% | -0.25% | 96% | 96% |

### CE6-14b (dimensions 32 and 64)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.45% | -0.29% | -0.26% | 98% | 90% | -0.45% | -0.29% | -0.26% | 98% | 90% |
| Class A2 | 0.02% | 0.12% | 0.14% | 98% | 94% | 0.02% | 0.12% | 0.14% | 98% | 94% |
| Class B | -0.01% | 0.09% | 0.13% | 99% | 97% | -0.01% | 0.09% | 0.13% | 99% | 97% |
| Class C | 0.02% | 0.02% | -0.10% | 98% | 95% | 0.02% | 0.02% | -0.10% | 98% | 95% |
| Class E | -0.02% | 0.03% | 0.02% | 98% | 91% | -0.02% | 0.03% | 0.02% | 98% | 91% |
| Overall | -0.07% | 0.01% | 0.00% | 98% | 94% | -0.07% | 0.01% | 0.00% | 98% | 94% |
| Class D | 0.02% | 0.01% | 0.00% | 98% | 96% | 0.02% | 0.01% | 0.00% | 98% | 96% |
| Class F | 0.00% | -0.03% | -0.01% | 98% | 95% | 0.00% | -0.03% | -0.01% | 98% | 95% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.75% | -0.06% | 0.06% | 123% | 100% | -0.55% | -0.50% | -0.44% | 104% | 97% |
| Class A2 | -0.69% | 0.53% | 0.64% | 123% | 100% | -0.09% | -0.02% | 0.11% | 103% | 99% |
| Class B | -0.50% | 0.74% | 0.26% | 125% | 101% | -0.08% | 0.07% | -0.26% | 103% | 98% |
| Class C | -0.24% | 0.58% | 0.67% | 126% | 96% | 0.01% | 0.08% | 0.01% | 99% | 94% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.52% | 0.49% | 0.41% | 124% | 99% | -0.15% | -0.06% | -0.15% | 102% | 97% |
| Class D | -0.35% | 0.33% | 0.56% | 125% | 97% | 0.05% | -0.25% | 0.06% | 100% | 96% |
| Class F | -0.10% | 0.35% | 0.34% | 117% | 95% | -0.03% | -0.01% | 0.07% | 99% | 94% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.81% | 0.78% | 1.04% | 137% | 103% | -0.05% | 0.32% | 0.04% | 104% | 96% |
| Class C | -0.44% | 1.12% | 1.40% | 136% | 100% | 0.07% | -0.09% | 0.32% | 100% | 97% |
| Class E | -0.25% | 1.73% | 1.16% | 119% | 94% | -0.03% | 0.49% | -0.14% | 100% | 92% |
| Overall | -0.55% | 1.13% | 1.19% | 132% | 100% | -0.01% | 0.22% | 0.09% | 102% | 95% |
| Class D | -0.58% | 1.65% | 1.24% | 132% | 97% | 0.04% | 0.57% | -0.14% | 98% | 93% |
| Class F | -0.41% | 0.70% | 0.43% | 126% | 94% | -0.23% | -0.03% | -0.30% | 100% | 92% |

|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.87% | 1.01% | 1.23% | 147% | 101% | -0.07% | -0.14% | -0.04% | 104% | 94% |
| Class C | -0.50% | 1.06% | 1.23% | 146% | 98% | 0.04% | 0.03% | 0.19% | 100% | 93% |
| Class E | -0.43% | 1.79% | 0.54% | 131% | 99% | -0.12% | 0.99% | -0.58% | 103% | 95% |
| Overall | -0.64% | 1.23% | 1.06% | 143% | 99% | -0.04% | 0.20% | -0.10% | 102% | 94% |
| Class D | -0.58% | 0.89% | 1.69% | 142% | 98% | 0.09% | -0.32% | 1.11% | 99% | 94% |
| Class F | -0.17% | 0.56% | 0.17% | 137% | 98% | 0.20% | 0.53% | -0.17% | 102% | 94% |

### CE6-14c (dimension 16 and 32)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | 0.02% | 0.08% | 93% | 88% | -0.07% | 0.02% | 0.08% | 93% | 88% |
| Class A2 | 0.25% | 0.26% | 0.26% | 95% | 91% | 0.25% | 0.26% | 0.26% | 95% | 91% |
| Class B | 0.20% | 0.32% | 0.23% | 96% | 91% | 0.20% | 0.32% | 0.23% | 96% | 91% |
| Class C | 0.11% | 0.11% | 0.01% | 96% | 93% | 0.11% | 0.11% | 0.01% | 96% | 93% |
| Class E | 0.08% | 0.07% | 0.18% | 95% | 90% | 0.08% | 0.07% | 0.18% | 95% | 90% |
| Overall | 0.12% | 0.17% | 0.15% | 95% | 91% | 0.12% | 0.17% | 0.15% | 95% | 91% |
| Class D | 0.07% | 0.07% | -0.02% | 96% | 96% | 0.07% | 0.07% | -0.02% | 96% | 96% |
| Class F | 0.08% | 0.08% | 0.03% | 96% | 94% | 0.08% | 0.08% | 0.03% | 96% | 94% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.27% | 0.33% | 0.58% | 112% | 99% | -0.06% | -0.12% | 0.08% | 95% | 95% |
| Class A2 | -0.46% | 0.53% | 0.58% | 115% | 100% | 0.15% | -0.01% | 0.05% | 96% | 98% |
| Class B | -0.29% | 0.82% | 0.84% | 118% | 99% | 0.13% | 0.16% | 0.32% | 98% | 97% |
| Class C | -0.13% | 0.71% | 0.96% | 120% | 98% | 0.12% | 0.22% | 0.29% | 95% | 95% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.28% | 0.64% | 0.77% | 117% | 99% | 0.09% | 0.08% | 0.21% | 96% | 96% |
| Class D | -0.30% | 0.58% | 0.48% | 122% | 99% | 0.11% | 0.00% | -0.03% | 97% | 98% |
| Class F | 0.01% | 0.42% | 0.43% | 112% | 97% | 0.08% | 0.06% | 0.15% | 95% | 97% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.56% | 0.81% | 0.95% | 125% | 103% | 0.20% | 0.35% | -0.04% | 95% | 97% |
| Class C | -0.38% | 0.87% | 1.20% | 129% | 98% | 0.13% | -0.34% | 0.12% | 94% | 95% |
| Class E | -0.10% | 1.62% | 0.92% | 112% | 95% | 0.12% | 0.39% | -0.37% | 94% | 92% |
| Overall | -0.39% | 1.03% | 1.03% | 123% | 99% | 0.16% | 0.13% | -0.07% | 95% | 95% |
| Class D | -0.47% | 0.95% | 1.69% | 129% | 100% | 0.15% | -0.12% | 0.30% | 96% | 96% |
| Class F | -0.15% | 0.82% | 0.79% | 119% | 96% | 0.03% | 0.07% | 0.05% | 95% | 95% |

|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.62% | 1.02% | 1.31% | 132% | 99% | 0.18% | -0.14% | 0.03% | 93% | 92% |
| Class C | -0.39% | 1.17% | 1.17% | 138% | 99% | 0.16% | 0.14% | 0.12% | 94% | 95% |
| Class E | -0.23% | 2.03% | 0.45% | 120% | 102% | 0.08% | 1.23% | -0.68% | 94% | 98% |
| Overall | -0.44% | 1.32% | 1.05% | 131% | 100% | 0.15% | 0.30% | -0.12% | 94% | 94% |
| Class D | -0.52% | 0.53% | 1.37% | 138% | 98% | 0.15% | -0.68% | 0.79% | 96% | 95% |
| Class F | -0.18% | 0.08% | 0.21% | 126% | 100% | 0.19% | 0.05% | -0.13% | 94% | 95% |

### CE6-14d (dimension 16, 32, and 64)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.38% | -0.20% | -0.28% | 94% | 86% | -0.38% | -0.20% | -0.28% | 94% | 86% |
| Class A2 | 0.18% | 0.21% | 0.21% | 95% | 91% | 0.18% | 0.21% | 0.21% | 95% | 91% |
| Class B | 0.16% | 0.22% | 0.26% | 96% | 91% | 0.16% | 0.22% | 0.26% | 96% | 91% |
| Class C | 0.10% | 0.03% | -0.01% | 95% | 92% | 0.10% | 0.03% | -0.01% | 95% | 92% |
| Class E | 0.07% | 0.09% | 0.15% | 95% | 89% | 0.07% | 0.09% | 0.15% | 95% | 89% |
| Overall | 0.05% | 0.09% | 0.08% | 95% | 90% | 0.05% | 0.09% | 0.08% | 95% | 90% |
| Class D | 0.06% | 0.03% | 0.06% | 96% | 92% | 0.06% | 0.03% | 0.06% | 96% | 92% |
| Class F | 0.06% | 0.09% | 0.15% | 96% | 92% | 0.06% | 0.09% | 0.15% | 96% | 92% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.76% | -0.02% | -0.08% | 120% | 99% | -0.56% | -0.47% | -0.58% | 102% | 95% |
| Class A2 | -0.61% | 0.51% | 0.61% | 121% | 99% | 0.00% | -0.04% | 0.08% | 102% | 97% |
| Class B | -0.39% | 0.68% | 0.63% | 122% | 99% | 0.03% | 0.01% | 0.11% | 101% | 97% |
| Class C | -0.13% | 0.72% | 0.95% | 124% | 95% | 0.12% | 0.22% | 0.29% | 98% | 93% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.44% | 0.51% | 0.57% | 122% | 98% | -0.07% | -0.04% | 0.01% | 101% | 95% |
| Class D | -0.30% | 0.52% | 0.42% | 123% | 96% | 0.10% | -0.05% | -0.09% | 98% | 95% |
| Class F | -0.05% | 0.42% | 0.43% | 115% | 95% | 0.03% | 0.06% | 0.16% | 98% | 94% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.70% | 0.73% | 0.82% | 134% | 101% | 0.05% | 0.27% | -0.17% | 102% | 95% |
| Class C | -0.33% | 1.08% | 1.47% | 134% | 96% | 0.19% | -0.14% | 0.39% | 98% | 93% |
| Class E | -0.22% | 1.68% | 1.34% | 118% | 93% | 0.01% | 0.45% | 0.04% | 99% | 90% |
| Overall | -0.46% | 1.09% | 1.17% | 130% | 98% | 0.09% | 0.18% | 0.07% | 100% | 93% |
| Class D | -0.47% | 0.76% | 1.44% | 131% | 96% | 0.15% | -0.30% | 0.06% | 97% | 93% |
| Class F | -0.22% | 0.55% | 0.52% | 124% | 95% | -0.04% | -0.19% | -0.22% | 99% | 93% |

|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.79% | 1.05% | 1.08% | 143% | 99% | 0.01% | -0.11% | -0.20% | 101% | 92% |
| Class C | -0.41% | 0.91% | 1.02% | 143% | 95% | 0.14% | -0.12% | -0.02% | 98% | 91% |
| Class E | -0.37% | 1.78% | 1.04% | 128% | 100% | -0.06% | 0.98% | -0.08% | 101% | 96% |
| Overall | -0.56% | 1.19% | 1.05% | 139% | 98% | 0.04% | 0.16% | -0.11% | 100% | 92% |
| Class D | -0.57% | 0.40% | 1.50% | 139% | 96% | 0.10% | -0.80% | 0.92% | 97% | 92% |
| Class F | -0.32% | 0.24% | 0.52% | 134% | 97% | 0.05% | 0.21% | 0.20% | 100% | 93% |

## Low QP (inter MTS disabled)

Note: time measurement for low QP values are not accurate.

### CE6-14a (dimension 32)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.01% | 86% | 100% |
| Class A2 | 0.16% | 0.20% | 0.20% | 84% | 99% |
| Class B | 0.01% | 0.01% | 0.01% | 89% | 102% |
| Class C | 0.01% | 0.00% | 0.00% | 87% | 102% |
| Class E | 0.00% | 0.05% | 0.05% | 87% | 100% |
| Overall | 0.03% | 0.04% | 0.05% | 87% | 101% |
| Class D | 0.00% | 0.02% | 0.02% | 84% | 100% |
| Class F (optional) | 0.01% | 0.01% | -0.02% | 90% | 101% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.11% | 0.09% | 96% | 104% |
| Class A2 | 0.03% | 0.04% | 0.04% | 95% | 103% |
| Class B | 0.01% | 0.02% | 0.04% | 92% | 103% |
| Class C | 0.01% | 0.01% | 0.01% | 97% | 106% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.04% | 0.04% | 95% | 104% |
| Class D | 0.00% | 0.03% | -0.01% | 91% | 101% |
| Class F (optional) | -0.08% | -0.14% | -0.05% | 89% | 103% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.03% | 0.04% | 85% | 97% |
| Class C | 0.00% | -0.01% | -0.03% | 100% | 107% |
| Class E | 0.01% | 0.00% | -0.01% | 89% | 104% |
| Overall | 0.00% | 0.01% | 0.01% | 91% | 102% |
| Class D | 0.01% | -0.03% | -0.06% | 98% | 105% |
| Class F (optional) | -0.07% | -0.01% | -0.04% | 107% | 118% |

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.02% | 0.02% | 95% | 109% |
| Class C | 0.00% | -0.02% | -0.03% | 97% | 103% |
| Class E | 0.01% | 0.02% | 0.01% | 99% | 102% |
| Overall | 0.01% | 0.01% | 0.00% | 97% | 105% |
| Class D | 0.00% | 0.04% | -0.02% | 89% | 99% |
| Class F (optional) | -0.10% | -0.12% | -0.02% | 95% | 105% |

### CE6-14b (dimensions 32 and 64)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.02% | 85% | 98% |
| Class A2 | 0.15% | 0.19% | 0.20% | 84% | 100% |
| Class B | 0.01% | 0.01% | 0.01% | 90% | 106% |
| Class C | 0.01% | 0.00% | 0.00% | 86% | 102% |
| Class E | 0.00% | 0.05% | 0.04% | 88% | 99% |
| Overall | 0.03% | 0.04% | 0.05% | 87% | 102% |
| Class D | 0.00% | 0.02% | 0.02% | 86% | 101% |
| Class F (optional) | 0.01% | 0.01% | -0.02% | 92% | 102% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.10% | 0.06% | 97% | 106% |
| Class A2 | 0.03% | 0.04% | 0.04% | 96% | 105% |
| Class B | 0.01% | 0.03% | 0.08% | 93% | 103% |
| Class C | 0.01% | 0.01% | 0.01% | 98% | 106% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.04% | 0.05% | 95% | 105% |
| Class D | 0.00% | 0.02% | 0.00% | 93% | 101% |
| Class F (optional) | -0.04% | -0.08% | -0.02% | 89% | 102% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.02% | 0.04% | 84% | 97% |
| Class C | 0.00% | -0.01% | -0.03% | 101% | 106% |
| Class E | 0.00% | 0.02% | 0.01% | 91% | 100% |
| Overall | 0.00% | 0.01% | 0.01% | 91% | 101% |
| Class D | 0.01% | -0.03% | -0.06% | 99% | 107% |
| Class F (optional) | -0.09% | -0.01% | -0.11% | 108% | 121% |

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.02% | 0.02% | 95% | 106% |
| Class C | 0.01% | -0.01% | -0.03% | 97% | 103% |
| Class E | 0.00% | 0.02% | 0.02% | 98% | 103% |
| Overall | 0.01% | 0.01% | 0.00% | 97% | 104% |
| Class D | 0.00% | 0.04% | -0.02% | 89% | 97% |
| Class F (optional) | -0.08% | -0.15% | -0.05% | 96% | 105% |

### CE6-14c (dimension 16 and 32)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.09% | 0.11% | 83% | 100% |
| Class A2 | 0.43% | 0.44% | 0.44% | 81% | 98% |
| Class B | 0.08% | 0.16% | 0.17% | 89% | 102% |
| Class C | 0.04% | 0.08% | 0.06% | 85% | 102% |
| Class E | 0.11% | 0.19% | 0.20% | 86% | 98% |
| Overall | 0.13% | 0.18% | 0.18% | 85% | 100% |
| Class D | 0.03% | 0.06% | 0.04% | 82% | 105% |
| Class F (optional) | 0.04% | 0.09% | 0.05% | 89% | 101% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.25% | 0.24% | 95% | 104% |
| Class A2 | 0.09% | 0.09% | 0.08% | 94% | 103% |
| Class B | 0.05% | 0.06% | 0.09% | 92% | 101% |
| Class C | 0.01% | 0.02% | 0.03% | 98% | 106% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.09% | 0.10% | 94% | 103% |
| Class D | 0.02% | 0.10% | -0.04% | 91% | 101% |
| Class F (optional) | -0.06% | -0.09% | -0.09% | 88% | 102% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.04% | 0.06% | 84% | 97% |
| Class C | 0.01% | 0.04% | -0.02% | 100% | 107% |
| Class E | 0.02% | 0.07% | 0.07% | 90% | 102% |
| Overall | 0.02% | 0.05% | 0.04% | 90% | 101% |
| Class D | 0.00% | -0.03% | 0.02% | 98% | 106% |
| Class F (optional) | -0.02% | -0.03% | -0.03% | 107% | 122% |

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | 0.05% | 0.05% | 93% | 105% |
| Class C | 0.02% | 0.00% | -0.01% | 96% | 104% |
| Class E | 0.05% | 0.07% | 0.08% | 97% | 103% |
| Overall | 0.04% | 0.04% | 0.04% | 95% | 104% |
| Class D | 0.02% | 0.00% | -0.02% | 89% | 98% |
| Class F (optional) | -0.09% | -0.07% | 0.02% | 94% | 107% |

### CE6-14d (dimension 16, 32, and 64)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.09% | 0.11% | 83% | 98% |
| Class A2 | 0.43% | 0.43% | 0.44% | 82% | 98% |
| Class B | 0.08% | 0.15% | 0.17% | 88% | 103% |
| Class C | 0.04% | 0.08% | 0.06% | 86% | 102% |
| Class E | 0.11% | 0.19% | 0.20% | 85% | 101% |
| Overall | 0.13% | 0.18% | 0.18% | 85% | 101% |
| Class D | 0.03% | 0.06% | 0.04% | 84% | 102% |
| Class F (optional) | 0.04% | 0.10% | 0.05% | 90% | 101% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.21% | 0.21% | 95% | 103% |
| Class A2 | 0.09% | 0.08% | 0.08% | 95% | 104% |
| Class B | 0.05% | 0.07% | 0.08% | 92% | 102% |
| Class C | 0.01% | 0.02% | 0.03% | 97% | 107% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.09% | 0.09% | 95% | 104% |
| Class D | 0.01% | 0.08% | -0.03% | 92% | 101% |
| Class F (optional) | -0.08% | -0.11% | -0.07% | 88% | 102% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.03% | 0.06% | 84% | 97% |
| Class C | 0.01% | 0.05% | -0.02% | 101% | 106% |
| Class E | 0.02% | 0.05% | 0.07% | 89% | 100% |
| Overall | 0.02% | 0.04% | 0.03% | 90% | 101% |
| Class D | 0.00% | -0.03% | 0.02% | 100% | 106% |
| Class F (optional) | -0.16% | -0.09% | -0.03% | 107% | 115% |

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | 0.05% | 0.07% | 95% | 106% |
| Class C | 0.02% | 0.00% | -0.01% | 97% | 104% |
| Class E | 0.04% | 0.07% | 0.07% | 99% | 103% |
| Overall | 0.04% | 0.04% | 0.04% | 96% | 105% |
| Class D | 0.02% | 0.00% | -0.02% | 90% | 100% |
| Class F (optional) | -0.11% | -0.11% | 0.04% | 95% | 107% |

## Low QP (inter MTS enabled)

Note: time measurement for low QP values are not accurate.

### CE6-14a (dimension 32)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.01% | 85% | 103% | 0.01% | 0.00% | 0.01% | 91% | 104% |
| Class A2 | 0.16% | 0.20% | 0.20% | 82% | 99% | 0.16% | 0.20% | 0.20% | 92% | 99% |
| Class B | 0.01% | 0.01% | 0.01% | 87% | 101% | 0.01% | 0.01% | 0.01% | 92% | 97% |
| Class C | 0.01% | 0.00% | 0.00% | 86% | 102% | 0.01% | 0.00% | 0.00% | 93% | 98% |
| Class E | 0.00% | 0.05% | 0.05% | 88% | 100% | 0.00% | 0.05% | 0.05% | 93% | 95% |
| Overall | 0.03% | 0.04% | 0.05% | 86% | 101% | 0.03% | 0.04% | 0.05% | 93% | 99% |
| Class D | 0.00% | 0.02% | 0.02% | 84% | 99% | 0.00% | 0.02% | 0.02% | 95% | 97% |
| Class F | 0.01% | 0.01% | -0.02% | 90% | 101% | 0.01% | 0.01% | -0.02% | 93% | 98% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.44% | 0.47% | 0.54% | 144% | 110% | 0.01% | 0.11% | 0.10% | 98% | 101% |
| Class A2 | -0.71% | -0.02% | -0.02% | 153% | 110% | 0.07% | 0.08% | 0.08% | 95% | 99% |
| Class B | -0.68% | 0.05% | 0.12% | 155% | 107% | 0.01% | 0.08% | 0.11% | 94% | 97% |
| Class C | -0.95% | 0.02% | -0.01% | 171% | 109% | 0.02% | 0.04% | 0.01% | 95% | 98% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.71% | 0.11% | 0.14% | 156% | 109% | 0.03% | 0.07% | 0.07% | 95% | 98% |
| Class D | -1.25% | -0.13% | -0.19% | 160% | 104% | 0.02% | -0.04% | -0.06% | 91% | 93% |
| Class F | -0.68% | -0.12% | -0.07% | 124% | 101% | -0.04% | -0.08% | -0.03% | 89% | 95% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.72% | -0.01% | 0.06% | 151% | 101% | 0.00% | 0.07% | 0.07% | 90% | 97% |
| Class C | -1.09% | -0.09% | -0.11% | 185% | 111% | 0.01% | 0.00% | 0.01% | 92% | 97% |
| Class E | -1.54% | -0.02% | -0.08% | 151% | 112% | 0.05% | 0.10% | 0.08% | 91% | 97% |
| Overall | -1.05% | -0.04% | -0.03% | 161% | 107% | 0.02% | 0.05% | 0.05% | 91% | 97% |
| Class D | -1.39% | -0.16% | -0.13% | 179% | 109% | 0.01% | -0.02% | 0.02% | 93% | 95% |
| Class F) | -0.92% | -0.07% | -0.05% | 162% | 120% | -0.05% | -0.10% | 0.02% | 93% | 96% |

|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.68% | -0.02% | 0.00% | 171% | 111% | 0.00% | 0.05% | 0.03% | 89% | 97% |
| Class C | -1.08% | -0.11% | -0.14% | 185% | 106% | 0.01% | 0.01% | 0.00% | 90% | 101% |
| Class E | -1.42% | -0.06% | -0.08% | 172% | 111% | 0.04% | 0.08% | 0.06% | 92% | 99% |
| Overall | -1.00% | -0.06% | -0.07% | 176% | 109% | 0.01% | 0.05% | 0.03% | 90% | 98% |
| Class D | -1.36% | -0.07% | -0.14% | 174% | 98% | 0.00% | 0.00% | -0.01% | 91% | 97% |
| Class F | -0.92% | -0.08% | 0.03% | 151% | 107% | 0.05% | 0.02% | 0.12% | 95% | 107% |

### CE6-14b (dimensions 32 and 64)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.02% | 88% | 101% | 0.01% | 0.00% | 0.02% | 94% | 102% |
| Class A2 | 0.15% | 0.19% | 0.20% | 85% | 98% | 0.15% | 0.19% | 0.20% | 94% | 99% |
| Class B | 0.01% | 0.01% | 0.01% | 89% | 102% | 0.01% | 0.01% | 0.01% | 94% | 98% |
| Class C | 0.01% | 0.00% | 0.00% | 88% | 104% | 0.01% | 0.00% | 0.00% | 96% | 100% |
| Class E | 0.00% | 0.05% | 0.04% | 89% | 101% | 0.00% | 0.05% | 0.04% | 94% | 95% |
| Overall | 0.03% | 0.04% | 0.05% | 88% | 101% | 0.03% | 0.04% | 0.05% | 95% | 99% |
| Class D | 0.00% | 0.02% | 0.02% | 85% | 101% | 0.00% | 0.02% | 0.02% | 96% | 99% |
| Class F | 0.01% | 0.01% | -0.02% | 89% | 100% | 0.01% | 0.01% | -0.02% | 92% | 97% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.45% | 0.43% | 0.51% | 148% | 111% | 0.01% | 0.08% | 0.07% | 101% | 102% |
| Class A2 | -0.73% | -0.02% | -0.03% | 160% | 111% | 0.06% | 0.07% | 0.07% | 99% | 100% |
| Class B | -0.69% | 0.05% | 0.11% | 157% | 106% | 0.00% | 0.07% | 0.09% | 95% | 97% |
| Class C | -0.95% | 0.02% | -0.01% | 176% | 111% | 0.02% | 0.04% | 0.01% | 98% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.72% | 0.10% | 0.13% | 161% | 109% | 0.02% | 0.07% | 0.06% | 98% | 99% |
| Class D | -1.26% | -0.15% | -0.18% | 164% | 105% | 0.01% | -0.06% | -0.05% | 93% | 94% |
| Class F | -0.61% | -0.04% | -0.05% | 133% | 105% | 0.03% | 0.00% | -0.01% | 96% | 99% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.72% | 0.00% | 0.06% | 153% | 101% | 0.00% | 0.08% | 0.07% | 92% | 97% |
| Class C | -1.09% | -0.10% | -0.12% | 190% | 112% | 0.01% | 0.00% | 0.00% | 95% | 97% |
| Class E | -1.56% | 0.00% | -0.06% | 156% | 110% | 0.03% | 0.12% | 0.10% | 94% | 95% |
| Overall | -1.05% | -0.03% | -0.03% | 165% | 107% | 0.01% | 0.06% | 0.05% | 93% | 97% |
| Class D | -1.39% | -0.14% | -0.17% | 185% | 111% | 0.01% | 0.00% | -0.02% | 96% | 97% |
| Class F | -0.83% | 0.02% | 0.01% | 169% | 126% | 0.04% | -0.01% | 0.08% | 97% | 102% |

|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.68% | -0.02% | 0.01% | 172% | 109% | 0.00% | 0.05% | 0.04% | 89% | 95% |
| Class C | -1.08% | -0.11% | -0.13% | 187% | 110% | 0.00% | 0.01% | 0.01% | 91% | 105% |
| Class E | -1.43% | -0.07% | -0.10% | 176% | 109% | 0.04% | 0.06% | 0.05% | 94% | 97% |
| Overall | -1.00% | -0.06% | -0.06% | 178% | 109% | 0.01% | 0.04% | 0.03% | 91% | 99% |
| Class D | -1.33% | -0.05% | -0.13% | 183% | 106% | 0.03% | 0.02% | 0.01% | 96% | 105% |
| Class F | -0.98% | -0.14% | -0.09% | 160% | 107% | 0.00% | -0.05% | 0.00% | 100% | 108% |

### CE6-14c (dimension 16 and 32)

### CE6-14d (dimension 16, 32, and 64)

# Patent rights declaration

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).