| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0538-v2 |
| --- | --- |

| Title: | CE6-1.4: Efficient Implementations of MTS with Transform Adjustments |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Amir Said, Hilmi E. Egilmez, Yung-Hsuan Chao, Vadim Seregin, Marta Karczewicz 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Email: | asaid@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports results of CE6-1.4, where a unified transformation approach is proposed to replace the large sized DCT-8 and DST-7 used as primary transforms in MTS of VTM-3.0. It describes modifications and improvements made in response to concerns and suggestions mentioned during a BoG discussion on primary transforms. The new method addresses all important issues, including using only the inverse DCT-2 at the decoder, and preserving low arithmetic bit precision. In addition, it now requires a lower number of arithmetic operations, and its total memory (beyond required for DCT-2) for computing transforms of dimension 16, 32, and 64, is only 64 bytes. VTM-3.0-based throughput measurements show the advantage over other proposals, and a SIMD implementation is provided as an example of how the simple computational structure can be easily and efficiently optimized in software and hardware. Simulation results show that under CTC, for MTS transforms of dimension 32 the B-D rates are 0.00%, -0.01%, -0.02%, and -0.04% for AI, RA, LB, and LP tests. Combining MTS transforms of dimension 32 and 64 yield (without increase on encoding or decoding times) gains of about 0.45% for class A2, and averages of -0.07%, -0.12%, 0.01%, and -0.01% for AI, RA, LB, and LP tests.

# Introduction

Transform “adjustments” were first proposed in JVET-J0066 as a method for replacing computationally costly trigonometric transforms, like the DST-7, and DCT-8, with approximations that could

- Provide the codec with the same type of transform diversity as that provided by the trigonometric transforms, and thus yield practically the same coding gains;

- Be computed using only an “adjustment” of very low complexity, plus the DCT-2 as the “main computational engine,” since it is the transform already used in HEVC, has been widely studied and optimized, and enables re-using previously developed hardware and software implementations.

The method defined in JVET-L0386 used adjustment filters before the DCT-3 (inverse DCT-2) at the forward transform. At the primary transform BoG discussions during the Macao meeting [JVET-L0685], this was considered a problem, since it would require implementing the forward DCT-2 at decoders, while currently some implementations only have the inverse DCT-2 at the decoder.

To address this problem a new implementation is proposed, as shown in Fig. 1, where the adjustments are computed after the forward DCT-2, and their inverse before the inverse DCT-2. In this new version those adjustments correspond to simply multiplying the vector with the 8 lowest-frequency transform coefficients by an 8×8 matrix. The same matrix is used for transforms of dimension 16, 32, and 64, and for the DST-7 and DCT-8 adjustments.

Fig. 1 – New implementation of transform adjustment that needs only the inverse DCT-2 at the decoder. For transforms of dimension 16, 32, and 64, the adjustment blocks correspond to multiplying the 8 lowest frequency transform coefficients by the same 8×8 matrix.

The simulation results (Section 7) show that that this alternative approach produces coding results similar or better than the previous proposal, with coding gains very similar to those obtained using the DST-7 and DCT-8 transforms.

More details about the implementation are provided in Section 3, and Section 2 contains a summary of the features provided by this approach and its implementation.

# Summary of features

As mentioned in the introduction, the new implementation addresses several issues discussed in the primary transform BoG [JVET-L0685] as well as CE6 discussions in the previous JVET meeting. The following is a summary.

- With the adoption of 8-bit coefficients for primary transforms in VTM-3, the use of the DCT-2 in this proposal now corresponds to current HEVC implementations, and previously optimized transform SW and HW can be readily re-used.

- No additional HW is needed for supporting the forward DCT at decoder-only implementations.

- Arithmetic precision is maintained:

  - The coefficients of the 8×8 adjustment matrix are also represented with 8 bits.

  - After the adjustment stages the computed values are re-normalized and “clamped” to guarantee that inputs to the inverse DCT-2 are limited to 16-bit integers.

- The band-matrix previously used for adjustment computations is replaced by a single small matrix, which was indicated as the preferred form of computation.

- All computations of the DCT-2 and the adjustments have very regular patterns, greatly simplifying efficient parallel implementation using SIMD instructions or custom HW (a SIMD-based code is shared as example).

- The additional memory (beyond that of used for the DCT-2) needed for computing the MTS transforms of dimensions 16, 32, and 64, is only 64 bytes. This represents a very significant reduction compared to the memory required to store the DST-7/DCT-8 matrices (cf. Section 4).

- Due to the “zero-out” technique used in DCT-2 of dimension 64, the worst-case transform complexity in VTM-3, which defines the custom HW costs, boils down to the matrix-based DST-7 and DCT-8 on blocks of size 32×32. In Section 6 it is shown how the worst-case number of multiplications per transform coefficient is reduced from 64 to 36 (i.e., approximately a 40% reduction).

- A comparison of simulation results in CE6-1.4c and CE6-1.4d show that MTS transforms of dimension 64 now have a reasonable computational complexity, which is practically equal to the DCT-2 transform of dimension 64 (both including “zero-out”).So, additional gains from extending MTS to larger blocks do not increase encoding or decoding times.

- Transform coefficient throughput measurements, based on the throughput profiler proposed in JVET-M0540, show significant improvements, especially at decoder inverse transformations (leading from 2 to 5 times decoder speedups).

- A separate proposal [JVET-M0539] (exploiting special properties of the DCT-2 and the corresponding adjustments for DST-7 and DCT-8,) describes how it is possible to significantly increase the throughput at the encoder side, because all four MTS combinations (DST-7 or DCT-8 for horizontal or vertical passes) can be computed with complexity similar to that of computing a single separable transformation.

# Implementation

For a transform of dimension N = 16, 32, or 64, if c(DCT-2) is the N-dimensional vector with coefficients of the DCT-2 transform (in increasing frequency), we can split this vector as

c(DCT-2) = [  x | y ],

where x has dimension 8 (i.e., the coefficients of the 8 lowest frequencies) and y has dimension N – 8. The transform adjustments corresponding to the DCT-8 and DST-7 MTS options are defined as

ĉ(DCT-8) = [  A(DCT-8) x | y ],

ĉ (DST-7) = [  A(DST-7) x | y ].

where A is an 8×8 matrix (symbol ĉ is used instead of c to indicate that those are not the true DCT-8 and DST-7 coefficients).

Matrix A should be orthogonal and designed to maximize coding gain. In this proposal the matrix used to compute the orthogonal transform associated with the DST-7 is scaled by 128, and defined by the C++ code

const TMatrixCoeff g_TAF_DST7[8][8] =

{

{ 118,-48,-12, -2, -2, -1, -1,  0 },

{  38,108,-54,-15, -8, -6, -3, -2 },

{  24, 36,107,-51,-16, -9, -4, -4 },

{  14, 23, 34,111,-44,-13, -8, -5 },

{  10, 16, 19, 29,114,-38,-14, -8 },

{   8, 13, 13, 14, 26,118,-33,-12 },

{   6,  9,  9, 11, 15, 24,120,-28 },

{   5,  8,  8,  8, 11, 14, 23,124 }

};

while the matrix corresponding to the DCT-8 computation is

const TMatrixCoeff g_TAF_DCT8[8][8] =

{

{ 118, 48,-12,  2, -2,  1, -1,  0 },

{ -38,108, 54,-15,  8, -6,  3, -2 },

{  24,-36,107, 51,-16,  9, -4,  4 },

{ -14, 23,-34,111, 44,-13,  8, -5 },

{  10,-16, 19,-29,114, 38,-14,  8 },

{  -8, 13,-13, 14,-26,118, 33,-12 },

{   6, -9,  9,-11, 15,-24,120, 28 },

{  -5,  8, -8,  8,-11, 14,-23,124 }

};

It can be observed that the elements of the two matrices have same magnitudes, but different signs, according to a chessboard pattern. This is not a coincidence and is a consequence of properties of the DCT-2, and DCT-8 and DST-7. Thus, only one of these matrices need to be stored, and used in the matrix-vector computations (as shown in the code for this CE). For various optimized HW or SW implementations it may be more useful to store both versions, yet it can be represented using a single 8x8 matrix requiring only 64 additional bytes of memory. Since the matrices above are scaled versions of orthogonal matrices, their inverses are defined by simply transposing (or left-side multiplication).

Note that in this proposal the same matrix coefficients are used for transforms of dimensions 16, 32, and 64, and thus the total memory requirement, beyond the DCT-2, is only 64 bytes.

# Complexity measures

## Operation counts for individual transforms

The operation counts for 1D forward/inverse transforms in this CE are shown in Table I, and include all arithmetic operations needed for implementing DCT-2 and adjustments. Note that counts for dimension 64 include the “zero-out” technique used in VTM-3.0, and the DCT-2 is implemented fully using the “partial-butterfly” factorization.

Table I: Operation counts for 1-D forward/inverse transforms

| Size | Number of Multiplications |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 4 | 8 | 16 | 32 | 64 |
| DST-7 | 8 | 64 | 150 | 406 | 747 |
| DCT-8 | 8 | 64 | 150 | 406 | 747 |
| DCT-2 | 5 | 24 | 86 | 342 | 683 |

| Size | Number of Additions |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 4 | 8 | 16 | 32 | 64 |
| DST-7 | 15 | 65 | 177 | 465 | 871 |
| DCT-8 | 15 | 65 | 177 | 465 | 871 |
| DCT-2 | 13 | 37 | 113 | 401 | 807 |

| Size | Number of Shifts |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 4 | 8 | 16 | 32 | 64 |
| DST-7 | 5 | 9 | 25 | 41 | 41 |
| DCT-8 | 5 | 9 | 25 | 41 | 41 |
| DCT-2 | 5 | 9 | 17 | 33 | 33 |

## Memory requirements for storing transform cores

The amounts of memory needed for transforms are shown in Table II. The symbol “+” in front of a number is used to indicate that, thanks to the recursive factorization of the DCT-2, the memory used for one dimension is re-used for larger dimensions. For instance, a DCT-2 of dimension 16 needs 64 bytes in addition to the 16 + 6 = 22 bytes required for dimensions 4 and 8, and a DCT-2 of dimension 32 needs 512 bytes in addition to256 + 64 + 16 + 6 = 342 bytes. Similarly, for HW implementations all the DCT-2 transform factors can be fetched from the same matrix stored in ROM.

Table II: Number of bytes for each transform matrix when transform adjustment is used for DST-7/DCT-8 including dimension 16 (i.e., 16, 32 and 64).

| Size | Number of bytes in transform matrices |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 4 | 8 | 16 | 32 | 64 | Total |
| DST-7 + DCT-8 | 16 | 64 | 64 |  |  | 144 |
| DCT-2 (SW) | 6 | +16 | +64 | +256 | +512 | 854 |
| DCT-2 (HW) | 512 |  |  |  | +512 | 1,024 |

Table III: Number of bytes for each transform matrix when transform adjustment is used for DST-7/DCT-8 with dimensions 32 and 64.

| Size | Number of bytes in transform matrices |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 4 | 8 | 16 | 32 | 64 | Total |
| DST-7 + DCT-8 | 16 | 64 | 256 | 64 |  | 400 |
| DCT-2 (SW) | 6 | +16 | +64 | +256 | +512 | 854 |
| DCT-2 (HW) | 512 |  |  |  | +512 | 1,024 |

# Throughput measurements

Using the method of JVET-M0540, the throughput of the proposed method was measured for the DST-7 and DCT-8 transforms, using 32 frames of the sequence “Tango2,” AI configuration, and QP = 32. Results are listed in Table III, which lists throughputs, and Table IV, which shows throughputs ratios, compared to VTM matrix-based computations.

Significant speed-ups can be observed, especially for the inverse transform. The forward transform can also be significantly accelerated using the unique properties of the DCT-2 and its transforms adjustments, using the method proposed in JVET-M0539. Its throughput measurements are also shown in Tables III and IV.

Note that the SIMD results are meant only as examples, to demonstrate that it is very easy to develop SIMD versions of the proposed methods using SSE instructions. They do not represent the most optimized SIMD code, which can use more powerful SIMD instructions, like AVX2.

Table IV: Transform computation throughput (106 coefficients per second)

| Transform method | Encoder |  |  |  |  |  | Decoder |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Forward |  |  | Inverse |  |  | Inverse |  |  |
|  | 16 | 32 | 64 | 16 | 32 | 64 | 16 | 32 | 64 |
| VTM (matrix) | 169 | 125 | 209 | 79 | 49 | 68 | 81 | 48 | 66 |
| CE6-1.4d | 208 | 172 | 334 | 184 | 219 | 338 | 224 | 268 | 377 |
| CE6-1.4d SIMD | 224 | 197 | 373 | 286 | 341 | 744 | 366 | 447 | 870 |
| JVET-M0539 | 414 | 333 | 1162 | 183 | 218 | 338 | 220 | 265 | 385 |
| JVET-M0539 SIMD | 438 | 379 | 1207 | 287 | 341 | 733 | 361 | 432 | 826 |

Table V: Transform computation throughput ratios, VTM matrix-based computation used as reference.

| Transform method | Encoder |  |  |  |  |  | Decoder |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Forward |  |  | Inverse |  |  | Inverse |  |  |
|  | 16 | 32 | 64 | 16 | 32 | 64 | 16 | 32 | 64 |
| CE6-1.4d | 1.23 | 1.38 | 1.60 | 2.32 | 4.48 | 4.99 | 2.77 | 5.55 | 5.74 |
| CE6-1.4d SIMD | 1.32 | 1.57 | 1.78 | 3.62 | 6.98 | 10.98 | 4.54 | 9.24 | 13.24 |
| JVET-M0539 | 2.44 | 2.67 | 5.56 | 2.32 | 4.46 | 4.99 | 2.73 | 5.48 | 5.86 |
| JVET-M0539 SIMD | 2.59 | 3.04 | 5.78 | 3.62 | 6.96 | 10.83 | 4.47 | 8.93 | 12.57 |

# Multiplication counts for hardware implementations

Custom hardware implementations are designed to satisfy some worst-case conditions. For transforms, it is necessary to first define a desired maximum frame rate at a given resolution, which specifies the minimum rate of transform computations, in transform throughputs (coefficients per second). The clock frequency, multiplication latency, number of multiplications per transforms, and coefficient throughputs are then used to define the number of parallel multipliers that are needed to be implemented, to support the frame rate.

Since multipliers are the most expensive elements in terms of silicon area (additions are much simpler and “cheaper”), the worst-case number of multiplications per transform coefficient is one of the most important factors to define the cost of implementing transforms in custom hardware.

This section presents the formulas needed to compute the number of multiplications per transform coefficient of the VVC codec, and the tables with results using DCT-2, matrix and transform adjustment computations.

The factors needed for the computation. for a separable transform in a block of dimensions W×H, are listed in Table V.

Table VI: Factors used for computing the number of multiplications per transform coefficient, separable transformations in blocks of size W×H.

| Pass | Transform lines | Coefficients per line | Multiplications per coefficient | Additional mult. per line |
| --- | --- | --- | --- | --- |
| Horizontal | H | W/Dz(W) | W/Dt(W) | A(W) |
| Vertical | W/Dz(W) | H/Dz(H) | H/Dt(H) | A(H) |

The functions used in Table IV are defined as

DzN=2,zero­out on dimension N,1,otherwise,

DtN=2,DCT­2 or TAF on dimension N,1,otherwise,

AN=64,TAF on dimension N,0,otherwise.

Note that Dt(N) assumes that in a hardware implementation the DCT-2 a first pass of partial-butterflies is used, followed by matrix computations.

Combining those factors results in a number of multiplications per transform coefficient equal to

M=HW2Dz(W)Dt(W)+A(W)+WDz(W)H2Dz(H)Dt(H)+AHWH

which can be simplified to

M=WDz(W)Dt(W)+HDz(W)Dz(H)Dt(H)+A(W)W+AHDz(W)H

The numbers of multiplications per transform coefficient needed for possible VTM-3 matrix-based computations are shown in Table VI, while the number corresponding to DCT-2 computations, with HW implementations using one level of partial-butterfly, are shown in Table VII. Note that those numbers are half of those defined by matrix-based computations.

The values for this proposal are listed in Tables VIII and IX. In all cases, we can observe that the design objectives of the proposed method are satisfied: the number of multiplications pers transform coefficient is only slightly larger than that required for the DCT-2 computations.

One very important fact for a custom hardware implementation is that the worst-case, which mostly defines the HW costs, is reduced from the 64 (32×32 MTS blocks in VTM-3.0) to at most 36. Note that even if the complexity of matrix-based is reduced on 32×32 MTS blocks, the multiplications per transform coefficient for 16×32, 32×16, 8×32, and 32×8 blocks is still be larger than the worst-case of this proposal.

Table VII: Number of multiplications per transform coefficient, separable transformation in blocks of size W×H, matrix-based computation.

| Matrix | No zero-out |  |  |  | Zero-out used when N = 64 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | W = 8 | W = 16 | W = 32 | W = 64 | W = 8 | W = 16 | W = 32 | W = 64 |
| H = 8 | 16 | 24 | 40 | 72 | 16 | 24 | 40 | 36 |
| H = 16 | 24 | 32 | 48 | 80 | 24 | 32 | 48 | 40 |
| H = 32 | 40 | 48 | 64 | 96 | 40 | 48 | 64 | 48 |
| H = 64 | 72 | 80 | 96 | 128 | 40 | 48 | 64 | 48 |

Table VIII: Number of multiplications per transform coefficient, separable transformation in blocks of size W×H, DCT-2 computation with single partial-butterfly implemented.

| DCT-2 | No zero-out |  |  |  | Zero-out used when N = 64 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | W = 8 | W = 16 | W = 32 | W = 64 | W = 8 | W = 16 | W = 32 | W = 64 |
| H = 8 | 8 | 12 | 20 | 36 | 8 | 12 | 20 | 18 |
| H = 16 | 12 | 16 | 24 | 40 | 12 | 16 | 24 | 20 |
| H = 32 | 20 | 24 | 32 | 48 | 20 | 24 | 32 | 24 |
| H = 64 | 36 | 40 | 48 | 64 | 20 | 24 | 32 | 24 |

Table IX: Number of multiplications per transform coefficient, separable transformation in blocks of size W×H, DCT-2 and TAF computation with single partial-butterfly implemented, zero-out on dimension 64.

| TAF | TAF used when N = 32 |  |  |  | TAF used when N = 32, 64 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | W = 8 | W = 16 | W = 32 | W = 64 | W = 8 | W = 16 | W = 32 | W = 64 |
| H = 8 | 16 | 24 | 26 |  | 16 | 24 | 26 | 21 |
| H = 16 | 24 | 32 | 34 |  | 24 | 32 | 34 | 25 |
| H = 32 | 26 | 34 | 36 |  | 26 | 34 | 36 | 26 |
| H = 64 |  |  |  |  | 25 | 33 | 35 | 25 |

Table X: Number of multiplications per transform coefficient, separable transformation in blocks of size W×H, DCT-2 and TAF computation with single partial-butterfly implemented, zero-out on dimension 64.

| TAF | TAF used when N = 16, 32 |  |  |  | TAF used when N = 16, 32, 64 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | W = 8 | W = 16 | W = 32 | W = 64 | W = 8 | W = 16 | W = 32 | W = 64 |
| H = 8 | 16 | 20 | 26 |  | 16 | 20 | 26 | 21 |
| H = 16 | 20 | 24 | 30 |  | 20 | 24 | 30 | 23 |
| H = 32 | 26 | 30 | 36 |  | 26 | 30 | 36 | 26 |
| H = 64 |  |  |  |  | 25 | 29 | 35 | 25 |

# Simulation results

## CTC (inter MTS disabled)

### CE6-14a (dimension 32)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.04% | -0.04% | 97% | 90% |
| Class A2 | 0.08% | 0.23% | 0.14% | 98% | 94% |
| Class B | 0.04% | 0.14% | 0.11% | 99% | 95% |
| Class C | 0.01% | 0.01% | -0.07% | 98% | 97% |
| Class E | 0.00% | -0.01% | 0.13% | 97% | 94% |
| Overall | 0.00% | 0.07% | 0.05% | 98% | 94% |
| Class D | 0.02% | 0.02% | -0.01% | 98% | 98% |
| Class F | 0.01% | 0.07% | 0.02% | 98% | 98% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | 0.02% | 0.09% | 99% | 98% |
| Class A2 | -0.01% | 0.03% | 0.28% | 100% | 99% |
| Class B | -0.01% | 0.21% | -0.01% | 101% | 99% |
| Class C | 0.03% | 0.00% | 0.21% | 98% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.08% | 0.13% | 100% | 99% |
| Class D | 0.02% | -0.36% | -0.11% | 100% | 100% |
| Class F | 0.02% | -0.01% | 0.03% | 98% | 98% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.19% | 0.00% | 100% | 101% |
| Class C | 0.04% | 0.19% | 0.24% | 99% | 99% |
| Class E | -0.12% | 0.44% | 0.14% | 98% | 94% |
| Overall | -0.02% | 0.09% | 0.11% | 99% | 99% |
| Class D | 0.02% | 0.35% | 0.11% | 99% | 101% |
| Class F | -0.08% | -0.05% | 0.08% | 97% | 95% |

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | 0.33% | 0.13% | 100% | 102% |
| Class C | 0.03% | 0.17% | 0.09% | 100% | 100% |
| Class E | -0.17% | 0.56% | -0.26% | 98% | 100% |
| Overall | -0.04% | 0.33% | 0.02% | 99% | 101% |
| Class D | -0.03% | 0.12% | 0.17% | 100% | 101% |
| Class F | -0.02% | -0.27% | -0.47% | 99% | 98% |

### CE6-14b (dimensions 32 and 64)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.45% | -0.29% | -0.26% | 98% | 90% |
| Class A2 | 0.02% | 0.12% | 0.14% | 98% | 94% |
| Class B | -0.01% | 0.09% | 0.13% | 99% | 97% |
| Class C | 0.02% | 0.02% | -0.10% | 98% | 95% |
| Class E | -0.02% | 0.03% | 0.02% | 98% | 91% |
| Overall | -0.07% | 0.01% | 0.00% | 98% | 94% |
| Class D | 0.02% | 0.01% | 0.00% | 98% | 96% |
| Class F | 0.00% | -0.03% | -0.01% | 98% | 95% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.44% | -0.85% | -0.75% | 102% | 99% |
| Class A2 | -0.07% | -0.02% | -0.01% | 101% | 99% |
| Class B | -0.07% | 0.17% | -0.23% | 101% | 99% |
| Class C | 0.01% | 0.00% | 0.05% | 97% | 96% |
| Class E |  |  |  |  |  |
| Overall | -0.12% | -0.12% | -0.22% | 100% | 98% |
| Class D | 0.02% | -0.36% | -0.13% | 98% | 96% |
| Class F | -0.04% | 0.01% | 0.00% | 97% | 95% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.18% | -0.13% | 101% | 102% |
| Class C | 0.04% | 0.20% | 0.35% | 98% | 94% |
| Class E | 0.05% | 1.14% | -0.21% | 96% | 91% |
| Overall | 0.01% | 0.28% | 0.01% | 99% | 97% |
| Class D | 0.04% | 0.29% | 0.69% | 98% | 95% |
| Class F | 0.04% | -0.24% | -0.34% | 97% | 95% |

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | 0.04% | -0.14% | 101% | 97% |
| Class C | 0.01% | 0.21% | 0.16% | 98% | 96% |
| Class E | -0.01% | 0.30% | -0.40% | 97% | 98% |
| Overall | -0.01% | 0.16% | -0.11% | 99% | 97% |
| Class D | -0.06% | -0.02% | 0.13% | 98% | 96% |
| Class F | 0.14% | -0.23% | 0.05% | 98% | 96% |

### CE6-14c (dimension 16 and 32)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | 0.02% | 0.08% | 93% | 88% |
| Class A2 | 0.25% | 0.26% | 0.26% | 95% | 91% |
| Class B | 0.20% | 0.32% | 0.23% | 96% | 91% |
| Class C | 0.11% | 0.11% | 0.01% | 96% | 93% |
| Class E | 0.08% | 0.07% | 0.18% | 95% | 90% |
| Overall | 0.12% | 0.17% | 0.15% | 95% | 91% |
| Class D | 0.07% | 0.07% | -0.02% | 96% | 96% |
| Class F | 0.08% | 0.08% | 0.03% | 96% | 94% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.13% | 0.05% | 99% | 98% |
| Class A2 | 0.05% | 0.08% | 0.12% | 100% | 99% |
| Class B | 0.08% | 0.29% | 0.25% | 101% | 100% |
| Class C | 0.08% | 0.01% | 0.22% | 98% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.09% | 0.18% | 99% | 99% |
| Class D | -0.01% | -0.17% | -0.25% | 99% | 99% |
| Class F | 0.06% | 0.12% | 0.11% | 97% | 97% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.04% | 0.37% | 100% | 102% |
| Class C | 0.08% | 0.12% | 0.22% | 99% | 100% |
| Class E | 0.03% | 0.57% | -0.15% | 97% | 96% |
| Overall | 0.04% | 0.20% | 0.19% | 99% | 100% |
| Class D | 0.08% | -0.13% | 0.42% | 99% | 99% |
| Class F | 0.06% | 0.33% | -0.43% | 97% | 95% |

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.15% | 0.09% | 98% | 98% |
| Class C | 0.02% | 0.09% | -0.01% | 99% | 98% |
| Class E | 0.01% | 0.35% | -0.05% | 98% | 102% |
| Overall | 0.01% | 0.18% | 0.02% | 98% | 99% |
| Class D | 0.04% | -0.48% | 0.78% | 99% | 100% |
| Class F | 0.11% | -0.10% | -0.74% | 97% | 98% |

### CE6-14d (dimension 16, 32, and 64)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.38% | -0.20% | -0.28% | 94% | 86% |
| Class A2 | 0.18% | 0.21% | 0.21% | 95% | 91% |
| Class B | 0.16% | 0.22% | 0.26% | 96% | 91% |
| Class C | 0.10% | 0.03% | -0.01% | 95% | 92% |
| Class E | 0.07% | 0.09% | 0.15% | 95% | 89% |
| Overall | 0.05% | 0.09% | 0.08% | 95% | 90% |
| Class D | 0.06% | 0.03% | 0.06% | 96% | 92% |
| Class F | 0.06% | 0.09% | 0.15% | 96% | 92% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.43% | -0.90% | -0.89% | 101% | 99% |
| Class A2 | 0.00% | -0.09% | 0.01% | 100% | 99% |
| Class B | 0.03% | 0.16% | -0.09% | 100% | 98% |
| Class C | 0.07% | 0.12% | 0.25% | 97% | 95% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | -0.11% | -0.14% | 99% | 98% |
| Class D | -0.03% | -0.17% | -0.17% | 97% | 95% |
| Class F | 0.04% | 0.08% | 0.11% | 96% | 95% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.30% | -0.21% | 101% | 99% |
| Class C | 0.04% | 0.00% | 0.33% | 97% | 96% |
| Class E | -0.01% | 0.39% | -0.05% | 96% | 93% |
| Overall | 0.01% | -0.03% | 0.01% | 98% | 96% |
| Class D | 0.09% | -0.30% | 0.30% | 98% | 95% |
| Class F | 0.06% | -0.41% | -0.28% | 97% | 95% |

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | 0.19% | -0.06% | 100% | 99% |
| Class C | 0.00% | -0.06% | 0.02% | 98% | 95% |
| Class E | -0.06% | 0.41% | -0.47% | 97% | 99% |
| Overall | -0.03% | 0.16% | -0.14% | 98% | 98% |
| Class D | 0.04% | -0.25% | 1.13% | 98% | 96% |
| Class F | 0.09% | -0.24% | -0.07% | 98% | 94% |

## CTC and inter MTS enabled

### CE6-14a (dimension 32)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.04% | -0.04% | 97% | 90% | -0.14% | -0.04% | -0.04% | 97% | 90% |
| Class A2 | 0.08% | 0.23% | 0.14% | 98% | 94% | 0.08% | 0.23% | 0.14% | 98% | 94% |
| Class B | 0.04% | 0.14% | 0.11% | 99% | 95% | 0.04% | 0.14% | 0.11% | 99% | 95% |
| Class C | 0.01% | 0.01% | -0.07% | 98% | 97% | 0.01% | 0.01% | -0.07% | 98% | 97% |
| Class E | 0.00% | -0.01% | 0.13% | 97% | 94% | 0.00% | -0.01% | 0.13% | 97% | 94% |
| Overall | 0.00% | 0.07% | 0.05% | 98% | 94% | 0.00% | 0.07% | 0.05% | 98% | 94% |
| Class D | 0.02% | 0.02% | -0.01% | 98% | 98% | 0.02% | 0.02% | -0.01% | 98% | 98% |
| Class F | 0.01% | 0.07% | 0.02% | 98% | 98% | 0.01% | 0.07% | 0.02% | 98% | 98% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.27% | 0.49% | 0.59% | 114% | 100% | -0.06% | 0.04% | 0.10% | 97% | 97% |
| Class A2 | -0.57% | 0.59% | 0.73% | 118% | 101% | 0.03% | 0.04% | 0.20% | 99% | 100% |
| Class B | -0.40% | 0.91% | 0.63% | 119% | 101% | 0.02% | 0.24% | 0.11% | 98% | 98% |
| Class C | -0.25% | 0.59% | 0.86% | 122% | 99% | 0.00% | 0.09% | 0.20% | 97% | 97% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.37% | 0.67% | 0.70% | 119% | 100% | 0.00% | 0.12% | 0.15% | 98% | 98% |
| Class D | -0.34% | 0.33% | 0.47% | 125% | 101% | 0.06% | -0.24% | -0.04% | 99% | 100% |
| Class F | -0.04% | 0.34% | 0.35% | 113% | 102% | 0.04% | -0.02% | 0.08% | 96% | 101% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.67% | 0.69% | 0.97% | 128% | 102% | 0.09% | 0.22% | -0.02% | 97% | 96% |
| Class C | -0.48% | 1.04% | 1.20% | 132% | 100% | 0.04% | -0.18% | 0.12% | 96% | 97% |
| Class E | -0.20% | 1.61% | 1.16% | 113% | 96% | 0.02% | 0.38% | -0.14% | 95% | 93% |
| Overall | -0.49% | 1.04% | 1.09% | 125% | 100% | 0.05% | 0.13% | -0.01% | 97% | 96% |
| Class D | -0.52% | 1.35% | 1.46% | 131% | 102% | 0.10% | 0.27% | 0.08% | 97% | 98% |
| Class F | -0.19% | 0.75% | 0.88% | 120% | 98% | -0.01% | 0.02% | 0.15% | 96% | 97% |

|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.71% | 0.96% | 1.15% | 136% | 100% | 0.09% | -0.20% | -0.12% | 96% | 92% |
| Class C | -0.46% | 1.14% | 1.02% | 141% | 102% | 0.08% | 0.11% | -0.02% | 96% | 97% |
| Class E | -0.33% | 1.52% | 0.82% | 122% | 101% | -0.02% | 0.72% | -0.29% | 96% | 97% |
| Overall | -0.53% | 1.16% | 1.03% | 134% | 101% | 0.06% | 0.13% | -0.13% | 96% | 95% |
| Class D | -0.61% | 1.45% | 1.31% | 142% | 103% | 0.06% | 0.22% | 0.72% | 99% | 100% |
| Class F | -0.18% | 0.34% | 0.12% | 129% | 100% | 0.19% | 0.31% | -0.25% | 96% | 96% |

### CE6-14b (dimensions 32 and 64)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.45% | -0.29% | -0.26% | 98% | 90% | -0.45% | -0.29% | -0.26% | 98% | 90% |
| Class A2 | 0.02% | 0.12% | 0.14% | 98% | 94% | 0.02% | 0.12% | 0.14% | 98% | 94% |
| Class B | -0.01% | 0.09% | 0.13% | 99% | 97% | -0.01% | 0.09% | 0.13% | 99% | 97% |
| Class C | 0.02% | 0.02% | -0.10% | 98% | 95% | 0.02% | 0.02% | -0.10% | 98% | 95% |
| Class E | -0.02% | 0.03% | 0.02% | 98% | 91% | -0.02% | 0.03% | 0.02% | 98% | 91% |
| Overall | -0.07% | 0.01% | 0.00% | 98% | 94% | -0.07% | 0.01% | 0.00% | 98% | 94% |
| Class D | 0.02% | 0.01% | 0.00% | 98% | 96% | 0.02% | 0.01% | 0.00% | 98% | 96% |
| Class F | 0.00% | -0.03% | -0.01% | 98% | 95% | 0.00% | -0.03% | -0.01% | 98% | 95% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.75% | -0.06% | 0.06% | 123% | 100% | -0.55% | -0.50% | -0.44% | 104% | 97% |
| Class A2 | -0.69% | 0.53% | 0.64% | 123% | 100% | -0.09% | -0.02% | 0.11% | 103% | 99% |
| Class B | -0.50% | 0.74% | 0.26% | 125% | 101% | -0.08% | 0.07% | -0.26% | 103% | 98% |
| Class C | -0.24% | 0.58% | 0.67% | 126% | 96% | 0.01% | 0.08% | 0.01% | 99% | 94% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.52% | 0.49% | 0.41% | 124% | 99% | -0.15% | -0.06% | -0.15% | 102% | 97% |
| Class D | -0.35% | 0.33% | 0.56% | 125% | 97% | 0.05% | -0.25% | 0.06% | 100% | 96% |
| Class F | -0.10% | 0.35% | 0.34% | 117% | 95% | -0.03% | -0.01% | 0.07% | 99% | 94% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.81% | 0.78% | 1.04% | 137% | 103% | -0.05% | 0.32% | 0.04% | 104% | 96% |
| Class C | -0.44% | 1.12% | 1.40% | 136% | 100% | 0.07% | -0.09% | 0.32% | 100% | 97% |
| Class E | -0.25% | 1.73% | 1.16% | 119% | 94% | -0.03% | 0.49% | -0.14% | 100% | 92% |
| Overall | -0.55% | 1.13% | 1.19% | 132% | 100% | -0.01% | 0.22% | 0.09% | 102% | 95% |
| Class D | -0.58% | 1.65% | 1.24% | 132% | 97% | 0.04% | 0.57% | -0.14% | 98% | 93% |
| Class F | -0.41% | 0.70% | 0.43% | 126% | 94% | -0.23% | -0.03% | -0.30% | 100% | 92% |

|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.87% | 1.01% | 1.23% | 147% | 101% | -0.07% | -0.14% | -0.04% | 104% | 94% |
| Class C | -0.50% | 1.06% | 1.23% | 146% | 98% | 0.04% | 0.03% | 0.19% | 100% | 93% |
| Class E | -0.43% | 1.79% | 0.54% | 131% | 99% | -0.12% | 0.99% | -0.58% | 103% | 95% |
| Overall | -0.64% | 1.23% | 1.06% | 143% | 99% | -0.04% | 0.20% | -0.10% | 102% | 94% |
| Class D | -0.58% | 0.89% | 1.69% | 142% | 98% | 0.09% | -0.32% | 1.11% | 99% | 94% |
| Class F | -0.17% | 0.56% | 0.17% | 137% | 98% | 0.20% | 0.53% | -0.17% | 102% | 94% |

### CE6-14c (dimension 16 and 32)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | 0.02% | 0.08% | 93% | 88% | -0.07% | 0.02% | 0.08% | 93% | 88% |
| Class A2 | 0.25% | 0.26% | 0.26% | 95% | 91% | 0.25% | 0.26% | 0.26% | 95% | 91% |
| Class B | 0.20% | 0.32% | 0.23% | 96% | 91% | 0.20% | 0.32% | 0.23% | 96% | 91% |
| Class C | 0.11% | 0.11% | 0.01% | 96% | 93% | 0.11% | 0.11% | 0.01% | 96% | 93% |
| Class E | 0.08% | 0.07% | 0.18% | 95% | 90% | 0.08% | 0.07% | 0.18% | 95% | 90% |
| Overall | 0.12% | 0.17% | 0.15% | 95% | 91% | 0.12% | 0.17% | 0.15% | 95% | 91% |
| Class D | 0.07% | 0.07% | -0.02% | 96% | 96% | 0.07% | 0.07% | -0.02% | 96% | 96% |
| Class F | 0.08% | 0.08% | 0.03% | 96% | 94% | 0.08% | 0.08% | 0.03% | 96% | 94% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.27% | 0.33% | 0.58% | 112% | 99% | -0.06% | -0.12% | 0.08% | 95% | 95% |
| Class A2 | -0.46% | 0.53% | 0.58% | 115% | 100% | 0.15% | -0.01% | 0.05% | 96% | 98% |
| Class B | -0.29% | 0.82% | 0.84% | 118% | 99% | 0.13% | 0.16% | 0.32% | 98% | 97% |
| Class C | -0.13% | 0.71% | 0.96% | 120% | 98% | 0.12% | 0.22% | 0.29% | 95% | 95% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.28% | 0.64% | 0.77% | 117% | 99% | 0.09% | 0.08% | 0.21% | 96% | 96% |
| Class D | -0.30% | 0.58% | 0.48% | 122% | 99% | 0.11% | 0.00% | -0.03% | 97% | 98% |
| Class F | 0.01% | 0.42% | 0.43% | 112% | 97% | 0.08% | 0.06% | 0.15% | 95% | 97% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.56% | 0.81% | 0.95% | 125% | 103% | 0.20% | 0.35% | -0.04% | 95% | 97% |
| Class C | -0.38% | 0.87% | 1.20% | 129% | 98% | 0.13% | -0.34% | 0.12% | 94% | 95% |
| Class E | -0.10% | 1.62% | 0.92% | 112% | 95% | 0.12% | 0.39% | -0.37% | 94% | 92% |
| Overall | -0.39% | 1.03% | 1.03% | 123% | 99% | 0.16% | 0.13% | -0.07% | 95% | 95% |
| Class D | -0.47% | 0.95% | 1.69% | 129% | 100% | 0.15% | -0.12% | 0.30% | 96% | 96% |
| Class F | -0.15% | 0.82% | 0.79% | 119% | 96% | 0.03% | 0.07% | 0.05% | 95% | 95% |

|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.62% | 1.02% | 1.31% | 132% | 99% | 0.18% | -0.14% | 0.03% | 93% | 92% |
| Class C | -0.39% | 1.17% | 1.17% | 138% | 99% | 0.16% | 0.14% | 0.12% | 94% | 95% |
| Class E | -0.23% | 2.03% | 0.45% | 120% | 102% | 0.08% | 1.23% | -0.68% | 94% | 98% |
| Overall | -0.44% | 1.32% | 1.05% | 131% | 100% | 0.15% | 0.30% | -0.12% | 94% | 94% |
| Class D | -0.52% | 0.53% | 1.37% | 138% | 98% | 0.15% | -0.68% | 0.79% | 96% | 95% |
| Class F | -0.18% | 0.08% | 0.21% | 126% | 100% | 0.19% | 0.05% | -0.13% | 94% | 95% |

### CE6-14d (dimension 16, 32, and 64)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.38% | -0.20% | -0.28% | 94% | 86% | -0.38% | -0.20% | -0.28% | 94% | 86% |
| Class A2 | 0.18% | 0.21% | 0.21% | 95% | 91% | 0.18% | 0.21% | 0.21% | 95% | 91% |
| Class B | 0.16% | 0.22% | 0.26% | 96% | 91% | 0.16% | 0.22% | 0.26% | 96% | 91% |
| Class C | 0.10% | 0.03% | -0.01% | 95% | 92% | 0.10% | 0.03% | -0.01% | 95% | 92% |
| Class E | 0.07% | 0.09% | 0.15% | 95% | 89% | 0.07% | 0.09% | 0.15% | 95% | 89% |
| Overall | 0.05% | 0.09% | 0.08% | 95% | 90% | 0.05% | 0.09% | 0.08% | 95% | 90% |
| Class D | 0.06% | 0.03% | 0.06% | 96% | 92% | 0.06% | 0.03% | 0.06% | 96% | 92% |
| Class F | 0.06% | 0.09% | 0.15% | 96% | 92% | 0.06% | 0.09% | 0.15% | 96% | 92% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.76% | -0.02% | -0.08% | 120% | 99% | -0.56% | -0.47% | -0.58% | 102% | 95% |
| Class A2 | -0.61% | 0.51% | 0.61% | 121% | 99% | 0.00% | -0.04% | 0.08% | 102% | 97% |
| Class B | -0.39% | 0.68% | 0.63% | 122% | 99% | 0.03% | 0.01% | 0.11% | 101% | 97% |
| Class C | -0.13% | 0.72% | 0.95% | 124% | 95% | 0.12% | 0.22% | 0.29% | 98% | 93% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.44% | 0.51% | 0.57% | 122% | 98% | -0.07% | -0.04% | 0.01% | 101% | 95% |
| Class D | -0.30% | 0.52% | 0.42% | 123% | 96% | 0.10% | -0.05% | -0.09% | 98% | 95% |
| Class F | -0.05% | 0.42% | 0.43% | 115% | 95% | 0.03% | 0.06% | 0.16% | 98% | 94% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.70% | 0.73% | 0.82% | 134% | 101% | 0.05% | 0.27% | -0.17% | 102% | 95% |
| Class C | -0.33% | 1.08% | 1.47% | 134% | 96% | 0.19% | -0.14% | 0.39% | 98% | 93% |
| Class E | -0.22% | 1.68% | 1.34% | 118% | 93% | 0.01% | 0.45% | 0.04% | 99% | 90% |
| Overall | -0.46% | 1.09% | 1.17% | 130% | 98% | 0.09% | 0.18% | 0.07% | 100% | 93% |
| Class D | -0.47% | 0.76% | 1.44% | 131% | 96% | 0.15% | -0.30% | 0.06% | 97% | 93% |
| Class F | -0.22% | 0.55% | 0.52% | 124% | 95% | -0.04% | -0.19% | -0.22% | 99% | 93% |

|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.79% | 1.05% | 1.08% | 143% | 99% | 0.01% | -0.11% | -0.20% | 101% | 92% |
| Class C | -0.41% | 0.91% | 1.02% | 143% | 95% | 0.14% | -0.12% | -0.02% | 98% | 91% |
| Class E | -0.37% | 1.78% | 1.04% | 128% | 100% | -0.06% | 0.98% | -0.08% | 101% | 96% |
| Overall | -0.56% | 1.19% | 1.05% | 139% | 98% | 0.04% | 0.16% | -0.11% | 100% | 92% |
| Class D | -0.57% | 0.40% | 1.50% | 139% | 96% | 0.10% | -0.80% | 0.92% | 97% | 92% |
| Class F | -0.32% | 0.24% | 0.52% | 134% | 97% | 0.05% | 0.21% | 0.20% | 100% | 93% |

## Low QP (inter MTS disabled)

### CE6-14a (dimension 32, low-QP time measurements not accurate)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.01% | 86% | 100% |
| Class A2 | 0.16% | 0.20% | 0.20% | 84% | 99% |
| Class B | 0.01% | 0.01% | 0.01% | 89% | 102% |
| Class C | 0.01% | 0.00% | 0.00% | 87% | 102% |
| Class E | 0.00% | 0.05% | 0.05% | 87% | 100% |
| Overall | 0.03% | 0.04% | 0.05% | 87% | 101% |
| Class D | 0.00% | 0.02% | 0.02% | 84% | 100% |
| Class F | 0.01% | 0.01% | -0.02% | 90% | 101% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.11% | 0.09% | 96% | 104% |
| Class A2 | 0.03% | 0.04% | 0.04% | 95% | 103% |
| Class B | 0.01% | 0.02% | 0.04% | 92% | 103% |
| Class C | 0.01% | 0.01% | 0.01% | 97% | 106% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.04% | 0.04% | 95% | 104% |
| Class D | 0.00% | 0.03% | -0.01% | 91% | 101% |
| Class F | -0.08% | -0.14% | -0.05% | 89% | 103% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.03% | 0.04% | 85% | 97% |
| Class C | 0.00% | -0.01% | -0.03% | 100% | 107% |
| Class E | 0.01% | 0.00% | -0.01% | 89% | 104% |
| Overall | 0.00% | 0.01% | 0.01% | 91% | 102% |
| Class D | 0.01% | -0.03% | -0.06% | 98% | 105% |
| Class F | -0.07% | -0.01% | -0.04% | 107% | 118% |

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.02% | 0.02% | 95% | 109% |
| Class C | 0.00% | -0.02% | -0.03% | 97% | 103% |
| Class E | 0.01% | 0.02% | 0.01% | 99% | 102% |
| Overall | 0.01% | 0.01% | 0.00% | 97% | 105% |
| Class D | 0.00% | 0.04% | -0.02% | 89% | 99% |
| Class F | -0.10% | -0.12% | -0.02% | 95% | 105% |

### CE6-14b (dimensions 32 and 64, low-QP time measurements not accurate)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.02% | 85% | 98% |
| Class A2 | 0.15% | 0.19% | 0.20% | 84% | 100% |
| Class B | 0.01% | 0.01% | 0.01% | 90% | 106% |
| Class C | 0.01% | 0.00% | 0.00% | 86% | 102% |
| Class E | 0.00% | 0.05% | 0.04% | 88% | 99% |
| Overall | 0.03% | 0.04% | 0.05% | 87% | 102% |
| Class D | 0.00% | 0.02% | 0.02% | 86% | 101% |
| Class F | 0.01% | 0.01% | -0.02% | 92% | 102% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.10% | 0.06% | 97% | 106% |
| Class A2 | 0.03% | 0.04% | 0.04% | 96% | 105% |
| Class B | 0.01% | 0.03% | 0.08% | 93% | 103% |
| Class C | 0.01% | 0.01% | 0.01% | 98% | 106% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.04% | 0.05% | 95% | 105% |
| Class D | 0.00% | 0.02% | 0.00% | 93% | 101% |
| Class F | -0.04% | -0.08% | -0.02% | 89% | 102% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.02% | 0.04% | 84% | 97% |
| Class C | 0.00% | -0.01% | -0.03% | 101% | 106% |
| Class E | 0.00% | 0.02% | 0.01% | 91% | 100% |
| Overall | 0.00% | 0.01% | 0.01% | 91% | 101% |
| Class D | 0.01% | -0.03% | -0.06% | 99% | 107% |
| Class F | -0.09% | -0.01% | -0.11% | 108% | 121% |

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.02% | 0.02% | 95% | 106% |
| Class C | 0.01% | -0.01% | -0.03% | 97% | 103% |
| Class E | 0.00% | 0.02% | 0.02% | 98% | 103% |
| Overall | 0.01% | 0.01% | 0.00% | 97% | 104% |
| Class D | 0.00% | 0.04% | -0.02% | 89% | 97% |
| Class F | -0.08% | -0.15% | -0.05% | 96% | 105% |

### CE6-14c (dimension 16 and 32, low-QP time measurements not accurate)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.09% | 0.11% | 83% | 100% |
| Class A2 | 0.43% | 0.44% | 0.44% | 81% | 98% |
| Class B | 0.08% | 0.16% | 0.17% | 89% | 102% |
| Class C | 0.04% | 0.08% | 0.06% | 85% | 102% |
| Class E | 0.11% | 0.19% | 0.20% | 86% | 98% |
| Overall | 0.13% | 0.18% | 0.18% | 85% | 100% |
| Class D | 0.03% | 0.06% | 0.04% | 82% | 105% |
| Class F | 0.04% | 0.09% | 0.05% | 89% | 101% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.25% | 0.24% | 95% | 104% |
| Class A2 | 0.09% | 0.09% | 0.08% | 94% | 103% |
| Class B | 0.05% | 0.06% | 0.09% | 92% | 101% |
| Class C | 0.01% | 0.02% | 0.03% | 98% | 106% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.09% | 0.10% | 94% | 103% |
| Class D | 0.02% | 0.10% | -0.04% | 91% | 101% |
| Class F | -0.06% | -0.09% | -0.09% | 88% | 102% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.04% | 0.06% | 84% | 97% |
| Class C | 0.01% | 0.04% | -0.02% | 100% | 107% |
| Class E | 0.02% | 0.07% | 0.07% | 90% | 102% |
| Overall | 0.02% | 0.05% | 0.04% | 90% | 101% |
| Class D | 0.00% | -0.03% | 0.02% | 98% | 106% |
| Class F | -0.02% | -0.03% | -0.03% | 107% | 122% |

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | 0.05% | 0.05% | 93% | 105% |
| Class C | 0.02% | 0.00% | -0.01% | 96% | 104% |
| Class E | 0.05% | 0.07% | 0.08% | 97% | 103% |
| Overall | 0.04% | 0.04% | 0.04% | 95% | 104% |
| Class D | 0.02% | 0.00% | -0.02% | 89% | 98% |
| Class F | -0.09% | -0.07% | 0.02% | 94% | 107% |

### CE6-14d (dimension 16, 32, and 64, low-QP time measurements not accurate)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.09% | 0.11% | 83% | 98% |
| Class A2 | 0.43% | 0.43% | 0.44% | 82% | 98% |
| Class B | 0.08% | 0.15% | 0.17% | 88% | 103% |
| Class C | 0.04% | 0.08% | 0.06% | 86% | 102% |
| Class E | 0.11% | 0.19% | 0.20% | 85% | 101% |
| Overall | 0.13% | 0.18% | 0.18% | 85% | 101% |
| Class D | 0.03% | 0.06% | 0.04% | 84% | 102% |
| Class F | 0.04% | 0.10% | 0.05% | 90% | 101% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.21% | 0.21% | 95% | 103% |
| Class A2 | 0.09% | 0.08% | 0.08% | 95% | 104% |
| Class B | 0.05% | 0.07% | 0.08% | 92% | 102% |
| Class C | 0.01% | 0.02% | 0.03% | 97% | 107% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.09% | 0.09% | 95% | 104% |
| Class D | 0.01% | 0.08% | -0.03% | 92% | 101% |
| Class F | -0.08% | -0.11% | -0.07% | 88% | 102% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.03% | 0.06% | 84% | 97% |
| Class C | 0.01% | 0.05% | -0.02% | 101% | 106% |
| Class E | 0.02% | 0.05% | 0.07% | 89% | 100% |
| Overall | 0.02% | 0.04% | 0.03% | 90% | 101% |
| Class D | 0.00% | -0.03% | 0.02% | 100% | 106% |
| Class F | -0.16% | -0.09% | -0.03% | 107% | 115% |

|  | Low delay P Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | 0.05% | 0.07% | 95% | 106% |
| Class C | 0.02% | 0.00% | -0.01% | 97% | 104% |
| Class E | 0.04% | 0.07% | 0.07% | 99% | 103% |
| Overall | 0.04% | 0.04% | 0.04% | 96% | 105% |
| Class D | 0.02% | 0.00% | -0.02% | 90% | 100% |
| Class F | -0.11% | -0.11% | 0.04% | 95% | 107% |

## Low QP (inter MTS enabled)

### CE6-14a (dimension 32, low-QP time measurements not accurate)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.01% | 85% | 103% | 0.01% | 0.00% | 0.01% | 91% | 104% |
| Class A2 | 0.16% | 0.20% | 0.20% | 82% | 99% | 0.16% | 0.20% | 0.20% | 92% | 99% |
| Class B | 0.01% | 0.01% | 0.01% | 87% | 101% | 0.01% | 0.01% | 0.01% | 92% | 97% |
| Class C | 0.01% | 0.00% | 0.00% | 86% | 102% | 0.01% | 0.00% | 0.00% | 93% | 98% |
| Class E | 0.00% | 0.05% | 0.05% | 88% | 100% | 0.00% | 0.05% | 0.05% | 93% | 95% |
| Overall | 0.03% | 0.04% | 0.05% | 86% | 101% | 0.03% | 0.04% | 0.05% | 93% | 99% |
| Class D | 0.00% | 0.02% | 0.02% | 84% | 99% | 0.00% | 0.02% | 0.02% | 95% | 97% |
| Class F | 0.01% | 0.01% | -0.02% | 90% | 101% | 0.01% | 0.01% | -0.02% | 93% | 98% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.44% | 0.47% | 0.54% | 144% | 110% | 0.01% | 0.11% | 0.10% | 98% | 101% |
| Class A2 | -0.71% | -0.02% | -0.02% | 153% | 110% | 0.07% | 0.08% | 0.08% | 95% | 99% |
| Class B | -0.68% | 0.05% | 0.12% | 155% | 107% | 0.01% | 0.08% | 0.11% | 94% | 97% |
| Class C | -0.95% | 0.02% | -0.01% | 171% | 109% | 0.02% | 0.04% | 0.01% | 95% | 98% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.71% | 0.11% | 0.14% | 156% | 109% | 0.03% | 0.07% | 0.07% | 95% | 98% |
| Class D | -1.25% | -0.13% | -0.19% | 160% | 104% | 0.02% | -0.04% | -0.06% | 91% | 93% |
| Class F | -0.68% | -0.12% | -0.07% | 124% | 101% | -0.04% | -0.08% | -0.03% | 89% | 95% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.72% | -0.01% | 0.06% | 151% | 101% | 0.00% | 0.07% | 0.07% | 90% | 97% |
| Class C | -1.09% | -0.09% | -0.11% | 185% | 111% | 0.01% | 0.00% | 0.01% | 92% | 97% |
| Class E | -1.54% | -0.02% | -0.08% | 151% | 112% | 0.05% | 0.10% | 0.08% | 91% | 97% |
| Overall | -1.05% | -0.04% | -0.03% | 161% | 107% | 0.02% | 0.05% | 0.05% | 91% | 97% |
| Class D | -1.39% | -0.16% | -0.13% | 179% | 109% | 0.01% | -0.02% | 0.02% | 93% | 95% |
| Class F) | -0.92% | -0.07% | -0.05% | 162% | 120% | -0.05% | -0.10% | 0.02% | 93% | 96% |

|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.68% | -0.02% | 0.00% | 171% | 111% | 0.00% | 0.05% | 0.03% | 89% | 97% |
| Class C | -1.08% | -0.11% | -0.14% | 185% | 106% | 0.01% | 0.01% | 0.00% | 90% | 101% |
| Class E | -1.42% | -0.06% | -0.08% | 172% | 111% | 0.04% | 0.08% | 0.06% | 92% | 99% |
| Overall | -1.00% | -0.06% | -0.07% | 176% | 109% | 0.01% | 0.05% | 0.03% | 90% | 98% |
| Class D | -1.36% | -0.07% | -0.14% | 174% | 98% | 0.00% | 0.00% | -0.01% | 91% | 97% |
| Class F | -0.92% | -0.08% | 0.03% | 151% | 107% | 0.05% | 0.02% | 0.12% | 95% | 107% |

### CE6-14b (dimensions 32 and 64, low-QP time measurements not accurate)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.02% | 88% | 101% | 0.01% | 0.00% | 0.02% | 94% | 102% |
| Class A2 | 0.15% | 0.19% | 0.20% | 85% | 98% | 0.15% | 0.19% | 0.20% | 94% | 99% |
| Class B | 0.01% | 0.01% | 0.01% | 89% | 102% | 0.01% | 0.01% | 0.01% | 94% | 98% |
| Class C | 0.01% | 0.00% | 0.00% | 88% | 104% | 0.01% | 0.00% | 0.00% | 96% | 100% |
| Class E | 0.00% | 0.05% | 0.04% | 89% | 101% | 0.00% | 0.05% | 0.04% | 94% | 95% |
| Overall | 0.03% | 0.04% | 0.05% | 88% | 101% | 0.03% | 0.04% | 0.05% | 95% | 99% |
| Class D | 0.00% | 0.02% | 0.02% | 85% | 101% | 0.00% | 0.02% | 0.02% | 96% | 99% |
| Class F | 0.01% | 0.01% | -0.02% | 89% | 100% | 0.01% | 0.01% | -0.02% | 92% | 97% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.45% | 0.43% | 0.51% | 148% | 111% | 0.01% | 0.08% | 0.07% | 101% | 102% |
| Class A2 | -0.73% | -0.02% | -0.03% | 160% | 111% | 0.06% | 0.07% | 0.07% | 99% | 100% |
| Class B | -0.69% | 0.05% | 0.11% | 157% | 106% | 0.00% | 0.07% | 0.09% | 95% | 97% |
| Class C | -0.95% | 0.02% | -0.01% | 176% | 111% | 0.02% | 0.04% | 0.01% | 98% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.72% | 0.10% | 0.13% | 161% | 109% | 0.02% | 0.07% | 0.06% | 98% | 99% |
| Class D | -1.26% | -0.15% | -0.18% | 164% | 105% | 0.01% | -0.06% | -0.05% | 93% | 94% |
| Class F | -0.61% | -0.04% | -0.05% | 133% | 105% | 0.03% | 0.00% | -0.01% | 96% | 99% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.72% | 0.00% | 0.06% | 153% | 101% | 0.00% | 0.08% | 0.07% | 92% | 97% |
| Class C | -1.09% | -0.10% | -0.12% | 190% | 112% | 0.01% | 0.00% | 0.00% | 95% | 97% |
| Class E | -1.56% | 0.00% | -0.06% | 156% | 110% | 0.03% | 0.12% | 0.10% | 94% | 95% |
| Overall | -1.05% | -0.03% | -0.03% | 165% | 107% | 0.01% | 0.06% | 0.05% | 93% | 97% |
| Class D | -1.39% | -0.14% | -0.17% | 185% | 111% | 0.01% | 0.00% | -0.02% | 96% | 97% |
| Class F | -0.83% | 0.02% | 0.01% | 169% | 126% | 0.04% | -0.01% | 0.08% | 97% | 102% |

|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.68% | -0.02% | 0.01% | 172% | 109% | 0.00% | 0.05% | 0.04% | 89% | 95% |
| Class C | -1.08% | -0.11% | -0.13% | 187% | 110% | 0.00% | 0.01% | 0.01% | 91% | 105% |
| Class E | -1.43% | -0.07% | -0.10% | 176% | 109% | 0.04% | 0.06% | 0.05% | 94% | 97% |
| Overall | -1.00% | -0.06% | -0.06% | 178% | 109% | 0.01% | 0.04% | 0.03% | 91% | 99% |
| Class D | -1.33% | -0.05% | -0.13% | 183% | 106% | 0.03% | 0.02% | 0.01% | 96% | 105% |
| Class F | -0.98% | -0.14% | -0.09% | 160% | 107% | 0.00% | -0.05% | 0.00% | 100% | 108% |

### CE6-14c (dimension 16 and 32, low-QP time measurements not accurate)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.09% | 0.11% | 83% | 98% | 0.06% | 0.09% | 0.11% | 89% | 99% |
| Class A2 | 0.43% | 0.44% | 0.44% | 83% | 98% | 0.43% | 0.44% | 0.44% | 93% | 99% |
| Class B | 0.08% | 0.16% | 0.17% | 87% | 102% | 0.08% | 0.16% | 0.17% | 92% | 98% |
| Class C | 0.04% | 0.08% | 0.06% | 87% | 102% | 0.04% | 0.08% | 0.06% | 95% | 98% |
| Class E | 0.11% | 0.19% | 0.20% | 86% | 101% | 0.11% | 0.19% | 0.20% | 91% | 96% |
| Overall | 0.13% | 0.18% | 0.18% | 86% | 101% | 0.13% | 0.18% | 0.18% | 92% | 98% |
| Class D | 0.03% | 0.06% | 0.04% | 82% | 100% | 0.03% | 0.06% | 0.04% | 92% | 98% |
| Class F | 0.04% | 0.09% | 0.05% | 89% | 101% | 0.04% | 0.09% | 0.05% | 92% | 98% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.57% | 0.14% | 0.20% | 151% | 105% | 0.12% | 0.16% | 0.18% | 91% | 96% |
| Class C | -0.86% | 0.08% | 0.08% | 169% | 107% | 0.11% | 0.10% | 0.10% | 94% | 96% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -1.18% | 0.00% | -0.07% | 159% | 104% | 0.09% | 0.08% | 0.06% | 91% | 93% |
| Class F | -0.55% | 0.04% | 0.04% | 124% | 102% | 0.09% | 0.08% | 0.09% | 89% | 96% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.62% | 0.06% | 0.12% | 144% | 99% | 0.10% | 0.13% | 0.13% | 87% | 95% |
| Class C | -1.02% | -0.01% | -0.03% | 183% | 110% | 0.08% | 0.09% | 0.10% | 91% | 95% |
| Class E | -1.43% | 0.19% | 0.11% | 147% | 105% | 0.17% | 0.31% | 0.26% | 88% | 91% |
| Overall | -0.95% | 0.07% | 0.07% | 157% | 104% | 0.11% | 0.16% | 0.15% | 88% | 94% |
| Class D | -1.34% | -0.12% | -0.06% | 181% | 109% | 0.06% | 0.02% | 0.09% | 93% | 95% |
| Class F | -0.95% | -0.03% | -0.03% | 160% | 124% | -0.08% | -0.06% | 0.04% | 91% | 100% |

|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.58% | 0.06% | 0.10% | 165% | 109% | 0.10% | 0.13% | 0.14% | 85% | 95% |
| Class C | -1.02% | -0.05% | -0.06% | 178% | 104% | 0.07% | 0.08% | 0.08% | 86% | 99% |
| Class E | -1.31% | 0.10% | 0.08% | 167% | 107% | 0.15% | 0.24% | 0.23% | 89% | 95% |
| Overall | -0.91% | 0.04% | 0.04% | 170% | 107% | 0.10% | 0.14% | 0.14% | 87% | 96% |
| Class D | -1.31% | -0.01% | -0.06% | 175% | 101% | 0.05% | 0.06% | 0.08% | 91% | 100% |
| Class F | -0.93% | -0.09% | -0.01% | 149% | 102% | 0.05% | 0.00% | 0.07% | 94% | 103% |

### CE6-14d (dimension 16, 32, and 64, low-QP time measurements not accurate)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.09% | 0.11% | 84% | 99% | 0.06% | 0.09% | 0.11% | 90% | 100% |
| Class A2 | 0.43% | 0.43% | 0.44% | 83% | 98% | 0.43% | 0.43% | 0.44% | 92% | 99% |
| Class B | 0.08% | 0.15% | 0.17% | 88% | 103% | 0.08% | 0.15% | 0.17% | 93% | 98% |
| Class C | 0.04% | 0.08% | 0.06% | 86% | 104% | 0.04% | 0.08% | 0.06% | 94% | 100% |
| Class E | 0.11% | 0.19% | 0.20% | 87% | 100% | 0.11% | 0.19% | 0.20% | 93% | 95% |
| Overall | 0.13% | 0.18% | 0.18% | 86% | 101% | 0.13% | 0.18% | 0.18% | 92% | 98% |
| Class D | 0.03% | 0.06% | 0.04% | 84% | 101% | 0.03% | 0.06% | 0.04% | 94% | 100% |
| Class F | 0.04% | 0.10% | 0.05% | 89% | 102% | 0.04% | 0.10% | 0.05% | 92% | 99% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.58% | 0.13% | 0.19% | 153% | 106% | 0.11% | 0.16% | 0.17% | 93% | 96% |
| Class C | -0.86% | 0.09% | 0.07% | 171% | 106% | 0.11% | 0.11% | 0.10% | 95% | 95% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | -1.17% | 0.00% | -0.05% | 160% | 104% | 0.10% | 0.08% | 0.08% | 91% | 93% |
| Class F | -0.59% | -0.01% | -0.01% | 129% | 104% | 0.05% | 0.04% | 0.04% | 93% | 97% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.62% | 0.05% | 0.12% | 148% | 97% | 0.10% | 0.13% | 0.14% | 89% | 93% |
| Class C | -1.02% | -0.03% | -0.04% | 184% | 109% | 0.09% | 0.07% | 0.09% | 91% | 95% |
| Class E | -1.43% | 0.18% | 0.12% | 154% | 107% | 0.17% | 0.30% | 0.28% | 93% | 93% |
| Overall | -0.95% | 0.06% | 0.07% | 161% | 104% | 0.11% | 0.15% | 0.16% | 91% | 94% |
| Class D | -1.35% | -0.09% | -0.05% | 179% | 109% | 0.05% | 0.05% | 0.09% | 93% | 95% |
| Class F | -0.89% | -0.07% | -0.01% | 161% | 119% | -0.01% | -0.10% | 0.06% | 92% | 95% |

|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over VTM-3.0 + inter MTS |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.59% | 0.06% | 0.10% | 166% | 107% | 0.10% | 0.13% | 0.14% | 86% | 93% |
| Class C | -1.02% | -0.04% | -0.06% | 183% | 108% | 0.07% | 0.08% | 0.08% | 89% | 103% |
| Class E | -1.32% | 0.15% | 0.10% | 170% | 108% | 0.14% | 0.29% | 0.25% | 91% | 96% |
| Overall | -0.91% | 0.05% | 0.05% | 173% | 108% | 0.10% | 0.15% | 0.14% | 88% | 97% |
| Class D | -1.30% | -0.02% | -0.01% | 172% | 101% | 0.06% | 0.05% | 0.13% | 90% | 100% |
| Class F | -0.92% | -0.12% | -0.01% | 158% | 108% | 0.06% | -0.03% | 0.08% | 99% | 109% |

# Patent rights declaration

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

It is assumed that “zero-out” is applied for transforms of dimension 64.

The CE6-1.4 code has a larger number because it is also able to support transforms of dimension 64 without zero-out. Table II contains the smallest memory requirement for VTM-3.0 with zero-out.

Encoder settings were 256 frames and temporal subsampling equal to 8.