| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0556-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-M0417 (CE8-related: Combination test of CE8.2.2 and CE8.2.5) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chen-Yen Lai No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 jenny.lai@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The document reports the results of crosschecking JVET-M0417 proposed by Alibaba Cloud Computing Ltd. JVET-M0417 reports the results of the combination of CE8.2.2 (palette mode and intra mode combination) and CE8.2.5 (separated palette design for the joint CUs containing both luma and chroma CBs).The verification task was done, and the BD-rate results in CPR off condition match exactly with those in JVET-M0417; however, the BD-rate results in CPR on condition are slightly different from those in JVET-M0417 because of a CPR-related bug in the VTM software. The encoding time percentages and decoding time percentages are not trustable.

# Verification

Alibaba Cloud Computing Ltd. provided the source code of their proposed method described in JVET-M0417 [1].

The combination is conducted under the the common test conditions (CTC) defined in [2], Class F, and Class SCC 1080p. It is performed on top of CE8.2.1 (JVET-M0050), and tested with/without CPR. Table 1 shows the results with CPR turned off in the setting and Table 2 shows the results with CPR turned on in the setting. Both results are compared with VTM3.0 and CE8.2.1.

The BD-rates of tests in CPR off condition from our experimental results match exactly of those in JVET-M0417; however, the BD-rates of tests in CPR on condition are slightly different from those in JVET-M0417, which is caused by a CPR-related bug in the VTM software. The encoding time percentages and decoding time percentages are not trustable.

Table 1: Results of the CE8.2.2+CE8.2.5 test (CPR is turned off)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over CE8.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.09% | 94% | 85% | -0.01% | -0.05% | 0.01% | 87% | 86% |
| Class A2 | 0.09% | 0.05% | 0.11% | 94% | 87% | -0.01% | -0.09% | 0.03% | 88% | 88% |
| Class B | 0.08% | 0.08% | 0.09% | 93% | 86% | -0.01% | -0.01% | 0.02% | 87% | 86% |
| Class C | 0.09% | 0.09% | 0.06% | 93% | 90% | -0.01% | 0.00% | 0.14% | 86% | 84% |
| Class E | 0.11% | 0.23% | 0.21% | 94% | 86% | -0.01% | 0.07% | 0.01% | 88% | 85% |
| Overall | 0.08% | 0.09% | 0.11% | 93% | 87% | -0.01% | -0.01% | 0.05% | 87% | 86% |
| Class D | 0.12% | 0.10% | 0.14% | 93% | 89% | 0.01% | 0.03% | 0.12% | 87% | 86% |
| Class F | -11.65% | -9.08% | -9.19% | 101% | 76% | -0.30% | -0.19% | -0.19% | 86% | 85% |
| Class SCC 1080p | -34.47% | -28.70% | -28.82% | 102% | 57% | -2.45% | -1.71% | -1.72% | 78% | 85% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  | Over CE8.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.10% | 0.06% | 0.10% | 96% | 78% | -0.02% | -0.11% | -0.03% | 92% | 79% |
| Class A2 | 0.09% | 0.13% | 0.24% | 95% | 78% | -0.02% | -0.10% | 0.01% | 91% | 78% |
| Class B | 0.12% | 0.26% | 0.00% | 95% | 73% | -0.02% | -0.02% | 0.10% | 90% | 73% |
| Class C | 0.17% | 0.09% | 0.26% | 95% | 79% | 0.00% | -0.01% | 0.05% | 88% | 74% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.12% | 0.15% | 0.14% | 95% | 76% | -0.01% | -0.05% | 0.05% | 90% | 75% |
| Class D | 0.23% | -0.10% | 0.25% | 96% | 76% | 0.03% | 0.06% | 0.00% | 88% | 74% |
| Class F | -8.94% | -8.23% | -8.68% | 97% | 61% | -0.54% | -0.26% | -0.42% | 88% | 61% |
| Class SCC 1080p | -17.87% | -17.28% | -17.34% | 93% | 54% | -2.81% | -3.20% | -3.42% | 88% | 58% |

Table 2: Results of the CE8.2.2+CE8.2.5 test (CPR is turned on)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  | Over CE8.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.18% | 0.01% | 96% | 87% | -0.01% | 0.00% | -0.07% | 87% | 82% |
| Class A2 | 0.09% | 0.10% | 0.11% | 94% | 86% | 0.00% | 0.03% | 0.00% | 85% | 82% |
| Class B | 0.08% | 0.12% | 0.11% | 92% | 87% | -0.01% | 0.04% | 0.05% | 87% | 85% |
| Class C | 0.10% | 0.20% | 0.10% | 92% | 83% | 0.02% | 0.09% | 0.03% | 86% | 82% |
| Class E | 0.13% | 0.18% | 0.02% | 90% | 82% | -0.01% | -0.10% | 0.00% | 85% | 81% |
| Overall | 0.09% | 0.15% | 0.08% | 93% | 85% | 0.00% | 0.02% | 0.01% | 86% | 83% |
| Class D | 0.08% | 0.31% | 0.06% | 92% | 83% | -0.01% | 0.12% | 0.11% | 85% | 80% |
| Class F | -4.49% | -2.41% | -2.46% | 94% | 79% | -0.03% | 0.05% | 0.05% | 85% | 82% |
| Class SCC 1080p | -13.42% | -8.20% | -8.17% | 93% | 70% | -0.92% | -0.31% | -0.36% | 82% | 82% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  | Over CE8.2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | 0.26% | 0.14% | 96% | 78% | 0.01% | 0.02% | 0.00% | 89% | 75% |
| Class A2 | 0.10% | -0.03% | 0.06% | 96% | 79% | -0.01% | -0.11% | 0.02% | 88% | 75% |
| Class B | 0.11% | 0.13% | 0.10% | 95% | 74% | 0.02% | 0.12% | -0.03% | 88% | 72% |
| Class C | 0.15% | -0.03% | 0.30% | 94% | 74% | -0.01% | -0.07% | 0.01% | 86% | 73% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.12% | 0.08% | 0.15% | 95% | 76% | 0.00% | 0.00% | 0.00% | 88% | 73% |
| Class D | 0.24% | 0.40% | 0.32% | 95% | 74% | -0.02% | 0.17% | 0.52% | 87% | 71% |
| Class F | -2.81% | -2.23% | -2.53% | 95% | 60% | -0.21% | 0.13% | 0.07% | 87% | 59% |
| Class SCC 1080p | -5.80% | -5.44% | -5.32% | 89% | 55% | -1.28% | -1.93% | -2.00% | 86% | 57% |

# References

- Y.-C. Sun, J. Lou, “CE8-related: Combination test of CE8.2.2 and CE8.2.5,” JVET-M0417, Jan. 2019.

- J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, the 12th JVET meeting, Oct. 2018, Macao, CN.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).