| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0558 |
| --- | --- |

| Title: | CE7-related: Template based Rice parameter derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz | Email: | martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes a unified template-based Rice parameter derivation for coding of abs_remainder and dec_abs_level syntax elements by introducing template-based Rice parameter derivation that is used for coding of dec_abs_level syntax element for coding of abs_remainder syntax element instead of current local HEVC style Rice parameter derivation. The simulations results show 0.XX% AI, 0.XX% RA, 0.XX% LB BD-Rate versus the VTM-3.0 anchor for common test conditions for Rice parameter derivation method.

# Introduction

In VTM-3.0 the coefficients that are completely coded in the bypass mode of the arithmetic coding engine using a Golomb-Rice code, i.e. dec_abs_level syntax element; the Rice parameter is derived based on the sum of absolute levels in a local template around the current transform coefficient level. The Rice parameter for coding the remainder (for coefficients that are not completely coded in bypass mode), i.e. abs_remainder syntax element, is derived similar to HEVC without referring to neighboring transform coefficient levels [1].

# Proposal

The proposal is integrated on top of VTM-3.0 [4].  For coding the non-binary syntax element abs_remainder [k], the sum of absolute values sumAbs in a local template is determined (the same template as in VTM-2 is used). The variable ricePar is determined by the table that is used for template-based Rice parameter derivation for coding of dec_abs_level syntax element in VTM-3.0. Since abs_remainder is half of remaining level after coding of sig_coeff_flag, abs_level_gt1_flag, par_level_flag, abs_level_gt3_flag, the Rice parameter from look-up table for coding of full levels in dec_abs_level coding is decremented by 1. This can be described by:

ricePar   = riceParTable[ min( 31, sumAbs ) ] 
ricePar = ricePar > 0 ? ricePar -1 : 0

where the riceParTable[] is given by

riceParTable[32] = {0,0,0,0,0,0,0,1,1,1,1,1,1,1,2,2,2,2,2,2,2,2,2,2,2,2,2,2,3,3,3,3};

# Results

The proposed scheme is implemented on top of VTM-3.0 software [4].  The simulations were carried out under the JVET common test conditions [3][5].

The test results for JVET common test conditions is tabulated in Table 1 for template based Rice parameter derivation for coding of abs_remainder syntax element.

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.02% | 101% | 102% |
| Class A2 | 0.02% | 0.03% | 0.05% | 101% | 101% |
| Class B | 0.01% | 0.03% | -0.03% | 102% | 102% |
| Class C | 0.01% | -0.08% | -0.04% | 101% | 101% |
| Class E | -0.02% | 0.02% | 0.02% | 101% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class D | 0.01% | 0.11% | -0.06% | 101% | 101% |
| Class F | 0.04% | 0.15% | 0.20% | 102% | 101% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM 3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusion

In this document a Rice parameter derivation method for coding of abs_remainder syntax element is presented that is based on neighbor template similar to the Rice parameter derivation for coding of dec_abs_level syntax.

# References

- H. Schwarz, M. Coban, C. Auyeung, “Description of Core Experiment 7 (CE 7): Quantization and coefficient coding,” JVET-L1027, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao, CN, October 2018.

- https://vcgit.hhi.fraunhofer.de/JVET-K-CE7/VVCSoftware_VTM/tree/CE7.1.2

- “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Macao, CN, October 2018.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).