| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0562 |
| --- | --- |

| Title: | Cross-check of JVET-M0436: AHG2: Regarding HMVP Table Size |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | Saurav Bandyopadhyay | Email: | saurav.bandyopadhyay@interdigital.com |
| Source: | InterDigital Inc. |  |  |

_____________________________

# Abstract

The document reports the cross-check for the results reported in input document JVET-M0436: AHG2: Regarding HMVP Table Size. The cross-check contribution reports the results of the proposed HMVP table size reduction from 6 to 5. This is because, a maximum of 5 HMVP candidates are allowed for adding the HMVP candidates to merge lists. The cross-check validated the experimental results.

# Introduction

The source code as well as the configuration, for JVET-M0436 [1], provided by the proponent LG Electronics were used for the cross-check. The test performed sets the MAX_NUM_HMVP_CANDS to 5 in the VTM3.0 software.

# Simulation Results

The simulation results reported in this contribution are based on the VTM-3.0 reference software and the JVET common test conditions [2] to collect the B-D rate, encoding and decoding time of the proposed techniques in the Random Access and Low Delay B configurations. The results are generated using configuration for JVET-M0436 that sets parameter MAX_NUM_HMVP_CANDS to 5.

Table 1. Random Access Main 10

|  | Over vtm-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 100% | 101% |

Low Delay B Main 10

|  | Over vtm-3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 100% | 100% |

# Conclusion

The results from this cross-check are equivalent to the results provided by the proponents in JVET-M0436.

# Reference

J. Zhao, “AHG2: Regarding HMVP Table Size”, JVET-M0436, Marrakech, MA, January 2019.

F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations”, JVET-L1010, Macao, CN, October 2018.

# Patent rights declaration(s)

InterDigital Inc. does not have any current or pending patent rights relating to the technology described in this contribution.