| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0573 |
| --- | --- |

| Title: | Crosscheck of JVET-M0480 (CE6-related: Implicit transform selection for Multi directional LM) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Kimihiko Kazui | Tel: Email: | +81 44 754 2049 kazui.kimihiko@fujitsu.com |
| Source: | FUJITSU LABORATORIES LTD. |  |  |

_____________________________

- Abstract

This contribution reports a crosscheck result of JVET-M0480 on “CE6-related: Implicit transform selection for Multi directional LM”. The code has been checked and the crosscheck in All-intra condition was carried out under the JVET common test condition (CTC). The results of JVET-M0480 are partially confirmed.

Introduction

In JVET-M0480[1], it is proposed to implicitly select the type of transform in Multi-Directional LM mode, considering the characteristics of residual signal. Specifically, DST-7 is always used for vertical direction in MDLM_T mode because only a vertically adjacent CU is used for deriving reference signal. Similarly, DST-7 is always used for horizontal direction in MDLM_L mode because only horizontally adjacent CU is used.

Simulation results

In this crosscheck, only the simulation of one specific testing condition is performed.

- All-intra condition

- (min, max) = (4, 16), where “min” and “max” are the minimum and maximum chroma CU size, respectively.

The result is summarized in Table 1. The BD-rate values are perfectly matched to those provided from the proponent. Similar running times (almost equal to 100%) are confirmed.

Table 1 Performance of JVET-M0480 (All-intra condition, (min, max) = (4, 16))

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.48% | -0.33% | 100% | 95% |
| Class A2 | 0.00% | -0.52% | -0.43% | 101% | 97% |
| Class B | 0.01% | -0.47% | -0.57% | 100% | 100% |
| Class C | 0.00% | -0.35% | -0.53% | 101% | 96% |
| Class E | 0.01% | -0.46% | -0.49% | 98% | 97% |
| Overall | 0.00% | -0.45% | -0.48% | 100% | 97% |
| Class D | 0.00% | -0.33% | -0.43% | 101% | 97% |
| Class F | 0.00% | -0.23% | -0.18% | 108% | 99% |

Conclusion

It was confirmed that the the simulation result of All-intra condition matches with those provided by the proponent.

References

[1] S. Iwamura, S. Nemoto, A. Ichigaya, “CE6-related: Implicit transform selection for Multi directional LM” JVET-M0480, January 2019

Patent rights declaration(s)

Fujitsu Limited does not have any current or pending patent rights relating to the technology described in this contribution.