| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0574 |
| --- | --- |

| Title: | Cross-check of JVET-M0264 (Non-CE4: Harmonization between HMVP and GBi) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jie Zhao 10225 Willow Creek Rd, San Diego, CA 92131, U.S.A | Tel: Email: | 1-201-816-2000 jie.zhao@lge.com |
| Source: | LG Electronics. |  |  |

_____________________________

# Abstract

This is cross-check report for JVET-M0264 “Non-CE4: Harmonization between HMVP and GBi”. We examined the code and performed the cross-check. Code is as described and result match with what is provided in JVET-M0264.

# Introduction

JVET-M0264 proposes to store the coded bi-prediction weight index in addition to the motion information for all HMVP candidates, and carry it through the HMVP table to be used in merge candidate generation process [1]. The software package was provided by proponent. We verified that the code is as what is described in JVET-M0264.

# Experimental Results

Code is compiled and tested following JVET common test condition [2], and compared with VTM3 anchor. Our results match with what is provided in JVET-M0264.

Table 1 Result from Cross-checker

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.04% | -0.06% | 100% | 101% |
| Class A2 | 0.00% | -0.03% | 0.06% | 100% | 101% |
| Class B | -0.03% | -0.01% | -0.05% | 100% | 99% |
| Class C | 0.00% | 0.01% | 0.03% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.02% | -0.01% | 100% | 101% |
| Class D | 0.01% | -0.03% | -0.04% | 101% | 104% |
| Class F | 0.00% | -0.01% | 0.05% | 101% | 102% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.25% | -0.10% | 101% | 103% |
| Class C | -0.02% | 0.21% | 0.04% | 101% | 102% |
| Class E | 0.04% | -0.36% | -0.10% | 101% | 101% |
| Overall | -0.01% | -0.12% | -0.05% | 101% | 102% |
| Class D | -0.05% | -0.16% | -0.11% | 101% | 103% |
| Class F | -0.06% | 0.02% | -0.14% | 101% | 103% |

# Reference

- J. Li, S. Wang, W. Gao, L. Zhang, K. Zhang, H. Liu, J. Xu, X. Xiu, J. Luo and Y. He, “Non-CE4: Harmonization between HMVP and GBi,” JVET-M0264, Marrakech, MA, Jan 2019.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team (JVET), JVET-L1010 Macao, CN, Oct 2018.