| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-Mxxx |
| --- | --- |

| Title: | Crosscheck of JVET-M0109 (CE12-related: block-based in-loop luma reshaping) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Cross-check Report |  |  |
| Author(s) or Contact(s): | Taoran Lu, Peng Yin 432 Lakeside Drive, Sunnyvale, CA, 94085 US | Tel: Email: | tlu@dolby.com pyin@dolby.com |
| Source: | Dolby Laboratories, Inc., |  |  |

_____________________________

# Abstract

This contribution reports the cross-checking results for JVET-M0109 (CE12-related: block-based in-loop luma reshaping). The simulation results reportedly match those provided by the proponents for both SDR content and HDR content.

# Introduction

JVET-M0109 [1] proposes a block-based implementation of the in-loop luma reshaping approach based on CE12-2 [2]. For each 4x4 block, linear model parameters are computed from three luma values and their mapped version. The linear model is then applied to the block samples.

# Experimental Results

The simulations were performed based on CE12-2 software [2]. Simulation results match those provided by the proponents for both SDR and HDR content. All the others are exactly the same. More details can be found in the spreadsheets in the contribution. The encoding and decoding time are not reliable. So they are not included in the excel sheet.

Table 1. BD-rate Results for SDR content

|  | All Intra Main10 |  |  |
| --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |
|  | Y | U | V |
| Class A1 | -1.56% | 0.32% | -0.93% |
| Class A2 | -1.53% | 1.27% | 1.39% |
| Class B | -0.60% | 1.20% | 0.78% |
| Class C | -0.28% | 2.43% | 2.53% |
| Class E | 0.03% | 2.13% | 2.18% |
| Overall | -0.74% | 1.49% | 1.22% |
| Class D | -0.11% | 1.63% | 1.49% |
| Class F | -0.12% | 1.43% | 1.51% |
|  | Random Access Main 10 |  |  |
|  | Over VTM-3.0 |  |  |
|  | Y | U | V |
| Class A1 | -0.83% | 2.74% | 2.74% |
| Class A2 | -1.20% | 2.71% | 2.83% |
| Class B | -1.44% | 0.31% | -0.20% |
| Class C | -1.08% | 1.15% | 1.71% |
| Overall | -1.18% | 1.50% | 1.50% |
| Class D | -0.94% | 2.08% | 2.22% |
| Class F | -0.76% | -0.59% | -0.41% |
|  | Low delay B Main10 |  |  |
|  | Over VTM-3.0 |  |  |
|  | Y | U | V |
| Class B | -1.01% | 2.34% | 2.17% |
| Class C | -0.39% | 2.13% | 2.17% |
| Class E | 0.02% | 0.02% | 0.02% |
| Overall | -0.54% | 1.69% | 1.63% |
| Class D | -0.79% | 2.62% | 3.21% |
| Class F | 0.26% | 1.11% | 1.09% |

Table 2. BD-rate Results for HDR content

|  | All Intra |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | wPsnrY | PSNRL100 | DE100 | wPsnrY | PSNRL100 | DE100 |
| Average HDR-B | -0.2% | -1.0% | 1.5% | 0.2% | 0.6% | 3.0% |

# References

- E, François, C. Chevance, F. Hiron, “CE12-related: block-based in-loop luma reshaping”, JVET-M0109, Jan 2019, Marrakech, MA.

- T. Lu, F. Pu, P. Yin, W. Husak, S. McCarthy, T. Chen, “CE12: Mapping functions (test CE12-1 and CE12-2)”, JVET-M0427, Jan 2019, Marrakech, MA.