| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, MA, 9–18 Jan. 2019 | Document: JVET-M0601 | Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-M0xxx |
| --- | --- | --- | --- |

| Title: | Crosscheck of JVET-M0228 (CE2-related: Affine mode simplifications) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Tsui-Shan Chang, Yu-Chen Sun, Jian Lou | Tel: Email: | +86-18667020363 tsuishan.cts@alibaba-inc.com |
| Source: | Alibaba Cloud Computing Ltd. |  |  |

_____________________________

- Abstract

This document reports crosscheck results of JVET-M0228 (CE2-related: Affine mode simplifications). The crosscheck results match the ones provided by the proponent.

Introduction

In JVET-M0228, two modifications are proposed. First, the motion vector comparisons between control points are removed when generating constructed affine merge candidates. Second, the affine motion model is used to derive the motion vectors for each 4x4 chroma subblock.

Simulation results

Simulation results with RA and LDB configurations for test 1 and test 2 over VTM-3.0 are shown below.

Table 1: Experimental results for Test 1 under RA

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.07% | 0.08% | 99% | 101% |
| Class A2 | -0.03% | 0.08% | 0.10% | 99% | 99% |
| Class B | -0.01% | 0.04% | -0.01% | 99% | 97% |
| Class C | 0.00% | 0.00% | 0.03% | 101% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.01% | 0.04% | 100% | 100% |
| Class D | 0.01% | -0.21% | -0.12% | 100% | 100% |
| Class F | -0.01% | -0.01% | 0.05% | 101% | 101% |

Table 2: Experimental results for Test 1 under LDB

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.19% | -0.02% | 99% | 98% |
| Class C | 0.01% | 0.24% | 0.15% | 99% | 94% |
| Class E | -0.03% | 0.50% | -0.06% | 99% | 98% |
| Overall | 0.01% | 0.12% | 0.03% | 99% | 96% |
| Class D | 0.02% | -0.13% | 0.29% | 100% | 100% |
| Class F | 0.00% | 0.24% | 0.45% | 100% | 97% |

Table 3: Experimental results for Test 2 under RA

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.00% | -0.05% | 98% | 99% |
| Class A2 | -0.01% | 0.07% | 0.13% | 99% | 99% |
| Class B | -0.01% | 0.13% | 0.04% | 99% | 96% |
| Class C | 0.02% | 0.00% | 0.07% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | 0.06% | 0.05% | 99% | 99% |
| Class D | 0.01% | -0.03% | -0.08% | 102% | 103% |
| Class F | 0.00% | 0.02% | 0.02% | 103% | 103% |

Table 4: Experimental results for Test 2 under LDB

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.21% | -0.04% | 103% | 101% |
| Class C | 0.03% | 0.10% | 0.14% | 101% | 97% |
| Class E | -0.03% | 0.23% | 0.22% | 100% | 94% |
| Overall | 0.01% | 0.00% | 0.09% | 102% | 98% |
| Class D | 0.04% | -0.32% | -0.18% | 102% | 102% |
| Class F | 0.09% | 0.35% | 0.25% | 101% | 94% |

References

- Y. –W. Chen, X. Wang, “CE2-related: Affine mode modifications,” JVET-M0228, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.