| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0602 |
| --- | --- |

| Title: | Cross-check of JVET-M0526: CE2-related: Further simplification of ATMVP collocated block derivation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | Saurav Bandyopadhyay | Email: | saurav.bandyopadhyay@interdigital.com |
| Source: | InterDigital Inc. |  |  |

_____________________________

# Abstract

The cross-check results in the input document JVET-M0526: CE2-related: Further simplification of ATMVP collocated block derivation are investigated in this report. The contribution reports results on the average and worst-case complexity reduction for ATMVP by disabling the reference list loop during the derivation of the co-located block in the case of bi-directional prediction. The cross-check validated the experimental results from the ATMVP simplifications.

# Introduction

The source code as well as the configuration, for JVET-M0526 [1], provided by the proponent SZ DJI Technology Co. Ltd. and Peking University were used for the cross-check. The contribution simplifies the computational complexity by disabling the reference list loop during co-located block derivation process during bi-directional prediction.

# Simulation Results

The simulation results reported in this contribution are based on the VTM-3.0 reference software and the JVET common test conditions [2] to collect the B-D rate, encoding and decoding time of the proposed techniques in the Random Access and Low Delay B configurations. The results are generated using configuration for JVET-M0526 provided by the proponents for the zero-out technique.

## SKIP_ATMVP_REFLIST_LOOP enabled

Random Access

|  | Over vtm3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.06% | -0.07% | 101% | 101% |
| Class A2 | 0.01% | 0.01% | 0.08% | 102% | 101% |
| Class B | 0.01% | 0.06% | -0.02% | 101% | 100% |
| Class C | 0.04% | 0.07% | 0.04% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.03% | 0.01% | 101% | 101% |
| Class D | 0.04% | 0.03% | 0.10% | 100% | 100% |
| Class F (optional) | 0.01% | 0.01% | 0.02% | 100% | 100% |

Low Delay B

|  | Over vtm3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | 0.03% | 0.24% | 0.23% | 100% | 97% |
| Class E | 0.03% | 0.09% | -0.35% | 100% | 97% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | 0.04% | -0.18% | 0.59% | 100% | 98% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

# Conclusion

The results from this cross-check are equivalent to the results provided by the proponents in JVET-M0526.

# Reference

S. Wang, X. Zheng, S. Wang, S. Ma, “CE2-related: Further simplification of ATMVP collocated block derivation”, JVET-M0526, Marrakech, MA, January 2019.

F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations”, JVET-L1010, Macao, CN, October 2018.

# Patent rights declaration(s)

InterDigital Inc. does not have any current or pending patent rights relating to the technology described in this contribution.