| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0603 |
| --- | --- |

| Title: | Crosscheck of JVET-M0192 (CE2-related: MV Derivation for Affine Chroma) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Guichun Li 2747 Park Boulevard, Palo Alto, CA 94306, U.S.A. | Tel: Email: | +1-650-798-3300 guichunli@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This document reports the crosscheck results for JVET-M0192, CE2-related: MV Derivation for Affine Chroma. The software was studied, and the implementation is as described in JVET-M0192. The simulation results match with the results provided by the proponent.

# Introduction

It was proposed in JVET-M0192 to simplify the MV derivation process for chroma sub-block of an affine CU. The software was provided by the proponent and we have verified that the code was implemented as described in JVET-M0192. It proposes several options simplify the MV derivation process and address the asymmetric rounding issue: 1) Use only the top-left luma sub-block’s MV as the representative MV 2) Use the average of the top-left and bottom-right luma sub-blocks’ MVs as the representative MV 3) Modify the current averaging process of four luma sub-block MVs to a symmetric one 4) Use the average of the top-left and bottom-right luma sub-blocks’ MVs as the representative MV while using symmetric rounding.

# Experimental Results

The software was built without issue. The simulations were conducted following JVET common test conditions [1]. VTM-3.0 is used as anchor for the tests.

4 tests were proposed to verify the corresponding proposed methods in JVET-M0192.

Our test results match with the results provided in JVET-M0192 for all 4 proposed tests.

Table 2.1 Simulation Result of test 1

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.03% | 0.08% | 98% | 98% |  |  |  |  |  |
| Class A2 | 0.09% | 0.76% | 0.60% | 98% | 98% |  |  |  |  |  |
| Class B | 0.01% | 0.27% | 0.09% | 98% | 96% | 0.04% | -0.14% | 0.06% | 99% | 97% |
| Class C | 0.03% | 0.05% | 0.05% | 99% | 98% | 0.04% | 0.10% | 0.05% | 101% | 95% |
| Class E |  |  |  |  |  | 0.09% | 0.13% | 0.38% | 98% | 94% |
| Overall | 0.03% | 0.25% | 0.18% | 98% | 97% | 0.05% | 0.01% | 0.14% | 99% | 96% |
| Class D | 0.00% | 0.04% | 0.04% | 99% | 98% | 0.03% | -0.29% | 0.19% | 99% | 97% |
| Class F | 0.02% | 0.31% | 0.33% | 99% | 98% | 0.02% | 0.46% | 0.62% | 99% | 95% |

Table 2.2 Simulation Result of test 2

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.04% | 0.01% | 97% | 98% |  |  |  |  |  |
| Class A2 | -0.01% | -0.03% | 0.07% | 97% | 97% |  |  |  |  |  |
| Class B | 0.00% | 0.04% | 0.07% | 98% | 96% | 0.03% | -0.18% | 0.47% | 100% | 98% |
| Class C | -0.01% | -0.08% | -0.09% | 99% | 98% | 0.02% | 0.29% | 0.20% | 99% | 95% |
| Class E |  |  |  |  |  | 0.01% | -0.03% | -0.01% | 98% | 93% |
| Overall | -0.01% | -0.02% | 0.01% | 98% | 97% | 0.02% | 0.01% | 0.26% | 99% | 96% |
| Class D | 0.01% | -0.06% | -0.07% | 99% | 99% | 0.03% | 0.42% | 0.19% | 99% | 98% |
| Class F | 0.01% | -0.01% | 0.00% | 99% | 98% | 0.13% | -0.28% | 0.11% | 98% | 95% |

Table 2.3 Simulation Result of test 3

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | 0.04% | 98% | 98% |  |  |  |  |  |
| Class A2 | -0.02% | 0.07% | 0.10% | 98% | 98% |  |  |  |  |  |
| Class B | -0.02% | 0.13% | -0.02% | 98% | 96% | 0.03% | -0.16% | -0.13% | 99% | 98% |
| Class C | 0.02% | 0.04% | -0.06% | 99% | 98% | 0.02% | 0.23% | 0.13% | 100% | 94% |
| Class E |  |  |  |  |  | 0.03% | 0.19% | 0.07% | 98% | 95% |
| Overall | 0.00% | 0.07% | 0.01% | 98% | 98% | 0.03% | 0.06% | 0.01% | 99% | 96% |
| Class D | 0.01% | -0.07% | -0.05% | 99% | 98% | -0.01% | 0.22% | 0.20% | 101% | 98% |
| Class F | 0.02% | 0.01% | 0.05% | 100% | 99% | -0.09% | 0.20% | 0.01% | 99% | 96% |

Table 2.4 Simulation Result of test 4

|  | Random access Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.00% | 0.00% | 98% | 99% |  |  |  |  |  |
| Class A2 | 0.00% | 0.10% | 0.10% | 98% | 98% |  |  |  |  |  |
| Class B | -0.01% | 0.07% | -0.01% | 99% | 96% | 0.02% | -0.22% | -0.04% | 99% | 97% |
| Class C | 0.01% | 0.07% | -0.03% | 99% | 98% | 0.06% | 0.24% | 0.36% | 100% | 94% |
| Class E |  |  |  |  |  | 0.00% | 0.10% | 0.08% | 99% | 93% |
| Overall | 0.00% | 0.06% | 0.01% | 99% | 98% | 0.03% | 0.01% | 0.12% | 99% | 95% |
| Class D | 0.00% | -0.09% | -0.01% | 100% | 100% | -0.01% | -0.18% | 0.78% | 100% | 97% |
| Class F | 0.00% | -0.01% | 0.01% | 99% | 98% | -0.04% | -0.11% | -0.43% | 99% | 94% |

# Reference

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, Document of Joint Video Experts Team (JVET), JVET-L1010 Macao, CN, Oct 2018.

- A. Tamse, M. W. Park, “CE2-related: MV Derivation for Affine Chroma”, Document of Joint Video Experts Team (JVET), JVET-M0192 Marrakech, MA, Jan 2019.