| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, Morocco, 9–18 Jan. 2019 | Document: JVET-M0611 |
| --- | --- |

| Title: | Crosscheck of JVET-M0177 (CE10.1.4: Simplification of combined inter and intra prediction) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Biao Wang | Tel: Email: | biao.wang@huawei.com |
| Source: | Huawei Technologies Co.,Ltd. |  |  |

_____________________________

# Abstract

This contribution reports a cross-checking of contribution JVET-M0177 (CE10.1.4: Simplification of combined inter and intra prediction) related to simplification on combined inter-intra prediction mode.

The BD-rate reported by the proponents and the cross-checker matches. The cross-checker received the source code before the meeting and performed the cross-check during the meeting. The encoding and decoding time measured by the cross-checker are in general matches.

# Introduction

The contribution disables Combined Inter-Intra prediction (CIIP) for coding units (CUs) with luma coding block (CB) sizes larger than 1024 luma samples for reducing intra prediction buffer size when CIIP is applied.

# Experimental Results

The cross-check test is performed on top of VTM-3.0 with Common Test Conditions (CTC) configurations. The results are shown in the following tables, and they match the results shown in JVET-M0296. Table 1 shows the results of the test in RA and LDB configurations

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | -0.09% | 0.05% | 98% | 100% |
| Class A2 | 0.00% | 0.02% | 0.07% | 99% | 100% |
| Class B | -0.02% | 0.05% | 0.00% | 99% | 100% |
| Class C | 0.00% | 0.05% | 0.09% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.02% | 0.05% | 99% | 100% |
| Class D | 0.01% | -0.02% | 0.03% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.03% | 99% | 102% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.26% | 0.27% | 99% | 100% |
| Class C | 0.03% | 0.21% | 0.16% | 100% | 100% |
| Class E | 0.09% | 0.00% | -0.13% | 99% | 101% |
| Overall | 0.04% | -0.04% | 0.13% | 99% | 100% |
| Class D | 0.02% | 0.37% | -0.17% | 100% | 101% |
| Class F | 0.05% | 0.27% | -0.18% | 99% | 101% |

Table 1: Test results of RA and LDB configurations of M0177.

# Reference

[1]“JVET common test conditions and software reference configurations for SDR video”, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring. JVET-L1010, Nov. 2018.

[2] “Versatile Video Coding (Draft 2)”, B. Bross, J. Chen, S. Liu doc. JVET-K1001, April 2018

[3] “Description of Core Experiment 3 (CE3):  Intra prediction and mode coding”, G. Van der Auwera, J. Heo, A. Filippov, doc. JVET-K1023, September 2018

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).