| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0634-v3 |
| --- | --- |

| Title: | Affine motion mode in intra coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Siqi Cao, Haiyang Han, Jun Wang, Fan Liang, Yuanfang Yu, Yang Liu | Email: | hanhy6@mail2.sysu.edu.cn wangj387@mail.sysu.edu.cn liangfan@china-avs.com.cn yuyuanfang@oppo.com serena@oppo.com |
| Source: | NELVT Guangzhou Research and Industrialization Center, Sun Yat-Sen University,OPPO |  |  |

_____________________________

- Abstract

In the contribution, the results of affine motion mode in intra coding are reported. This proposed technique combines the affine motion mode with current picture referencing (CPRAffine). The use of CPRAffine is signaled by using a reference picture index pointing to this current reference picture. In this way, the syntax structure and decoding process of CPRAffine mode are aligned with the regular intra mode. The proposed test reportedly shows BD-rate gain under AI for VTM anchors are:

- -0.08%/-0.06%/-0.00% for Y, U, V component of Class F, separately.

Introduction

Current picture referencing (CPR) method was previously proposed in JVET-K0076 and implemented on the BMS software version 1.0 [1] to achieve efficient content coding. Affine motion inter prediction method was proposed in MPEG doc. m37525 [2] to address the need for non-translational motion coding in inter coding. In this contribution, it is proposed an affine motion prediction method for a block in intra coding based on CPR mode. The basic idea is that when the CPR mode is used then the CPRAffine mode is performed to match a non-translational block in the current picture.

Algorithm details

Encoder aspect:

The encoder performs RD check only for block sizes with both side equal to 16. After CPR mode, the CPRAffine mode is performed using affine motion search. The CPRAffine mode is the same with the affine motion inter prediction and the only outstanding difference is that the reference picture is the current picture itself. No more flag need to be coded.

Decoder aspect:

The bitstream syntax in this approach shares the same syntax with the affine motion inter prediction. If the mode is affine motion inter prediction and the reference picture is the current picture then perform the decoding process unified with inter coding.

Simulations results

The simulation was carried out under the JVET common test condition [3]. In addition to the sequences in class F are to be tested.

Results of VTM3.0 with CPR Affine vs VTM3.0 without CPR Affine

|  |  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| F | BasketballDrillText | -0.01% | -0.02% | -0.01% |  |  |
|  | ArenaOfValor | -0.00% | -0.02% | -0.00% |  |  |
|  | SlideEditing | -0.21% | -0.15% | -0.18% |  |  |
|  | SlideShow | -0.10% | -0.04% | -0.20% |  |  |
|  | All Average | -0.08% | -0.06% | -0.00% | 213% |  |

References

- https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0/

- S. Lin, H. Chen, H. Zhang, S. Maxim, H. Yang, J. Zhou, “Affine transform prediction for next generation video coding”, MPEG doc. m37525 and ITU-T SG16 Doc. COM16–C1016, Oct. 2015.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, JVET-J1010, San Diego, CA, April 2018

Patent rights declaration(s)

OPPO and NELVT Guangzhou Research and Industrialization Center may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sun Yat-Sen University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).