| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0646-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-M0251 (Non-CE7: Last position coding for large block-size transforms) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Heiko Schwarz Fraunhofer HHI Einsteinufer 37, 10587 Berlin Germany | Tel: Email: | +49 30 31002 226 heiko.schwarz@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports the crosscheck results for version 5 of JVET-M0251 “Non-CE7: Last position coding for large block-size transforms”.  The cross checker reports that the source code, the description, and the provided Working Draft text were carefully analyzed.  The cross checker confirms that the software implementation is consistent with the description and the provided Working Draft text. The simulation results provided in JVET-M0251 could be reproduced exactly.

# Summary of Proposal JVET-M0251

In VTM-3.0, the following issue can be observed. For MxN block with M>32 or N>32, all transform coefficient levels outside the Min(M,32)xMin(N,32) top-left region are forced to be equal to 0. Nonetheless, the syntax allows to code transform coefficients unequal to 0 outside this region. The proposal in the input document JVET-M0251 [1] solves this issue as follows:

- the coding of the last significant coefficient position is modified in a way that only positions inside the top-left Min(M,32)xMin(N,32) region can be transmitted;

- the coded subblocks flags outside the top-left Min(M,32)xMin(N,32) region are not coded, but inferred to be equal to 0. This is equivalent to restricting the scan region to the top-left Min(M,32)xMin(N,32) region.

The suggested modifications have the effect that the top-left Min(M,32)xMin(N,32) region of larger blocks is coded in exactly the same way as an Min(M,32)xMin(N,32) block.

# Source Code Analysis

The description in JVET-M0251 [1], the Working Draft text provided in version 5 of JVET-M0251, and the source code provided by LGE have been carefully analyzed.  The source code was found to be consistent with the description and the provided Working Draft text.

# Simulation results

The cross check results for the common test conditions [2] are summarized in Table 1.  The results reported in JVET-M0251 could be reproduced exactly.

Table 1: Summary of crosscheck results (using the fix avoiding usage of uninitialized memory).

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | -0.07% | 0.03% | 100% | 101% |
| Class A2 | 0.02% | 0.00% | -0.03% | 100% | 100% |
| Class B | 0.01% | 0.05% | -0.02% | 100% | 102% |
| Class C | 0.00% | 0.01% | -0.03% | 100% | 101% |
| Class E | 0.02% | -0.02% | -0.03% | 100% | 99% |
| Overall | 0.01% | 0.00% | -0.02% | 100% | 101% |
| Class D | 0.00% | 0.01% | -0.01% | 100% | 100% |
| Class F (optional) | -0.01% | 0.03% | 0.02% | 100% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.09% | 0.00% | 99% | 104% |
| Class A2 | 0.01% | 0.07% | 0.03% | 100% | 102% |
| Class B | -0.01% | -0.02% | -0.04% | 100% | 100% |
| Class C | 0.00% | 0.02% | -0.01% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.01% | -0.01% | 100% | 101% |
| Class D | 0.00% | 0.03% | 0.06% | 100% | 100% |
| Class F (optional) | -0.01% | -0.03% | -0.07% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.31% | -0.05% | 100% | 106% |
| Class C | -0.01% | 0.19% | 0.20% | 100% | 100% |
| Class E | 0.11% | 0.29% | -1.33% | 100% | 99% |
| Overall | 0.03% | 0.01% | -0.29% | 100% | 102% |
| Class D | -0.01% | 0.26% | -0.07% | 100% | 98% |
| Class F (optional) | 0.06% | 0.02% | 0.13% | 100% | 101% |

# References

- J. Choi, J. Heo, S. Yoo, J. Choi, L. Li, J. Lim, S. Kim, “Non-CE7: Last position coding for large block-size transforms,” Joint Video Experts Team, doc. JVET-M0251, Marrakech, MA, Jan. 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Joint Video Experts Team, doc. JVET-L1010, Macau, CN, Oct. 2018.