| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0688 |
| --- | --- |

| Title: | Cross-check of JVET-M0305: "CE7-related: Joint coding of chrominance residuals" |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Christophe Gisquet Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: |  |
| Source: | Canon |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results of JVET-M0305 on joint coding of chrominance residuals. It is reported that similar performance to the ones obtained by the proponents is obtained. The software has been examined and seems to implement what is described in the contribution.

# Summary of JVET-M0305

## Method

In JVET-M0305, a new CABAC-coded flag with a single context at the TU level is added to signal the new residual coding method, when there is chroma residual. In this case, a joint residual is coded, and Cb residual is set to it, and Cr set as its opposite.

## Verification of source code

The proposed modifications are implemented on top of VTM-3.0. The source code have been analysed and each modification seems match the description.

The biggest code additions, as can be expected, are RDO tests during intra or inter coding. Some comments are present to explain for instance some storing operations. The only tuning, as commented in the code, is that the lambda is also modified to 75% of the value used for chroma analysis with the rationale that each coefficient is used for both Cb and Cr residuals.

# Experimental results

The experimental results are reported in the attached Excel files and the results match exactly with those provided by the proponent for the 4 different macro settings. But the processing runtimes for encoding and decoding are not very accurate in these excel files.

The experimental results are provided in this contribution for AI, RA, LDB and LDP configurations.

It should be noted that the decoding numbers are inaccurate.

# Conclusion

Our cross-check validates the obtained results.