| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, MA, 9–18 Jan. 2019 | Document: JVET-M0721-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-M0444 (CE4-related: Simplified symmetric MVD based on CE4.4.3) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Huanbang Chen Timofey Solovyev | Tel: Email: | +8615017554651 chenhuanbang@huawei.com solovyev.timofey@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

This contribution is a crosscheck report of Crosscheck of JVET-M0444 (CE4-related: Simplified symmetric MVD based on CE4.4.3). The simulations were carried out based on the common test condition (CTC). The test results are matched for all points and the software matches the description in the proposal.

Introduction

JVET-M0444 proposes a simplified symmetric MVD mode for bi-prediction based on the corresponding CE test CE4.4.3. With the symmetric MVD mode, MVD of list 1 is not signaled but derived from the signaled MVD of list 0, and reference picture indices of both list 0 and list 1 are not signaled but derived based on the POC of reference pictures and the current picture. Two items are proposed in this contribution: (1) Disabling bi-directional optical flow (BDOF) for symmetric MVD mode;(2) encoder optimizations to improve the performance. The simulation results reportedly show on average 0.33% BD rate saving with 105% encoding time for RA configuration over VTM 3.0.Simulation results

Table 1. Summary results of the proposed algorithm

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.34% | -0.44% | -0.46% | 106% | 100% |
| Class A2 | -0.42% | -0.41% | -0.41% | 106% | 100% |
| Class B | -0.34% | -0.32% | -0.33% | 106% | 100% |
| Class C | -0.25% | -0.27% | -0.22% | 105% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.33% | -0.35% | -0.34% | 106% | 100% |
| Class D | -0.17% | -0.29% | -0.18% | 105% | 100% |
| Class F (optional) | -0.33% | -0.33% | -0.29% | 105% | 100% |

References

- J. Luo, and Y. He, “CE4-related: Simplified symmetric MVD based on CE4.4.3”, JVET-M0444, Jan. 2019, Marrakesh, MA.

Patent rights declaration(s)

Huawei Technologies Co., Ltd does not have any current or pending patent rights relating to the technology described in this contribution.