| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0754 |
| --- | --- |

| Title: | Cross-check of JVET-M0345: CE4-related: Remove redundancy between TMVP and ATMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | Saurav Bandyopadhyay | Email: | saurav.bandyopadhyay@interdigital.com |
| Source: | InterDigital Inc. |  |  |

_____________________________

# Abstract

The cross-check results in the input document JVET-M0345: CE4-related: Remove redundancy between TMVP and ATMVP. The contribution aims to reduce the redundancy in operation between TMVP and ATMVP. It proposes to skip TMVP process in merge mode when CU size is equal to 8x8 and skip TMVP process in merge mode when the width or height of CU is 4 along with previous condition. The cross-check validated the experimental results from the TMVP and ATMVP redundancy removal.

# Introduction

The source code as well as the configuration, for JVET-M0345 [1], provided by the proponent SZ DJI Technology Co. Ltd. and Peking University were used for the cross-check. The contribution reduces the redundancy between TMVP and ATMVP from the motion derivation process at 8x8 coding block and by discarding usage of TMVP block sizes smaller than 8x8.

# Simulation Results

The simulation results reported in this contribution are based on the VTM-3.0 reference software and the JVET common test conditions [2] to collect the B-D rate, encoding and decoding time of the proposed techniques in the Random Access and Low Delay B configurations. The results are generated using configuration for JVET-M0345 provided by the proponents for the TMVP and ATMVP redundancy removal.

## Skip TMVP in merge mode when CU size is equal to 8x8

Random Access

|  | Over vtm3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.00% | 0.00% | 101% | 102% |
| Class A2 | -0.04% | -0.01% | 0.04% | 101% | 100% |
| Class B | -0.04% | 0.01% | 0.02% | 101% | 100% |
| Class C | 0.00% | -0.01% | -0.03% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.00% | 0.01% | 101% | 101% |
| Class D | 0.02% | -0.21% | 0.01% | 100% | 100% |
| Class F (optional) | -0.02% | -0.03% | -0.03% | 100% | 100% |

Low Delay B

|  | Over vtm3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.18% | 0.05% | 100% | 99% |
| Class C | 0.02% | 0.16% | 0.19% | 100% | 97% |
| Class E | -0.01% | -0.03% | -0.38% | 100% | 98% |
| Overall | 0.00% | -0.03% | -0.01% | 100% | 98% |
| Class D | -0.03% | -0.16% | -0.04% | 100% | 97% |
| Class F (optional) | 0.07% | -0.20% | 0.08% | 100% | 96% |

## Skip TMVP in merge mode when the width or height of CU is 4

Random Access

|  | Over vtm3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.04% | -0.01% | 101% | 101% |
| Class A2 | -0.07% | -0.10% | -0.04% | 101% | 101% |
| Class B | -0.06% | -0.05% | -0.06% | 100% | 100% |
| Class C | -0.01% | -0.05% | 0.05% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -0.04% | -0.02% | 101% | 100% |
| Class D | 0.11% | -0.10% | -0.04% | 100% | 100% |
| Class F (optional) | -0.03% | -0.01% | 0.03% | 100% | 100% |

Low Delay B

|  | Over vtm3.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | -0.29% | 0.22% | 100% | 99% |
| Class C | 0.10% | 0.12% | 0.17% | 100% | 97% |
| Class E | -0.07% | 0.33% | 0.08% | 100% | 97% |
| Overall | 0.00% | 0.00% | 0.17% | 100% | 98% |
| Class D | 0.19% | -0.26% | 0.08% | 101% | 97% |
| Class F (optional) | -0.04% | 0.25% | 0.08% | 100% | 96% |

# Conclusion

The results from this cross-check are equivalent to the results provided by the proponents in JVET-M0345.

# Reference

S. Wang, X. Zheng, J. Cui, S. Wang, S. Ma, “CE4-related: Remove redundancy between TMVP and ATMVP”, JVET-M0345, Marrakech, MA, January 2019.

F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations”, JVET-L1010, Macao, CN, October 2018.

# Patent rights declaration(s)

InterDigital Inc. does not have any current or pending patent rights relating to the technology described in this contribution.