| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0759-v1 |
| --- | --- |

| Title: | CE5: Report of subtest 3 on complexity and throughput aspects for hardware |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Benno Stabernack Fraunhofer HHI Ted Hsieh Qualcomm Inc. | Email: | benno.stabernack@ hhi.fraunhofer.de thsieh@qti.qualcomm.com |
| Source: | HHI, Qualcomm |  |  |

_____________________________

# Abstract

This is the report of CE5 subtest 2, on an in-depth analysis of complexity and throughput aspects of a hypothetical hardware implementation of the proposed arithmetic coding engines.

# In-depth analysis of hardware implementation aspects

## Scope

The scope of the experiment/analysis is to identify potential throughput issues which will arise if the arithmetic coding engine and its different variants which have been proposed will be implemented as a hardware architecture using state of the art silicon technology.  The work will be carried out as a paper pencil analysis.

## Methodology

Since it is not feasible to implement a complete arithmetic coding engine using a hardware description language for the purpose of simulation, the actual analysis will be carried out on the basis of well-known numbers from already available implementations e.g. HEVC CABAC decoders. Parameter of interest is the number of BINs per cycle(BIN/cycl).On the basis of the actual modification of the base architecture (HEVC CABAC decoder) proposed in the contribution, the impact of the modifications will be discussed.

## Description of the base architecture

The analysis is based on a real hardware implementation of a HEVC CABAC decoder, which is capable of reaching a throughput of 1 bin/cycle at a given reasonable clock frequency.

Fig. 1 shows the internal architecture of the slice segment data unit for the CABAC entropy decoder. Each syntax processing unit on the left side contains a finite state machine (FSM) which controls the pipeline of the context model (CM) and the arithmetic decoding unit (ADU) for the corresponding syntax group. One cycle after context selection the decoded bin value is available for de-binarization to receive the final syntax element.

The CM in Fig. 1 stores all context models in a (7x154 bit for HEVC) register array, which can be accessed by an address multiplexer. Since the final number of context models in VVC and corresponding bits for each context model is not known at the time being, we assume that under ideal condition to handle all context models in local cache / register file.

An approach to reduce the critical path between the CM and the ADU is to insert a pipeline register after the context memory for the selected state and most probable symbol. Due to pipeline stalls, which occur in context preload, all syntax processing units need a highly optimized address preload scheme for each syntax element using ADU decision mode.

To avoid data hazards in the context selection pipeline for two consecutive bins using the same context model, a direct feedback multiplexer ((d) in Fig. 1) is implemented in the CM. Thus, the recently updated context can be accessed without a pipeline stall. This approach is called context model forwarding.

Fig 1: HEVC arithmetic decoder base architecture

The maximum throughput of the proposed base CABAC architecture is restricted to one bin per cycle, which corresponds up to 300 Mbins per second with respect to the system clock of 300 MHz clock frequency synthesized for 28 nm TSMC technology. Other implementation with similar architectural approaches can be found in [1], [2] and [3].

## Modifications to the base architecture

In the following chapters, the modifications to the base architecture and their impact on the throughput parameters will be discussed, which are basically the probability estimator which offers one or two probabilities per state and the kind of range generation.

## HHI JVET-K0430 State-based probability estimator

An extension of the state-based probability estimator of VTM-1.0 to two states per context model has been proposed in JVET-K0430. The transition table size is reduced from 64 to 32 elements and the two states per context model require 8 and 12 bit, respectively. Experimental results for the VTM configuration show overall luma BD rate reductions of -0.67%, -0.45%, and -0.41% for AI, RA, and LB, respectively

The probability estimation method of this proposal stores two signed integer state variables S0 and S1 per context model which require 8 and 12 bit, respectively.

The lookup table ‘Next’ is defined as follows and can be found in Fig 2 (LUT 32x8 ROM):

Next[32] = { 171, 151, 132, 114, 98, 82, 68, 56, 45, 35, 28, 21, 16, 11, 8, 5, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 4, 0 };

The critical path of state-based estimator is depicted in Fig 2 and consists basically of an adder followed by a table lookup which gives the result into an adder. Since these components are not complex in terms of gate count, the assumption is, that the overall performance of an CABAC decoder will not degrade by the usage of the proposed method.

Fig 2: Block diagram state-based probability estimator based on JVET-K0430

The derivation of the range subinterval of the LPS employs the lookup table ‘rangeTabLps’ of VTM, but with dimensions 32x16 and with adapted values in order to reflect the correct probability values:

R_lps = rangeTabLps[ abs( (S0 << 4) + S1 ) >> 7][(range >> 4) & 15];

In the following figure the block diagram of the subinterval derivation as proposed in JVET-K0430 is depicted. The critical path is here given by an adder followed by a look up table with size of 256x8 bits.

Fig 3: Block diagram range subinterval derivation for LPS

The complexity is as well low and comparable to the subinterval derivation as used in HEVC.

## HHI JVET-L0426 Coding interval Subdivision

CABAC range sub-interval derivation as described in JVET-K0430 is performed by a lookup in a 2-dimensional table, which outputs the coding interval associated with the LPS (Least Probable Symbol). CE5 includes experiments on different lookup table sizes (e.g. 32x8, 16x8 etc.) and on replacing the table lookup by range probability product computation. An alternative implementation, that uses a combination of both approaches, is proposed. The coding performance is virtually the same.

The interval subdivision is realized using a one dimensional lookup table (probTabLPS) of size 32 (or 16) and a multiplication operation which is equivalent to using a 32x8 (or 16x8) table.

Fig 4 shows the block diagram of the method used for coding interval subdivision. The critical path is build out of an adder and a lookup table which gives its results into a multiplier. Compared to the method for subinterval derivation given in in JVET-K0430, this method is more complex and restricted by a longer critical path, mainly determined by the multiplier.

Fig 4: Block diagram Coding interval Subdivision

## Qualcomm JVET-L0115, Double Window Probability Estimator with Adaptive Window Shifter Parameters

The CABAC engine in JVET-L0115 uses a “parallel double window” to estimate the binary symbol probabilities. These probability estimations are updated using the recursive equations, as below.

bin = 1    →		U[k+1] = U[k] + ((2Bu – U[k]) >> a),

V[k+1] = V[k] + ((2Bv – V[k]) >> b),

bin = 0    →		U[k+1] = U[k] – (U[k] >> a),

V[k+1] = V[k]  – (V[k]) >> b),

P[k]      = U[k] + (V[k]) >> (Bu – Bv)),  where

U[k] and V[k] represent the probabilities estimated by the two windows, respectively. P[k] represents the probability estimate used for binary arithmetic coding. Parameters Bu and Bv are the same for all contexts, which are 10 and 14, respectively.

The two positive integer window shift parameters a and b, which can be different for each context, are in the interval 2, 3, …, 9, and can be implemented by a look-up table(LUT). In each entry of the LUT, a 4-bit parameter c is stored to represent a and b, as shown in below equations, respectively. Since the value of c for each context are never modified, in custom hardware implementation they can be stored in less expensive and power-efficient read-only memory (ROM).

a = (c >> 2) + 2

b = a + 1 + (c & 3)

The HW data path of the proposed probability estimator is given in Figure 5. The computations needed for the probability estimator’s data path are not complex since the corresponding logics are adder and multiplexer only. In terms of window shift parameters (a, b), they are derived based on c by simple equations (adders involved only), and since c(4 bits) is fixed for each context model,  a LUT/ROM-based access could be used to generate it. In general, since no critical path involved in above design, the corresponding CABAC decoder performance should not be impacted.

Fig 5: Probability estimator’s data path in JVET-L0115

## Qualcomm JVET-L0116,  Single Window Probability Estimator with Adaptive Window Shift Parameter

JVET-L0116 is similar as JVET-L0115, but with one-window probability estimation.

bin = 1    →		U[k+1] = U[k] + ((2Bu – U[k]) >> a)

bin = 0    →		U[k+1] = U[k] – (U[k] >> a), where

U[k] represents the probability estimate used for binary arithmetic coding. Parameters Bu is 14 for all contexts. The window shift parameter a, which is fixed for each context, is represented by 3 bits, and can be accessed through a LUT.

The HW data path of the proposed probability estimator is depicted in Figure 6. The corresponding data path is basically similar as JVET-L0115(i.e. roughly half of the double window approach) and the complexity is less compared with JVET-L0115. Therefore, no impacted of the CABAC decoder performance is assumed.

Fig 6: Probability estimator’s data path in JVET-L0116

Basically, the hardware implementations of the above various probability estimators are very simple, so the completion of the operation should be finished within one clock cycle at a reasonable clock frequency.

## Summary for subtest 3

| JVET Contribution | BIN/Cycl | Impact on throughput compared to ideal reference implementation |
| --- | --- | --- |
| HHI JVET-K0430 | 1 | No impact on throughput. |
| HHI JVET-L0426 | 1 | No impact on throughput. |
| HHI JVET-L0429 | 1 | No impact on throughput. |
| Qualcomm JVET-L0115 | 1 | No impact on throughput. |
| Qualcomm JVET-L0116 | 1 | No impact on throughput. |

# References

[1] 	Y. Choi and J. Choi, “High-throughput CABAC codec architecture for HEVC,” Electronics Letters, vol. 49, no. 18, pp. 1145–1147, August 2013.

[2] 	Y.H. Chen, and V Sze, "A 2014 Mbin/s deeply pipelined CABAC decoder for HEVC," IEEE international conference on image processing (ICIP), pp. 2110 – 2114, 2014.

[3]	Jan Hahlbeck and Benno Stabernack, A 4k capable FPGA based high throughput binary arithmetic decoder for H.265/MPEG-HEVC, IEEE International Conference on Consumer Electronics - Berlin 2014, Berlin, Germany, September 2014.