| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0792 |
| --- | --- |

| Title: | CE10-related: Combined test of multi-hypothesis inter prediction and OBMC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Chun-Chia Chen, Yu-Cheng Lin, Yu-Wen Huang, Shaw-Min Lei, No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan Xiaoyu Xiu, Yuwen He 9276 Scranton Road, Suite 300, San Diego, CA 92121, USA Martin Winken, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +886-3-5670766 +49-30-31002-570 {zhi-yi.lin, peter.chuang, chingyeh.chen, cw.hsu, chunchia.chen, yucheng.lin, yuwen.huang, shawmin.lei} @mediatek.com Yuwen.He@InterDigital.com martin.winken@hhi.fraunhofer.de |
| Source: | MediaTek Inc., InterDigital Communications, Inc., Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document presents experimental results for a combination of the coding tools multi-hypothesis inter prediction and OBMC.  These tools have been tested separately within CE10.  In the combined test, the usage of these tools is mutually exclusive per PU, i.e. if multi-hypothesis prediction is used, OBMC is disabled.  And, vice versa, the additional hypotheses which may be present in neighboring blocks are not used in the OBMC process.

In one test, the combination of multi-hypothesis variant CE10.1.2.a and OBMC variant CE10.2.4 is tested. The following results are reported, with the individual tool results shown again here for convenience:

|  |  |  | RA |  |  |  |  | LB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| CE10.1.2.a | -0.29% | -0.16% | -0.12% | 107% | 97% | -0.41% | -0.04% | -0.04% | 110% | 99% |
| CE10.2.4 | -0.49% | -0.91% | -0.87% | 106% | 105% | -0.45% | -0.62% | -0.34% | 108% | 110% |
| CE10.1.2.a + CE10.2.4 | -0.78% | -1.00% | -1.03% | 113% | 101% | -0.95% | -0.74% | -0.60% | 118% | 108% |

In a second test, the combination of multi-hypothesis variants CE10.1.2.c and CE10.1.2.d as well as OBMC variant CE10.2.1 is tested. The following results are reported for this test:

|  |  |  | RA |  |  |  |  | LB |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | Cb | Cr | Enc | Dec | Y | Cb | Cr | Enc | Dec |
| CE10.1.2.c + CE10.1.2.d | -0.19% | 0.02% | 0.02% | 105% | 99% |  |  |  |  |  |
| CE10.2.1 | -0.27% | -0.58% | -0.62% | 104% | 103% | -0.36% | -0.50% | -0.51% | 106% | 105% |
| CE10.1.2.c + CE10.1.2.d + CE10.2.1 | -0.47% | -0.51% | -0.55% | 110% | 103% |  |  |  |  |  |

# Introduction

Within Core Experiment (CE) 10 [1], the coding tools multi-hypothesis inter prediction (MHIP) and overlapped block motion compensation (OBMC) have been tested separately in different variants.  In this document, a combination of these tools is studied.  Since it is not feasible to evaluate every variant of MHIP in conjunction with every variant of OBMC, two corner cases are considered here:

- Test 1 (CE10.1.2.a and CE10.2.4): In this test, the variants of MHIP and OBMC which have individually shown the best coding gain are tested jointly.

- Test 2 (CE10.1.2.c+CE10.1.2.d and CE10.2.1): In this test, for MHIP, a combination of the variants CE10.1.2.c and CE10.1.2.d is used (i.e., max 2 reference frames and only applied for luma). For OBMC, the variant CE10.2.1 is used.  This test is intended to reflect a lower complexity variant than test 1.

Details on OBMC can be found in [3], [4], and details on MHIP can be found in [5]

Note that usage of MHIP and OBMC is mutually exclusive per PU.  That means that for a PU which uses MHIP, OBMC is disabled.  In AMVP mode, the obmc_flag is omitted if an additional prediction hypothesis is signaled.  In merge mode, there is no obmc_flag and OBMC is not applied if the selected merging candidate has an additional prediction hypothesis.  On the other hand, in the OBMC process, additional prediction hypotheses from neighboring blocks are also not used for the blending.

# Experimental results

## Test 1: Combination of CE10.1.2.a and CE10.2.4

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.43% | -0.98% | -1.01% | 111% | 101% |
| Class A2 | -0.76% | -1.13% | -0.94% | 113% | 105% |
| Class B | -1.01% | -1.02% | -1.11% | 113% | 100% |
| Class C | -0.78% | -0.90% | -1.01% | 114% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.78% | -1.00% | -1.03% | 113% | 101% |
| Class D | -0.63% | -0.76% | -0.40% | 114% | 108% |
| Class F | -0.81% | -0.83% | -0.75% | 111% | 101% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -1.03% | -0.78% | -0.48% | 118% | 105% |
| Class C | -0.90% | -0.41% | -0.74% | 119% | 110% |
| Class E | -0.88% | -1.11% | -0.62% | 117% | 111% |
| Overall | -0.95% | -0.74% | -0.60% | 118% | 108% |
| Class D | -0.65% | -0.94% | -0.43% | 118% | 114% |
| Class F | -1.62% | -1.63% | -1.51% | 116% | 104% |

## Combination of CE10.1.2.c and CE10.1.2.d and CE10.2.1

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.23% | -0.41% | -0.42% | 108% | 100% |
| Class A2 | -0.39% | -0.41% | -0.35% | 109% | 105% |
| Class B | -0.58% | -0.44% | -0.58% | 110% | 102% |
| Class C | -0.60% | -0.75% | -0.77% | 110% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0.47% | -0.51% | -0.55% | 110% | 103% |
| Class D | -0.54% | -0.75% | -0.53% | 111% | 107% |
| Class F | -0.61% | -0.41% | -0.37% | 109% | 105% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-3.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.71% | -0.47% | -0.58% | 114% | 113% |
| Class E | -0.47% | -0.54% | -0.58% | 112% | 108% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.76% | -0.99% | -0.23% | 113% | 115% |
| Class F | -0.70% | -0.67% | -0.63% | 111% | 105% |

# References

- C.-W. Hsu, M. Winken, “Description of Core Experiment 10: Combined and multi-hypothesis prediction,” JVET-L1030, Macao, CN, October 2018.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring (editors), “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, Macao, CN, October 2018.

- Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, C.-C. Chen, Y.-C. Lin, Y.-W. Huang, S.-M. Lei (MediaTek), X. Xiu, Y. He (InterDigital), “CE10.2.1: Uni-prediction-based CU-boundary-only OBMC,” JVET-M0178, Marrakech, MA, January 2019.

- Y.-C. Lin, C.-C. Chen, C.-W. Hsu, Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek), “CE10.2.4: Subblock OBMC with integer-MV-based OBMC at CU boundaries,” JVET-M0181, Marrakech, MA, January 2019.

- M. Winken, H. Schwarz, D. Marpe, T. Wiegand (HHI), “CE10: Multi-hypothesis inter prediction (Test 10.1.2),” JVET-M0425, Marrakech, MA, January 2019.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).