| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0800 |
| --- | --- |

| Title: | Cross-check report of JVET-M0085 on Fast algorithm for DST-4/DCT-4 as alternative transforms for MTS (CE6-related) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Karam NASER | Tel: Email: | Karam.naser@technicolor.com |
| Source: | Technicolor |  |  |

_____________________________

# Abstract

This contribution reports a cross-checking of contribution JVET-M0085 on “Fast algorithm for DST-4/DCT-4 as alternative transforms for MTS”. The cross-checker could reportedly exactly reproduce the results from the two proposed tests reported in JVET-M0297 for AI, RA and LDB configurations. The measured encoder and decoder run time ratios are reported to be of same order as the anchor (VTM3.0 with inter MTS enabled). It is also reported that the source code implementation was checked and is conform to the description.

# Summary of Proposal JVET-L0304

The proposal JVET-L0304 is about a fast implementation of DST4/DCT4 which are used as alternative transform of DST7/DCT8 (resp.) for MTS design. The fast implementation is based on the symmetry of the transform basis functions that can bring 75% reduction of the number of multiplications.

# Source Code Analysis

The source code was analysed and is confirmed to be in line with the description of the proposal.

# Simulation Results

Results were checked for AI, RA and LDB configurations. They are summarized in following tables and in attached xls files. Perfect matching with proponent results was obtained. Encoding and decoding runtime seem to be similar to the anchors one.

Table 1 Simulation results under CTC of Test1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0_interMTS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.66% | 0.55% | 0.71% | 100% | 90% |
| Class A2 | 0.50% | 0.23% | 0.21% | 99% | 92% |
| Class B | 0.33% | -0.01% | 0.04% | 100% | 93% |
| Class C | -0.24% | -0.27% | -0.32% | 100% | 96% |
| Class E | -0.01% | -0.24% | -0.17% | 100% | 93% |
| Overall | 0.23% | 0.03% | 0.07% | 100% | 93% |
| Class D | -0.27% | -0.42% | -0.35% | 99% | 96% |
| Class F | -0.12% | -0.21% | -0.04% | 100% | 96% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM 3.0_interMTS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.32% | 0.27% | 0.44% | 101% | 97% |
| Class A2 | 0.29% | 0.24% | 0.16% | 100% | 99% |
| Class B | 0.18% | 0.14% | 0.01% | 100% | 96% |
| Class C | -0.18% | 0.01% | -0.05% | 100% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.13% | 0.15% | 0.11% | 100% | 97% |
| Class D | -0.24% | -0.48% | -0.36% | 100% | 97% |
| Class F | -0.03% | -0.12% | -0.05% | 100% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM 3.0_interMTS |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class E | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |