| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0803 |
| --- | --- |

| Title: | Cross-check of contribution JVET-M0392 (Non-CE3: Extended Mode-Dependent Intra Smoothing) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Michael Schäfer Jonathan Pfaff | Tel: Email: | michael.schaefer@hhi.fraunhofer.de jonathan.pfaff@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution reports a cross-checking of contribution JVET-M0392 related to Extended Mode-Dependent Intra Smoothing.

The BD-rate gains reported by the proponents and the cross-checker match. The cross-checker received the source code before the meeting and performed the cross-check during the meeting. The encoding and decoding times measured by the cross-checker are not reliable.

# Introduction

The authors of JVET-M0392 propose a different reference sample filter for blocks, whose sum of width and height is greater than 32, and only if MDIS condition are true. Further, an interpolation filter is applied to the filtered reference samples. The first test uses a Gaussian filter with coefficients given by an LUT. The second test uses the intra chroma interpolation filter instead of the Gaussian filter. Both tests were cross-checked.

# Test Result

The cross-check test is performed on top of VTM-3.0 with VTM-3.0 configurations. The results are is shown in the following tables, and they match the results shown in JVET-M0392. Tables 1.1 to 2.2 show the results of each test in AI and RA configuration.

Table 1.1 Result of Test 1 for VTM AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.38% | -0.34% | -0.25% | 104% | 96% |
| Class A2 | -0.07% | 0.03% | 0.00% | 103% | 97% |
| Class B | -0.07% | -0.02% | 0.01% | 103% | 97% |
| Class C | 0.00% | -0.02% | -0.10% | 103% | 98% |
| Class E | 0.06% | -0.07% | 0.16% | 103% | 98% |
| Overall | -0.09% | -0.07% | -0.03% | 103% | 97% |
| Class D | 0.02% | -0.05% | 0.11% | 103% | 100% |
| Class F (optional) | 0.04% | 0.00% | 0.06% | 102% | 99% |

Table 1.2 Result of Test 1 for VTM RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.20% | -0.34% | -0.23% | 104% | 99% |
| Class A2 | -0.03% | 0.06% | 0.20% | 104% | 104% |
| Class B | -0.06% | -0.04% | -0.11% | 105% | 98% |
| Class C | 0.00% | -0.10% | 0.00% | 105% | 96% |
| Class E |  |  |  |  |  |
| Overall | -0.07% | -0.10% | -0.04% | 104% | 99% |
| Class D | -0.04% | -0.24% | -0.18% | 105% | 101% |
| Class F (optional) | -0.03% | -0.07% | -0.10% | 104% | 98% |

Table 2.1 Result of Test 2 for VTM AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.25% | -0.21% | -0.09% | 104% | 98% |
| Class A2 | -0.02% | 0.08% | 0.05% | 103% | 99% |
| Class B | -0.02% | -0.01% | 0.02% | 103% | 97% |
| Class C | 0.01% | 0.01% | -0.14% | 103% | 99% |
| Class E | 0.07% | 0.03% | 0.08% | 103% | 99% |
| Overall | -0.04% | -0.02% | -0.02% | 103% | 98% |
| Class D | 0.03% | 0.02% | -0.09% | 103% | 101% |
| Class F (optional) | 0.01% | 0.11% | 0.11% | 102% | 98% |

Table 2.2 Result of Test 2 for VTM RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0.11% | -0.20% | -0.08% | 104% | 97% |
| Class A2 | -0.02% | 0.18% | 0.09% | 104% | 103% |
| Class B | -0.04% | 0.04% | -0.06% | 104% | 101% |
| Class C | 0.00% | 0.04% | 0.29% | 104% | 96% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | 0.02% | 0.06% | 104% | 99% |
| Class D | -0.01% | -0.24% | 0.04% | 105% | 102% |
| Class F (optional) | -0.01% | 0.04% | -0.04% | 104% | 99% |

# Reference

[1]	F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, 12th JVET Meeting, Macao, China, October 2018.

[2]	A. Fillipov et al., “Non-CE3: Extended Mode-Dependent Intra Smoothing”, Document of Joint Video Experts Team (JVET), JVET-M0392-v1, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019