| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0822 |
| --- | --- |

| Title: | Non-CE8: Encoder optimization for palette mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongtao Wang, Yung-Hsuan Chao, Vadim Seregin, Marta Karczewicz 5775 Morehouse Drive San Diego, CA, U.S.A. 92121 | Tel: Email: | hongtaow@qti.qualcomm.com yunghsua@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

# Abstract

This contribution proposes an encoder optimization for palette mode. The results show more than 20% encoding time reduction for AI for screen content sequences. When tested with CPR enabled, the partial results show that the encoding time is reduced from 157% to 115% with only 0.14% (Y) BD rate loss for TGM420 sequences.

# Introduction

In the 12th JVET meeting in Macao, CN, palette mode is been studied on top of VTM. The method provides high coding efficiency for coding screen content sequences with low complexity on decoder side. However, some encoder complexity is also introduced. In this proposal, we propose an encoder only optimization to reduce the encoder complexity.

# Proposed encoder optimization for palette mode

In this proposal, three encoder-only modifications are included on top of CE8.2.1 palette base code.

- For intra slice, palette mode is checked before intra mode on encoder side. The RD cost of palette is compared with the first intra mode (in the loop of full RD checking). If palette mode has lower cost than the first intra mode, the rest of the intra modes will be skipped. Otherwise, intra mode checking will be continued until all intra modes selected for full RD check are processed. The method is similar as in the encoder modification proposed in JVET-M0056.

- In the current VTM3.0 software, an encoder fast algorithm is applied for BT/TT partition. In this fast algorithm, encoder will skip checking BT/TT split if the maximum QT depth derived from QT split check is larger than the QT depth of the current CU + 1. The code is shown as below:

case CU_HORZ_SPLIT:

case CU_TRIH_SPLIT:

if( cuECtx.get<bool>( QT_BEFORE_BT ) && cuECtx.get<bool>( DID_QUAD_SPLIT ) )

{

if( cuECtx.get<int>( MAX_QT_SUB_DEPTH ) > partitioner.currQtDepth + 1 )

{

if( featureToSet >= 0 ) cuECtx.set( featureToSet, false );

return false;

}

}

break;

case CU_VERT_SPLIT:

case CU_TRIV_SPLIT:

if( cuECtx.get<bool>( QT_BEFORE_BT ) && cuECtx.get<bool>( DID_QUAD_SPLIT ) )

{

if( cuECtx.get<int>( MAX_QT_SUB_DEPTH ) > partitioner.currQtDepth + 1 )

{

if( featureToSet >= 0 ) cuECtx.set( featureToSet, false );

return false;

}

}

break;

In our proposed method, the maximum QT depth derived from QT split will be based on the RD

optimization without palette mode.

- For AI configuration, the ratio of area using palette mode is checked in the first I slice. If the area using palette mode is smaller than 1% of the total area, palette will be disabled for the rest of the sequence. For RA and LD configuration, the ratio of area using palette mode will be checked for each IRAP frame.  If the area using palette mode is smaller than 1% of the total area in the IRAP frame, palette mode will be disabled until the next IRAP frame is processed.

# Simulation results

The proposed encoder optimization is tested on top of palette base code in CE8.2.1. In AI configuration, parallelization is not used in the simulation. The anchor is VTM3.0. In this proposal, we test CE8.2.1 with proposed encoder speed up with CPR enabled.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.21% | -0.32% | -0.30% | 140% | 107% |
| Class C | 0.02% | -0.18% | -0.17% | 144% | 109% |
| Class E | -0.51% | -0.59% | -0.51% | 143% | 108% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.00% | 0.00% | 0.23% | 140% | 104% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| TGM | -44.61% | -41.02% | -41.24% | 115% | 80% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.10% | 0.03% | -0.32% | 103% | 102% |
| Class C | 0.37% | -0.05% | 0.19% | 108% | 105% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | 0.41% | -0.18% | 0.32% | 108% | 104% |
| Class F (optional) | -11.87% | -11.53% | -11.88% | 111% | 102% |
| TGM | -25.64% | -24.44% | -24.76% | 101% | 94% |

# Reference

- F. Bossen, J.Boyce, X. Li, V. Seregin and K. Suehring,  “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, 12th meeting, Macao, CN, October 2018.

- Y.-C. Sun, J. Lou (Alibaba), Y.-H. Chao, H. Wang, V. Seregin, M. Karczewicz (Qualcomm), “CE8: Palette Mode in HEVC (test 8.2.1)”, 13th meeting, Marrakech, MA, Jan.  2019.

- F. Henry, A. Mohsen (Orange), P. Philippe, G. Clare (bcom), “CE8: BDPCM with LOCO-I and independently decodable areas (test 8.3.1a)”, 13th meeting, Marrakech, MA, Jan.  2019.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).