| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0823 |
| --- | --- |

| Title: | CE4-related: Encoder optimization of CE4.4.5 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jingya Li Ru-ling Liao Chong Soon Lim | Tel: Email: | +65-6550-5423 jingya.li@sg.panasonic.com chongsoon.lim@sg.panasoni.com |
| Source: | Panasonic Corporation |  |  |

_____________________________

# Abstract

This contribution provides the simulation results of encoder optimization algorithm used in JVET-M0312 CE4: MMVD improvement (test 4.4.5). Experimental results show 0.11% (RA) and 0.08% (LDB) luma BD-rate reduction with 3-5% increase in encoding timing time and no impact on decoding time.

# Algorithm description

Firstly, the early termination for MMVD which is included in VTM-3.0 is disabled, (the early termination used for MMVD in VTM3.0 is: when the SATD cost of a MMVD candidate with short distance is smaller than a threshold, the MMVD candidate with larger distance will not be tested.)

Secondly, an new encoder fast algorithm is applied to MMVD. This fast algorithm includes 2 parts (1) During pre-selection of MMVD candidate using SATD cost, BDOF is disabled for motion compensation. After best MMVD candidates are selected, BDOF is applied to those candidates. (2) During pre-selection of MMVD candidate using SATD cost, chroma motion compensation is skipped. After best MMVD candidates are selected, chroma motion compensation is calculated for to those candidates.

# Experimental Results

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.15% | -0.09% | 100% | 98% |
| Class A2 | -0.19% | -0.20% | -0.19% | 101% | 97% |
| Class B | -0.10% | -0.12% | -0.26% | 104% | 97% |
| Class C | -0.11% | -0.12% | -0.14% | 107% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.11% | -0.14% | -0.18% | 103% | 98% |
| Class D | -0.16% | -0.43% | -0.26% | 108% | 98% |
| Class F | -0.12% | -0.17% | -0.17% | 108% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.07% | -0.31% | 0.06% | 104% | 99% |
| Class C | -0.13% | -0.06% | -0.30% | 106% | 100% |
| Class E | -0.04% | 0.16% | -0.12% | 105% | 100% |
| Overall | -0.08% | -0.11% | -0.10% | 105% | 100% |
| Class D | -0.12% | -0.22% | -0.30% | 107% | 100% |
| Class F | -0.18% | -0.28% | -0.24% | 106% | 100% |

# Conclusions

In this contribution, the encoder optimization for MMVD mode is proposed. It shows 0.11% B-D rate reduction in RA configuration and 0.08% B-D rate reduction in LDB configuration. We recommend to adopt the proposed algorithm into the next version of VTM.

# References

- F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 12th Meeting: Macao, CN, 3–12 Oct. 2018.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).