| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0827-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-M0310 (CE2-related: Using shorter-tap filter for 4x4 sized partition) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yu-Cheng Lin No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 yucheng.lin@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

The document reports the results of crosschecking JVET-M0310 proposed by Panasonic. JVET-M0310 proposes to use 6-tap filter for 4x4 sized partitions including 4x4 CU and sub-block of affine. The verification task was done, and it is observed that there is 0.01% BD-rate difference in Y in class A1 and V in class A2 in Random Access (RA) in Test 1. After investigation, this should be caused by a PSNR calculation accuracy problem. Therefore, the BD-rate result can be treated as match.

# Verification

Panasonic provided the source code of their proposed method described in JVET-M0310 [1].

Two tests are conducted under the the common test conditions (CTC) defined in [2] and are carried out by using the VTM3.0 software [3]. In both tests, the VTM3.0 is used as the anchor.

It is observed that there is 0.01% BD-rates difference in Y in class A1 and V in class A2 in RA in Test 1. After investigation, this should be caused by a PSNR calculation accuracy problem. Therefore, the BD-rate result can be treated as match.

Table 1. Simulation results of Test 1 compared to VTM3.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.05% | -0.04% | 101% | 103% |
| Class A2 | 0.09% | 0.09% | 0.21% | 101% | 100% |
| Class B | 0.05% | 0.08% | -0.10% | 101% | 101% |
| Class C | 0.16% | 0.10% | 0.09% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | 0.06% | 0.02% | 100% | 101% |
| Class D | 0.25% | 0.15% | 0.38% | 100% | 100% |
| Class F (optional) | 0.05% | -0.04% | 0.03% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.09% | -0.08% | 0.17% | 100% | 101% |
| Class C | 0.22% | 0.45% | 0.35% | 102% | 101% |
| Class E | 0.09% | 0.17% | 0.50% | 101% | 101% |
| Overall | 0.14% | 0.16% | 0.31% | 101% | 101% |
| Class D | 0.42% | 0.48% | 0.46% | 101% | 101% |
| Class F (optional) | 0.19% | 0.02% | -0.17% | 101% | 100% |

Table 2. Simulation results of Test 2 compared to VTM3.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.13% | -0.10% | 101% | 104% |
| Class A2 | 0.30% | 0.06% | 0.08% | 101% | 103% |
| Class B | 0.35% | 0.16% | 0.02% | 101% | 100% |
| Class C | 0.65% | 0.33% | 0.29% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.35% | 0.13% | 0.08% | 101% | 102% |
| Class D | 1.27% | 0.68% | 0.91% | 100% | 100% |
| Class F (optional) | 0.12% | 0.08% | 0.15% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.65% | -0.04% | 0.33% | 101% | 101% |
| Class C | 0.90% | 0.71% | 0.56% | 101% | 99% |
| Class E | 0.73% | 0.45% | -0.05% | 101% | 101% |
| Overall | 0.75% | 0.33% | 0.31% | 101% | 100% |
| Class D | 2.48% | 1.24% | 1.46% | 100% | 99% |
| Class F (optional) | 0.51% | 0.42% | 0.24% | 101% | 100% |

# References

- J. Li, R.-L Liao, C. S. Lim, “CE2-related: Using shorter-tap filter for 4x4 sized partition,” Document of Joint Video Experts Team, JVET-M0310

- J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-L1010, the 12th JVET meeting, Oct. 2018, Macao, CN.

- VTM3.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).