| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0831 |
| --- | --- |

| Title: | Cross-check of JVET-M0470 (CE7-related: Golomb-Rice/exponential Golomb coding for abs_remainder and dec_abs_level syntax elements) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | Karl Sharman Steve Keating | Email: | karl.sharman@sony.com steve.keating@sony.com |
| Source: | Sony Europe Ltd |  |  |

_____________________________

# Abstract

This contribution provides a cross-check of the proposal document JVET-M0470 (CE7-related: Golomb-Rice/exponential Golomb coding for abs_remainder and dec_abs_level syntax elements).

The original proposal has two aspects relating to the Golomb Rice/Exponential Golomb coding scheme used in the coding of coefficients: using a fixed Golomb Rice prefix length of 5+1, rather than the current variable system, and limiting the maximum code length to 32 bits. Both of these are related to aspects of the HEVC design that have since been adjusted in the current version of the VVC draft (and VTM3.0).

The cross-check indicates that the code supplied implements the required operation, the draft text changes have been studied, and the results match those given in the contribution (v2), with the changes in BD-rate attributed to just the first aspect. It is stated that the single length Golomb Rice prefix length of 5+1 is a simplification over the current VVC draft. It is also stated that that the maximum code length exceeds 32 bits and that the proposal achieves the aim to limit the maximum code length to 32 bits.

# Introduction

This contribution is a cross-check of the proposal document JVET-M0470 (Cross-check of JVET-M0470 (CE7-related: Golomb-Rice/exponential Golomb coding for abs_remainder and dec_abs_level syntax elements).

The original proposal has two aspects, as described below.

# Aspect 1: Fixed Golomb Rice Length

The current VVC draft (and VTM3.0) use a variable Golomb Rice prefix length, depending on the Golomb Rice Parameter. In this proposal, the table is removed and a fixed value of 5+1 is used. (This document uses 5+1, as ‘5’ is the value used in the software, whereas the specification uses a value of 6, but uses cRiceParam+1 for controlling the exp-Golomb; mathematically they are equivalent.)

Replacing the table with a fixed code is a simplification. Note that in HEVC this prefix length was fixed at a value of 3+1.

The code supplied correctly implements this function.

# Aspect 2: Maximum code length

For extreme cases, it is possible for the current VVC draft design to describe coefficients that would take more than 32-bits when coded in the bit-stream. These cases are extremely rare, and extremely unlikely to occur in practise. The proposal limits the maximum code length to 32-bits, using the scheme employed for higher bit-depths in HEVC RExt.

Limiting the code length to 32 simplifies the worst case scenario.

The code supplied correctly implements this function. All (currently) possible values have been evaluated.

# Simulations

Simulations have been run, using the standard CTCs:

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM3.0-Benchmark-VTM-JVET-K1010 |  |  |  |  | Over HM16.20-HM420-JCTVC-AF1100 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.05% | 0.07% | 101% | 101% | -21.65% | -36.76% | -33.56% | 1061% | 152% |
| Class A2 | 0.00% | 0.03% | -0.02% | 101% | 101% | -20.85% | -23.28% | -17.94% | 1763% | 154% |
| Class B | 0.00% | -0.02% | 0.02% | 101% | 101% | -17.30% | -23.08% | -29.53% | 1968% | 155% |
| Class C | 0.00% | -0.03% | -0.08% | 100% | 102% | -17.62% | -22.68% | -25.64% | 2687% | 162% |
| Class E | 0.00% | -0.03% | 0.16% | 100% | 101% | -20.90% | -25.40% | -28.27% | 1488% | 138% |
| Overall | 0.00% | -0.02% | 0.02% | 100% | 101% | -19.29% | -25.69% | -27.20% | 1783% | 153% |
| Class D | 0.00% | 0.07% | 0.00% | 100% | 101% | -14.46% | -18.45% | -19.91% | 3056% | 164% |
| Class F | 0.00% | -0.16% | 0.09% | 101% | 102% | -17.76% | -25.02% | -27.31% | 1625% | 150% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM3.0-Benchmark-VTM-JVET-K1010 |  |  |  |  | Over HM16.20-HM420-JCTVC-AF1100 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | 0.03% | 0.14% | 100% | 101% | -28.65% | -42.53% | -43.70% | 443% | 136% |
| Class A2 | -0.03% | 0.15% | 0.16% | 100% | 101% | -32.76% | -38.85% | -33.88% | 472% | 134% |
| Class B | 0.01% | 0.05% | -0.12% | 100% | 101% | -27.64% | -40.10% | -41.07% | 471% | 134% |
| Class C | 0.02% | 0.12% | 0.14% | 100% | 101% | -22.58% | -32.08% | -34.10% | 560% | 136% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.01% | 0.09% | 0.06% | 100% | 101% | -27.51% | -38.20% | -38.30% | 488% | 135% |
| Class D | 0.00% | -0.18% | -0.09% | 100% | 100% | -21.81% | -29.67% | -31.06% | 550% | 137% |
| Class F | -0.07% | 0.02% | -0.01% | 100% | 100% | -21.96% | -30.06% | -31.63% | 289% | 122% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM3.0-Benchmark-VTM-JVET-K1010 |  |  |  |  | Over HM16.20-HM420-JCTVC-AF1100 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% | -0.28% | -0.11% | 100% | 101% | -21.73% | -32.35% | -32.37% | 442% | 123% |
| Class C | 0.01% | 0.05% | 0.03% | 100% | 102% | -18.61% | -27.40% | -28.82% | 505% | 128% |
| Class E | 0.05% | -0.01% | 0.22% | 101% | 103% | -23.21% | -30.49% | -32.55% | 247% | 109% |
| Overall | 0.02% | -0.10% | 0.02% | 100% | 102% | -21.06% | -30.23% | -31.23% | 399% | 121% |
| Class D | 0.07% | -0.24% | 0.44% | 100% | 102% | -18.16% | -24.05% | -25.32% | 460% | 128% |
| Class F | 0.09% | -0.17% | -0.33% | 101% | 101% | -21.99% | -32.29% | -34.23% | 261% | 106% |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM3.0-Benchmark-VTM-JVET-K1010 |  |  |  |  | Over HM16.20-HM420-JCTVC-AF1100 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.05% | 0.13% | 0.26% | 100% | 101% | -26.30% | -35.28% | -35.28% | 373% | 128% |
| Class C | 0.00% | 0.24% | -0.12% | 100% | 100% | -20.83% | -28.25% | -29.89% | 414% | 132% |
| Class E | 0.07% | -0.40% | 0.03% | 100% | 99% | -26.03% | -34.02% | -35.75% | 196% | 112% |
| Overall | -0.01% | 0.03% | 0.08% | 100% | 100% | -24.41% | -32.62% | -33.60% | 329% | 125% |
| Class D | -0.05% | -0.29% | 0.12% | 100% | 101% | -19.92% | -25.15% | -26.31% | 368% | 131% |
| Class F | 0.02% | -0.53% | -0.65% | 100% | 101% | -21.47% | -31.57% | -33.68% | 217% | 111% |

The results match those given by the proponent, albeit with minor differences for Random-access, attributed to the method of combining parallel simulations.

The differences observed are likely to only be due to the change to using a fixed length prefix.

# Draft text

The draft text changes have been checked. They add a new limited EGk binarization process, which is broadly the same as that process used in HEVC, albeit accommodating the change from a Cmax of 4 to 6.

The text also makes the change to fix the prefix length to 6. This modification addresses a bug in the text, where the previous variable length was not fully described: a fixed value of 4 was used in two places, whereas it should have been variable.

# Conclusion

The code supplied implements the required operation, the draft text has been studied, and the results match those given in the contribution (v2), with the changes in BD-rate attributed to just the first aspect.

The single length Golomb Rice prefix length of 5 is a simplification over the current VVC draft, and the proposal limits the maximum code length to 32 bits.