| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0834 |
| --- | --- |

| Title: | Crosscheck of JVET-M0464 (Non-CE8: Unified Transform Type Signalling and Residual Coding for Transform Skip, test TSRC-CCB3 and CCB2) |  |  |
| --- | --- | --- | --- |
| Status: | Input |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Sunmi Yoo Jaehyun Lim 19 Yangjaedaero-11gil, Seocho-gu Seoul 06772, South Korea | Tel: Email: | +82-2-6912-6561 sunmi.yoo@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In this document, a crosscheck result of JVET-M0464 [1] is provided. The simulation results are provided in cases that the number of context coded bins (CCB) per pixel of the residual sub-block syntax elements is 3 and 2. The source code is also examined.

It is reported that Y BD-rate changes of CCB3 method are like below :

Average : -0.16% (AI)

Class F : -6.83% (AI)

TGM : -20.84% (AI)

Moreover, Y BD-rate changes of CCB2 method are like below :

Average : -0.16% (AI)

Class F : -4.75% (AI)

TGM : -14.56% (AI)

It is confirmed that the reported result of the proponent and the result from the cross-checker matches from each other.

Introduction

JVET-M0464 proposes two aspects, one is the transform skip (TS) signalling and the other is a coefficient coding for TS coded blocks. In the first aspect, the transform skip flag is one of MTS indices. When the MTS flag is equal to one, it means the current block is transform-skipped. The MTS index greater than 1 corresponds to each of non-zero MTS indices in the current VTM 3.0. Since the TS signal is harmonized with MTS and MTS is not used in chroma coding, TS for chroma is no more available in JVET-M0464. Another aspect is that the coefficient coding for TS is additionally introduced. If the current block is a TS block, syntax elements for the last coefficient position information are no more transmitted. Moreover, there are syntax element changes in the coefficient group. For example, sign flags for each coefficient is included in the first scan pass and it is context-coded, and gtX flag (X = 5, 7, 9) syntax elements are newly introduced. All syntax elements in the coefficient group are context-coded. In the crosscheck requested methods, in order to resolve the throughput issue the context coded bins (CCB) per pixel is restricted to 3 or 2 at the encoder side.

Crosscheck result

The crosscheck simulation is done under the CTC [2], using VTM-3.0 [3] software.

Table 1 JVET-M0464 CCB3 (All intra)

Table 2 JVET-M0464 CCB3 (Random access)

Table 3 JVET-M0464 CCB3 (Low delay B)

Table 4 JVET-M0464 CCB2 (All intra)

Table 5 Table 4 JVET-M0464 CCB2 (Low delay B)

Conclusion

The experimental results from the proponent and the crosschecker are identical from each other. The crosscheck is confirmed.

Reference

- B. Bross, T. Nguyen, P. Keydel, H. Schwarz, D. Marpe, T. Wiegand, “Non-CE8: Unified Transform Type Signalling and Residual Coding for Transform Skip,” JVET-M0464, 13th Meeting: Marrakech, MA, 9–18 Jan. 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-L1010, 12th Meeting: Macao, CN, 3–12 Oct. 2018.

- VTM-3.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-3.0

Patent rights declaration(s)

LG Electronics Inc. does not have any current or pending patent rights relating to the technology described in this contribution.