| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakesh, MA, 9–18 Jan. 2019 | Document: JVET-M0856 |
| --- | --- |

| Title: | Cross-check of JVET-M0446: CE1: Rectangular virtual pipeline data unit (test 1.1.1) and supplementary results |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | A. Wieckowski Fraunhofer HHI Einsteinufer 37 10587 Berlin | Tel: Email: | +49(30) 31002 833 adam.wieckowski@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports the cross-check results for the supplementary results reported in input document JVET-L0446: CE1: Rectangular virtual pipeline data unit (test 1.1.1) and supplementary results. The contribution describes how to unify efficient usage of VPDUs used for CTUs contained within a picture and at picture boundary by allowing rectangular VPDUs.

# Introduction

The test was performed on top of proponents CE1 contribution (test 1.1.1) with the following configuration:

## Test #1

#define CE1_RECT_VPDU                                     1

#define ENC_FAST_RECT_VPDU                         0

#define BOUNDARY_VPDU_FORCE_SPLIT        0

## Test #2

#define CE1_RECT_VPDU                                     0

#define ENC_FAST_RECT_VPDU                         0

#define BOUNDARY_VPDU_FORCE_SPLIT        1

# Results

## Test #1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.52% | -0.70% | -0.50% | 121% | 98% |
| Class A2 | -0.56% | -0.52% | -0.36% | 114% | 100% |
| Class B | -0.23% | -0.13% | -0.29% | 114% | 99% |
| Class C | -0.02% | -0.04% | -0.05% | 107% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.30% | -0.30% | -0.28% | 113% | 99% |
| Class D | -0.03% | -0.11% | -0.03% | 107% | 101% |
| Class F (optional) | -0.14% | -0.15% | -0.07% | 111% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.18% | -0.56% | -0.25% | 111% | 95% |
| Class C | -0.01% | -0.02% | 0.12% | 105% | 106% |
| Class E | -0.54% | -0.72% | -0.38% | 114% | 104% |
| Overall | -0.21% | -0.42% | -0.16% | 110% | 101% |
| Class D | -0.05% | 0.05% | 0.65% | 102% | 105% |
| Class F (optional) | -0.24% | -0.58% | -0.50% | 106% | 100% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.16% | 0.04% | -0.20% | 109% | 96% |
| Class C | -0.02% | 0.10% | 0.11% | 104% | 103% |
| Class E | -0.51% | -0.54% | -1.04% | 111% | 98% |
| Overall | -0.20% | -0.08% | -0.31% | 108% | 99% |
| Class D | -0.05% | -0.19% | 0.62% | 102% | 104% |
| Class F (optional) | -0.16% | -0.49% | -0.49% | 105% | 101% |

## Test #2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class F (optional) | 0.00% | 0.00% | 0.00% | 99% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.06% | 0.10% | 0.03% | 100% | 99% |
| Class A2 | 0.00% | -0.05% | 0.04% | 100% | 101% |
| Class B | 0.06% | 0.06% | 0.10% | 100% | 99% |
| Class C | 0.02% | 0.07% | 0.05% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.05% | 0.06% | 100% | 99% |
| Class D | 0.05% | 0.08% | 0.03% | 101% | 100% |
| Class F (optional) | 0.03% | 0.02% | 0.03% | 100% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.02% | 0.14% | 0.18% | 100% | 105% |
| Class E | 0.19% | -0.16% | -0.17% | 100% | 104% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.09% | 0.26% | 0.00% | 100% | 105% |
| Class F (optional) | 0.00% | 0.00% | 0.13% | 100% | 99% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.04% | 0.26% | 0.04% | 99% | 96% |
| Class C | 0.00% | -0.06% | -0.19% | 100% | 104% |
| Class E | -0.02% | 0.45% | -0.50% | 100% | 99% |
| Overall | 0.01% | 0.20% | -0.17% | 100% | 99% |
| Class D | 0.03% | -0.05% | 0.30% | 100% | 105% |
| Class F (optional) | 0.08% | -0.32% | 0.35% | 100% | 101% |

# Conclusion

The partial results obtained in the cross-check match the results presented in JVET-M0446-v2. The cross-checker confirms that the provided source-code used for simulations enforces edges at VPDU boundaries at picture boundary. It was later commented by some experts, that the block scan order might not follow the VPDU concept, which the cross-checker confirms.

# Reference

- M. Xu, X. Li, S. Liu, “CE1: Rectangular virtual pipeline data unit (test 1.1.1) and supplementary results” JVET-M0446, Jan. 2019, Marrakesh, MA.

# Patent rights declaration(s)

Fraunhofer HHI does not have any current or pending patent rights relating to the technology described in this contribution.