| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0861 |
| --- | --- |

| Title: | Crosscheck of JVET-M0854: (CE4-related: Combination of CE4.4.4a and CE4.4.5b) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Vadim Seregin | Tel: Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution reports cross-checking results for JVET-M0854, which combines CE4.4.4a and CE4.4.5b methods. Test results match the results provided by the proponent.

# Introduction

The software was sent by the proponent, which implements a combination of CE4.4.4a and CE4.4.5b tests.

CE4.4.4a proposes to use additional directional candidates, the candidates are as follows:

| Direction index | 000 | 001 | 010 | 011 | 100 | 101 | 110 | 111 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x-axis | +2 | –2 | 0 | 0 | +1 | -1 | -1 | +1 |
| y-axis | 0 | 0 | +2 | –2 | +1 | -1 | +1 | -1 |

CE4.4.5.b proposes to use adaptive distance based MMVD candidates with two tables shown below, the switch is based on the resolution (Test 1) or by introducing PPS flag (Test 2).

| Distance index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pixel distance | 1/4-pel | 1/2-pel | 1-pel | 2-pel | 4-pel | 8-pel | 16-pel | 32-pel |

| Distance index | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pixel distance | 1-pel | 2-pel | 4-pel | 8-pel | 16-pel | 32-pel | 64-pel | 128-pel |

# Test results

The proposed method was implemented on top of VTM-3.0 software and tested under common test conditions described in JVET-L1010.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.52% | -0.54% | -0.69% | 99% | 99% |
| Class A2 | -0.52% | -0.68% | -0.73% | 100% | 99% |
| Class B | -0.17% | -0.05% | -0.10% | 102% | 99% |
| Class C | -0.19% | -0.14% | -0.09% | 106% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.31% | -0.30% | -0.34% | 102% | 99% |
| Class D | -0.31% | -0.25% | -0.11% | 106% | 99% |
| Class F | -0.10% | -0.09% | -0.08% | 107% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.07% | -0.37% | -0.28% | 102% | 99% |
| Class C | -0.06% | 0.13% | 0.26% | 103% | 102% |
| Class E | -0.11% | -0.01% | 0.02% | 102% | 107% |
| Overall | -0.08% | -0.11% | -0.03% | 103% | 102% |
| Class D | -0.19% | 0.05% | -0.24% | 102% | 99% |
| Class F | -0.05% | 0.06% | -0.39% | 105% | 101% |

Test 1.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.52% | -0.54% | -0.69% | 99% | 98% |
| Class A2 | -0.52% | -0.68% | -0.73% | 100% | 99% |
| Class B | -0.17% | -0.05% | -0.10% | 102% | 98% |
| Class C | -0.19% | -0.14% | -0.09% | 105% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.31% | -0.30% | -0.34% | 102% | 98% |
| Class D | -0.31% | -0.25% | -0.11% | 106% | 98% |
| Class F | -0.10% | -0.09% | -0.08% | 107% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.07% | -0.37% | -0.28% | 103% | 99% |
| Class C | -0.06% | 0.13% | 0.26% | 103% | 102% |
| Class E | -0.11% | -0.01% | 0.02% | 101% | 101% |
| Overall | -0.08% | -0.11% | -0.03% | 102% | 101% |
| Class D | -0.19% | 0.05% | -0.24% | 103% | 99% |
| Class F | -0.04% | 0.06% | -0.39% | 104% | 99% |

Test 2.