| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0862-v1 |
| --- | --- |

| Title: | BoG report on CE2 related contributions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Yuwen He | Email: | yuwen.he@interdigital.com |
| Source: | BoG |  |  |

_____________________________

# Summary

Three sessions were held, 2030 ~ 2230 on Jan. 10, 0900 ~ 0230 and 2045 ~ 2330 on Jan. 11, for discussing 36 technical contributions in five categories,

- Affine motion compensation (6)

- Affine merge mode enhancement (6)

- Sub-block merge mode (4)

- Complexity reduction (13)

- ATMVP simplification (7)

## Recommendations

### Recommended adoptions to VTM

Normative changes

Bugfix of VTM SW

Encoder optimization

### Recommended CE2 tests from following contributions

Affine motion compensation:

Affine merge mode:

Sub-block merge mode:

Complexity reduction for sub-block modes:

(1) sub-block selection based on uni/bi-prediction: one test for JVET-M0226 (2.4.1.a, 2.4.1.b, 2.4.1.c), JVET-M0150 (2.4.3.b), JVET-M0472 (2.4.4.a, 2.4.4.b, 2.4.4.c and 2.4.4.d)

(2) sub-block MV clipping: JVET-M0309 (2.4.2), JVET-M0702

(3) adaptive sub-block size selection and interpolation filter switching: JVET-M0400, JVET-M0310

(4) padding based MC method for small block: JVET-M0248

(5) conformance method: JVET-M0049, JVET-M0049 plus the formular in JVET-M0400

Local buffer storage reduction:

JVET-M0110, JVET-M0168, JVET-M0270, JVET-M0266

ATMVP related:

## Open issues

### Requested review in track

### Revisit in track

JVET-M0432: CE2-related: Combination of CE2.2.3.d and affine inheritance from motion data line buffer

### Contribution not reviewed yet

# Affine motion compensation and affine motion vector coding (6)

JVET-M0145 Non-CE2: Motion vector clipping in affine sub-block motion vector derivation [P. Hanhart, Y. He (InterDigital)]

In affine motion prediction, the motion field is derived for each sub-block based on the control point motion vectors. Even if the control point motion vectors are in the range specified by the motion field storage bit depth, the derived sub-block motion vectors may be outside of the range of values that can be represented within the motion field bit depth, resulting in arithmetic underflow and/or overflow. However, no motion vector clipping to the motion field storage bit depth (e.g., 16-bit in VTM) is considered in the VTM-3.0 software nor in the current VVC draft. To ensure the same behavior between different implementations and to avoid encoder crash in some cases, it is proposed to apply clipping to motion field storage bit depth to both sub-block motion vectors and control point motion vectors after the sub-block motion vector derivation in affine mode. Simulation results show that the proposed fix provides similar performance as VTM-3.0 in terms of BD-rate and timing.

It is reported that there is no range control for sub-block MVs in affine prediction, which may cause crash. It is proposed to introduce clipping on derived MVs in affine prediction. RA/LB(0.00%, 0.00%)

Discussion:

Several proposals are related.

Q: Where to clip the MVs? At MC stage, or at the MV reconstruction stage?

A: At the MV reconstruction stage.

It is commented that it should be done on both stages.

Revisit after reviewing other related proposals

Discussed together with .JVET-M0434.

It was agreed that this is a straight-forward bug-fix.

Decision: Adopt

JVET-M0192 CE2-related: MV Derivation for Affine Chroma [A. Tamse, M. W. Park, K. Choi (Samsung)]

In this contribution, it is proposed to simplify the MV derivation process for chroma sub-block of an affine CU. The size of chroma sub-block is 4x4 samples, which corresponds to 8x8 luma samples. To obtain the MV for the chroma sub-block, the MVs of each of the four 4x4 luma sub-blocks are averaged and rounded. Furthermore, the rounding process employed for computing the average of the four MVs is not symmetric for positive and negative numbers, because the current implementation uses right shifting by 2 after adding an offset of 2 irrespective of sign of the input. This contribution proposes several options simplify the MV derivation process and address the asymmetric rounding issue: 1) Use only the top-left luma sub-block’s MV as the representative MV 2) Use the average of the top-left and bottom-right luma sub-blocks’ MVs as the representative MV 3) Modify the current averaging process of four luma sub-block MVs to a symmetric one 4) Use the average of the top-left and bottom-right luma sub-blocks’ MVs as the representative MV while using symmetric rounding. All the proposed methods show marginal performance change.

Method 1: use top-left MV of 8x8 luma for the MV of 4x4 chroma;

Method 2: use the average of top-left and bottom-right MVs of 8x8 luma as the MV of 4x4 chroma;

Method 3: add symmetric rounding to chroma MV derivation;

Method 4: 2+3

Comment: method 4 has better performance compared to method 1 because of MV averaging. Method 1 and method 2 compared to the method in VTM-3.0 may have some loss for some sequences. The symmetric rounding is used for MV rounding (MVP, affine MV derivation).

Recommendation: Adopt Method 3

JVET-M0247 CE2-related: Joint test of AMVR for Affine Inter mode (Test 2.1.1 and Test 2.1.2) [H. Liu, K. Zhang, L. Zhang, J. Xu (Bytedance), J. Luo, Y. He, X. Xiu (InterDigital)]

This contribution reports results of joint test of CE2.1.1 and CE2.1.2. In this joint test, Adaptive Motion Vector Resolution (AMVR) is extended to affine inter mode, wherein MV precision of an affine-coded CU could be 1/16-pel, 1/4-pel or 1-pel. Simulation results reportedly show 0.32% and 0.23% luma BD-rate saving for Random Access (RA) and Low Delay B (LB) configuration, respectively.

Compared to CE2-1-2 (JVET-M0246) which has been adopted, 0.1% gain is achieved for RA and 0.03% coding gain is achieved for LDB with 3% encoding time increase for RA, 0% encoding time increase for LDB.

The only difference from the CE2-1-2 is encoder optimization: After affine ME in VTM-3, further refine around derived control point in affine ME.

It was asked about how accurate the runtime is. The proponent mentioned the runtime is reliable.

Crosschecking document has not been uploaded yet.

One expert expressed support to adopt.

Recommendation: adopt JVET-M0247 (encoder adoption) after crosschecker confirm the performance.

JVET-M0268 Non-CE2: Interweaved Prediction for Affine Motion Compensation [K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang, P. Zhao, D. Hong(Bytedance)]

With the affine motion compensation (AMC) in the VTM, a coding-block is divided into 4×4 sub-blocks, and each sub-block is assigned with individual motion vectors derived by one affine model. In this contribution, an interweaved prediction approach is proposed for AMC. With the interweaved prediction, a coding block is divided into sub-blocks with two different dividing patterns. Then two auxiliary predictions are generated by AMC with the two dividing patterns. The final prediction is calculated as a weighted-sum of the two auxiliary predictions. All sub-blocks are 4×4 with both the two dividing patterns. The bandwidth is not increased by interweaved prediction since the area used to do MC for all sub-blocks is fetched together as a whole in AMC. Two tests are conducted in this contribution. In Test 1, interweaved prediction is only applied on uni-prediction. In Test 2, interweaved prediction is applied on both uni-prediction and bi-prediction. Simulation results reportedly show 0.27% and 0.38% BD-rate savings for Test 1 and Test 2, respectively, under Random Access (RA) configurations. The BD-rate saving on sequence SlideShow is 2.13% in Test 2.

Test 1: only apply the approach to uni-prediction, 101% decoding time

Test 2: apply both uni- and bi-prediction, 102% decoding time

Significant coding gains are reported for RA: Cacuts: -1.13% for Test 1, -1.64% for Test 2. SlideShow: -1.40% for Test 1, -2.13% for Test 2;

The approach is applied to both luma and chroma.

Comment:

change all uni-prediction to bi-prediction for affine mode with test 1; the approach has a flag in tile group header to enable this feature;

Cross-checker confirmed that the encoding and decoding time match with that reported by proponents.

Q: What’s the performance for chroma?

A: Not tested.

The approach uses sample based weight for the averaging.

Expert want to know the subjective quality impact of proposed approach compared to VTM-3.0. The affine MC method in VTM-3.0 may cause quality issue especially for rotation.

Q: How the method handle VPDU boundary for large CU?

Recommendation: Further study in CE

JVET-M0462 CE2-related: 4x4 chroma affine motion compensation and motion vector rounding unification [L. Pham Van, W.-J. Chien, H. Huang, V. Seregin, M. Karczewicz (Qualcomm)]

This contribution proposes two simplifications for motion vector processing. In the first aspect, the motion vector of a 4x4 affine chroma block is obtained by averaging motion vectors of two diagonal corresponding luma blocks instead of four. This solution leads to an average (Y, U, V) BD-rate change of (0.00%, 0.06%, 0.01%) and (0.03%, 0.01%, 0.12%) under RA and LDB, respectively. The second simplification unifies the motion vector rounding using the TMVP rounding format. This unification results in an average (Y, U, V) BD-rate change of (0.00%, 0.07%, 0.05%) and (-0.01%, -0.08%, 0.12%) under RA and LDB, respectively. The combination of these two approaches leads to an average (Y, U, V) BD-rate change of (0.00%, 0.08%, 0.06%) and (-0.01%, 0.02%, 0.01%) under RA and LDB, respectively.

Refer to meeting notes of JVET-M0192.

JVET-M0467 CE2-related: Symmetric MVD for Affine Bi-prediction Coding [J. Luo, Y. He (InterDigital)] (Chaired by K. Zhang)

This contribution proposes to a symmetric MVD mode for affine bi-prediction mode. With the proposed symmetric MVD mode, affine control point MVDs of list-1 is not signalled but derived from the signalled MVD of list-0, and reference picture indices of both list-0 and list-1 are not signalled but derived based on the POC of reference pictures and the current picture. The simulation results reportedly show on average 0.13% BD rate saving with 106% encoding time for RA configuration over VTM-3.0.

Extend to idea of symmetric MVD to affine prediction.

With an enhanced affine ME method, RA 0.16% 108%

Recommendation:

Study in CE

# Affine merge mode (6)

JVET-M0166 CE2-related: Simplification of constructed affine merging candidate derivation [Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

JVET-M0217 CE2-related: Constructed affine merge candidate simplification [L Li, J Nam, N Park, H Jang, J Lim, S Kim (LGE)]

JVET-M0266 CE2-related: History-based affine merge candidates [K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang, P. Zhao, D. Hong(Bytedance)]

In this contribution, history-based affine merge candidates are proposed to be inserted into the sub-block-based affine merge candidate list. After decoding an affine-coded CU, the affine parameters are added to a history table. When constructing the sub-block prediction affine merge candidate list, the affine parameters stored in the history table are combined with MVs of neighbouring blocks to generate the history-based affine merge candidates. Simulation results reportedly show -0.13% BD-rate changing under Random Access (RA) configuration with almost no impact on encoding and decoding time.

- The method stores non-translational affine parameters instead of CPMV for affine inheritance, it can save local buffer storage for affine inheritance in affine merge and AMVP.

- The method uses affine HMVP for affine inheritance, the HMVP candidate is added after constructed affine candidates for affine merge mode.

Recommendation: Further study in CE

JVET-M0434 CE2-related: Constraint on constructed affine merge candidates [G. Li, X. Xu, X. Li, S. Liu (Tencent)]

In this document, a constraint on constructed affine merge candidates is proposed to limit the constructed affine model parameters such that the control point motion vectors are within a reasonable range. The proposed method was implemented on VTM-3.0. It was reportedly observed that the method has negligible changes on BD-rate performance, at -0.02% on RA, and 0.00% on LDB.

Problem: constructed affine merge candidate from spatial and temporal corner MVs may have very large affine model parameters, and it causes very large MVD due to MV prediction.

The method only constrains non-translational affine parameters. If the affine candidate does not satisfy the constraint, it will not be used for affine merge and affine MVP.

It is commented that (1) the threshold (2) is good enough or not since there are some loss (0.00% for LDB on average, but peak loss 0.12%) for some sequences in LDB; (2) what is the performance if only applying this approach at encoder?

Recommendation: Further study in CE

JVET-M0756 CE2.2.7 related: Affine temporal constructed candidates without pruning [F. Galpin, A. Robert, F. Leleannec, T. Poirier (Technicolor)] [late]

JVET-M0839 CE2-related: On number of fast merge candidates for Affine Merge mode [A. Robert, F. Le Léannec, F. Galpin (Technicolor)] [late]

# Sub-block merge mode [PMVP, triangular mode] (4)

JVET-M0167 CE2-related: Decoupling of SbTMVP and affine merge candidate derivation in subblock merge mode [Y.-L. Hsiao, T.-D. Chuang, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)

JVET-M0273 CE2-related: Early awareness of accessing temporal blocks in sub-block merge list construction [	L. Zhang, K. Zhang, H. Liu, J. Xu, Y. Wang, P. Zhao, D. Hong(Bytedance)]

JVET-M0382 CE2-related: Modification of Triangle and MMVD merge indexes coding [G. Laroche, C. Gisquet, P. Onno, J. Taquet (Canon)]

JVET-M0490 	CE2-related: Simplified context model for triangular prediction mode [M. Gao, X. Li, S. Liu (Tencent)]

# Complexity reduction (13)

Worst case memory bandwidth reduction:

| Proposal | Bandwidth measurement | Others (additional operations per CU to determine SB size, MC fetching unit) Multiplication, addition, comparison, look-up size | Operations of SB MV derivation per 8x8 block (e.g. clipping) | Complexity of MC per 8x8 bi-prediction: Multiplication, addition |
| --- | --- | --- | --- | --- |
| Translational model | 10.69 / 14.69 | No | N/A | Multiplication: 57 Addition: |
| Affine model | 9.09 / 18.19 | No | No clipping | Affine model: Multiplication: 65.5 Addition: 59.125 |
| 2.4.1.a | 9.09 / 13.38 | 2 comparisons per CU (1 for width/height + 1 for uni/bi) | Same | Same |
| 2.4.1.b | 9.09 / 13.38 | 1 comparison per CU (1 for uni/bi) | Same | Same |
| 2.4.1.c | 9.09 / 13.38 | 1 comparison per CU (1 for uni/bi) | Same | Same |
| 2.4.3.b | 9.09 / 10.09 | 1 comparison per CU (1 for uni/bi) | Same |  |
| 2.4.4.a | 9.09/13.38 | 1 comparison per CU (1 for uni/bi) | same | same |
| 2.4.4.b | 9.09/13.38 | 1 comparison per CU (1 for uni/bi) | same | Affine model: Multiplication: 51,5 (reduced by 21%) Addition: 46,875 (reduced by 26%) |
| 2.4.4.c | 9.09/13.38 | 2 comparisons per CU (1 for width/height + 1 for uni/bi) | same | same |
| 2.4.4.d | 9.09/10.09 | 1 comparison per CU (1 for uni/bi) | same | Affine model: Multiplication: 51,5 (reduced by 21%) Addition: 46,875 (reduced by 26%) |
| 2.4.2 (fetching larger region, 4x4 interpolation) | 8.56/10.09 (luma worst case is calculated using restricted MV range, chroma worst case is calculated in the same way as VTM3, clipped MV of luma is used for calculating chroma MV) | Multiplication: 0 Shift: 14 Addition: 9 Number of comparison: 16 Look up table size: 210 bit (7*30) | 4 clipping | No change |
| 2.4.3.a | 9.09 / 13.38 | 3 comparisons per CU (1 for width/height + 1 for uni/bi + 1 for restriction) | Same | Same |
| 2.4.3.c | 9.09 / 11.06 | 13 comparisons per CU (1 for uni/bi + 12 for restriction) | Same | Same |
| 2.4.5 | 5.53 / 11.06 | No | MV clipping for subblk MVs + Find min every 4 subblk MVs (note: 9x9 luma) 14 comparisons for each 8x8 block | Same |
| JVET-M0049 CE2-related: A restriction on memory bandwidth consumption of affine mode | 5.53 / 11.06 | No | Same | Same |
| JVET-M0248 AHG16: Motion compensation with padded samples for small coding units | Translational model: ? / 10.09 Affine Model: ? / 16.81 | N/A | N/A | Translational model: Multiplication: 68 Addition: 58 Affine model: Multiplication: 70 Addition: 59.5 |
| JVET-M0310 CE2-related: Using shorter-tap filter for 4x4 sized partition | Translation and affine mode: 6.59 / 13.19 | Same | Same | Affine 4x4 bi: Mul: 50, Add: 43.75 4x4 uni: Mul: 26.5, Add: 23 |
| JVET-M0348 CE2/4-related: Further reducing VVC memory bandwidth worst case by combining 4x4/4x8/8x4 bi-prediction with AMVR | Transaltional model: 10.69 / 10.09 | No | No | Translational model: Same for 8x8 bi-prediction |
| JVET-M0400 CE2-related: Worst-case memory bandwidth reduction for VVC | 9.09 / 11.06 | 13 comparisons per CU (1 for uni/bi + 12 for restriction) | Same | a. Same b. same |
| JVET-M0702 CE2-related: Adaptive sub-block MV clip for affine blocks [late] | Tx=0,Ty=0: 5.05/10.09 (same as anchor 8x8) Tx=1,Ty=0: 5.28/10.56 Tx=0,Ty=1: 5.28/10.56 Tx=1,Ty=1: 5.53/11.06 | No | 23 comparisons per 8x8 | Same |

Recommendation: Further study in two directions: (1) using conformance method like JVET-M0049 to constrain worst case memory bandwidth; (2) to reduce worst case memory bandwidth and computation complexity of small block MC (e.g. 4x4, 4x8 and 8x4), such as using shorter interpolation filter. The following proposals are recommended for further study in CE.

(1) sub-block selection based on uni/bi prediction: one test for JVET-M0226 (2.4.1.a, 2.4.1.b, 2.4.1.c), JVET-M0150 (2.4.3.b), JVET-M0472 (2.4.4.a, 2.4.4.b, 2.4.4.c and 2.4.4.d)

(2) sub-block MV clipping: JVET-M0309 (2.4.2), JVET-M0702

(3) adaptive sub-block size selection and interpolation filter switching: JVET-M0400, JVET-M0310

(4) padding based MC method for small block: JVET-M0248

(5) conformance method: JVET-M0049, JVET-M0049 plus the formula in JVET-M0400

The results only applying to affine mode need to be provided to compare the performance. Other results such as applying the method to all inter modes are optional.

Use memory patterns to measure worst case memory bandwidth in the CE:

Panasonic (J. Li) provides some memory patterns and worksheet for measurement in all CE tests. It can measure luma, chroma and overall bandwidth.

Reduction of local buffers

|  | Local storage (bits per CTU: 36 bits per CPMV, 3 bits for CU width, 3 bits for CU height, 10 bits for CU top-left x and y, 1 bit for affine type) | Line buffer for (bits per 8x8) | Additional logics |
| --- | --- | --- | --- |
| Affine model in VTM-3.0 | 7689 (33*(36*3*2 + 3*2 +10 + 1)) | 8 (width, x) |  |
| 2.4.7 | 3961 (17*(36*3*2 + 10 + 3*2 + 1)) | Same | No |
| 2.4.8 | 7359 (33*(36*2*3 + 3*2 + 1)) | Same | No |
| JVET-M0110 Alignment of affine control-point motion vector and subblock motion vector | 1. 8x8 grid 2937 (33*(36*2+11 +3*2)) 2. 16x16 grid 1513 (17*(36*2 + 11 + 3*2)) | Same | No |
| JVET-M0168 CE2-related: Simplifications for inherited affine candidates | Test 1: same as VTM-3.0 Test 2: same as VTM-3.0 Test 3: 2376 ((128/8 * 2 + 1) * (18*2*2)) Test 4: 2376 ((128/8 * 2 + 1) * (18*2*2)) | 0 |  |
| JVET-M0270 CE2-related: An alternative storing method for affine inheritance | 2112 (33*(32*2 )) (Percentage: 27.6%) | 0 | Remove redundant affine parameter derivation in VTM-3. Add one checking when inherited crossing CTU rows |
| JVET-M0432 CE2-related: Combination of CE2.2.3.d and affine inheritance from motion data line buffer | 1165 (233*5) |  |  |
| JVET-M0266 |  |  |  |

Revisit JVET-M0432 in track;

Recommend the following proposals related to local buffer storage reduction for CE tests:

JVET-M0110, JVET-M0168, JVET-M0270, JVET-M0266

JVET-M0049 CE2-related: A restriction on memory bandwidth consumption of affine mode [M. Zhou (Broadcom)]

To constrain the memory bandwidth consumption of the affine mode at PU level, it is proposed to impose a bitstream constraint to disallow the affine model parameters that can lead to the reference block bounding box size exceeding a pre-defined threshold.

- For the setting in which the threshold of luma reference block bounding box is set to 16*16 for 2x2 sub-block vectors of bi-directional affine mode, and to16*12 for 2x1 sub-block vectors of unidirectional affine mode, the BD-rate difference is 0.05% in RA, 0.04% in LD_B and -0.02% in LD_P, respectively.

- For the setting of threshold 17*17 for 2x2 sub-block vectors of bi-directional affine mode and  17*13 for 2x1 sub-block vectors of unidirectional affine mode, the BD-rate difference is 0.01% in RA, 0.06% in LD_B and 0.01% in LD_P, respectively.

- For the setting in which the threshold of luma reference block bounding box is set to 16*16 for 2x2 sub-block vectors of bi-directional affine mode, and  to 16*12 for both 2x1 and 1x2 sub-block vectors of unidirectional affine mode, the BD-rate difference is 0.06% in RA, 0.02% in LD_B and -0.01% in LD_P, respectively.

For the setting of threshold 17*17 for 2x2 sub-block vectors of bi-directional affine mode and  17*13 for both 2x1 and 1x2  sub-block vectors of unidirectional affine mode, the BD-rate difference is 0.00% in RA, 0.03% in LD_B and 0.01% in LD_P, respectively.

Method 1: limit 2x1 sub-block group in a bounding window for uni-prediction; limit 2x2 sub-block group in a bounding window for bi-prediction;

Test 1: limit 2x1 sub-block group to 16x12 memory access region for uni-prediction, and limit 2x2 sub-block group to 16x16 memory access region for bi-prediction, BD rate loss is 0.05% and 0.04% for RA and LDB;

Test 2: limit 2x1 sub-block group to 17x13 memory access region for uni-prediction, and limit 2x2 sub-block group to 17x17 memory access region for bi-prediction, BD rate loss is 0.01% and 0.06% for RA and LDB;

Method 2: limit 2x1 sub-block group in a bounding window and limit 1x2 sub-block group in a bounding window for uni-prediction; limit 2x2 sub-block group in a bounding window for bi-prediction;

Test3: 16x16, BD rate loss 0.06% and 0.02% for RA and LDB;

Test4: 17x17, BD rate loss 0.00% and 0.03% for RA and LDB;

Feature: apply conformance constraint at encoder side

Comments:

(1) apply the constraint both to encoder and decoder;

(2) For affine mode, there is computation complexity issue too because 4x4 sub-block is used for MC;

(3) Experts think this approach is good.

Recommendation: This approach should be seriously considered if VVC wants to address worst case memory bandwidth issue of affine mode.

JVET-M0110 CE2-related: Alignment of affine control-point motion vector and subblock motion vector [H. Huang, W.-J. Chien, V. Seregin, H. Wang, M. Karczewicz (Qualcomm)]

This contribution proposes to use the subblock motion vectors as the affine control-point motion vectors (CPMVs). The affine motion inheritance uses the subblock motion vectors from the neighboring coded block, and they are not necessary control-point motion vectors. It’s reported that the BD-rates in random access common test condition are -0.01%, 0.01%, 0.00%, and are 0.03%, -0.17%, -0.03% in low delay B.

BD rate: -0.01% (peak loss 0.16% class D BQSquare) for RA; 0.03% (peak loss 0.24% class D BQSquare) for LDB;

Recommendation: Further study in CE with 16x16 grid

Following the additional test is suggested:

- Test on shifting CPMV by (2, 2) and changing MVP derivation order for AMVP and constructed merge candidates;

JVET-M0168 CE2-related: Simplifications for inherited affine candidates [Y.-L. Hsiao, T.-D. Chuang, C.-W. Hsu, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)

This contribution proposes simplifications for inherited affine candidates at coding tree unit (CTU) row boundaries and inside the current CTU row. Compared with the inherited affine candidates in VTM3.0, it is asserted that the CU width information line buffer and the CPMV buffer can be removed. At CTU row boundaries, it is proposed to use two or three subblock motion vectors (MVs) with a fixed distance for affine candidate inheritance, where an accurate distance is used to derive the affine motion and coding unit (CU) width information is not needed anymore for saving the CU width information line buffer. Inside the current CTU row (i.e., not at CTU row boundaries), it is proposed to use two or three subblock MVs with half CU height or half CU width for affine candidate inheritance, where control point motion vectors (CPMVs) are not needed anymore for saving the CPMV buffer. Results are summarized as follows.

Proposed inherited affine candidate derivation at CTU row boundaries by using two subblock MVs with a fixed distance equal to 16
VTM3.0-RA: {Y-BD-rate=0.00%, EncT=100%, DecT=101%}
VTM3.0-LB: {Y-BD-rate=0.04%, EncT=100%, DecT=100%}

Proposed inherited affine candidate derivation at CTU row boundaries by using two subblock MVs when the MVs are bi-prediction and using three subblock MVs when the MVs are uni-prediction with a fixed distance equal to 16
VTM3.0-RA: {Y-BD-rate=-0.02%, EncT=100%, DecT=101%}
VTM3.0-LB: {Y-BD-rate=0.01%, EncT=100%, DecT=100%}

Proposed inherited affine candidate derivation inside the current CTU row by using subblock MVs with half CU width or half CU height and inherited affine candidate derivation at CTU row boundaries by using two subblock MVs with a fixed distance equal to 16
VTM3.0-RA: {Y-BD-rate=0.00%, EncT=100%, DecT=100%}
VTM3.0-LB: {Y-BD-rate=0.02%, EncT=101%, DecT=100%}

Proposed Inherited affine candidate derivation inside the current CTU row by using subblock MVs with half CU width or half CU height and inherited affine candidate derivation at CTU row boundaries by using two subblock MVs when the MVs are bi-prediction and using three subblock MVs when the MVs are uni-prediction with a fixed distance equal to 16
VTM3.0-RA: {Y-BD-rate=-0.02%, EncT=100%, DecT=100%}
VTM3.0-LB: {Y-BD-rate=0.02%, EncT=101%, DecT=101%}

Method 1: reduce the CU width and x position in the line buffer; BD rate 0.00% for RA, 0.04% for LDB;

Method 2: derive 6-parameter affine model at CTU row boundary without additional storage, but need additaional controlling logics; BD rate -0.02% for RA, 0.01% for LDB;

Method 3: use sub-block MV for affine inheritance; BD rate for the test combining method 1 and 3: 0.00% for RA, 0.02% for LDB;

Recommendation: Further study in CE for method 1 and method 3 aligned with JVET-M0110.

JVET-M0228 CE2-related: Affine mode simplifications [Y.-W. Chen, X. Wang (Kwai Inc.)]

JVET-M0248 AHG16: Motion compensation with padded samples for small coding units [H. Liu, J. Chon, H.-C. Chuang, L. Zhang, K. Zhang, J. Xu (Bytedance)]

In current VVC, bi-predicted 4x16, 16x4, 4x8 and 8x4 coding units (CU) and uni-predicted 4x4 CUs require even higher memory bandwidth than 8x8 bi-predicted CUs. This contribution proposes to fetch only some of the reference pixels and pad other reference pixels in the motion compensation process. With proposed method, it is asserted that the worst-case memory bandwidth occurs in 8x8 bi-predicted CUs which is aligned with HEVC design. Simulation results reportedly show negligible impact on coding performance.

Solution:

- 4x4 uni-predicted CU: for luma, the bottom row is padded; for chroma, no padding is required.

- 4x8/8x4 bi-predicted CU: for luma, the top row and the bottom 2 rows are padded, and the left column and the right column are padded; for chroma, the bottom row and the right column are padded.

- 4x16/16x4 bi-predicted CU: for luma, the bottom row and the right column are padded; for chroma, the bottom row is padded.

Padding is applied to the MC of both luma and chroma components, and padding number for luma and chroma may be different.

Performance:

Worst case memory bandwidth with padding is equivalent to 8x8 Bi

BD rate: 0.01% for RA, 0.04 for LDB; peak loss: 0.15% for RA (class D RaceHorse), 0.28% for LDB (class F SlideShow)

Comments from experts: all memory bandwidth reduction related proposals should evaluate memory bandwidth reduction by considering memory pattern. Panasonic will provide some memory patterns for CE2 group to select memory patterns for evaluation.

Recommendation: further study in CE

JVET-M0270 CE2-related: An alternative storing method for affine inheritance [K. Zhang, L. Zhang, H. Liu, J. Xu, Y. Wang, P. Zhao, D. Hong(Bytedance)]

In this contribution, two methods are proposed to modify the affine inheritance process. Method 1 is proposed to address the line-buffer memory issue. It is proposed to inherit the affine model from MVs of a neighbouring 4×4 block and its adjacent 4×4 block. Therefore, the side information required to be stored in the line-buffer, including the CU width and the x-component of the CU top-left coordinate, are totally removed from line buffer. Method 2 is proposed to address the In-CTU-buffer memory issue. It is proposed to store affine parameters instead of CPMVs, CU top-left coordinate, CU width and CU height. It is reported that the additional line-buffer memory required by affine inheritance can be reduced from 8192 bits (or 2K bytes in a byte-alignment implementation) to 0. And the additional In-CTU-buffer memory required by affine inheritance can be reduced by about 70%. Simulation results reportedly show 0.01%, 0.01% and 0.02% BD-rate changing under Random Access (RA) configuration with Method 1, Method 2 and Method 1+2, respectively.

Method 1: remove all line buffer storage; and MV of above CTU row needed to be stored in cache is reduced from (62+34) to (2+34);

Method 2: reduce the local buffer and remove redundant calculations for affine model parameter derivation;

Comment: class E has some loss for the LDB in the combined test (method 1 and method 2).

Recommendation: further study in CE; suggest the additional test using different precisions.

JVET-M0310 CE2-related: Using shorter-tap filter for 4x4 sized partition [	J. Li, R.-L. Liao, C. S. Lim (Panasonic)]

This contribution uses 6-tap filter is used for 4x4 sized partitions including 4x4 CU and sub-block of affine. it reduces worst case memory bandwidth of 4x4 sized CU by 67% and complexity of affine to 65% number of multiplications and 62% number of additions. Experimental result shows 0.08% luma BD-rate change in RA and 0.14% luma BD-rate change in LD_B. Ignorable change in encoding and decoding time are observed.

Solution: propose 6 tap filter for 4x4 MC. Worst case memory bandwidth: 6.59 / 13.19 (VTM-3.0 (4x4) 9.09/18.19)

Performance: BD rate 0.08% for RA (peak: 0.58% for class D BQSquare), 0.14% for LDB (1.14% for class D BQSquare)

Recommendation: further study in CE

JVET-M0311 CE2-related: Memory bandwidth reduction for affine mode with less dependency [J. Li, R.-L. Liao, C. S. Lim (Panasonic)]

JVET-M0348 CE2/4-related: Further reducing VVC memory bandwidth worst case by combining 4x4/4x8/8x4 bi-prediction with AMVR

In current VVC standard, 4x4 bi-prediction inter CUs are disallowed because of the memory bandwidth issue. And 4x8/8x4 bi-prediction inter CUs are allowed, which causes worst-case memory bandwidth increase. This contribution proposes an improvement method that combines 4x4/4x8/8x4 bi-prediction inter CUs with full pixel and 4-pixel AMVR, which allows 4x4/4x8/8x4 bi-prediction and avoid the increase of memory bandwidth compare to HEVC. There are two test conditions conducted, (a) 4x4 bi-directional inter CUs + full pixel and 4-pixel AMVR mode; (b) 4x4/4x8/8x4 bi-directional inter CUs + full pixel and 4-pixel AMVR mode. The proposed method (a) can achieve 0.01% and 0.03% bit-rate reduction with no encoding and decoding time increase respectively under RA and LDB cases. The proposed method (b) can reduce the worst-case memory access bandwidth by about 32% compared to the current VVC, with 0.01% and 0.03% bit-rate reduction and xxx% encoding and xxx% decoding time reduction respectively under RA and LDB cases.

Solution: for bi-prediction AMVP mode, only allows 1-pel and 4-pel for 4x4/4x8/4x16 and 8x4/16x4 inter mode by changing AMVR sigaling (only the first bin is used); for normal bi-prediction inter merge mode with CU size 4x4/4x8/4x16 and 8x4/16x4, rounds MV to 1-pel. For those inter modes, it can save MC computation complexity because interpolation is skipped.

Performance: For normal inter mode, worst memory bandwidth is 10.69(Uni) / 10.09 (Bi); need to revisit after full results are ready.

JVET-M0400 CE2-related: Worst-case memory bandwidth reduction for VVC [W.-J. Chien, L. Pham Van, H. Huang, V. Seregin, M. Karczewicz (Qualcomm)]

This contribution proposes two different approaches to reduce the worst-case memory bandwidth of motion compensation. The first solution (block size restriction) is to restrict bi-predictions that have memory bandwidth exceeding a predefined level. In also this approach, the sub-block size of affine prediction can be set to be higher than 4x4 to ensure the memory requirement. In the second solution (switch filter), the luma interpolation of a prediction can be performed by the chroma interpolation filter to ensure the memory bandwidth requirement. The average luma BD-rate changes of the tests that achieve the worst-case bandwidth of 47% (changes only applied to affine based on condition), 14% (changes only applied to affine based on condition and 4x8 non-affine mode) and 8% (always applied to affine and 4x8/8x4/4x16/16x4) the worst-case of HEVC are:

Using block size restriction:

RA: 0.03%, 0.12% and 0.51%, respectively.

LDB: -0.01%, 0.13%, and 0.53%, respectively.

Using switch filter:

RA: -0.01%, 0.02, and 0.26%, respectively.

LDB: 0.02%, 0.01%, and 0.45%, respectively.

For non-affine mode, the interpolation filter is determined by CU size; for affine mode, the interpolation filter is determined by CPMV and CU size.

Comment: this method may switch the interpolation filter for the same affine CU size; the method depends on affine or non-affine mode.

Recommendation: further study in CE

JVET-M0432 CE2-related: Combination of CE2.2.3.d and affine inheritance from motion data line buffer [G. Li, X. Xu, X. Li, S. Liu (Tencent), J. Zhao, S. Kim (LGE)]

This document reports test results of combining CE2.2.3.d (replacing original affine model inheritance with inheritance from affine HMVP) and keeping original affine model inheritance from motion data line buffer. It was observed that the BD-rate performance compared with VTM-3.0 is 0.03% and 0.07% on RA and LDB, respectively. Compared to CE2.2.3.d, the BD-rate performance is -0.04% and -0.02% on RA and LDB, respectively.

Compared to CE2.2.3.d, the current approach enables affine inheritance from line buffer at CTU row boundary. The improvement is -0.04% for RA and -0.02% for LDB.

It saves 84% for local buffer storage. Over VTM-3.0, there is a small loss: 0.03% and 0.07% for RA and LDB.

Recommendation: Revisit in Track B.

JVET-M0477 CE2: Simplification of Affine constructed merge candidates (Test 2.2.6.c and 2.2.6.d) and supplementary results [Y.-C. Yang, Y.-J. Chang (Foxconn)]

JVET-M0702  CE2-related: Adaptive sub-block MV clip for affine blocks [X. Li, M. Gao, S. Liu (Tencent)][late]

In this contribution, adaptive sub-block MV clip is proposed for affine blocks to reduce worst case memory bandwidth while keeping coding efficiency based on constraining max difference among 4x4 level MVs within an 8x8 sub-blocks to no larger than 1 pixel. It is further proposed to use different thresholds for x and y components for better trade-off. It is reported 0.16%/0.06%/0.03%, 0.09%/0.03%/0.01%, 0.11%/0.03%/0.03%, and 0.01%/-0.02%/0.00% luma BD-rate increase over VTM-3.0 for RA, LDB, and LDP when {Tx=0, Ty=0}, {Tx=1, Ty=0}, {Tx=0, Ty=1}, and {Tx=1, Ty=1}, respectively. With the proposed methods, the worst case memory bandwidth for affine blocks is reportedly reduced to 7.03, 7.5, 7.5 and 8 luma samples/pixel when {Tx=0, Ty=0}, {Tx=1, Ty=0}, {Tx=0, Ty=1}, and {Tx=1, Ty=1}, respectively. Note that 7.03 luma samples/pixel is the memory bandwidth for regular 8x8 bi-predicted blocks. SIMD implementation of the proposed method is also provided.

Compared to CE2.4.5 ({Tx=1, Ty=1}), applying nearest neighbor rounding additionaly, performance is imporved with same setting {Tx=0, Ty=0}.

Recommendation: further study in CE

# ATMVP simplification (7)

JVET-M0116 CE2-related: ATMVP simplification [R. Yu, D. Liu, K. Andersson, R. Sjöberg (Ericsson)]

JVET-M0204 CE2-related: Simplification of ATMVP [M. Park, M. W. Park, S. Jeong, A. Tamse, K. Choi (Samsung)]

JVET-M0240 CE2-related: Simplification of subblock-based temporal merging candidates [H. Lee, S.-C. Lim, J. Lee, J. Kang, H. Y. Kim (ETRI)]

JVET-M0338 Non-CE2 : Simplified neighboring spatial coding unit derivation for SbTMVP [H.Jang, J.Nam, S.Kim, J.Lim(LGE)]

JVET-M0343 Non-CE2 : Simplified subblock motion derivation for SbTMVP [H.Jang, J.Nam, S.Kim, J.Lim(LGE)]

JVET-M0515 Non-CE2.5: ATMVP Collocated Block Derivation from History-based Candidate [C.-C. Chen, W.-J. Chien, Y. Zhang, C.-H. Hung, Y. Han, H. Huang, M. Karczewicz (Qualcomm)]

JVET-M0526 CE2-related: Further simplification of ATMVP collocated block derivation [S.H. Wang (Peking University), X. Zheng (DJI), S.S. Wang, S.W. Ma]