| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0866-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-M0832 (Non-CE3: On block size restrictions for PDPC with disabled linear filtering for PDPC in the case of skew non-diagonal modes) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ling Li 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 | Tel: Email: | +82-2-6912-6561 l.li@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This is a crosscheck report of JVET-M0832 (Non-CE3: On block size restrictions for PDPC with disabled linear filtering for PDPC in the case of skew non-diagonal modes). The simulations were carried out based on the common test condition (CTC). The test results are matched for all points and the software matches the description in the proposal.

Proposed Method

In this contribution, we test the combinations of Tests 3, 5 and 7 described in JVET-M0122 with disabled linear filtering for PDPC in the case of skew non-diagonal modes as proposed in JVET-M0238 (see Table 1).

Table 1. Summary of the proposed tests

| Test # | Description |
| --- | --- |
| 1 | Test 3 of JVET-M0122 with disabled linear filtering for PDPC in the case of skew non-diagonal modes as proposed in JVET-M0238 |
| 2 | Test 5 of JVET-M0122 with disabled linear filtering for PDPC in the case of skew non-diagonal modes as proposed in JVET-M0238 |
| 3 | Test 7 of JVET-M0122 with disabled linear filtering for PDPC in the case of skew non-diagonal modes as proposed in JVET-M0238 |

Experimental Results

Table 2 Experiment result for test 1 of all intra (AI)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.05% | -0.10% | 0.00% | 102% | 100% |
| Class A2 | 0.04% | 0.04% | -0.03% | 100% | 100% |
| Class B | 0.04% | 0.03% | 0.00% | 101% | 102% |
| Class C | 0.03% | -0.02% | -0.18% | 101% | 105% |
| Class E | 0.07% | -0.05% | 0.01% | 102% | 102% |
| Overall | 0.03% | -0.02% | -0.04% | 101% | 102% |
| Class D | 0.05% | -0.10% | 0.02% | 103% | 102% |
| Class F | -0.12% | -0.17% | -0.12% | 101% | 104% |

Table 3 Experiment result for test 1 of random access (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.03% | 0.13% | 100% | 99% |
| Class A2 | 0.01% | 0.03% | 0.13% | 100% | 100% |
| Class B | 0.03% | 0.13% | 0.03% | 100% | 101% |
| Class C | 0.01% | -0.15% | -0.04% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.01% | 0.05% | 100% | 101% |
| Class D | 0.02% | -0.45% | -0.32% | 101% | 102% |
| Class F | -0.10% | -0.15% | -0.05% | 100% | 102% |

Table 4 Experiment result for test 2 of all intra (AI)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | -0.02% | -0.07% | 100% | 100% |
| Class A2 | -0.05% | 0.03% | -0.08% | 99% | 101% |
| Class B | 0.03% | -0.03% | -0.02% | 101% | 103% |
| Class C | 0.00% | -0.11% | -0.10% | 101% | 104% |
| Class E | 0.02% | -0.03% | 0.01% | 101% | 102% |
| Overall | -0.01% | -0.04% | -0.05% | 101% | 102% |
| Class D | 0.07% | -0.09% | -0.08% | 103% | 101% |
| Class F | -0.19% | -0.23% | -0.19% | 101% | 102% |

Table 5 Experiment result for test 2 of random access (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.08% | 0.11% | 99% | 97% |
| Class A2 | -0.04% | 0.11% | 0.15% | 99% | 98% |
| Class B | 0.01% | 0.14% | -0.01% | 100% | 101% |
| Class C | 0.06% | -0.19% | 0.02% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.04% | 0.06% | 100% | 100% |
| Class D | 0.03% | -0.36% | -0.25% | 100% | 101% |
| Class F | -0.23% | -0.20% | -0.11% | 100% | 102% |

Table 6 Experiment result for test 3 of all intra (AI)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | 0.02% | 0.03% | 101% | 100% |
| Class A2 | -0.05% | -0.02% | -0.11% | 100% | 100% |
| Class B | 0.01% | 0.00% | -0.03% | 100% | 103% |
| Class C | -0.03% | -0.07% | -0.17% | 101% | 104% |
| Class E | 0.00% | -0.09% | -0.12% | 102% | 103% |
| Overall | -0.03% | -0.03% | -0.08% | 101% | 102% |
| Class D | 0.02% | -0.09% | -0.14% | 102% | 101% |
| Class F | -0.15% | -0.16% | -0.01% | 101% | 103% |

Table 7 Experiment result for test 3 of random access (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.08% | 0.10% | 99% | 97% |
| Class A2 | -0.04% | 0.13% | 0.10% | 99% | 97% |
| Class B | -0.01% | 0.16% | 0.07% | 100% | 101% |
| Class C | -0.01% | -0.13% | -0.07% | 101% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.06% | 0.05% | 100% | 100% |
| Class D | 0.02% | -0.39% | -0.12% | 101% | 104% |
| Class F | -0.16% | -0.11% | -0.11% | 100% | 102% |

Summary

The test results are matched for all points and the software matches the description in the proposal.

Patent rights declaration(s)

LG Electronics does not have any current or pending patent rights relating to the technology described in this contribution.