| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0873-v1 |
| --- | --- |

| Title: | BoG report on CE10 related contributions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Chih-Wei Hsu Martin Winken | Email: | cw.hsu@mediatek.com martin.winken@hhi.fraunhofer.de |
| Source: | BoG |  |  |

_____________________________

Summary

The CE10-related documents were categorized as follows:

- CIIP modifications (12)

- OBMC (2)

- TPM modifications (23)

- LIC (6)

- Other (1)

The BoG met on 13 Jan. 2019 from 6:00pm-10:30pm, 14 Jan. 2019 from 1:00pm-x:xpm

Recommendations

Recommended adoptions to VTM

Normative changes

Recommendations:

TPM:

- Adopt syntax change of JVET-M0118

  - Same as in M0185, M0190, M0216 (the first aspect), M0234 (change corresponding to the result table 7 and 8), M0317 (section 2.2), M0328 (test E)

- Adopt using only weight group (second weight as in JVET-M0328)

- Adopt using only one context model for merge_triangle_flag (JVET-M0490)

Recommended CE10 tests from following contributions

TPM:

- Merging candidate list construction:

  - Re-use general merging candidate list construction as common part

  - Study different variants thereof within CE

- Signalling:

  - All proposals use 3 syntax elements for the signaling (split direction and 2 merge indices)

  - Study different variants thereof within CE

- Study combination of triangular prediction and MMVD (within CE about motion vector coding)

Open issues

Revisit in track

CIIP Modifications (12)

JVET-M0082 CE10-related: Simplification of Multi hypothesis intra prediction [Y. Kato, K. Abe, T. Toma (Panasonic)]

In this contribution, striped noise in coded images by VTM3.0 is reported and a simplification of multi hypothesis intra prediction (MHIntra) is proposed as a potential solution of it. In the current MHIntra, intra prediction block is divided into 4 regions when the intra mode is vertical or horizontal, and different weights are used for blending with inter prediction block in each region. In this proposal, MHIntra mode is used only when intra mode is DC or Planar for which uniform weight is used for the blending. The subjective image quality problem is reportedly solved by this method. Simulation results show that BD-rate is 0.02% loss for RA, 0.11% loss for LDB with the VTM configurations.

JVET-M0808 Cross-check result of JVET-M0082 (CE10-related: Simplification of Multi hypothesis intra prediction) [K. Kawamura, S. Naito (KDDI)] [late]

JVET-M0096 CE10.1-related: Inter-intra prediction [L. Pham Van, G. Van der Auwera, A. K. Ramasubramonian, V. Seregin, M. Karczewicz (Qualcomm)]

This contribution proposes a modified blending technique for inter-intra prediction. Firstly, only the intra planar mode is allowed in the inter-intra mode. Secondly, the weights for planar and merge predicted samples and neighbouring reconstructed samples are adaptively selected in the blending process. It is asserted that the proposed modifications simplify the inter-intra prediction and improve the performance. The average luma BD-rate gain is 0.10%, 0.15%, and 0.22% for the RA, LDB, and LDP configurations, respectively. The average encoding time is reportedly reduced without an average increase of decoding time. (EncT, DecT) is (96%, 99%), (97%, 100%), and (98%, 98%) under the RA, LDB, and LDP respectively.

JVET-M0681 Crosscheck of JVET-M0096 (CE10.1-related: Inter-intra prediction) [F. Galpin, T. Poirier (Technicolor)] [late] [miss]

JVET-M0183 CE10-related: Simplification of MPM generation for CIIP [M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

This contribution proposes to simplify the most probable mode (MPM) generation for combined inter intra prediction (CIIP) in two aspects. Aspect 1 is that when generating the MPM list for CIIP, the default mode of the referenced-neighbouring coding unit (CU) is aligned with that for non-CIIP intra mode, i.e., changed from DC to planar. Aspect 2 is to further simplify the intra mode signalling for CIIP based on Aspect 1. The number of MPMs for CIIP is reduced from three to one. It is reported that, compared to VTM3.0, the proposed method results in -0.01% and 0.03% luma BD-rates for RA and LB, respectively.

JVET-M0693 Crosscheck of JVET-M0183 (CE10-related: Simplification of MPM generation for CIIP) [Z. Zhang, K. Andersson, R. Sjöberg (Ericsson)] [late] [miss]

JVET-M0695 Crosscheck of JVET-M0183 (CE10-related: Simplification of MPM generation for CIIP) [K. Andersson (Ericsson)] [late] [miss]

JVET-M0232 CE10-related: Simplification of CIIP Intra mode coding [Y.-W. Chen, X. Wang (Kwai Inc.)]

In this contribution, a modification of intra mode coding for combined inter and intra prediction (CIIP) is proposed. In the current CIIP scheme, most probable mode (MPM) mechanism is used to predict the intra modes of CIIP. In this contribution, we propose to remove the MPM mechanism for the intra mode coding of CIIP. Instead, the truncated unary codewords are used for the 4 intra modes of CIIP. The results reportedly show that the proposed simplification incurs 0.00% and 0.02% BD-rate increase for RA and LDB, respectively.

JVET-M0552 Crosscheck of JVET-M0232 (Non-CE10: Simplification of CIIP Intra mode coding) [M.-S. Chiang (MediaTek)] [late] [miss]

JVET-M0276 CE10-related: MPM list alignment between CIIP and intra mode [J. Li, S. Wang, W. Gao (Peking University), L. Zhang, K. Zhang, H. Liu, J. Xu (Bytedance)]

Combined Inter-Intra mode Prediction (CIIP) utilizes a new MPM list construction process to select intra prediction mode which is different from that developed for intra mode coding. In this contribution, it is proposed to reuse the MPM list construction process for intra mode for CIIP. Only the first four or first one mode of the intra MPM list are allowed for coding a CIIP coded block. Simulation results reportedly show that the unification of MPM list achieves 0.02% and 0.01% BD rate savings for RA Main10 with four and one allowed modes, respectively.

JVET-M0755 Crosscheck of JVET-M0276 (CE10-related: MPM list alignment between CIIP and intra mode) [Y.-W. Chen (Kwai Inc.)] [late] [miss]

JVET-M0294 CE10-related: Modification for blocks applied with Combined Inter-Intra prediction [B. Wang, A.M. Kotra, S. Esenlik, H. Gao, J. Chen (Huawei)]

This contribution proposes two modifications for blocks applied with combined inter-intra prediction (for simplification, these blocks are named as combined prediction blocks). The first modification occurs to MPM (Most Probable Mode) list construction, where combined prediction blocks are considered as inter blocks always. The second modification is applied for deblocking filter, where combined prediction blocks are considered as intra blocks in boundary strength derivation. Two solutions are proposed: the first solution applies modification only for MPM list construction, the second solution applies both modification of MPM list construction and boundary strength derivation. The BD-rate results over the VTM3.0 anchor are:

- -0.01% (RA), 0.01% (LDB), -0.04% (LDP) for luma component with the first solution

- 0.00% (RA), 0.00% (LDB), -0.07% (LDP) for luma component with the second solution

AI results are the same to the anchor as modifications have no effect on I frames, wherein no combined prediction blocks exist.

JVET-M0587 Crosscheck of JVET-M0294 (CE10-related: Modification for blocks applied with Combined Inter-Intra prediction) [K. Zhang (Bytedance)] [late] [miss]

JVET-M0296 CE10-related: Simplification on combined inter-intra mode prediction [B. Wang, S. Esenlik, A.M. Kotra, H. Gao, J. Chen (Huawei)]

This contribution proposes a simplification for blocks applied with combined inter-intra prediction (for simplicity, these blocks are named as combined prediction blocks). In VTM3.0, for performing combined inter-intra prediction, a block shape based 3/4 MPM (Most Probable Modes) list is used and the list is constructed using neighboring blocks. In the present contribution, a simplified and block shape independent 3 MPM list is used. Experimental results shows that the simplification brings the coding gain of:

- -0.02% for Random Access, 0.00% for Low delay B, -0.06% for Low delay P

AI results are the same to the anchor as the simplification have no effect on I frames, wherein no combined prediction blocks exist

JVET-M0734 Crosscheck of JVET-M0296 (CE10-related: Simplification on combined inter-intra mode prediction) [L. Zhao (Tencent)] [late] [miss]

JVET-M0331 CE10-related: A simplification method for Multi-intra-inter mode scheme [L. Xu, F. Chen, L. Wang (Hikvision)]

This contribution presents a simplification for multi-intra-inter mode. In the Lth JVET meeting, the combination of inter and intra prediction is adopted. However, if inter prediction is bi-prediction, three motion compensations (MCs), including two inter MCs and one additional intra compensation, are required. In this contribution, only uni-prediction merge candidates will be used for this mode. As a result, at most one MC and one intra compensation are needed to conduct for multi-intra-inter mode in the decoder. It is reported that, compared to VTM3.0, the proposed method only brings 0.06% loss for RA configuration and 0.04% loss for LB configuration.

JVET-M0533 Crosscheck of JVET-M0331 (CE10-related: A simplification of inter prediction information derivation for Multi-intra-inter mode scheme) [X. Chen (HiSilicon)] [late] [miss]

JVET-M0390 CE10 related: Multi-hypothesis with uni-directional inter prediction restriction [T. Poirier, E. François, K. Naser (Technicolor)]

This contribution describes a simplification of multi-hypothesis intra-inter prediction by restricting inter-prediction to be uni-directional. Another simplification removes the specific syntax for narrow CUs. The proposed method has been implemented on top of VTM-3.0. The first modification is reported to result in PSNR-Y, U, V BD-rate variations of 0.10%, 0.01%, 0.02% with 98% and 103% encoding and decoding times in RA configuration, 0.06%, 0.16%, 0.20% with 104% and 106% encoding and decoding times in LDB configuration. The second one removes the intra direction restriction for narrow CUs on top of the first restriction. The second modification on top of the first is reported to result in PSNR-Y, U, V BD-rate variations of 0.09%, -0.01%, 0.03% with 94% and 97% encoding and decoding times in RA configuration, 0.06%, 0.07%, 0.07% with 103% and 105% encoding and decoding times in LDB configuration.

JVET-M0742 Crosscheck of JVET-M0390 (CE-10: related multi-hypothesis with uni-directional inter prediction restriction) [H. Wang (Qualcomm)] [late] [miss]

JVET-M0454 CE10-related: Multi-Hypothesis Intra with Weighted Combination [A. Seixas Dias, G. Kulupana, S. Blasi (BBC)]

This contribution presents a modification to the Multi-Hypothesis Intra tool included in VTM 3.0. In this contribution, an Intra prediction can be combined with a merge Inter prediction using a weighted average. To allow this weighted combination of Intra and Inter predictions, additional syntax elements are signalled indicating if unequal weights are used and if so, which pair of weights is needed. This contribution is built on top of the method in JVET-L0375, where only Planar mode is allowed for Intra prediction when used in combination with an Inter prediction. Average BD-rates of -0.11% (Y), 0.00% (U) and -0.01% (V) are reported for RA with 103% encoding time and 100% decoding time with respect to VTM 3.0. For LD, the reported BD-rates are 0.05% (Y), 0.21% (U) and 0.19% (V) with 102% and 101% encoding and decoding times, respectively.

JVET-M0610 Crosscheck of JVET-M0454 (CE10-related: Multi-Hypothesis Intra with Weighted Combination) [B. Wang (Huawei)] [late] [miss]

JVET-M0492 CE10-related: Simplified multi-hypothesis intra-inter mode [L. Zhao, X. Zhao, X. Li, S. Liu (Tencent)]

In this contribution, a simplified multi-hypothesis intra-inter mode is proposed. Firstly, the size of intra candidate mode list of intra-inter mode is reduced from 4 to 1 such that if the signaled intra-inter mode flag is true, extra intra mode coding is not needed. Secondly, for the intra-inter prediction process, PDPC filter is directly applied to the inter prediction samples to get the final prediction values, and intra smoothing filter, intra prediction process, and weighted intra and inter prediction process are avoided. It is reportedly shown that, the proposed method achieved 0.05% and -0.05% luma BD-rates with 99% and 99% encoding time and 98% and 98% decoding time for RA and LDB respectively.

JVET-M0589 Crosscheck of JVET-M0492 (CE10-related: Simplified multi-hypothesis intra-inter mode) [H. Dou, Z. Deng, L. Xu (Intel)] [late] [miss]

JVET-M0525 CE10-related: Simplification of intra prediction in CIIP [P.-H. Lin, Y.-J. Chang (Foxconn)]

In VTM3.0, combined inter and intra prediction (CIIP) combines an intra prediction and a merge indexed prediction. When it applies the CIIP, the available luma intra prediction modes are set to 4, which are Planar, DC, horizontal and vertical modes. This contribution proposes a selection scheme to simplify the CIIP by keeping only 1 available intra prediction mode for a CU. Whether the PDPC and reference smoothing filter are enabled or not is also studied. The results show that 0.07%/0.04%/0.06% BD-rate change in RA condition and 0.15%/0.09%/0.29% BD-rate change in LB condition with slightly encoding time decreasing are observed if the PDPC and the reference smoothing filter are enabled, and 0.12%/0.03%/0.01% BD-rate in RA condition and 0.19%/0.21%/0.26% BD-rate in LB condition with slightly encoding time decreasing are observed if the PDPC and the reference smoothing filter are disabled.

JVET-M0578 Crosscheck of JVET-M0525 (CE10-related: Simplification of intra prediction in CIIP) [M.-S. Chiang (MediaTek)] [late] [miss]

OBMC (2)

JVET-M0357 CE10-related: Reduction of the worst-case memory bandwidth and operation number of OBMC [Y. Kidani, K. Kawamura, K. Unno, S. Naito (KDDI)]

JVET-M0828 Crosscheck of JVET-M0357 (CE10-related: Reduction of the worst-case memory bandwidth and operation number of OBMC) [Y.-C. Lin (MediaTek)] [late] [miss]

JVET-M0792 CE10-related: Combined test of multi-hypothesis inter prediction and OBMC [Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, C.-C. Chen, Y.-C. Lin, Y.-W. Huang, S.-M. Lei (MediaTek), X. Xiu, Y. He (InterDigital), M. Winken, H. Schwarz, D. Marpe, T. Wiegand (HHI)] [late] [miss]

JVET-M0838 CE 10 related: M0390-M0096 combination [F. Galpin, T. Poirier, E. François (Technicolor)] [late]

TPM (23)

JVET-M0118 CE10-related: A fix for merge triangle flag signaling [R. Yu (Ericsson)]

In VTM-3.0, the merge_triangle_flag is reported to be redundantly signaled when a block uses MMVD (merge with motion vector difference) or multi-hypothesis intra inter prediction. The contribution describes a fix for the redundant signaling. The fix was tested under the VTM-3.0 random access (RA) and low delay B (LDB) common test configurations. The BD-rate results are reported to be -0.07% for RA, -0.14% for LDB. It is proposed to adopt the fix into next VTM.

It is remarked that there are several proposal for resolving this issue.

Recommend adoption

JVET-M0631 Crosscheck of JVET-M0118 (CE10-related: A fix for merge triangle flag signalling) [M.-S. Chiang (MediaTek)] [late] [miss]

The document reports the results of crosschecking JVET-M0118 proposed by Ericsson. JVET-M0118 proposes to fix the redundant signaling for merge_triangle_flag. The verification task was done, and the BD-rate results match exactly with those in JVET-M0118.

JVET-M0184 CE10-related: Simplification of triangle merging candidate list derivation [T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

In VTM3.0 triangle prediction, a triangle merging candidate list is introduced. In the triangle merging candidate list derivation, at most 94 times of motion vector (MV) pruning, 7 times of MV averaging, and 7 times of MV scaling are required. To reduce the complexity, it is proposed to discard the current triangle merging candidate list and reuse the existing merging candidate list of non-triangle prediction to generate a uni-prediction MV for each of the two triangular partitions. In the proposed triangle prediction, two merging candidates from the existing merging candidate list are first selected without syntax change (i.e., still using one merge_triangle_idx to select one candidate for each of the two triangular partitions). Next, to ensure uni-prediction for each triangular partition, if the first merging candidate is bi-prediction, the list 0 (L0) MV is kept for the first triangular partition, and the list 1 (L1) MV is discarded; similarly, if the second merging candidate is bi-prediction, the L1 MV is kept for the second triangular partition, and the L0 MV is discarded. Finally, an offset MV of (1/4 pel, 0) is added to the merging candidate of the second partition if the two merging candidates of the two triangular partitions are the same. In this way, the complexity of the proposed triangle merging candidate list derivation is kept as low as that of the existing merging candidate list derivation plus only two inter prediction indicator (inter_pred_idc, to indicate L0, L1, or bi-prediction) comparisons, one MV comparison, and one MV addition. Simulation results reportedly show that the average luma BD-rates are 0.02% and 0.04% in RA and LB cases, respectively.

Comments:

- It is agreed by several experts that this is an issue.

- Question is how to generate triangular merging candidate list.

- There seem to be more than one approach to do this.

- This proposal is using regular merging candidate list construction.

Revisit

JVET-M0729 Crosscheck of JVET-M0184 (CE10-related: Simplification of triangle merging candidate list derivation) [M. Winken (HHI)] [late] [miss]

This is crosscheck report for JVET-M0184.  The software package was provided by the proponent and was studied.  The software changes seem consistent with the description in JVET-M0184.  The implemented method was evaluated for JVET Common Test Condition (CTC) for SDR video, Random Access (RA) and Low Delay B (LDB) encoder configurations.  It is reported that the R-D data generated by the crosschecker exactly matches that provided by the proponent.

JVET-M0185 CE10-related: Syntax redundancy removal in triangle prediction [M.-S. Chiang, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

This contribution proposes to remove syntax redundancy in triangle prediction. In VTM3.0, triangle prediction does not coexist with combined inter intra prediction (CIIP) and merge with motion vector difference (MMVD). Therefore, it is proposed to avoid signalling merge_triangle_flag, the flag that indicates if triangle merge mode is enabled for a coding unit (CU), when CIIP or MMVD is enabled for the CU. It is reported that, compared to VTM3.0, the proposed method achieves -0.07% and -0.14% luma BD-rates for RA and LB, respectively.

Same as JVET-M0118

JVET-M0582 Cross-check of JVET-M0185 (CE10-related: Syntax redundancy removal in triangle prediction) [K. Andersson, R. Yu (Ericsson)] [late] [miss]

JVET-M0185 proposes to avoid signalling of triangle prediction flag when MMVD or CIIP is used. The description matches the software and the objective performance are matched. It can be noted that same approach is proposed in JVET-M0118.

JVET-M0190 CE10-related: Redundant syntax reduction for triangle prediction [Y. Ahn, D. Sim (Digital Insights)]

This contribution proposes to reduce redundant syntax of triangle prediction when the MMVD or multi hypothesis prediction is enabled. It has been observed that the flag for triangle prediction is still signaled when the MMVD or multi hypothesis prediction is enabled. In this contribution, the triangle prediction flag is skipped when the MMVD or multi hypothesis prediction is enabled.

Same as JVET-M0118

JVET-M0749 Cross-check of JVET-M0190 (CE10-related: Redundant syntax reduction for triangle prediction) [S.-C. Lim, H. Lee, J. Lee, J. Kang (ETRI)] [late] [miss]

This document presents a cross-check report on redundant syntax reduction for triangle prediction as proposed in JVET-M0190. It is confirmed that the software implementation is consistent with the description and all the test results are matched with those provided by the proponent.

JVET-M0194 CE10-related: Triangle Prediction Mode Harmonization [A. Tamse, M. W. Park, S. Jeong, M. Park, K. Choi (Samsung)]

Triangle prediction mode consists of candidate list generation as the first step of the process. The candidate list generation itself consists of generation of an MV list by checking the neighboring CUs and temporal MV. Based on this list, another list of uni-directional MVs is constructed, which is used in turn to create a combination of candidates. The first step of generating MV list by accessing neighbors and temporal MVs is very similar to the regular merge list construction process. Hence to avoid having additional logic module just for triangular prediction mode, it is proposed to harmonize the list construction process with the regular merge list construction process by using the regular merge list construction process for triangular mode list as well. Furthermore it is seen that only the first few candidates of the regular merge list are sufficient for the triangle prediction mode process. Since the regular merge list construction also uses HMVP and average candidates, an additional restriction is also tested which allows only the spatial and temporal candidates. This contribution also proposes a simplified pruning operation during construction of uni-directional MV list based on the spatial and temporal MV list from the previous step. The performance change after harmonization is -0.03% in Y, 0.03% in U and 0.00% in V without any change in runtime on top of VTM3.0 under RA test conditions.

- 4 methods are proposed to simplify the triangle merging candidate list construction process.

- It is commented that the proposed method still is complicated.

- This proposal is using part of regular merging candidate list construction.

JVET-M0207 CE10-related: Joint optimizations of Triangular prediction unit mode and Multi-Hypothesis prediction mode [S. Jeong, M. W. Park, A. Tamse, M. Park, K. Choi (Samsung)]

This contribution proposes optimization tests related to Triangular prediction unit mode (Triangle prediction mode) and Multi-Hypothesis (MH) prediction mode. This includes two optimization tests. The first optimization test is the removal of unnecessary flag signalling when MH mode flag is ON. Currently, when MH mode flag is ON, Triangular prediction unit mode flag is signalled. Since MH mode doesn’t use Triangular prediction unit, Triangular mode flag does not have to be signalled. Experimental results shows -0.02% luma BD-rate change in RA and -0.03% luma BD-rate change in LDB. The second optimization test is the unification of enable condition between MH mode and Triangular prediction unit mode. Currently, after skip and merge flag, there are MH mode flag and Triangular prediction mode flag. These two tools’ enable conditions are different with each other. After the unification of two tools’ enable conditions, two tools can be conducted with one enable condition without performance loss. Experimental results shows -0.01% luma BD-rate change in RA and 0.01% luma BD-rate change in LDB. The combination test of two optimization tests is also conducted. Experimental results shows -0.03% luma BD-rate change in RA and -0.04% luma BD-rate change in LDB.

- The first test appears to solve a bug in the VVC.

- The second test reduces the maximum size for TPM to 64x64 (from previously 128x128).

- It is remarked that 128x128 TPM might also violate VPDU condition. It is answered that this not the case, as it can be done on a 64x64 block basis.

- It is questioned if the alignment of “enable conditions” as done in test 2 is really required.

- A non-proponent says that disabling triangular prediction on 128x128 blocks can reduce decoder complexity.

- Proponent of TPM would like clarification on this assumed decoder complexity reduction.

- It is suggested to ask more experts for their opinion on that.

Recommendation: - Test 1 is part of JVET-M0118 (which is recommended to adopt), no action on test 2

JVET-M0775 Crosscheck of JVET-M0207: CE10-related: Joint optimizations of Triangular prediction unit mode and Multi-Hypothesis prediction mode [L. Pham Van, W.-J. Chien, M. Karczewicz (Qualcomm)] [late]

This contribution reports the results of the crosscheck of the JVET- M0207: CE10-related: Joint optimizations of Triangular prediction unit mode and Multi-Hypothesis prediction mode. Results match the ones presented by the proponents in JVET-M0207 in both three sub-tests. Encoder and decoder run times provided by the cross-checker is inaccurate.

JVET-M0216 CE10-related: syntax clean-up on triangle prediction [L. Li, J. Nam, N. Park, H. Jang, J. Lim, S. Kim (LGE)]

This contribution proposes syntax clean-up on triangle prediction. Two aspects are proposed:

1) merge_triangle_flag is not parsed when mmvd_flag is 1 or mh_intra_flag is 1.

2) Condition for parsing merge_triangle_flag: sps_triangle_enabled_flag && slice_type == B &&  cbWidth * cbHeight >= 64.

The experiment result reportedly shows overall -0.07% and -0.14% luma BD-rate changes for RA and LDB, respectively.

- Is like JVET-M0118 (recommended for adoption)

- Is already incorporated into newest VVC draft text.

JVET-M0233 CE10-related: Triangle prediction merge list construction [X. Wang, Y.-W. Chen (Kwai Inc.)]

In this contribution, some modifications are proposed on the merge candidate list construction process for triangle prediction mode. Firstly, it is proposed to use the regular merge candidate list motion vectors for deriving the uni-prediction merge list for triangle prediction. Secondly, L0 and L1 motion vectors from the regular candidate list are selected in a simple interleaving manner into the triangle merge list. The reported results show -0.05% and 0.02% BD-rate impact with the proposed simplifications for RA and LDB conditions respectively. Results are also provided for the proposal without using motion vector pruning in forming the uni-prediction merge list wherein L1 motion vectors are not used for low delay case.

- This also addresses the issue of triangular merging candidate list construction.

- This proposal is using regular merging candidate list construction.

JVET-M0674 Crosscheck of JVET-M0233 (Non-CE10: Triangle prediction merge list construction) [S. Esenlik (Huawei)] [late] [miss]

JVET-M0234 CE10-related: Triangle prediction merge index coding [X. Wang, Y.-W. Chen (Kwai Inc.)]

In this contribution, some modifications are proposed to the triangle prediction merge index coding. In the current design, a 40x3 size table has to be saved to map 40 group of (partition orientation, merge index 1, merge index 2) values to table index values for signaling. According to this proposal, partition orientation is signaled separately from merge indexes. Another separate flag is signaled to indicate how merge index 1 and 2 are mapped to the two triangle partitions. With the proposed method, the table size is effectively reduced to 10x2. The reported results show no performance impact with the proposed simplifications. In addition, a bug fix is reported in the contribution on triangle mode flag signaling, which results in minor coding performance gain.

Comments:

- It is remarked that there are other proposals which completely remove this look-up table.

JVET-M0675 Crosscheck of JVET-M0234 (Non-CE10: Triangle prediction merge index coding) [S. Esenlik (Huawei)] [late] [miss]

JVET-M0271 CE10-related: Merge list construction process for triangular prediction mode [L. Zhang, K. Zhang, H. Liu, J. Xu, Y. Wang, P. Zhao, D. Hong (Bytedance)]

Triangular prediction mode (TPM) was adopted in the last JVET meeting. TPM constructs the merge candidate list in a different way from that designed for regular merge list (used by non-affine, non-TPM mode). The current TPM merge list requires to access spatial and temporal blocks to obtain regular merge candidates with full pruning among regular spatial merge candidates. The obtained regular merge candidates are further traversed 4 times to derive the uni-prediction TPM candidates, and full pruning is applied to each TPM candidate. In this contribution, three methods are presented with the first one to unify the TPM merge list with the regular merge list and the other two to directly apply pruning to TPM candidates with a limitation of pruning operations. The difference between the second and the third method is whether to access one or two temporal blocks. All of the three methods only need to traverse the regular merge candidate once. Simulation results reportedly show that all of the three methods achieves 0.03% BD rate reduction for RA Main10 configurations.

- The regular merging list construction process is used partially.

- It is remarked that it is important to consider number of comparisons and number of stages in order to assess the complexity.

JVET-M0588 Crosscheck of JVET-M0271 (CE10-related: Merge list construction process for triangular prediction mode) [H. Dou, Z. Deng, L. Xu (Intel)] [late] [miss]

This contribution reports a cross-checking of contribution JVET-M0271 related to merge list construction process for triangular prediction mode.

It is reported that the result obtained during this cross-checking matches the result described in JVET-M0271.

JVET-M0283 CE10-related: Reduction of motion predictor pruning in Triangle Merge mode [A. Robert, F. Le Léannec, T. Poirier, F. Galpin (Technicolor)]

This contribution describes two simple motion predictor pruning processes to be applied in Triangle Merge modes. The proposed processes have been implemented in the JVET VTM3.0. For the first version of pruning reduction, the reported simulation results keep the BD-rate performances in RA and LDB configurations. Then, for the second and more drastic reduction, the reported simulation results show an average luma BD-rate loss of 0.03% in RA configuration and of 0.14% in LDB configuration. The encoding and decoding times stay identical to the VTM-3.0 ones with both versions. The worth case complexity in terms of integer comparisons required by the pruning is decreased from 276 to 144 comparisons for the first version (-48%) and to only 36 comparisons for the second version (-87%).
Combined with the proposals of JVET-M0281 CE4.1.5 and JVET-M0282 CE2.2.8 that performs motion predictor pruning for AMVP and Merge modes and for Affine AMVP and Affine Merge modes respectively, the reported simulation results show, with the first version, an average luma BD-rate gain of -0.04% in RA configuration and of -0.04% in LDB configuration and, with the second version, the same BD-rate performances for RA configuration and an average luma BD-rate loss of 0.08% in LDB configuration. The encoding and decoding times also stay identical to the VTM-3.0 ones with both versions. The total worth case complexity in terms of integer comparisons required by the pruning in all coding modes is reduced, at decoding, from 276 to 198 comparisons (-28%) with both versions, and, at encoding, from 750 to 752 or 644 comparisons with the two versions respectively (0% or -14%).

- This proposal also uses the regular merging candidate list construction process.

JVET-M0286 Non-CE4: Simplifications for triangular prediction mode [T. Solovyev, S. Esenlik, S. Ikonin, J. Chen (Huawei)]

This contribution presents simplifications for the triangular prediction mode. The following modifications are proposed:

- The additional uni-prediction merge candidate list dedicated to triangular prediction mode is removed. Instead, regular merge list is used by converting bi-prediction candidates to uni-prediction candidates.

- Table based mode coding scheme is removed by coding the split direction and merge indices separately.

The simulations results for combined solution reportedly show -0.06% and 0.02% change in luma BD-rate for RA and LDB configurations respectively, without significant change in encoding and decoding times. It is asserted that the proposed modifications eliminate the additional logic necessary for the triangular candidate list construction and simplify the binarization of the triangular prediction mode coding.

- For the modified signaling, small coding gains are reported.

- The triangular merging candidate list construction follows the approach done for 4x4 blocks (using the regular merge list).

- Both aspects of TPM (merging candidate list construction and signaling) seem to deserve further study.

JVET-M0793 Crosscheck of JVET-M0286 (Non-CE4: Simplifications for Triangular Prediction Mode) [C.-C. Chen, W.-J. Chien (Qualcomm)] [late] [miss]

JVET-M0317 CE10-related: Simplification of triangular prediction unit mode [R.-L. Liao, J. Li, C. S. Lim (Panasonic)]

In this contribution, three modifications are proposed for the triangular prediction unit mode. Firstly, the derivation process of candidate list for triangular prediction unit is unified with that for normal merge mode. Secondly, the redundant triangular prediction unit mode flag, which is signaled when either merge motion vector difference mode or multi-hypothesis intra mode is used, is removed. Thirdly, the de-blocking filter is not applied to transform unit boundaries within the coding unit coded in triangular prediction unit mode when there is no residue. With all these modifications, it is reported that as compared to VTM-3.0, 0.05% B-D rate reduction with 102% encoding time and 97% decoding time is achieved in random access configuration. 0.07% B-D rate reduction with 102% encoding time and 100% decoding time is achieved in low delay B configuration.

This proposal has 3 parts:

- A modification to the triangular merging candidate list construction process.

- The syntax redundancy removal as on M0118

- A modification to the deblocking of PUs in TPM which have width or height equal to 128.

Comments:

- What happens if motion vectors of TPM are inherited by ATMVP? Should be deblocked then? There would be no blending/weighting in this case.

- It it suggested to have the 3rd aspect studied in the CE on deblocking filter before making a decision.

JVET-M0710 Crosscheck of JVET-M0317 (CE10-related: Simplification of triangular prediction unit mode) [F. Chen (Hikvision)] [late] [miss]

This contribution is a crosscheck report of JVET-M0317. It is confirmed that the results of the proposed method match with the proponent’s results, and the implementation is consistent with the document description.

JVET-M0328 CE10-related: Simplified triangle prediction unit mode [F. Chen, L. Wang (Hikvision)]

Although the triangle prediction unit mode has been adopted in the VTM 3.0, it still has some redundant processes. In this contribution, several redundancies are proposed to be removed. First, the candidate list construction is simplified from two aspects: the redundant pruning for spatial candidates is removed, and some unnecessary candidates are also removed. Second, the two weight groups during the motion compensation are reduced to one group. Third, the redundant triangle prediction flag is not signaled when MMVD or MHIntra mode is enabled.

It is reported that the first simplification on the candidate list construction only brings 0.00% and 0.02% loss for RA and LB compared to VTM-3.0, and the second simplification on the candidate list construction only brings 0.00% and 0.03% loss for RA and LB compared to VTM-3.0. And when only keeping one weight group, it only brings 0.01% and 0.03% loss for RA and LB compared to VTM-3.0. Moreover, when the redundant triangle prediction flag is not signaled, it can provide 0.07% and 0.14% gains for RA and LB compared to VTM-3.0.

- This proposal also addresses the issue of triangular merging candidate list construction, but it differs from other proposals in this area in that it does not use the regular merging candidate list.

- The second aspect of this proposal addresses the weighting/blending.

- The third aspect seems to be identical to JVET-M0118 (syntax redundancy removal)

Comments:

- It seems reasonable to reuse the regular merging candidate list construction process, unless there is evidence that another method is beneficial.

- The proponents of TPM comment that the two weighting groups were originally introduced for subjective quality. But this has not been studied recently.

- Suggestion: Use only the second weight.

JVET-M0666 Crosscheck of JVET-M0328 (CE10-related: Simplified triangle prediction unit mode) [R.-L. Liao, C. S. Lim (Panasonic)] [late] [miss]

This contribution is a crosscheck report of JVET-M0328. It is confirmed that the all results of the proposed method are match with the proponent’s results, and the implementation is consistent with the document description

JVET-M0329 CE10-related: Modified enabling condition for triangle prediction unit mode [F. Chen, L. Wang (Hikvision)]

Currently, the triangle prediction unit mode is just enabled under condition that the frame is B frame and the area of current block is larger or equal than 64. In this contribution, the frame type condition is modified as that the frame is inter frame, and the size condition is modified from two folds: the minimum size and the maximum size. For the minimum size, it is modified as that both the width and the height of current block should be larger than 4. For the maximum size, the condition that both the width and the height of current block should be less than 128 is added. It is reported that the first modification about frame type can provide 1.56% gains for LP compared to VTM-3.0 with around 8% encoding time and no decoding time increase. And the second modification about minimum size can bring 0.05% and 0.11% loss for RA and LB respectively compared to VTM-3.0. While the second modification about maximum size can bring 0.01% gains and 0.01% loss for RA and LB respectively compared to VTM-3.0.

The proposal has a 3rd aspect: a modified RDO for triangular: 0.05% coding loss for RA (92% enc time), 0.17% coding loss for LB (91% enc time)

Comments:

- It is remarked that the comparison seems to be somewhat unfair, as it would introduce bi-prediction to P frames.

- It is remarked that an encoder optimization should not lead to coding loss.

- It is remarked that is not clear if we’re saving anything with the proposed block size restriction.

No action for this constraint.

- It is questioned if the 128xN and Nx128 blocks with triangular mode would violate the VPDU. To be discussed offline.

JVET-M0692 Crosscheck of JVET-M0329 (CE10-related: Modified enabling condition for triangle prediction unit mode) [S.H. Wang (Peking University), X. Zheng, S.S. Wang, S.W. Ma] [late] [miss]

This contribution is a crosscheck report for JVET-M0329 (CE10-related: Modified enabling condition for triangle prediction unit mode). The simulations were carried out based on the common test condition (CTC). Based on the partial results, the test results are almost matched except that the encoding time shows some inconsistency due to the instability computing performance.

JVET-M0352 CE10-related: Simplification of triangular partitions [D. Park, Y. Yoon, J.-G. Kim (KAU), J. Lee, J. Kang (ETRI)]

This contribution presents simplifications of triangular partitions based on VTM-3.0. The first simplification is to remove the reference picture mapping process in the motion vector storage of triangular partitions. The second one is to reduce the number of combinations of motion information for each partition along with modified binarization methods for the reduced combination indications. In this contribution, the performances on the 3 tests as shown follows are reported

- Test 1: Simplification of motion vector storage skipping reference picture mapping process

  - (RA) 0.00% BD-rate with 100% encoding time and 100% decoding time

  - (LDB) 0.17% BD-rate with 100% encoding time and 100% decoding time

- Test 2: Reduction of the number of motion information combinations

  - (RA) 0.01% BD-rate with 96% encoding time and 100% decoding time

  - (LDB) 0.03% BD-rate with 94% encoding time and 100% decoding time

- Test 3: Test1 + Test2

  - (RA) 0.02% BD-rate with 96% encoding time and 100% decoding time

  - (LDB) 0.20% BD-rate with 94% encoding time and 100% decoding time

  - Comments:

- One participant said it should be encouraged to study encoder-only speed-ups (without normative changes).

- Another participant encouraged to also study encoder speed-up and consider the signaling.

JVET-M0722 Crosscheck of JVET-M0352 (CE10-related: Simplification of triangular partitions) [G. Laroche (Canon)] [late] [miss]

This contribution reports the crosscheck results of JVET-M0352 on modifications of triangular partitions. The crosscheck results match the ones provided by the proponent. The software has been examined and match the description.

JVET-M0349 CE10-related: Simplification of triangle prediction merging candidate list derivation [X. W. Meng (Peking University), X. Zheng (DJI), S.S. Wang, S.W. Ma (Peking University)]

This contribution proposes a simplification method of triangle prediction merge list construction in VTM-3.0[1]. The proposed method reusing inter merge list as triangle prediction’s merge list. It is reported that as compared to VTM-3.0, xxx% B-D rate saving with xxx% encoding time and xxx% decoding time is achieved in random access configuration. 0.09% B-D rate increase with xxx% encoding time and xxx% decoding time is achieved in low delay B configuration.

- This proposal also uses the regular merging candidate list construction process.

JVET-M0399 CE10-related: Modifications of Triangular PU Mode [H. Wang, W.-J. Chien, V. Seregin, Y.-H. Chao, H. Huang, M. Karczewicz (Qualcomm)]

This contribution presents 3 changes to the triangular PU mode. The changes include 1) encoding of triangular PU motion information, 2) generation of uni-prediction motion vector candidate set and 3) motion vector storage. Compared to VTM-3.0, the change 1) achieved -0.01% in terms of average luma BD-rate for both randomaccess and lowdelay. The change 2) achieved -0.05% (RA) and 0.00% (LB) in terms of average luma BD-rate. Without MV pruning in the second step, the performance of change 2) is -0.03%(RA) and 0.25%(LB). The change 1) + 2) achieved -0.06% (RA) and -0.04% (LB) in terms of average luma BD-rate. And the change 1) + 2) + 3) achieved -0.05% (RA) and -0.04% (LB) in terms of average luma BD-rate.

- 1) changes the signalling of the triangular merge mode

- 2) changes the merging candidate list construction

- 3) is about complexity reduction (don’t do mapping in certain cases)

JVET-M0694 Crosscheck of JVET-M0399 (CE10-related: Modifications of Triangular PU Mode) [T. Poirier (Technicolor)] [late] [miss]

The cross-checker reports that the source code provided by Qualcomm was carefully analysed and found to be consistent with the description in JVET-M0399. The cross checker could exactly reproduce the results reported in JVET- M0399 as the “proposed technique”. The measured encoder and decoder run time ratios are similar to those reported in JVET-M0399.

JVET-M0438 CE10-related: Size constraint on Triangular Prediction [J. Zhao, S. Kim (LGE)]

In this document, it is proposed to disable Triangular prediction if either width or height of a block is 128 to reduce Triangular prediction complexity. Experimental result show that the coding impact is -0.01%, 0.00%, -0.01% for Y, U, V, components respectively in RA case, and 0.01%,0.00%,0.09% for Y, U,V, component respectively in LD case, and there is about 1% run time reduction for both encoder and decoder.

No action.

JVET-M0663 Crosscheck of JVET-M0438 (CE10-related: Size constraint on Triangular Prediction) [C.-W. Kuo, R.-L. Liao, C. S. Lim (Panasonic)] [late] [miss]

This contribution is a cross-check report of JVET-M0438: “CE10-related: Size constraint on Triangular Prediction”. The code has been checked and the crosscheck was carried out under the JVET common test condition (CTC). According to the results of experiments, the results of JVET-M0438 are confirmed.

JVET-M0448 CE4-related: Triangle merge index signaling [M. Xu, X. Li, S. Liu (Tencent)]

Triangle prediction was adopted in VTM-3.0. In triangle prediction, a triangle merge index is signaled, then the triangle direction and the merge indices for the two triangle PUs in a CU are derived from a lookup table (LUT).  In this proposal, a signaling scheme is presented without using LUT. The proposed methods lead to -0.01% and 0.01% BD-rate difference on average for RA, respectively. Both methods show no gain or loss on average for LB.

Revisit

JVET-M0836 Crosscheck of JVET-M0448 (CE4-related: Triangle merge index signaling) [X. Wang (Kwai Inc.)] [late] [miss]

JVET-M0490 CE2-related: Simplified context model for triangular prediction mode [M. Gao, X. Li, S. Liu (Tencent)]

This contribution proposes not using spatial neighbors as the context model for triangular prediction unit mode. It is reported that 0.01% and 0.01% luma BD-rate increase over VTM-3.0 is observed for RA and LD, respectively.

Recommend adoption

JVET-M0753 Crosscheck of JVET-M0490 (CE2-related: Simplified context model for triangular prediction mode) [R.-L. Liao, C. S. Lim (Panasonic)] [late] [miss]

JVET-M0836 Crosscheck of JVET-M0448 (CE4-related: Triangle merge index signaling) [X. Wang (Kwai Inc.)] [late] [miss]

JVET-M0581 CE10-related: Bi-directional motion vector storage for triangular prediction [M. Bläser, J. Sauer (RWTH Aachen University)] [late]

This contribution presents a method of storing bi-directional motion vectors for uni-directional triangular prediction. The presented method results in a BD-rate coding efficiency loss of 0.08% for RA and 0.16% for LB, with marginal increase in encoder complexity for RA, and 128% increase in encoder complexity for LB. No significant increase in decoder complexity is reported.

No action recommended.

JVET-M0829 Crosscheck of JVET-M0581 (CE10-related: Bi-directional motion vector storage for triangular prediction) [H. Gao (Huawei)] [late] [miss]

This document reports the cross-check results for the results reported in input document JVET-M0581 CE10-related: Bi-directional motion vector storage for triangular prediction. The crosschecked contribution present a method of storing bi-directional motion vectors for uni-directional triangular prediction.

JVET-M0736 CE10-related: Triangular prediction with MMVD [M. Bläser, J. Sauer (RWTH Aachen University)] [late]

This contribution proposes the combination of triangular prediction with merge with motion vector difference (MMVD) techniques. The proposed method achieves a BD-rate change of -0.14% for RA and -0.33% for LB with an increase of 157% for RA and 224% for LB in encoder complexity. No significant increase in decoder complexity is reported.

It is remarked that this is interesting and should be studied. However, the current VTM obviously has a bug which should be fixed.

Recommend: Further study within CE

JVET-M0830 Crosscheck of JVET-M0736 (CE10-related: Triangular prediction with MMVD) [H. Gao (Huawei)] [late] [miss]

This document reports the cross-check results for the results reported in input document JVET-M0736 CE10-related: Triangular prediction with MMVD. This crosschecked contribution proposes the combination of triangular prediction with merge with motion vector difference (MMVD) techniques.

LIC (6)

JVET-M0088 CE10-related: LIC restriction for pipeline structure [K. Abe, T. Toma (Panasonic)]

This contribution is based on the LIC which is described in CE10.5.2 (JVET-M0087). CE10.5.2 can achieve the basement pipeline structure with LIC, but it still has memory bandwidth issue and stage cycle issue coming form pipeline restriction. This contribution provides the solution for these issues by introducing the LIC restriction for the combination with VPDU split, ATMVP, and MH-Intra. Simulation results reportedly show that the proposed LIC provides 0.49% BD-rate gain for RA, 0.47% BD-rate gain for LDB.

JVET-M0371 Crosscheck of JVET-M0088 (CE10-related: LIC restriction for pipeline structure) [P. Bordes (Technicolor)] [late]

The cross-checker reports that the source code provided by Panasonic was carefully analysed and found to be consistent with the description in JVET-M0088. The cross checker could exactly reproduce the results reported in JVET- M0088 as the “proposed technique”. The measured encoder and decoder run time ratios are similar to those reported in JVET-M0088.

JVET-M0115 CE10-related: pipeline reduction for LIC and GBI [P. Bordes, F. Galpin, T. Poirier (Technicolor)]

This contribution proposes to reduce pipeline for LIC by disabling GBI when LIC is enabled at CU level.

The proposed process has been implemented both in the JVET VTM3.0 on top of regular LIC (CE10.5.2) and combined with JVET-M0224 (LIC simplifications). The reported simulation results show an average BD-rate gain of -0.54%,-0.26%,-0.33% in Y,Cb,Cr in RA configuration and of -0.48%,-0.54%,-0.46% in LDB configuration implemented on top of (CE10.5.2), and an average BD-rate gain of -0.59%,-0.32%,-0.42% in Y,Cb,Cr in RA configuration and of -0.53%,-0.59%,-0.69% in LDB configuration when combined with JVET-M0224. The encoding and decoding times are slightly reduced compared to the regular LIC.

JVET-M0570 Crosscheck of JVET-M0115 (CE10-related: pipeline reduction for LIC and GBI) [T. Chujoh (Sharp)] [late]

The cross checker reports that the source code provided by Technicolor who proposes JVET-M0115 has been carefully analysed and has been found to be consistent with the description in JVET-M0115. The cross checker could reproduce the results reported in JVET-M0115. The difference from measured encoder and decoder run time ratios to those reported in JVET-M0115 is acceptable.

JVET-M0182 CE10-related: Simplification of local illumination compensation [C.-M. Tsai, C.-C. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

This contribution proposes methods to simplify the local illumination compensation (LIC) in BMS2.2. Three tests are conducted. In Test 1, to reduce encoding time, two methods are included: 1) reusing the LIC-off motion search results for testing LIC-on, 2) disallowing bi-prediction with unequal coding unit (CU) weights and subblock-based temporal motion vector prediction (SbTMVP) when LIC is on, where the first method is encoder-only while the second method is normative. In Test 2, to alleviate the latency issue for LIC in hardware decoders, LIC is applied only with the offset (i.e., the weight is fixed to 1.0). In addition, Test 1 is included in Test 2. In Test 3, to improve the coding efficiency of Test 2 and keep the latency issue alleviated, top-and-left template, top template, and left template are used to derive the linear model parameters of LIC at the top left corner, top boundary, and left boundary of the current coding tree unit (CTU), and only the offset parameter is derived if the current template within the current CTU. Besides, Test 3 also includes Test 1. Results of these three tests compared with VTM3.0 are reported as follows:

Test 1: LIC fast encoding + disallowing bi-prediction with unequal CU weights when LIC is on
RA BD-rates = -0.40% (Y), -0.23% (U), -0.28% (V); EncT=105%; DecT=100%
LB BD-rates = -0.42% (Y), -0.36% (U), -0.33% (V); EncT=105%; DecT=100%

Test 2: Test 1 + offset-only LIC
RA BD-rates = -0.14% (Y), -0.22% (U), -0.24% (V); EncT=105%; DecT=101%
LB BD-rates = -0.05 % (Y), -0.31 % (U), -0.29 % (V); EncT=105%; DecT=101%

Test 3: Test 1 + adaptive template shape LIC at CTU boundary + offset-only LIC within CTU
RA BD-rates = -0.14% (Y), -0.17% (U), -0.15% (V); EncT=105%; DecT=101%
LB BD-rates = -0.11% (Y), -0.30% (U), -0.42% (V); EncT=106%; DecT=101%

JVET-M0596 Crosscheck of JVET-M0182 (CE10-related: Simplification of local illumination compensation) [Y. Yasugi, T. Ikai (Sharp)] [late] [miss]

The contribution reports a crosscheck of JVET-M0182 (CE10-related: Simplification of local illumination compensation). The crosscheck was carried out under the JVET common test condition. Experimental results (partial) match with those provided by the proponent.

JVET-M0224 CE10-related: Local Illumination compensation simplifications [S. Bandyopadhyay, X. Xiu, Y. He (InterDigital)]

This document proposes simplifications to Local Illumination Compensation (LIC) based on CE 10.5.2. LIC is used to compensate for the illumination changes at the motion compensation stage that exists in-between the temporal reference block and the current coding block based on a linear model. Traditionally, the LIC parameters e.g. scaling factor and the offset are separately estimated for each prediction direction when bi-prediction is applied. In the simplified method, the bi-prediction reference samples of the template samples are first generated using bi-predictive weighted average (BPWA) based on the motion vectors of the current block. Then the LIC parameters are derived by minimizing the difference between the template reference samples and the template samples. In additional to disabling LIC for affine merge & triangle uni-prediction modes, which is the same as that in CE 10.5.2, the proposed simplifications further disable LIC for ATMVP sub-block merge mode.

The simplification results in PSNR-Y, U, V BD-rate variations of {-0.63%, -0.35%, -0.45%} gain in RA configuration over VTM3.0 and {-0.07%, -0.04%, -0.09%} gain over CE10.5.2 in RA configuration.

JVET-M0714 Crosscheck of JVET-M0224 (CE10-related: Local Illumination compensation simplifications) [P. Bordes (Technicolor)] [late] [miss]

The cross-checker reports that the source code provided by InterDigital was carefully analysed and found to be consistent with the description in JVET-M0224. The cross checker could exactly reproduce the results reported in JVET- M0224 as the “proposed technique”. The measured encoder and decoder run time ratios are similar to those reported in JVET-M0224.

JVET-M0450 CE10-related: LIC inheritance restrictions and interaction with GBI [M. Xu, X. Li, X. Xu, S. Liu (Tencent)]

The LIC tool tested in CE10.5.2 is explicitly signaled or implicitly derived. When it is derived from a spatial or temporal neighboring block, the LIC flags needs to be stored in temporal and spatial buffers. This contribution describes three methods to restrict the LIC flag derivation, so that the storage for LIC flag in certain buffers is no longer required. In method 1, LIC flag is not allowed to inherit from a temporal neighbor, with 0.00%, -0.02%, and 0.01% BD-rate difference on average compared with CE10.5.2 for RA, LB, and LP, respectively. In method 2, LIC flag is not allowed to inherit from a different CTU row, with 0.07%, 0.03%, and 0.07% BD-rate loss on average compared with CE10.5.2 for RA, LB, and LP, respectively. In method 3, LIC flag is not allowed to inherit from a different CTU, with 0.09%, 0.04%, and 0.09% BD-rate loss on average compared with CE10.5.2 for RA, LB, and LP, respectively. In addition, it is proposed to disable GBI when LIC is on. It shows 0.XX% and 0.XX% BD-rate loss on average for RA and LB, respectively.

JVET-M0500 CE10-related: Unidirectional illumination compensation [V. Seregin, W.-J. Chien, T. Hsieh, N. Hu, M. Karczewicz (Qualcomm)]

This contribution presents test results for local illumination compensation (LIC) tool, which was implemented in the BMS software and is also studied in CE10. In the proposal, LIC is applied to AMVP, merge and MMVD tools, and it is disabled for other inter tools and bi-directional prediction. Test results show luma BD rate reduction of -0.51% with 116% encoder and 97% decoder runtime for RA configuration, and -0.38% with 113% encoder and 97% decoder runtime for LDB configuration.

JVET-M0633 Crosscheck of JVET-M0500 (CE10-realated: Unidirectional illumination compensation) [K. Abe (Panasonic)] [late] [miss]

This contribution is a cross-check report of JVET-M0500[1]. It is confirmed that the cross-check results are identical with the proponent’s results.

Other (1)

JVET-M0424 CE10-related: On enhancement of 4-tap interpolation filters [M. Sychev, J. Chen (Huawei)] [late] [miss]

JVET-M0752 Crosscheck of JVET-M0424 [A. Henkel (HHI)] [late] [miss]