| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 13th Meeting: Marrakech, MA, 9–18 Jan. 2019 | Document: JVET-M0887-v1 |
| --- | --- |

| Title: | Crosscheck of additional tests in JVET-M0147 (CE9: Results of DMVR related Tests CE9.2.1 and CE9.2.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Tomohiro Ikai | Tel: Email: | +81-43-299-8526 chujoh.takeshi@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

The cross checker reports that the source code provided by Ittiam who proposes two additional tests in JVET-M0147 has been carefully analysed and has been found to be consistent with the description in JVET-M0147. The cross checker could reproduce the results reported in JVET-M0147. The difference from measured encoder and decoder run time ratios to those reported in JVET-M0147 is acceptable.

Summary of additional tests in JVET-M0147

Additional tests, which are CE9.2.1g with refined MVs used only for temporal prediction (and not for spatial prediction and de-blocking), are added to JVET-M0147. The following define statement in Typedef.h is changed.

define  DMVR_JVET_LAG2_K0041   0  // use of refined MVs across Top left and Top CTB boundaries

On the above condition, two distortion measurements are tested.

Test 1: MR-SAD

Test2: SAD.

Simulation Results

Their software is based on CE9.2.1g software. The anchor is VTM-3.0. The results were consistent with the results of JVET-M0147. The difference from measured encoder and decoder run time ratios to those reported in JVET-M0147 is acceptable.

Table 1 Summary of experimental results compared to anchor (Test 1)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.12% | -1.30% | -1.54% | 104% | 124% |
| Class A2 | -1.57% | -2.05% | -2.03% | 103% | 128% |
| Class B | -0.77% | -1.03% | -1.09% | 103% | 125% |
| Class C | -0.82% | -1.02% | -1.02% | 103% | 122% |
| Class E |  |  |  |  |  |
| Overall | -1.01% | -1.29% | -1.35% | 103% | 125% |
| Class D | -0.75% | -1.02% | -1.01% | 102% | 122% |
| Class F (optional) | -0.37% | -0.56% | -0.45% | 103% | 134% |

Table 2 Summary of experimental results compared to anchor (Test 2)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-3.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.94% | -1.20% | -1.44% | 103% | 120% |
| Class A2 | -1.51% | -2.04% | -1.98% | 102% | 124% |
| Class B | -0.67% | -0.94% | -1.04% | 103% | 121% |
| Class C | -0.76% | -1.00% | -1.03% | 102% | 119% |
| Class E |  |  |  |  |  |
| Overall | -0.92% | -1.23% | -1.31% | 103% | 121% |
| Class D | -0.75% | -1.08% | -0.96% | 102% | 119% |
| Class F (optional) | -0.32% | -0.48% | -0.38% | 103% | 131% |

Conclusion

In this contribution, the additional test results of JVET-M0147 have been confirmed.

Reference

[1]	S. Sethuraman, CE9: Results of DMVR related Tests CE9.2.1 and CE9.2.2, Joint Video Experts Team Document, JVET-M0147, Marrakech, Jan. 2019.

[2]	F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, JVET common test conditions and software reference configurations for SDR video, Joint Video Experts Team Document, JVET-L1010, Macao, Oct. 2018.

Patent rights declaration(s)

Sharp Corporation does not have any current or pending patent rights relating to the technology described in this contribution.